# 3GPP TSG-RAN WG2 Meeting #131		                  								R2-2505371

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.2.4

Source:	Robert Bosch GmbH

Title:	Discussions on ambient IoT data transmission

Document for:	Discussion and Decision

# Introduction

In the RAN2#130 meeting, it was agreed that:

Agreements

-	For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not.   FFS for CFRA

-	FFS for need for release message

# Ambient IoT AS ID

After reception of msg1 from a device in a reader, the reader decides to response to one of the devices with the transmission of msg2 and echoing back the same random number of msg1. The echoed random number will help the device to recognize that it can proceed with msg3 and data transmission. In this case, the reader assigns an AS ID to the device to be used by the device in the following message transmissions. The AS ID can be

- The same 16 bits random number or at least n bits of it

- A shorter ID than the random number assigned by the reader

Observation 1: A device needs to recognize the correlation between AS ID and random number.

Proposal 1: The device should be informed by the reader if it can use random number as AS ID after reception of msg2 through one of the R2D messages for e.g., msg2.

Proposal 2: For simplicity, the device can use n bit out of 16 bits of random number as AS ID and the reader should configure the value of n.

After reception of msg2 in the device, the device can use AS ID for its further transmissions. However, the device may release the AS ID if:

- It has no data to transmit or

- Due to energy status such as the need for energy harvesting cannot transmit further data messages

Observation 2: The AS ID should be released if a device cannot transmit any data.

Proposal 3: To release AS ID by a device, a reader may use one of the following options:

Option A: Reader may use an explicit R2D message to ask for the release of AS ID by a device.

Option B: Reader may re-transmit msg2 with the same AS ID and possibly a flag to inform the device that it cannot use the AS ID anymore.

Since ambient IoT devices may not receive continuously due to their energy status and the need for energy harvesting, a device may fail to receive any AS ID release message from a reader. In this case, the AS ID will not be released and may collide with the transmission of other devices if the reader allocates the AS ID to other devices.

Observation 3: A device may fail to receive the AS ID release message and use it for its future transmissions.

Proposal 4: Device can confirm the release of its AS ID using a D2R message.

# Conclusions

Based on the discussions in this contribution, we have the following observation and proposals:

Observation 1: The device need to recognize the correlation between AS ID and random number.

Observation 2: The AS ID should be released if a device cannot transmit any data.

Observation 3: A device may fail to receive the AS ID release message and use it for its future transmissions.

Proposal 1: The device should be informed by the reader if it can use random number as AS ID after reception of msg2 through one of the R2D messages for e.g., msg2.

Proposal 2: For simplicity, the device can use n bit out of 16 bits of random number as AS ID and the reader should configure the value of n.

Proposal 3: To release AS ID by a device, a reader may use one of the following options:

Option A: Reader may use an explicit R2D message to ask for the release of AS ID by a device.

Option B: Reader may re-transmit msg2 with the same AS ID and possibly a flag to inform the device that it cannot use the AS ID anymore.

Proposal 4: Device can confirm the release of its AS ID using a D2R message.