3GPP TSG-RAN WG2 Meeting #131		R2-2505374

Bengaluru, India, Aug 25th – 29th, 2025

Title: 	XR Rate control details

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.7.6

Document for:	Discussion and decision

# Introduction

At RAN2#129-bis it was agreed that the XR rate control mechanism uses the per QoS flow-based indication in the MAC CE. Subsequently, at RAN2#130, it was agreed that the final decision for the format of the recommended bit rate MAC CE will be selected from the following options:

We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead:

    - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit)

    - Implicit, e.g. index or mapping

In this contribution, we discuss the detailed format of the MAC CE for recommended bit rate query and propose the way forward.

# Discussion

If an explicit DRB ID and QFI are used in the MAC CE, then the signalling can have full flexibility to signal all possible DRBs and QFI combinations. However, this will have a significant overhead in the MAC CE. Such full flexibility is not really needed given that only a few DRBs/QoS flows will carry the XR traffic at any given time. Thus, to limit the signalling overhead, it is recommended that only a subset of DRBs/QoS flows will be subject rate control. Furthermore, we should select a limit on maximum number of DRBs/QoS flows that could be subject to rate control. 8 DRBs and 8 QoS flows seem to be a reasonable starting point. So, we propose the following:

Proposal 1 (MAC-3, MAC-4): Only a sub-set of DRBs/QoS flows should be configured to be subject to rate control (by RRC)

Proposal 2 (MAC-3, MAC-4): A maximum of 8 DRBs and 8 QoS flows per DRB may be configured to be subject to rate control at any given point in time

With the above consideration, we can then use a bit-map based approach to indicate the recommended bit rate for a given QoS flow on a given DRB.

Proposal 3 (MAC-3, MAC-4): The format of enhanced recommended bit rate MAC CE as shown below is proposed to be adopted:

Figure 1: Enhanced Recommended bit rate MAC CE

-	Dd: This field indicates whether recommend bit rate for one of the QoS flows mapped to DRB ID d is included in this MAC CE, where d is in the ascending order of the DRB ID among the DRBs configured for enhanced rate control mechanism.

-	Qd,q: This field indicates that recommended bit rate is included in the MAC CE for a QoS flow number q mapped to DRB d, for which Dd is set to 1, where q is in the ascending order of the QoS flow IDs among the QoS flows configured for enhanced rate control mechanism within the DRB.

-	Recommended Bit Rate d,q: This field indicates the recommended bit rate value for the QoS flow Qd,q.

# Conclusion

The following proposals are made in this contribution:

Proposal 1 (MAC-3, MAC-4): Only a sub-set of DRBs/QoS flows should be configured to be subject to rate control (by RRC)

Proposal 2 (MAC-3, MAC-4): A maximum of 8 DRBs and 8 QoS flows per DRB may be configured to be subject to rate control at any given point in time

Proposal 3 (MAC-3, MAC-4): The format of enhanced recommended bit rate MAC CE as shown below is proposed to be adopted:

Figure 1: Enhanced Recommended bit rate MAC CE

-	Dd: This field indicates whether recommend bit rate for one of the QoS flows mapped to DRB ID d is included in this MAC CE, where d is in the ascending order of the DRB ID among the DRBs configured for enhanced rate control mechanism.

-	Qd,q: This field indicates that recommended bit rate is included in the MAC CE for a QoS flow number q mapped to DRB d, for which Dd is set to 1, where q is in the ascending order of the QoS flow IDs among the QoS flows configured for enhanced rate control mechanism within the DRB.

-	Recommended Bit Rate d,q: This field indicates the recommended bit rate value for the QoS flow Qd,q.

# References