3GPP TSG-RAN WG2 Meeting #131	R2-2505445

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.7.5

Source:	Apple

Title:	Remaining Issues of RLC-AM Enhancements for Rel-19 XR

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

The following open issues have been identified for Rel-19 RLC-AM enhancements [1]:

- [RLC-10] 		Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard.

- [RLC-11] 		FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”

- [RLC-12] 		FFS how to handle PDCP SN gap report handling during UE mobility

- [RLC-14] 		If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, whether the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers.

This paper presents some of our views on these open issues.

# Discussions

## (RLC-11) Poll Retransmission

The open issue (RLC-11) has been discussed in RAN2 #130, we have made the following working assumption:

Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

First of all, we would like to point out that this open issue is relevant to the scenarios where the poll retransmit timer is expired and all the RLC SDUs (or segments) awaiting acknowledgements are already outdated (i.e. the corresponding PDCP SDU has been discarded). Clearly, this situation can happen before the transmitter receives any status report, because both packet discarding and poll retransmit timer expiry can occur at any time, even prior to any status report triggering at the receiver side. Furthermore, this could be a common scenario in the context of XR, where a large number of packets belonging to the same PDU Set are discarded simultaneously

In our views, this is not about whether the window is stalled or not. The more important question is, why we still need to consider retransmit the poll when we already know that there is no more RLC SDU awaiting acknowledgement?

Observation 1: Packet discarding can happen much earlier than the reception of status report. Therefore, status report triggering by the receiver cannot prevent the cases where the transmitter does not have a RLC SDU to retransmit the poll when the t-PollRetransmit expires.

Observation 2: The more important question is why we still consider poll retransmission when already know there is no RLC SDU (or segments) awaiting acknowledgement.

From our perspective, even if there is a RLC SDU that can be used to retransmit the poll upon expiry of t-PollRetransmit, the transmitter does not need to retransmit the poll. According to TS 38.322, when a poll is triggered and sent, the transmitter should start or restart t-PollRetransmit, and set the state variable POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer. Furthermore, the transmitter can stop and reset t-PollRetransmit when it receives a STATUS report that indicates the status of RLC SDU corresponding to POLL_SN:

- if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit.

The reason why we keep the t-PollRetransmit running is because the transmitter would like to continue the polling procedure (i.e. to transmit the poll when it needs to), until it knows the status of each of the RLC SDU with SN up to POLL_SN. And therefore, if the transmitter obtains a status report that includes at least POLL_SN, the transmitter can stop and resets t-PollRetransmit. The transmitter is now aware of the status of each RLC SDU with SN up to POLL_SN based on such status report, so retransmission of the poll is no longer needed.

When a packet is discarded by the upper layers, similarly the transmitter is no longer interested about whether the receiver has successfully received the discarded packets or not, because no retransmission will be performed for their RLC SDUs regardless of their status. Of course, the transmitter would still need to receive the status report from the receiver so it can advance the transmitting window, but the receiver will anyway provide such status report proactively when it detects some packets are already discarded (based on expiry of t-RxDiscard). Thus, there is no need to retransmit the poll if all RLC SDUs with SN up to POLL_SN are either acknowledged or discarded.

Observation 3: By following the legacy principle, there is no need to retransmit the poll if the TX already knows the status of RLC SDUs with SN up to POLL_SN (e.g. positively/negatively acknowledged or discarded)

Hence, we think t-PollRetransmit should be stopped if the status of RLC SDUs up to POLL_SN are confirmed. During RAN2 #130, some companies have argued that it is fine to keep the t-PollRetransmit running, even if the transmitter already knows the status of RLC SDUs up to POLL_SN. We would like to point out some technical problems if t-PollRetransmit continues to run in such cases:

- It may result in excessive polling that leads to some additional signaling overhead of unnecessary or pre-mature status reports from the receiver, which may further prevent the receiver from providing the more critical status report (the one that is triggered by expiration of t-RxDiscard ) quickly due to t-StatusProhibit. More specifically, if the poll is retransmitted upon expiry of t-PollRetransmit, the receiver may be forced to send a status report (and start t-StatusProhibit ) by this poll before detecting that some RLC SDUs have been discarded. When the receiver is finally ready to send a status report triggered by detection of discarding upon t-RxDiscard expiry, it may not be able to send it if the t-StatusProhibit started earlier is still running. Obviously, the transmitting window advancement would be further delayed in this case, due to interference caused by unnecessary polling.

- From implementation perspective, a timer expiry and its associated system routines consume CPU resources, for example (depending on the actual timer management scheme), due to additional event handling in the operating system, additional task context switches, or even timer interrupts, which account to UE power consumption. In the cases of concern, the UE already knows that the poll retransmission is not needed (because of the status of all RLC SDUs up to POLL_SN are confirmed), expiry of t-PollRetransmit would only cause some unnecessary interrupt that can impact the operation, as some of the other tasks may have to be pre-empted by the CPU due to such interrupt.

Observation 4: If the t-PollRetransmit continues to run when all the RLC SDUs up to POLL_SN are either acknowledged or discarded by PDCP, potentially there are some drawbacks:

- Additional latency for the status report to advance the TX window, and

- Unnecessary interruption of UE processing due to timer expiry

With this in mind, we do not see any reason to keep the timer running and to retransmit the poll, as it does not bring any benefit, and instead it may cause some unnecessary impacts that can degrade system performance. Thus, in our views, the open issue RLC-11 can be resolved by simply allowing the UE to stop a running t-PollRetransmit when it knows all RLC SDUs up to POLL_SN are either acknowledged or discarded. By doing so, the concerned situation of RLC-11 - “No SDU to retransmit the poll” can be prevented and will not happen.

Proposal 1: (RLC-11) When all RLC SDUs with SNs up to POLL_SN are already positively/negatively acknowledged or discarded by PDCP, the transmitter should stop and reset the running t-PollRetransmit.

## (RLC-10) Dependency between stopReTxDiscardedSDU and t-RxDiscard

The open issue (RLC-10) raises the question about whether the feature of stopping retransmission of discarded RLC SDUs should be always configured jointly with receiver discard timer. In our views, these two features are not closely related. From a UE perspective, the enablement of stopReTxDiscardedSDU allows the UE to refrain from performing unnecessary retransmission in UL, while t-RxDiscard is used for the UE to detect if any DL packet has been discarded by the gNB. Since these two features are used for traffics in different directions, we tend to think this is more flexible if these features can be configured independently.

Proposal 2: (RLC-10) There is no need to couple the configurations of stopReTxDiscardedSDU and t-RxDiscard.

## (RLC-14) Discarding of RLC SDUs Submitted to Lower Layer

Currently, it is specified that the TX side of RLC should discard SDUs that has not been submitted the lower layer, if a discard indication is received from the upper layer. When stopReTxDiscardedSDU is configured, since a RLC SDU that has been submitted to the lower layer will not be retransmitted anyway when the corresponding PDCP SDU is discarded, it seems there is no need to keep the RLC SDU in the buffer. In other words, it could be discarded by the RLC as well. However, we think when the transmitter should discard a RLC SDU should be an implementation issue. This is akin to the situation where the transmitter receives the positive acknowledgement for a RLC SDU, where we do not specify whether the UE should discard this RLC SDU or not. With this consideration, we suggest leaving case concerned in the open issue (RLC-14) to implementation as well.

Proposal 3: (RLC-14) If stopReTxDiscardedSDU is configured, whether a RLC SDU that has been submitted to the lower layer should be discarded when receiving a discarding indication from PDCP, is an implementation issue.

## (RLC-12) PDCP SN Gap Report During Mobility

Previously RAN2 has reached the following agreement for this issue:

- RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements.  If it is a problem further discussions are needed on the alternatives.

We have always assumed that proper network coordination can handle the issue, without any UE impact. So far, we are not seeing any critical problem with such approach. Considering this is the last meeting to wrap up Rel-19, we think there is no need for RAN2 to discuss this issue for the time being.

Proposal 4: (RLC-12) There is no need to further consider issues on PDCP SN Gap Reporting during mobility.

# Conclusions

This paper presented our views on RLC-AM enhancements for Rel-19. We have the following observations and proposals:

Observation 1: Packet discarding can happen much earlier than the reception of status report. Therefore, status report triggering by the receiver cannot prevent the cases where the transmitter does not have a RLC SDU to retransmit the poll when the t-PollRetransmit expires.

Observation 2: The more important question is why we still consider poll retransmission when already know there is no RLC SDU (or segments) awaiting acknowledgement.

Observation 3: By following the legacy principle, there is no need to retransmit the poll if the TX already knows the status of RLC SDUs with SN up to POLL_SN (e.g. positively/negatively acknowledged or discarded)

Observation 4: If the t-PollRetransmit continues to run when all the RLC SDUs up to POLL_SN are either acknowledged or discarded by PDCP, potentially there are some drawbacks:

- Additional latency for the status report to advance the TX window, and

- Unnecessary interruption of UE processing due to timer expiry

Proposal 1: (RLC-11) When all RLC SDUs with SNs up to POLL_SN are already positively/negatively acknowledged or discarded by PDCP, the transmitter should stop and reset the running t-PollRetransmit.

Proposal 2: (RLC-10) There is no need to couple the configurations of stopReTxDiscardedSDU and t-RxDiscard.

Proposal 3: (RLC-14) If stopReTxDiscardedSDU is configured, whether a RLC SDU that has been submitted to the lower layer should be discarded when receiving a discarding indication from PDCP, is an implementation issue.

Proposal 4: (RLC-12) There is no need to further consider issues on PDCP SN Gap Reporting during mobility.

# References

- Rapporteur summary of discussion [POST130][508][XR] RLC Running CR and Open Issues (vivo)

- 3GPP RAN2 #130 Chair’s Notes, Malta, May2025.