3GPP TSG-RAN WG2 Meeting #131	R2-2505446

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.7.6

Source:	Apple

Title:	Remaining Issues of UL Rate Control for Rel-19 XR

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

The following open issues have been identified for Rel-19 UL Rate Control [1]:

- [MAC-3]	Design of identifier for QoS flows indicated in the UL Rate Control MAC CE.

- [MAC-4]	Format of the UL Rate Control MAC CE.

- [MAC-5]	Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

This paper presents some of our views on these open issues.

# Discussions

## (MAC-3) QoS Flow Identification in UL Rate Control MAC CE

Regarding the QFI indication in the MAC CE, in RAN2 #130 we have agreed the following [2]:

We rule out explicit signalling of PDU session ID + QFI

We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead:

    - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit)

    - Implicit, e.g. index or mapping

For XR applications, quite often we need to deal with data associated to multiple modalities that are mapped to different QoS flows (e.g. video, audio, haptics etc). When congestion is present, it may be desirable to adjust the bit rates of multiple QoS flows simultaneously. Thus, the signaling overhead can scale up easily with the number of concerned QoS flows in the MAC CE, if explicit indication of QoS flow ID is used. Thus, we think RAN2 should only focus on implicit indication of QFI.

Proposal 1: (MAC-3) For identifier of QoS flows in the UL Rate Control MAC CE, RAN2 should focus on implicit indication of QFI.

There are two approaches based on implicit indications have been discussed during RAN2 #130:

- Option 1: RRC Configuration of applicable QFI + MAC CE with QFI bitmap

- Option 2: MAC CE with both DRB ID and QoS flow bitmap

With Option 1, the network predefines a set of QFI that are subject to bit rate adaptation via RRC. Then, in the MAC CE a bitmap can be used to identify which of the QFI (within the predefined set) are recommended to adjust the bit rate. In cases there are a large number of QoS flows that are subject to rate adaptation, it can result in excessive signalling overhead in both RRC and MAC.

With Option 2, there is no need to pre-define the set of QFI. Instead, The MAC CE contains a DRB ID, along with a QFI bitmap of QoS flows mapped to the identified DRB. Both RRC and MAC CE signalling overhead can be reduced with this approach. Hence, Option 2 would be more efficient when only certain QoS flows within one DRB are desired to adjust the rate. Furthermore, Option 2 is also more aligned with the legacy bit rate recommendation MAC CE which already contains a LCH ID, so the additional efforts for implementation of this feature could be minimized.

Based on the discussion above, we think RAN2 should adopt the approach with DRB ID + QoS flow bitmap.

Proposal 2: (MAC-3) The QoS flows in the UL Rate Control MAC CE can be identified via a DRB ID along with a bitmap of QoS flows mapped to the indicated DRB.

## (MAC-4) UL Rate Control MAC CE Format

By following proposal 2, we think the UL rate control/query MAC CE should at least include the following fields:

- One DRB ID

- One bitmap indication of concerned QoS flow IDs that are mapped to the indicated DRB

- A list of recommended/preferred rates corresponding to the concerned QoS flow IDs

Figure 1 An exemplary MAC CE structure for XR rate control/query

Proposal 3: (MAC-4) The MAC CE for UL rate control/query should include the following:

- One DRB ID

- One bitmap indication of concerned QoS flow IDs that are mapped to the indicated DRB

- A list of recommended/preferred rates corresponding to the concerned QoS flow IDs

## (MAC-5) Insufficient Resource for UL Rate Query

The open issue [MAC-5] raised a concern when the UE does not have a sufficient UL-SCH resource that can accommodate the UL rate query MAC CE. From our perspective, there may be additional UE complexity if the UE needs to decide which (also how many) pending query(ies) should be sent by evaluating the available resource. Additionally, we tend to think the network can make a better decision about whether a query can be permitted, if it knows the full picture about the bit rate that is preferred by each QoS flow. When only partial information related to certain QoS flows is provided to the network, it may be insufficient for the network to determine a more appropriate UL rate control recommendation.

Furthermore, we have made the following agreement in RAN2 #130 [2]:

(MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow.

Since the procedure for legacy Recommended bit rate query has been agreed to be the baseline,  we think we should just follow the legacy behavior in this case. According to TS 38.321, the MAC entity would only transmit the Recommendation bit rate query when there is a UL resource that can accommodate this MAC CE:

If the MAC entity has UL resources allocated for new transmission the MAC entity shall: 1> for each Recommended bit rate query that the Recommended Bit Rate procedure determines has been triggered and not cancelled: 2> if bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query is configured, and it is not running; and 2> if the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate a Recommended bit rate MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1: 3> instruct the Multiplexing and Assembly procedure to generate the Recommended bit rate MAC CE for the logical channel and the direction of this Recommended bit rate query; 3> start the bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query; 3> cancel this Recommended bit rate query.

Thus, by following the legacy procedure, we think such optimization is not needed. That is, the UE should just send the UL rate query MAC CE when the allocated UL resource is sufficient to carry this MAC CE with all pending queries.

Proposal 4: (MAC-5) The UE should only transmit UL rate query MAC CE when the allocated UL resource can accommodate all pending queries.

# Conclusions

This paper aims to address the remaining open issues on UL rate control with the following proposals:

Proposal 1: (MAC-3) For identifier of QoS flows in the UL Rate Control MAC CE, RAN2 should focus on implicit indication of QFI.

Proposal 2: (MAC-3) The QoS flows in the UL Rate Control MAC CE can be identified via a DRB ID along with a bitmap of QoS flows mapped to the indicated DRB.

Proposal 3: (MAC-4) The MAC CE for UL rate control/query should include the following:

- One DRB ID

- One bitmap indication of concerned QoS flow IDs that are mapped to the indicated DRB

- A list of recommended/preferred rates corresponding to the concerned QoS flow IDs

Proposal 4: (MAC-5) The UE should only transmit UL rate query MAC CE when the allocated UL resource can accommodate all pending queries.

# References

- Rapporteur summary of discussion [POST130][505][XR]MAC Running CR and Open Issues Qualcomm)

- RAN2 #130 Chairman’s Notes