3GPP TSG-RAN WG2 Meeting #131	                                   R2-2505518

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.6.3

Source: 	OPPO

Title:  	Open issues for event-triggered L1 measurement reporting

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the remaining open issues for L1 event-triggered measurement reporting.

Discussion

MAC&RRC-LTM2

In current MAC running CR, for the generated MR MAC CE which triggered by LTM2, there is a note captured as following：

NOTE 3:	For the measurement report triggered by LTM2, the RS with Type of 00 is the current beam, which is always included in the last octet.

And in the event triggered L1 measurement report configuration, UE may be configured to report the measurement result of current RS of the serving cell by reportCurrentBeam.

For LTM2, it is natural that UE always report the current beam for LTM2, while the relation between the “Note” and the “reportCurrentBeam” indication is unclear due to the ambiguity of the role of the “current beam”. For example:

- Understanding 1: If the “current beam” of LTM2 is considered as triggered beam (type1 or type2), NW shall always set “reportCurrentBeam” as false to avoid duplicated current beam measurement reporting;

- Understanding 2: If the “current beam” is considered as current beam, NW should guarantee that UE is always configured with “reportCurrentBeam” for LTM2. Otherwise, UE may not report serving beam at all.

Either of the understanding requires modification on the field description of “reportCurrentBeam”. And understanding 1 is aligned with other LTM event. To unify the MR content of all LTM event, it is suggested to modify the presence condition for reportCurrentBeam for LTM2, i.e., the reportCurrentBeam is not configured for LTM2.

The reportCurrentBeam is not configured for LTM2.

MAC-8

| MAC-8 | FFS the coexistence between event triggered report and mTRP, e.g. how UE determine the current beam or candidate beam in this case? | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

In previous RAN1 meeting, there is a conclusion on that it is up to RAN2 to decide whether the coexistence of LTM event triggered reporting and mTRP operation at serving cells is supported or not.

Conclusion

The following is up to RAN2: Coexistence of LTM event triggered reporting and mTRP operation at serving cells

In general, we understand supporting LTM event triggered L1 measurement reporting should not cause any restrictions to serving cell configuration. If the coexistence is not supported, it means NW should disable the mTRP related configuration when providing an event triggered L1 measurement reporting configuration for LTM candidates. And LTM support subsequent cell switch execution, the event configuration is provided to UE during LTM preparation period. That is, the system may need to turn off the mTRP operation for a long period as UE may keep LTM config among multiple cell coverages.

As we agreed, mTRP MIMO 2TA can be configured with inter-CU LTM candidate and CLTM candidate, and LTM support subsequent cell switch execution. We understand current agreements have implicitly indicated the coexistence between event triggered L1 MR and mTRP at serving cell. Otherwise, NW has to reconfigure the LTM candidate/serving cell to disable mTRP function.

Coexistence of LTM event triggered reporting and mTRP operation at serving cell is supported.

Supporting the serving cell to configure mTRP requires minimal additional specification effort, which is how to define the beam of the serving cell since there are two indicated TCI states in this case.

In previous meeting, RAN2 agreed that the current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. The straightforward way is to consider the best beam as current beam for the LTM event evaluation.

If mTRP is configured at serving cell, the current beam corresponds to best beam of the two indicated beams for LTM event evaluation.

Furthermore, for event triggered MR MAC CE, NW may config UE to report the current beam in the triggered MR MAC CE. It is questionable on how to indicate NW which “current beam” is used and reported in the event triggered MR MAC CE. 1-bit indication may be required to differentiate which TRP the “current beam” belongs to.

If mTRP is configured at serving cell, the TRP info of the current beam used for LTM event evaluation is included in event triggered L1 MR MAC CE.

MAC-24

| MAC-24 | It is still working assumption on the details for truncated L1 MR MAC CE Editor’s NOTE: It is working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

In last meeting, the issue on how to handle the current serving beam in the truncated MR MAC CE has discussed the following working assumption has been reached, i.e. the UE always includes the current serving beam in a truncated MR MAC CE.

Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

In our understanding, the working assumption is reasonable as the “reportCurrentBeam” is configured by NW. UE should follow NW indication and if the grant is not enough to accommodate the triggered beam and current beam, UE should not send the truncated MAC CE. If NW expect to allow UE truncate the MR MAC CE without considering the current beam measurement, NW can set “reportCurrentBeam” to False.

Confirm the following working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	The reportCurrentBeam is not configured for LTM2.

Proposal 2	Coexistence of LTM event triggered reporting and mTRP operation at serving cell is supported.

Proposal 3	If mTRP is configured at serving cell, the current beam corresponds to best beam of the two indicated beams for LTM event evaluation.

Proposal 4	If mTRP is configured at serving cell, the TRP info of the current beam used for LTM event evaluation is included in event triggered L1 MR MAC CE.

Proposal 5	Confirm the following working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx