3GPP TSG-RAN WG2 #131                                          	    R2-2505544

Bengaluru, India, August 25-29, 2025

Agenda item:	   8.6.3

Source: 		Qualcomm Incorporated

Title: 	Discussion on L1 event-triggered measurement reporting

Document for:	Discussion

# 1	Introduction

RAN2#129 has agreed on the following for L1 event-triggered measurements for LTM [1]:

Agreements on stage-3 details - 16 beams in total including the current beam in MR MAC CE. - The beam offset for current beam of serving cell, i.e. Obs in the event evaluation inequality, is needed for the LTM event evaluation. Obs offset is configured per-cell, while applied at the level of indidual beam. Obs offset is applied to LTM3. - RAN2 assumes there is no L1-SINR quantity for L1 event triggered MR. - It is up to the NW configuration whether the UE can include measurements for LTM candidates in the MR MAC CE triggered by LTM2, i.e. based on the current configuration in RRC on the number of beams for LTM2 included in the MAC CE. - For MR triggered by LTM2, if measurements for LTM candidates is configured to be included in the MR MAC CE, which LTM candidate beam should be included: Only RS(s) associated with the report ID for LTM2. - The triggered MR is not cancelled when transmitting truncated MAC CE. The triggered MR is cancelled if regular MAC CE is reported, as in current MAC running CR. - When a beam (or multiple beam) meets the entry condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the leaving condition, then, the corresponding MR should be cancelled. - When a beam (or multiple beam) meets the leaving condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the entry condition, then, the corresponding MR should be cancelled. - When MR is triggered, if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, UE will trigger RACH procedure, as in legacy. - (RRC-10) Follow RAN1 agreement which indicate that at least one candidate RS (within the LTM CSI resource configuration) shall be configured for event LTM2. Agreements on L1 event-triggered MR: - UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell. - Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE. - Confirm working assumption, i.e. four codepoints to represent one of {type1, type2, type3, type4}, as an agreement. For detailed description of type 1, it can be handled as part of running CR preparation. - (MAC-6) For the truncated MR MAC CE, the UE determines the beam to be included in the MAC CE, based on the priority: Type 1 beam > Type 2 beam > Type 3 beam > Type 4 beam. If the UE cannot include all of the beams with the same priority, the beams are selected in the (truncated) MR MAC CE based on measured RS quality. Among all selected beams, the UE puts the best quality beam as the reference beam as the first one included in the (truncated) MR MAC CE. - Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE. - Same priority between regular MR MAC CE and truncated MR MAC CE. It is up to UE implementation how to handle/include if more than one triggering events are pending and UL grant is not sufficient for all regular L1 MR MAC CEs. - No prohibit timer is applied to L1 event-triggered MR.

This paper discusses further aspects related to L1 event-triggered measurement report enhancements for LTM.

# 2	mTRP support

In RAN1#120, the coexistence of LTM event-triggered reporting and mTRP operation at serving cells was discussed, and it was concluded that it is up to RAN2 to decide the support of coexistence between LTM event-triggered reporting and mTRP [3]. And the following notes were made during the previous RAN2 meetings, and the post email discussion has been initiated.

RAN2#129b chair’s note - Will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR. RAN2#130 chair’s note - Simple solution should be preferable and considering next meeting is the last meeting for WI completion, everything (including companies’ consensus, one solution, and the corresponding TP) should be ready for next meeting. [POST130][117][MOB] (Nokia) Scope: Discuss issues of coexistence between L1 event triggered MR and mTRP (source cell). - Whether to allow coexistence between event triggered MR and mTRP (source cell)? - How to define the current beam? Intended outcome: Summary discussion. Deadline: Long email discussion

While many companies support the coexistence between LTM L1 event-triggered measurement reporting and mTRP in the email discussion, this coexistence discussion has some concerning points.

The mTRP support for LTM L1 event-triggered measurement reporting has been discussed without any RAN1 discussion/support. Since Rel-19 UE-initiated/event-driven beam reporting in MIMO WID has explicitly excluded the mTRP support (i.e., only for sTRP), RAN1 had no strong reason to support the mTRP for LTM event-triggered measurement reporting, hence it was left up to RAN2 decision. Under this circumstance, any solution derived solely within RAN2 may not work as expected, as the discussions have been somewhat rushed.

In the ongoing email discussion, for the current serving beam selection in mTRP, the best beam chosen between two current beams got the most support. But it’s unclear to us whether the event evaluation based on the best beam would provide the expected advantage of event-triggered reporting or not. As an example, if one serving beam is consistently worse than the other beam, then the event may not be triggered at all due to the stronger beam. Even though the event is triggered, there’s still question of which RSRP values would have to be reported based on one of two serving beams.

Given the insufficient discussion/support from RAN1 and the lack of enough discussions within RAN2, we believe it’s logical to not support mTRP in LTM event-triggered measurement reporting in Rel-19.

Observation 1: The mTRP discussion for LTM event-triggered MR doesn’t have any reference discussion and/or support from RAN1 MIMO UE-initiated/event-driven beam reporting which focuses only on sTRP. Therefore, the solution being discussed is forced to be simple due to the lack of time which limit the technical benefits of mTRP support.

Proposal 1: RAN2 agree to not support the coexistence between mTRP and LTM event-triggered measurement reporting in Rel-19.

# 3	Measurement reporting triggers and MR MAC CE generation (Open issue MAC-22)

In [POST130][111][MOB] email discussion for MAC, the following item is discussed for handling multiple MR triggering and the corresponding L1 MR MAC CE generation.

Open issue MAC-22: whether multiple MR should be triggered and wether only one or multiple L1 measurement report MAC CE should be generated in case multiple beams associated with an LTM report configuration satisfies triggering condition for TTT.

Since in RAN2#128 it was agreed that TTT is evaluated per beam and the measurement report is triggered by beam that satisfied the condition for the whole duration of TTT, it is possible that multiple beams fulfil the event conditions with a small time gap (e.g., due to different measurement schedule), which could lead to multiple measurement reports triggered within a short period of time.

When multiple measurement reports are triggered, however, it doesn’t necessarily mean that the L1 MR MAC CE has to be generated for each triggered measurement report. As there will be a time delay from the event detection through the dedicated SR transmission to the UL grant from L1 MR MAC CE (assuming there’s no on-going UL grant), all triggered measurement reports corresponding to the same event-triggered CSI report configuration can be combined into a single L1 MR MAC CE, which is already captured in the current MAC running CR.

Observation 2: As per previous RAN2 agreements, it is expected that multiple measurement reports can be triggered in a staggered manner within a time window depending on the measurement schedule. But those triggered measurement reports could be still combined into one L1 MR MAC CE depending on the UL grant schedule.

Proposal 2: RAN2 agree to confirm the current MAC running CR clause 5.x.3 and 5.x.4, i.e., multiple measurement reporting procedures can be triggered for an event-triggered CSI report configuration. However, only a single MR MAC CE is generated after aggregating all initiated measurement reporting instances.

# 4	L1 MR MAC CE

## 4.1	Minimum payload size for truncated MR MAC CE

In RAN2#130, the minimum payload size for truncated L1 MR MAC CE was discussed and the following working assumption was made.

Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

During the discussion, there was a concern that the truncated MR MAC CE won’t be transmitted if (i) the current serving beam report is configured and (ii) the available UL grant is not enough to carry the current serving beam measurement (i.e., the grant size is short of 1 byte). Although the scenario is theoretically possible, the chance of running into this situation would be quite low. Since the logical channel priority of MR MAC CE is same as that of (enhanced) BFR MAC CE, the insufficient UL grant size for the truncated MR MAC CE can be avoided via NW scheduling implementation. Therefore, the working assumption can become an agreement with both cases covered, i.e., whether the current beam reporting is configured or not.

Observation 3: The minimum payload size of truncated MR MAC CE needs to be based on the RRC configuration (e.g., current beam reporting). Since MR MAC CE has the same priority as (enhanced) BFR MAC CE, the chance of running into a corner case (e.g., not being able to send truncated MR MAC CE due to lack of UL grant size) will be very low.

Proposal 3: RAN2 to confirm WA with the following minimum payload definitions for truncated MR MAC CE.

  - {Report ID + Beam type + RSRItriggered_beam + RSRPtriggered_beam + RSRPcurrent_serving_beam}, if current serving beam report is enabled

  - {Report ID + Beam type + RSRItriggered_beam + RSRPtriggered_beam}, otherwise

## 4.2	Handling pending event report(s) and/or remaining beam result(s) following truncated L1 MR MAC CE

In RAN2#129b, the following agreement was made, and it is also discussed in the MAC running CR email discussion.

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

When UE sends a truncated L1 MR MAC CE due to the lack of sufficient PUSCH grant, UE may be in a situation where there are still one or more of pending beam results from this truncated MR MAC CE (i.e., some beams could not be included into this truncated L1 MR MAC CE) and/or one or more of pending reports (i.e., these triggered events’ reports were dropped/deprioritized due to lack of PUSCH grant and event prioritization).

In the current L1 MR MAC CE design, there is no indication of such pending reports and/or beam results to gNB, and these pending measurement reports cannot be indicated by BSR, either. Therefore, in the worst case where there’s no dedicated SR, UE has no on-going PUSCH traffic, and multiple events are triggered, UE may need to send SR multiple times to send those triggered L1 MR MAC CEs, which could take a long time for UE to send all of them. Considering the motivation of event-triggered measurement report is to enable a timely transmission of measurement results for LTM cell switch, it would be beneficial to have an explicit indication of pending event reports and/or beam results in L1 MR MAC CE.

# 5	LTM event evaluation (Different HYS/TTT values for leaving condition)

In RAN2#127b, it was agreed that a measurement report can be sent when the leaving condition is met based on NW configuration, and in RAN2#128, it was agreed that TTT is applied to the leaving condition.

In L3 event-based measurement reports, one set of HYS and TTT values are configured per event configuration, and they are applied to both entering and leaving conditions as illustrated in Figure 1. Applying the same HYS and TTT values to both entering and leaving conditions wouldn’t make much difference in triggering the L3 measurement reports and subsequent L3 HOs because (i) the cell metric is used for the evaluation hence the metric is less sensitive to the beam level metric differences, (ii) the measurement report through RRC signaling has a larger time scale than lower layer signaling (UCI or event-triggered L1 measurement MAC CE), and (iii) L3 HO has a longer overall switching latency than LTM switch (i.e., from the measurement report trigger to the cell switch complete).

Observation 5a: For L3 measurement report, the same HYS and TTT are applied to both entering and leaving conditions. Since L3 measurement report and HO deal with the cell level measurement (not beam level), it won’t be sensitive to identifying the leaving condition quicker.

Figure 1. Illustration of entering and leaving condition in L3 measurement Event A3

In event-triggered L1 measurement reports for LTM, however, the situation becomes different. For a seamless LTM cell switch, it is crucial to keep track of the best candidate beam among those who fulfilled the entering condition for the potential cell switch so that serving cell can perform the candidate TCI state activation and/or early UL synchronization with that chosen beam. While a candidate beam satisfying the entering condition can be chosen conservatively via HYS/TTT, any candidate beam that is no longer good for cell switch (i.e., leaving condition) should be identified by serving cell through a measurement report as quickly as possible. In this aspect, applying the same HYS and TTT could delay the leaving condition to be triggered significantly.

Observation 5b: For L1 measurement report, keeping track of the best candidate beam is crucial due to the possible TCI state activation and/or early UL synchronization. Therefore, any candidate beam that is not satisfying the better quality than the serving beam (i.e., leaving condition) needs to be identified as quickly as possible.

In order to expedite the event-triggered L1 measurement report for leaving condition, there can be two options to be considered:

- Option 1: Keep one set of HYS and TTT for both entering and leaving conditions, and set them to smaller values

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

Option 1 can be the simplest solution without any specification impact, but the smaller HYS and TTT values would impact the entering condition to be triggered quicker as well, which could lead to unexpectedly frequent measurement reports triggered due to potential noisy L1 measurements (without any spec-defined filtering applied). Both Option 2 and 3 provide the configurability of different HYS and/or TTT values between entering and leaving conditions, but Option 3 could be preferrable due to its flexibility in not providing additional HYS and/or TTT for leaving condition if NW chooses not to.

Observation 5c: In order to achieve rather conservative entering condition but a bit aggressive leaving condition, separate configurations of HYS and/or TTT for leaving condition is desirable.

Proposal 5: Support the configurability of different HYS and/or TTT values between entering and leaving conditions with following options:

- Option A: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option B: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

# Conclusion

This contribution discussed aspects related to measurement enhancements for LTM. The following observations and proposals have been made:

Observation 1: The mTRP discussion for LTM event-triggered MR doesn’t have any reference discussion and/or support from RAN1 MIMO UE-initiated/event-driven beam reporting which focuses only on sTRP. Therefore, the solution being discussed is forced to be simple due to the lack of time which limit the technical benefits of mTRP support.

Proposal 1: RAN2 agree to not support the coexistence between mTRP and LTM event-triggered measurement reporting in Rel-19.

Observation 2: As per previous RAN2 agreements, it is expected that multiple measurement reports can be triggered in a staggered manner within a time window depending on the measurement schedule. But those triggered measurement reports could be still combined into one L1 MR MAC CE depending on the UL grant schedule.

Proposal 2: RAN2 agree to confirm the current MAC running CR clause 5.x.3 and 5.x.4, i.e., multiple measurement reporting procedures can be triggered for an event-triggered CSI report configuration. However, only a single MR MAC CE is generated after aggregating all initiated measurement reporting instances.

Observation 3: The minimum payload size of truncated MR MAC CE needs to be based on the RRC configuration (e.g., current beam reporting). Since MR MAC CE has the same priority as (enhanced) BFR MAC CE, the chance of running into a corner case (e.g., not being able to send truncated MR MAC CE due to lack of UL grant size) will be very low.

Proposal 3: RAN2 to confirm WA with the following minimum payload definitions for truncated MR MAC CE.

  - {Report ID + Beam type + RSRItriggered_beam + RSRPtriggered_beam + RSRPcurrent_serving_beam}, if current serving beam report is enabled

  - {Report ID + Beam type + RSRItriggered_beam + RSRPtriggered_beam}, otherwise

Observation 4: The presence of pending beam results (beam measurements that are dropped via truncation) and/or pending reports (triggered event reports that are dropped due to event prioritization) cannot be signalled to gNB, which could take a long time for UE to send all triggered event reports to the gNB.

Proposal 4: RAN2 agree to support explicit indication(s) of pending beam results and/or pending reports in L1 MR MAC CE.

Observation 5b: For L1 measurement report, keeping track of the best candidate beam is crucial due to the possible TCI state activation and/or early UL synchronization. Therefore, any candidate beam that is not satisfying the better quality than the serving beam (i.e., leaving condition) needs to be identified as quickly as possible.

Observation 5a: For L3 measurement report, the same HYS and TTT are applied to both entering and leaving conditions. Since L3 measurement report and HO deal with the cell level measurement (not beam level), it won’t be sensitive to identifying the leaving condition quicker.

Observation 5c: In order to achieve rather conservative entering condition but a bit aggressive leaving condition, separate configurations of HYS and/or TTT for leaving condition is desirable.

Proposal 5: Support the configurability of different HYS and/or TTT values between entering and leaving conditions with following options:

- Option A: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option B: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

# References

- Chairman Notes; TSG RAN WG2 Meeting #130