3GPP TSG-RAN WG2 #131                                          	    R2-2505546

Bengaluru, India, August 25-29, 2025

Agenda item:	   8.6.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on inter-CU LTM

Document for:	Discussion

# 1	Introduction

RAN2#129b has agreed on the following for inter-CU LTM [1]:

Agreements on stage-3 details - The Semi-persistent CSI-RS resource set for candidate cell MAC CE is initially deactiated upon (re-)configuration by upper layer. - (RRC-2) The appliedLTM-CandidateId is included in both MN and SN RRCReconfigurationComplete message (Rel18 specification should be okay). - For RRC-3, keep current text in the running RRC CR. - (RRC-6) In inter-MN LTM failure (LTM cell switch failure) the UE fall back to the previous source PCell configuration before selecting a cell (same as legacy procedure). - (RRC-7) The legacy sk-counter is used for inter-MN LTM (this means sk-counter value can be re-used). Agreements on inter-CU LTM: - RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell. - If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency. - RAN2 not support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE. - The Enhanced LTM CSC MAC CE always contains an NCC value field. The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

This paper discusses further aspects related to the SP CSI-RS/CSI-IM management.

# 2	SP CSI-RS/CSI-IM resource management

## 2.1	Common SP CSI-RS activation/deactivation for RRM measurement and early CSI acquisition

In legacy SP CSI-RS resource activation/deactivation, the common activation/deactivation MAC CE (i.e., SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE) is used for SP CSI-RS resources across different report quantities (e.g., CSI-RSRP or CRI-RI-PMI-CQI).

For RRM measurement and early CSI acquisition in LTM, the same principle can be applied such that SP CSI-RS Resource Set Activation/Deactivation for Candidate Cell MAC CE can be used. During the email discussions, there were some concerns about using the common activation/deactivation MAC CE for both cases (e.g., SP CSI-RSs from multiple candidate cells within LTM CSI resource configuration for early CSI acquisition or UE-side behavioural ambiguity between RRM measurement and early CSI acquisition after SP CSI-RS activation), but those are not real issues.

Firstly, NZP-CSI-RS resource for RRM measurement (i.e., L1-RSRP measurement for BM) requires a configuration for repetition, and RAN1#120b agreed to support only ‘Repetition=OFF’ for L1-RSRP measurement in Rel-19. On the other hand, NZP-CSI-RS resource for early CSI acquisition (i.e., RI/CQI/PMI measurement) won’t have the repetition configuration. Therefore, there will be no ambiguity in UE behaviour when it receives SP CSI-RS activation for candidate cell MAC CE.

Secondly, LTM CSI resource configuration can have multiple NZP-CSI-RS resources from a single candidate cell as well as multiple NZP-CSI-RS resources from multiple candidate cells. Given that LTM CSI-IM resource set configuration includes only one candidate cell’s CSI-IM resource set ID, this would impose implicit restriction onto LTM CSI resource configuration for CMR to have NZP-CSI-RS resources from a single candidate cell.

To sum up, LTM NZP-CSI-RS resource set (LTM-NZP-CSI-RS-ResourceSet-r19) is expected to be configured like below:

- For RRM measurement, NZP-CSI-RS resources are from one or more candidate cells with a single CSI-RS port & repetition set to OFF.

- For early CSI acquisition, NZP-CSI-RS resources are from one candidate cell with >= 1 CSI-RS port & repetition not configured.

Observation 1: Similar to legacy SP CSI-RS activation/deactivation MAC CE, the new SP CSI-RS activation/deactivation for candidate cell MAC CE can be used for both RRM measurement and early CSI acquisition. LTM NZP-CSI-RS resource set and the associated NZP-CSI-RS resources have distinct information for UE to distinguish between RRM measurement and early CSI acquisition.

Proposal 1: RAN2 agree to use SP CSI-RS resource set activation/deactivation for candidate cell(s) MAC CE commonly for both RRM measurements (L1-RSRP) and early CSI acquisition (CQI/RI/PMI).

## 2.2	SP CSI-IM activation/deactivation for candidate cell

In RAN1#121, it was agreed to support a list of IMRs for LTM early CSI acquisition, and during the email discussion for MAC running CR, the activation/deactivation of SP CSI-IM resources is discussed.

RAN1#121 agreements [2] A list of interference measurement resources for candidate cells is supported for LTM CSI acquisition - If this list is not configured, CMR is used for interference measurement R1-2503243 [3] (consolidated Rel-19 higher layer parameters) snippet

When it comes to activation/deactivation of SP CSI-IM resource set from candidate cell, the same approach as in Rel-15 SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE can be adopted, i.e., both SP CSI-RS and SP CSI-IM can be activated via a single MAC CE.

Since in RAN2#130, it was decided to use LTM-CSI-ResourceConfigId to specify the list of NZP-CSI-RS resources from candidate cells (i.e., LTM-NZP-CSI-RS-ResourceSet) to be activated/deactivated, LTM-CSI-ResourceConfigId can be also used to specify the CSI-IM resources from a candidate cell.

Observation 2: Similar to the legacy, SP CSI-RS Resource Set Activation/Deactivation for Candidate Cell MAC CE can be extended to activated/deactivate SP CSI-IM resource set from a candidate cell.

Proposal 2: RAN2 agree to extend SP CSI-RS resource set activation/deactivation for candidate cell MAC CE to support SP CSI-IM resource set for candidate cell by adding two new fields - ‘IM’ and ‘LTM CSI Resource Config ID for IMR’.

## 2.3	Activated SP CSI-RS/CSI-IM resource handling after LTM cell switch

In RAN2#130, it was agreed that UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch and the target cell case was left as FFS.

In LTM CSI report framework, each cell (serving or candidate) is responsible for creating its own LTM CSI Report Config and LTM CSI Resource Config, where LTM CSI Resource Config contains NZP-CSI-RS Resource configurations from serving and/or neighbouring candidate cells. This means each cell could have completely independent LTM CSI Resource configurations as well as legacy CSI Resource configurations, therefore one or more of SP CSI-RS resources which are activated by the previous serving cell may not be part of LTM CSI Resource configurations nor legacy CSI Resource configurations.

Observation 3a: In general, each cell has its own LTM CSI Resource configurations so that keeping SP CSI-RS resources activated after LTM cell switch may not be possible at all in some cases.

Additionally, RAN1#121 discussed the similar issue of defining active CSI-RS resources after CSC (snippet copied below), and the conclusion was made to assume there’s no active CSI-RS ports/resources for candidate cells including target cell after CSC.

Observation 3b: RAN1#121 made a conclusion that active CSI-RS ports/resources for candidate cells including target cell are not defined after CSC.

##### R1-2502627 [4] (FL summary 4 of Measurements related enhancements for LTM) ##### [FL proposal 5-12-v2] Definition of active CSI-RS resources after CSC until the completion of LTM procedure - Alt .1: Both periodic and semi-persistent CSI-RS resource(s) for the CSI-RS resource(s) associated with the target cell are counted as active - Alt. 2: CSI-RS resource(s) for target cell provided by LTM-CSI-SSB-ResourceSet should be counted as active based on the new rule below - Periodic CSI-RS - it is associated with the indicated TCI state in cell switch command and, - it is associated with an active CSI report configuration. - Semi-persistent CSI-RS - it is activated, - it is associated with the indicated TCI state in cell switch command and, - it is associated with an active CSI report configuration. - Alt.3: Active CSI-RS ports/resources for the candidate cells including target cell are not defined after CSC until the completion of LTM procedure - FL note: no other CSI process than is conducted during LTM procedure and neither of serving cell and candidate cell can manage the CPU during this transient period RAN1#121 conclusion [2] Definition of active CSI-RS resources after CSC until the completion of LTM procedure: - Active CSI-RS ports/resources for the candidate cells including target cell are not defined after CSC until the completion of LTM procedure

Based on above observations, we believe it’s more logical for UE to deactivate SP CSI-RS resources for target cell after LTM cell switch. In addition to SP CSI-RS, SP CSI-IM is currently discussed in the RAN2 email discussion, and the handling of SP CSI-IM should be no different from SP CSI-RS.

Proposal 3: RAN2 agree that UE deactivates all activated SP CSI-RS and SP CSI-IM resources for candidate cells including target cell from after LTM cell switch, which is applicable to both RRM measurement and early CSI acquisition.

## 2.4	Granularity of SP CSI-RS/CSI-IM resources in activation/deactivation MAC CE

In RAN2#130, it was agreed that LTM-CSI-ResourceConfigId is included in SP CSI-RS activation/deactivation MAC CE, and the following MAC CE format has been captured in the MAC running CR.

Figure 1. SP CSI-RS Resource Set Activation/Deactivation for Candidate Cell MAC CE in MAC running CR

While above MAC CE format mimics the legacy SP CSI-RS/CSI-IM resource set activation/deactivation MAC CE that manages all NZP-CSI-RS resources in one shot (i.e., all NZP-CSI-RS resources are activated or deactivated as a whole), that approach may not work well for LTM SP CSI-RS resource management.

Firstly, serving cell may not have a clear view on UE’s mobility trajectory such that LTM CSI resource configuration may need to include a broader set of candidate cells and their NZP-CSI-RS resources. Under this circumstance, the legacy approach of one-shot activation/deactivation would end up causing unnecessary activation of SP CSI-RS resources of no interest (w.r.t. spatial beam direction and/or distance from candidate cells) and excessive UE measurement overhead for those activated SP CSI-RS resources.

Observation 4a: LTM CSI resource configuration is likely to include a broader set of candidate cells and their NZP-CSI-RS resources, therefore one-shot activation of those NZP-CSI-RS resources within LTM CSI resource configuration would cause unnecessary activation of NZP-CSI-RS resources of no interest and excessive UE measurement overhead.

Secondly, to overcome the abovementioned issue, serving cell may want to create many LTM CSI resource configurations, each of which covers a small number of SP CSI-RS resources from a few candidate cells. While this kind of LTM CSI resource configuration is possible, this would lead to another problem of needing to create those many LTM CSI report configurations. As shown in the LTM-CSI-ReportConfig snippet below, each LTM CSI report can have only one LTM CSI resource configuration as a channel measurement resource, therefore configuring multiple LTM CSI resources to cope with one-shot activation/deactivation overhead is not a feasible solution.

Observation 4b: Multiple LTM CSI resource configurations, each of which includes a small set of NZP-CSI-RS resources, could avoid abovementioned issues, but it would require many LTM CSI report configurations as each LTM CSI report configuration can have only a single LTM CSI resource configuration, which is not efficient either.

Figure 2. LTM-CSI-ReportConfig & LTM-CSI-ResourceConfig IE in RRC running CR

In order to efficiently handle possibly large number of SP CSI-RS resources from multiple candidate cells within an LTM CSI resource configuration, it would be logical to support the selective activation/deactivation of NZP-CSI-RS resource IDs within the LTM CSI resource configuration, where, for activation only selected NZP-CSI-RS resource IDs are assigned TCI state IDs.

Proposal 4: RAN2 agree to support the selective activation/deactivation of SP CSI-RS resources where only selected NZP-CSI-RS resource IDs (e.g., referred by the ordinal index to LTM CSI-RS Resource List) are activated (assigned TCI state IDs) or deactivated.

When selective activation/deactivation of SP CSI-RS resources is supported, the delta update of SP CSI-RS resources would provide more flexibility and less signalling overhead in maintaining the SP CSI-RS resources, i.e., when activation/deactivation MAC CE is received, UE keeps the existing activation status and apply the activation/deactivation change indicated by MAC CE on top of it.

Observation 5: Along with the selective activation/deactivation, the delta update of SP CSI-RS resources can provide more flexibility and less signalling overhead for SP CSI-RS resource management.

Proposal 5: RAN2 agree to support the delta activation/deactivation of SP CSI-RS resources, where UE keeps additional SP CSI-RS resources are newly activated or partial activated SP CSI-RS resources are deactivated while UE keep the activation status for other SP CSI-RS resources than the specified.

In case of SP CSI-IM, LTM CSI-IM resource set has a single LTM candidate cell ID and its CSI-IM resource set ID, therefore we don’t see a strong need of supporting selective activation/deactivation of individual CSI-IM resources. Just like the legacy SP CSI-IM activation/deactivation, all CSI-IM resources within CSI-IM resource set can be activated or deactivated.

Observation 6: LTM CSI-IM resource set has a single candidate cell and its CSI-IM resource set ID, therefore there’s no strong need of supporting the selective activation/deactivation.

Proposal 6: RAN2 agree to support the ‘one-shot’ activation/deactivation of SP CSI-IM resources where all CSI-IM resource IDs within CSI-IM resource set ID (referred by LTM CSI resource ID) are activated or deactivated.

# 3	Conclusion

This contribution discussed aspects related to SP CSI-RS/CSI-IM in inter-CU LTM. The following observations and proposals have been made:

Observation 1: Similar to legacy SP CSI-RS activation/deactivation MAC CE, the new SP CSI-RS activation/deactivation for candidate cell MAC CE can be used for both RRM measurement and early CSI acquisition. LTM NZP-CSI-RS resource set and the associated NZP-CSI-RS resources have distinct information for UE to distinguish between RRM measurement and early CSI acquisition.

Proposal 1: RAN2 agree to use SP CSI-RS resource set activation/deactivation for candidate cell(s) MAC CE commonly for both RRM measurements (L1-RSRP) and early CSI acquisition (CQI/RI/PMI).

Observation 2: Similar to the legacy, SP CSI-RS Resource Set Activation/Deactivation for Candidate Cell MAC CE can be extended to activated/deactivate SP CSI-IM resource set from a candidate cell.

Proposal 2: RAN2 agree to extend SP CSI-RS resource set activation/deactivation for candidate cell MAC CE to support SP CSI-IM resource set for candidate cell by adding two new fields - ‘IM’ and ‘LTM CSI Resource Config ID for IMR’.

Observation 3a: In general, each cell has its own LTM CSI Resource configurations so that keeping SP CSI-RS resources activated after LTM cell switch may not be possible at all in some cases.

Observation 3b: RAN1#121 made a conclusion that active CSI-RS ports/resources for candidate cells including target cell are not defined after CSC.

Proposal 3: RAN2 agree that UE deactivates all activated SP CSI-RS and SP CSI-IM resources for candidate cells including target cell from after LTM cell switch, which is applicable to both RRM measurement and early CSI acquisition.

Observation 4a: LTM CSI resource configuration is likely to include a broader set of candidate cells and their NZP-CSI-RS resources, therefore one-shot activation of those NZP-CSI-RS resources within LTM CSI resource configuration would cause unnecessary activation of NZP-CSI-RS resources of no interest and excessive UE measurement overhead.

Observation 4b: Multiple LTM CSI resource configurations, each of which includes a small set of NZP-CSI-RS resources, could avoid abovementioned issues, but it would require many LTM CSI report configurations as each LTM CSI report configuration can have only a single LTM CSI resource configuration, which is not efficient either.

Proposal 4: RAN2 agree to support the selective activation/deactivation of SP CSI-RS resources where only selected NZP-CSI-RS resource IDs (e.g., referred by the ordinal index to LTM CSI-RS Resource List) are activated (assigned TCI state IDs) or deactivated.

Observation 5: Along with the selective activation/deactivation, the delta update of SP CSI-RS resources can provide more flexibility and less signalling overhead for SP CSI-RS resource management.

Proposal 5: RAN2 agree to support the delta activation/deactivation of SP CSI-RS resources, where UE keeps additional SP CSI-RS resources are newly activated or partial activated SP CSI-RS resources are deactivated while UE keep the activation status for other SP CSI-RS resources than the specified.

Proposal 6: RAN2 agree to support the ‘one-shot’ activation/deactivation of SP CSI-IM resources where all CSI-IM resource IDs within CSI-IM resource set ID (referred by LTM CSI resource ID) are activated or deactivated.

# References

- Chairman Notes; TSG RAN WG2 Meeting #130

- Chairman Notes; TSG RAN WG1 Meeting #121

- R1-2502627, Higher layer parameters for Rel-19 NR mobility enhancements Ph4

- R1-2504889, FL summary 4 of Measurements related enhancements for LTM