3GPP TSG-RAN WG2 Meeting #131	R2-2505561

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.2.4

Source:	Nokia

Title:	Open issues in AIoT

WID/SID:	Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses several open issues as identified by the Rapporteur in [POST130][027][AIoT] MAC Running CR (Huawei) and concerns more specifically aspects of the inventory/command use case.

# 2	Discussion

RAN1 agreed that the interval between R2D transmission and the starting time of the D2R in time domain is within the range of 30us–1.33ms. At the same time, RAN2 made the following agreements:

Agreements - The device is expected to send a MAC response to the reader in the D2R occasion. The MAC response contains the NAS message if available at the D2R occasion. If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed. - Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response. RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command). Ask if this can be handled by CT1.

If a device fails to generate a command response within the permitted time frame (eg, the command requires extensive memory access and/or processing), the device sends an empty response with the sole purpose to satisfy the PHY requirements.

Observation 1: Device sends an empty MAC SDU if command response is not generated within transmission deadline.

However, the device may have the response data ready shortly after the “dummy” transmission is completed, and the question is now how to retrieve it.

One approach is to leave the task of retrieving a delayed command response to the application layer. For example, the AIoT-AF may issue a new command to read the response to the previous command. This approach however does not solve the underlying problem of the device being too slow in responding to (complex) commands. Even the sheer size of the response to be read from memory and the associated (signal) processing may take more time than permitted. The AIoT-AF may thus find itself in a long or even perpetual retrieval process.

It is therefore more favourable to leave this task to the lower layers, namely MAC. As per 38.391, the MAC already includes signalling options for D2R segmentation, including the “More Data Indication” (MDI) field. If a device expects a command to yield a response (eg, based on application-layer/implementation feedback, or proactively), raising the MDI flag allows the reader to poll the delayed command response in the future.

Proposal 1: D2R segmentation signalling is used to indicate the presence of delayed command response to enable its future polling by the reader.

In total, the reader needs to distinguish four transmission cases:

no data to be sent (in current or later transmission),

current transmission delivers all buffer data (once-off delivery),

current transmission is insufficient to clear buffer (segmentation needed),

data expected to be sent later (eg, delayed command response).

Observation 2: The combination of “More Data Indication” (MDI) and “SDU Length” (SDU_L) permits to encode four transmission cases involving (i) partial / complete delivery of (ii) current / future data.

The presence of a response may be indicated to the device within the command itself (eg in its very first part so that the device can retrieve this information prior its mandatory response transmission). Alternatively, a device implementation  may proactively trigger future polling as only indicate definitive response absence later.

To reflect this approach, we make the following proposal:

Proposal 2: Device uses MAC parameters “More Data Indication” (MDI) and “SDU Length” (SDU_L) to indicate to the reader any outstanding data (including future CMD response) beyond current MAC SDU transmission as follows:

- no data to be sent (in current or later transmission)					MDI=0  	SDU_L=0,

- current transmission delivers all buffer data							MDI=0  	SDU_L>0,

- current transmission is segmented 										MDI=1  	SDU_L=0,

- data expected to be sent in future (buffer empty) 					MDI=1  	SDU_L>0.

With the MDI flag raised to 1, the reader understands that another transmission from the corresponding device must be initiated to poll the outstanding data, be is the remainder of a segmented transmission or future data to be generated by the device.

Observation 3: If device indicates MDI=1, the reader needs to actively poll the outstanding data from the device.

The following proposal captures the approach of leaving the task of outstanding data polling to the MAC layer rather than the slower and less flexible application layer.

Proposal 3: MAC design enables best-effort measures for simple and efficient delivery (polling) of outstanding device data, irrespective of possible application-layer mechanisms of such type.

In this context, RAN2 needs to decide if reader polling must be done within the same session (identified by the same transaction ID), or even under a new paging process. An important observation is to be made in this context.

Observation 4: Renewed CBRA paging with the same transaction ID does not release the AS IDs if the previous transaction was not deemed as failed.

Proposal 4: RAN2 to decide if reader polling of outstanding device data is permitted

- only in the context of the same transaction ID, or

- also after new paging (eg, selectively involving "MDI=1" devices). FFS transaction ID and AS ID reuse in polling-only scenarios.

A related question is how many devices are polled. If the same transaction (session) is used, devices may be polled individually. For example, poling may be triggered by indicating “Received Data Size=0” to the device after having received an empty SDU. Multi-device paging with repeated transaction ID may be the basis for more efficient polling, possibly based on a new message type.

Proposal 5: RAN2 to decide if the polling of outstanding data is done on the basis of single-device or multi-device communications.

The current specification clearly defines the release of AS IDs as dependent on the transaction usage whereby the case of repeated transaction ID usage requires further development. In view of the above proposal, it is desirable to simplify the specification as well as provide robust design by refining a new signal that controls the release of AS IDs.

While it may be desirable to maintain AS ID even beyond a given paging instance as described above, it is also necessary to ensure that all AS IDs are correctly released, eg in case the reader does not or cannot find the opportunity to poll a given device. With no further action from the reader, the device may freeze in the “polling” state. Selective AS ID release in devices for example reporting MDI=1 may be even more desirable. The unicast / groupcast / broadcast nature of this signal is another aspect to be studied.

Proposal 6: RAN2 to adopt a mechanism for release of AS IDs, involving

- all devices active under specific transaction ID, and/or

- devices reporting MDI=1 status.

FFS unicast / groupcast / broadcast options.

The device itself should have be able to determine when polling is expected, for example to release data from the buffer if no polling is to be expected anymore. Protection from the corner case of no reader-side action is thus needed. To this end, a device may use standard mechanisms such as a pre-defined time limit (possibly implemented as a number of access occasions to address limited time-keeping capabilities of certain AIoT devices).

Proposal 7: There is a time limit (eg a specific number of contiguous access occasions) over which a device reports MDI=1 status expects to be polled for outstanding data. After this time limit elapses, the device will no longer guarantee the data availability (data release left to implementation and/or application layer).

Since repeated communications and polling in general are prone to replay and spoofing attacks, it is critical to ensure robust design also in this space.

Proposal 8: RAN2 to ensure that the polling of outstanding data is protected against replay / spoofing attack, including the option of sending an LS to SA3.

# 3	Conclusion

Always echo in the conclusion all proposals made above.

This document has made the following observations:

Observation 1: Device sends an empty MAC SDU if command response is not generated within transmission deadline.

Observation 2: The combination of “More Data Indication” (MDI) and “SDU Length” (SDU_L) permits to encode four transmission cases involving (i) partial / complete delivery of (ii) current / future data.

Observation 3: If device indicates MDI=1, the reader needs to actively poll the outstanding data from the device.

Observation 4: Renewed CBRA paging with the same transaction ID does not release the AS IDs if the previous transaction was not deemed as failed.

And proposed the following:

Proposal 1: D2R segmentation signalling is used to indicate the presence of delayed command response to enable its future polling by the reader.

Proposal 2: Device uses MAC parameters “More Data Indication” (MDI) and “SDU Length” (SDU_L) to indicate to the reader any outstanding data (including future CMD response) beyond current MAC SDU transmission as follows:

- no data to be sent (in current or later transmission)					MDI=0  	SDU_L=0,

- current transmission delivers all buffer data							MDI=0  	SDU_L>0,

- current transmission is segmented 										MDI=1  	SDU_L=0,

- data expected to be sent in future (buffer empty) 					MDI=1  	SDU_L>0.

Proposal 3: MAC design enables best-effort measures for simple and efficient delivery (polling) of outstanding device data, irrespective of possible application-layer mechanisms of such type.

Proposal 4: RAN2 to decide if reader polling of outstanding device data is permitted

- only in the context of the same transaction ID, or

- also after new paging (eg, selectively involving "MDI=1" devices). FFS transaction ID and AS ID reuse in polling-only scenarios.

Proposal 5: RAN2 to decide if the polling of outstanding data is done on the basis of single-device or multi-device communications.

Proposal 6: RAN2 to adopt a mechanism for release of AS IDs, involving

- all devices active under specific transaction ID, and/or

- devices reporting MDI=1 status.

FFS unicast / groupcast / broadcast options.

Proposal 7: There is a time limit (eg a specific number of contiguous access occasions) over which a device reports MDI=1 status expects to be polled for outstanding data. After this time limit elapses, the device will no longer guarantee the data availability (data release left to implementation and/or application layer).

Proposal 8: RAN2 to ensure that the polling of outstanding data is protected against replay / spoofing attack, including the option of sending an LS to SA3.