3GPP TSG-RAN WG2 Meeting #131	        											                         R2-2505568

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.9.4

Source:	MediaTek Inc.

Title:	Remaining open issues of PWS for NB-IoT

Document for:	Discussion and Decision

# Introduction

New objective was added to Rel-19 IoT NTN work item in RAN#105 [1]:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we discuss the remaining open issues in this objective.

# Discussion

In the previous RAN2 meeting, it has been agreed that we will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed.

It is clear that the SIB1-NB contains the scheduling information of system information block. UE cannot acquire SIB10/11/12 without this scheduling information.

The scheduling periodicity of SIB1-NB is 2560ms. The smallest NPDSCH repetition number in the 2560ms is 4, which means the UE can get a SIB1-NB within 640ms in worse case. The latency for receiving the PWS message is acceptable.

Therefore, we think SIB1-NB acquisition after receiving the PWS notification is needed.

Proposal 1: SIB1-NB acquisition is needed after receiving the PWS notification.

In the previous RAN2 meeting, the following agreement was made:

- Regarding support continued reception of PWS segmentation of a message from different cells, can come back in the next meeting with an actual TP. In any case we will not target other scenarios than the intra-gNB case

Certain companies are considering the possibility of introducing continued reception of PWS segmentation from different cells. However, we believe that switching cells during PWS message reception in NTN is rare, given the large cell coverage in NTN. Considering the limited time remaining for RAN2 discussions and the low expected benefit, we propose not supporting continued reception of PWS segmentation for PWS in Rel-19.

Proposal 2: Continued reception of PWS segmentation from different cells is NOT supported in Rel-19.

# Conclusion

Proposal 1: SIB1-NB acquisition is needed after receiving the PWS notification.

Proposal 2: Continued reception of PWS segmentation from different cells is NOT supported in Rel-19.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)