3GPP TSG-RAN WG2 Meeting #131	R2-2505616

Bangalore, India, August. 25th – 29rd, 2025

Agenda Item	: 8.13.2

Source	: LG Electronics Inc.

Title	: Discussion on discovery and relay reslection

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we propose the discovery response message sending condition at the intermediate Relay UE. Also, we investigate how to build up a tree-like topology and whether to include RRC state information in the discovery message.

2.	Discussion

2.1 Whether the intermediate Relay UE can send response message without forwarding solicitation message

According to the SA2 spec (TS 23.304), an intermediate Relay UE can send the response message without forwarding the received solicitation message if the intermediate Relay UE already has a PC5 link. The relevant SA2 spec is as follows:

6.3.2.5.3 Procedure for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B <omitted> 3a. If an indication that multi-hop relay is supported is contained in the received Solicitation message, the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1a, of a 5G ProSe Intermediate UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe Intermediate UE-to-Network Relay (if any), the 5G ProSe Intermediate UE-to-Network Relay may decide to send a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe Intermediate UE-to-Network Relay shall drop the received Solicitation message if the hop count (corresponding to the number of Relays included in the message) has reached the Hop-Limit of the received Solicitation message or the (pre)configured maximum number of hops associated with the RSC. The 5G ProSe Intermediate UE-to-Network Relay may send a Response message when it has already found or established PC5 link with 5G ProSe UE-to-Network Relay(s), without sending Solicitation message. i.e. steps 4a-7a are skipped and step 8a is performed directly. The response message additionally contains the User Info ID of UE-to-Network Relay, path information to the UE-to-Network Relay which is an (ordered) list of User Info ID of intermediate UE-to-Network Relay(s). If the same information on User Info IDs of Remote UE and UE-to-Network Relay is received from different ProSe UEs, the 5G ProSe Intermediate UE-to-Network Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g. hop count, delay, channel quality of received messages, etc.). <omitted>

RAN2 can follow the same procedure as the SA2 spec for the discovery response message delivery. So, RAN2 confirms whether the discovery response message can be generated at the intermediate Relay UE, not the last Relay UE, without forwarding the solicitation message toward the last Relay UE, if the intermediate Relay UE already has a PC5 link with its parent UE. When the intermediate Relay UE sends the response message without forwarding the solicitation message, the intermediate Relay UE has to follow the PC5 AS layer condition (e.g., PC5-RSRP) in the same way as the last Relay UE.

Proposal 1: RAN2 confirms the SA2’s understanding such as:

- For discovery model B, if an intermediate Relay UE already has an PC5 connection with a parent UE, the intermediate Relay UE can send a response message toward the Remote UE without forwarding the solicitation message to its parent UE.

Proposal 2: For the discovery model B, if the intermediate Relay UE can generate the response message without forwarding the received solicitation message, the intermediate Relay UE needs to check the PC5 AS condition (e.g., PC5-RSRP) before sending a discovery response message towards the Remote UE.

2.2 The way to support tree-like topology

According to the RAN2 agreement, RAN2 decided not to support the topologies in Figure 1. RAN2 agreed to support only the tree-like topologies. However, RAN2 has not discussed how to prevent building the topologies in Figure 1.

Figure 1. the topologies not to be supported in the L2 U2N multi-hop relay

In the discovery model A, the intermediate Relay UE forwards the received announcement message after PC5 connection establishment with the parent UE. Once the intermediate Relay UE makes a PC5 connection with one parent UE, the intermediate Relay UE should not forward other discovery announcement messages except the announcement message via the PC5 link with the current parent UE. Once the intermediate Relay UE makes a PC5 connection with one parent UE, the intermediate Relay UE should not make the other PC5 link with the other parent UE.

Proposal 3: To support tree-like topology, an intermediate Relay UE having the PC5 connection with a parent UE should forward only the received discovery announcement messages from the parent UE.

Proposal 4: To support tree-like topology, once the intermediate Relay UE establishes a PC5 connection with one parent UE, the intermediate Relay UE should not establish other PC5 connection when receiving the announcement message from the other relay UE.

In the discovery model B, the Remote UE decides the whole path. If the intermediate Relay UE doesn’t have a PC5 connection with a parent UE, the intermediate Relay UE can forward the received solicitation and response messages. However, if the intermediate Relay UE has a PC5 connection with a parent UE, the intermediate Relay UE can send the response message to the Remote UE without forwarding the solicitation message to the parent UE according to the session 6.3.2.5.3 in TS 23.304. The problem happens when two different Remote UE establishes the PC5 link with one intermediate Relay UE simultaneously, which intermediate Relay UE doesn’t have PC5 link with a parent UE and when each Remote UEs has a purpose to make connection with different last Relay UEs. In this case, the intermediate Relay UE should select one path to connect with the parent UE. Some mechanism may be required to inform the other Remote UE(s) that the desired path of the Remote UE cannot be supported, or just release the PC5 with the other Remote UE. This operation relies on the upper layer operation.

Proposal 5: To support tree-like topology, once the intermediate Relay UE has a PC5 connection with a parent UE, the intermediate Relay UE should not establish another PC5 connection with other parent UEs for the purpose of relay.

Proposal 6: In the discovery model B, if an intermediate Relay UE receives a PC5 connection request from two different Remote UE simultaneously, in which each Remote UE has the purpose to have connection with different last Relay UE, how the intermediate Relay UE selects the last Relay UE depends on the upper layer operation.

Proposal 7: Send LS to SA2 to inform proposal 3, proposal 4, proposal 5 and proposal 6.

2.3 Including RRC state in the Discovery message

For the fast connection setup, whether to include the RRC state in the Discovery message was discussed at the last meeting. If we decide to include the RRC state in the Discovery message, there are some issues. One of them is which discovery message include the RRC state, for example, announcement message, solicitation message, and/or response message. Another issue is whether all the RRC states of the intermediate Relay UEs and the last Relay UE along the path should be included in the discovery message. The third issue seems to be how to handle when the RRC state in the Discovery message is changed.

Before handling the above issues, we need to think whether the inclusion of the RRC state in the Discovery message will be an essential feature for the fast connection setup. If the RRC state is included in the Discovery message, the Remote UE has a chance to choose the path with as many intermediate Relay UEs in RRC_CONNECTED state as possible. However, if the discovery message includes only the RRC state of the first Relay UE, the Remote UE cannot know the other RRC states of the intermediate Relay UEs. In this case, the Remote UE cannot differentiate which path has many RRC_CONNECTED state intermediate Relay UEs. Also, the RRC state is not a deterministic value. The RRC state of the intermediate Relay UE and/or the last Relay UE can be changed. When the Remote UE selects the path, even though the first Relay UE is in RRC_CONNECTED, the RRC state of the first intermediate Relay UE can be changed when the Remote UE tries to establish a connection with the gNB via it. In this case, the RRC state information will not be helpful for the fast connection. Regardless of the RRC state of the first Relay UE, the Remote UE can perform a connection establishment with the gNB. Also, whether the Remote UE uses the RRC state information for relay selection is up to the Remote UE implementation. In these terms, we think including the RRC state of the intermediate Relay UE is too much of an enhancement.

Proposal 8: Including the RRC state in the discovery message looks like a useless enhancement for the following reasons:

- The RRC state of the intermediate Relay UEs can be changed at any time. In other words, it’s not a deterministic value.

- If the RRC state of only the first Relay UE is included in the discovery message, the Remote UE cannot differentiate which path has many RRC_CONNECTED intermediate Relay UEs.

- It is up to Remote UE’s implementation whether the RRC state information of the intermediate Relay UEs used for the relay (re)selection (i.e., it’s not an essential element for fast relay (re)selection).

- Regardless of the RRC state of the intermediate Relay UEs, the Remote UE can try connection establishment.

Even though above objective reasons, if we have to include the RRC state in the discovery message, we have to discuss which discovery message includes the RRC state. For example, the discovery announcement message and the discovery response message may include the RRC state of the first intermediate Relay UE. In the Discovery model B, the Remote UE selects the path based on the received response message. So, if the RRC states of the intermediate Relay UEs should be included in the discovery message, the RRC states may be included in the response message.

The other issue is whether to include all the intermediate Relay UE’s RRC states or include only the first Relay UE’s RRC state in the discovery message. If only the first Relay UE’s RRC state is included in the Discovery message, the intermediate Relay UE receives the discovery message from its parent Relay UE replaces the RRC state to the RRC state of itself. We prefer this way because it uses small data volume and if the first Relay UE is in RRC_CONNECTED state, it is clear that the RRC state of all the intermediate Relay UEs along the path toward gNB will be in RRC_CONNECTED.

Proposal 9: If there exist clear reasons that the RRC states of the intermediate Relay UEs should be included in the Discovery message, we support including the RRC state of the only first Relay UE in the announcement message in the Discovery model A and the response message in the Discovery model B.

Regardless of the RRC sate of the intermediate Relay UEs, it’s possible the Remote UE makes the connection establishment with the gNB. So, including the RRC state of the intermediate Relay UE in the discovery message can be optional feature. This may affect the UE capability.

Proposal 10: If there exist clear reasons that the RRC states of the intermediate Relay UEs should be included in the Discovery message, it will be an optional feature (it's not an essential one).

3.	Conclusion

Proposal 1: RAN2 confirms the SA2’s understanding such as:

- For the discovery model B, if an intermediate Relay UE already has an PC5 connection with a parent UE, the intermediate Relay UE can send a response message toward the Remote UE without forwarding the solicitation message to its parent UE.

Proposal 2: For the discovery model B, if the intermediate Relay UE can generate the response message without forwarding the received solicitation message, the intermediate Relay UE needs to check the PC5 AS condition (e.g., PC5-RSRP) before sending a discovery response message towards the Remote UE.

Proposal 3: To support tree-like topology, an intermediate Relay UE having the PC5 connection with a parent UE should forward only the received discovery announcement messages from the parent UE.

Proposal 4: To support tree-like topology, once the intermediate Relay UE establishes a PC5 connection with one parent UE, the intermediate Relay UE should not establish other PC5 connection when receiving the announcement message from the other relay UE.

Proposal 5: To support tree-like topology, once the intermediate Relay UE has a PC5 connection with a parent UE, the intermediate Relay UE should not establish another PC5 connection with other parent UEs for the purpose of relay.

Proposal 6: In the discovery model B, if an intermediate Relay UE receives a PC5 connection request from two different Remote UE simultaneously, in which each Remote UE has the purpose to have connection with different last Relay UE, how the intermediate Relay UE selects the last Relay UE depends on the upper layer operation.

Proposal 7: Send LS to SA2 to inform proposal 3, proposal 4, proposal 5 and proposal 6.

Proposal 8: Including the RRC state in the discovery message looks like a useless enhancement for the following reasons:

- The RRC state of the intermediate Relay UEs can be changed at any time. In other words, it’s not a deterministic value.

- If the RRC state of only the first Relay UE is included in the discovery message, the Remote UE cannot differentiate which path has many RRC_CONNECTED intermediate Relay UEs.

- It is up to Remote UE’s implementation whether the RRC state information of the intermediate Relay UEs used for the relay (re)selection (i.e., it’s not an essential element for fast relay (re)selection).

- Regardless of the RRC state of the intermediate Relay UEs, the Remote UE can try connection establishment.

Proposal 9: If there exist clear reasons that the RRC states of the intermediate Relay UEs should be included in the Discovery message, we support including the RRC state of the only first Relay UE in the announcement message in the Discovery model A and the response message in the Discovery model B.

Proposal 10: If there exist clear reasons that the RRC states of the intermediate Relay UEs should be included in the Discovery message, it will be an optional feature (it's not an essential one).

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNe..