3GPP TSG-RAN WG2 Meeting #131	R2-2505648

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	8.7.6

Source:	InterDigital

Title:	Discussion on UL congestion signaling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The rapporteur of email discussion “[POST130][505][XR] MAC running CR and open issues (Qualcomm)” has listed the following open issues for UL rate control:

[MAC-3] Design of identifier for QoS flows indicated in the UL Rate Control MAC CE. [MAC-4] Format of the UL Rate Control MAC CE. [MAC-5] Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

This TDoc discusses these open issues.

# 2	Discussion

## 2.1	[MAC-3] Identification of a QoS flow for rate control

From RAN2 point of view, the per QoS flow indication for XR rate control was agreed and it should be discussed how the QoS flow is identified in the Uplink Rate Control MAC CE. Since different PDU sessions can use the same QFI for different QoS flows, the QFI itself would not suffice as an identification of a QoS flow.

LCID is already known to the MAC entity and since the Uplink Rate Control MAC CE is rather a small MAC CE, it seems simplest to use the LCID + QFI combination to identify a specific QoS flow for UL rate control.

Proposal 1: Use the LCID + QFI (6 + 6 bits) combination to identify a specific QoS flow for UL rate control.

However, whether to allow full flexibility where the UL rate control MAC CE could provide throttling information over multiple DRBs and QoS flows requires discussion. One option is that one UL rate control MAC CE can provide rate control information for QoS flows within a DRB (ie., only one LCID indication would be required in this case). This seems to suffice in most of the practical scenarios where typically only default DRB is deployed for a PDU session. Another option could allow full flexibility where multiple pairs of LCID + QFI can be provided. In any case, it seems useless to provide same LCID more than once in the UL rate control MAC CE in either of the options, which would allow some overhead savings when many QoS flows need to be indicated at the same time.

Proposal 2: For a DRB, only one LCID indication within the UL rate control MAC CE is required which can follow number of QFIs for QoS flows for which the rate control information is provided for the given DRB.

## 2.2	[MAC-4] Format of the UL Rate Control MAC CE

As discussed in the previous paragraph, allowing the UL Rate Control MAC CE to provide throttling information over multiple DRBs should be discussed. Given that in practical scenarios, not many DRBs are employed in practice, the provided throttling information would usually be limited to QoS flows within a single DRB – this is the approach used also with the legacy Recommended bit rate MAC CE. If gNB wishes to provide information of QoS flows over multiple DRBs, multiple UL Rate Control MAC CEs could be provided (as in legacy).

Proposal 3: [MAC-4] The new UL Rate Control MAC CE can provide UL rate control information for multiple QoS flows within a single DRB.

Extension bit can be further employed to indicate if a further pair of QFI (QoS Flow ID) + Bit Rate information is provided in the MAC CE.

Proposal 4: [MAC-4] Use an extension bit to indicate a further pair of QFI+Bit Rate fields follows in the UL Rate Control MAC CE.

Based on the above 4 proposals, the new UL Rate Control MAC CE could look as follows:

Figure 2.2-1: Proposed UL Rate Control MAC CE format

Proposal 5: Agree on the MAC CE format as proposed in the Figure 2.2-1.

## 2.2	[MAC-5] Reporting of UL Rate Control MAC CE in UL

An open issue [MAC-5] was raised on when the UE could not fit the UL Rate Control MAC CE in the available UL-SCH resources, whether a truncated reporting format should be introduced. Since the UL rate control was in the beginning meant for NW to provide throttling information for UL traffic and since the UE query on preferred bit rate was agreed only after in RAN2, it seems unnecessary to think about optimizations for the UE query. We think the legacy principle where the UL Rate Control MAC CE is only reported when sufficient UL-SCH resources are available should be kept.

Proposal 6: UE reports the UL Rate Control MAC CE only when sufficient UL-SCH resources are available.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: Use the LCID + QFI (6 + 6 bits) combination to identify a specific QoS flow for UL rate control.

Proposal 2: For a DRB, only one LCID indication within the UL rate control MAC CE is required which can follow number of QFIs for QoS flows for which the rate control information is provided for the given DRB.

Proposal 3: [MAC-4] The new UL Rate Control MAC CE can provide UL rate control information for multiple QoS flows within a single DRB.

Proposal 4: [MAC-4] Use an extension bit to indicate a further pair of QFI+Bit Rate fields follows in the UL Rate Control MAC CE.

Proposal 5: Agree on the MAC CE format as proposed in the Figure 2.2-1.

Proposal 6: UE reports the UL Rate Control MAC CE only when sufficient UL-SCH resources are available.