3GPP TSG-RAN WG2 Meeting #131												R2-2505661

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.11.2

Source: 	Sony

Title:	Remaining issues for Random Access in SBFD Operation

Document for:	Discussion and Decision

# Introduction

RAN1 and RAN2 made significant work to progress the SBFD specification. The following agreements are related to the discussion of SBFD random-access procedure.

RAN1#116 Agreement:

- For RRC connected mode UEs, at least cell-specific configuration on time and frequency (working assumption) location of SBFD subbands is supported within a TDD carrier.​

RAN1#117 Agreement:

- For RRC connected mode UEs, SBFD subband time period:

  - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

  - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- For cell-specific indication of SBFD subband frequency location, adopt the following option:

  - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

RAN1#116bis Agreement:

- Confirm the working assumption:​

  - Working assumption​

    - For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.​

RAN1#117 Agreement:

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

RAN1#117 Agreement:

- Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

RAN1#118 Working Assumption:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

RAN2#127 Agreements:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

RAN2 to strive for a common SBFD CBRA framework independent of RRC state.

FFS whether/how early indication is used during a SBFD RA procedure.

RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

RAN2#127bis Agreements:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

The following two RACH configuration options are considered for SBFD based random access:

    - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

    - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration.

For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

The RACH configuration for SBFD is transmitted via SIB1.

FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

RAN2#128 Aagreements:

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

RAN2#129 Aagreements:

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell.

When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration.

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

FFS whether RO type selection is performed before or after the RA type selection.

FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

- RAN2#129 Aagreements:

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold.

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random-Access resources.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

In this contribution, we discuss some remaining issues for random access in SBFD operation and provide some proposals at the end.

## Random Access for SBFD Subband

## Open issue: RRC-1 RRC signalling design for the NT indicates RO types to the UE

The initial RO Type selection for CBRA:

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

- The network may indicate the prioritization of the additional ROs over legacy RO for load balancing. For example, in the early deployment of the SBFD feature where number of SBFD-aware UEs are small, the network can mandate all SBFD-aware UEs to select the additional ROs more often compared to legacy non-SBFD ROs, at least in the first attempt. It means that after certain (configured) number of times of RACH attempt in SBFD RACH occasions, the UE is allowed to switch to legacy RACH occasions. However, in the later stage when the deployment of the SBFD feature is popular (i.e., a significant number of UEs supports the feature), it is not desirable to assign all SBFD-aware UEs to the additional RO as it will be more overloaded compared to the legacy RO. In this case, the network should adjust the percentage/portion of SBFD-aware UEs using either the additional RO or legacy RO semi-statically.

- Proposal 1: For initial CBRA transmission, the network should indicate the percentage/portion of SBFD-aware UEs using either the additional RO or legacy RO semi-statically.

- In RAN2#129 meeting, it is agreed that for initial CBRA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE. Detailed signalling is FFS.

- Based on the above RAN2 agreement, 1-bit indication (ON: all SBFD UEs to use additional RO or OFF: all SBFD UEs to use legacy RO) is not good for load balancing, hence multilevel indication is needed. For example:

- In the early deployment of the SBFD feature where number of SBFD-aware UEs are small, the network can mandate for all SBFD-aware UEs to select the additional ROs, i.e., usage of additional RO 100% and legacy RO 0%.

- When the deployment of the SBFD feature is popular (i.e., a significant number of UEs supports the feature), the network can, for load balancing purpose, adjust the percentage of SBFD-aware UEs using either additional RO or legacy RO, i.e., usage of additional RO 50% and legacy RO 50%.

- The network can indicate unequal percentage of SBFD-aware UEs for additional RO and legacy RO, i.e., usage of additional RO 80% and legacy RO 20%.

- In some cases, the network can mandate all SBFD-aware UEs to select the legacy RO, i.e., usage of additional RO 0% and legacy RO 100%.

Based on the above deign principles, we think the indication should be semi-static signalling as dynamic change of this partitioning is not required in a short period. Hence, signalling should be cell specific with four different values, indicated by 2-bits representing four different values as captured above.

Proposal 2: Based on proposal 1, the network indicates the percentage of SBFD-aware UEs using additional RO where indication is 2-bits representing four different values: {100%, 80%, 50%, 0%}.

RNTI collision/confusion:

For SBFD with configuration 2, how to handle the case where the legacy ROs (e.g. in Flexible Symbols) overlap with additional ROs needs to be discussed in RAN1 or RAN2. As the possible impact relates to RNTI collision/confusion, it should be discussed RAN2.

The legacy RA-RNTI/MsgB-RNTI are calculated as follows (TS 38.321):

The RA-RNTI/ MSGB-RNTI associated with the PRACH occasion in which the Random-Access Preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211, f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

From our perspective, it is necessary to have a standardized solution as network implementations cannot solve the RNTI issue without additional complexity and restrictions on the configurable RACH resources. For example, allocating different preamble list to the legacy RACH configuration and additional RACH configuration would limit the available resources, hence impacts the RACH capacity, and negates the benefit of the SBFD feature of increasing the network RACH capacity. Note that the RACH duration in an occasion can be as small as 2 OFDM symbols, especially in FR2, making highly likely that the starting symbols of the legacy RACH configuration and additional RACH configuration to overlap in time.

A simple solution would make a significant difference in terms of flexibility of configuring the RACH resources, reducing the complexity, increasing RACH resources and minimising the latency due to RNTI collisions. For example, for potential solution, there are a lot of unused RNTIs in the current equation based on the s_id depending on the RACH duration on each occasion. If there is a shift of s_id into another unused symbol index (0 ≤ s_id < 14). This will increase the number of available RNTIs as well as avoiding RNTI collisions, without increasing RNTI space. The offset can be signalled in SIB1 as a part of PRACH resource configuration for SBFD-aware UEs. However, it seems most of the companies prefer to have a fixed value of s_id +1 so we are willing to accept it.

Proposal 3: To avoid RNTI collision/confusion, simply introduce adding fixed offset for the legacy symbol index (s_id) in the RNTI equation, that is s_id + 1 for SBFD-aware UEs.

## Conclusions

In this contribution, we have discussed some remaining issues for random access in SBFD operation and we have the following proposals.

- Proposal 1: For initial CBRA transmission, the network should indicate the percentage/portion of SBFD-aware UEs using either the additional RO or legacy RO semi-statically.

Proposal 2: Based on proposal 1, the network indicates the percentage of SBFD-aware UEs using additional RO where indication is 2-bits representing four different values: {100%, 80%, 50%, 0%}.

Proposal 3: To avoid RNTI collision/confusion, simply introduce adding fixed offset for the legacy symbol index (s_id) in the RNTI equation, that is s_id + 1 for SBFD-aware UEs.

## References

# [1] RP-232763, 3GPP TR 38.858 V2.0.0, Study on Evolution of NR Duplex Operation.

# [2] RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RAN#104.