3GPP TSG-RAN2 Meeting #131	R2-2505667

# Bengaluru, India, 25 - 29 August 2025

Agenda Item:	8.11.3

Source:	Ericsson

Title:	DC and CSI-RS measurements in SBFD

Document for:	Discussion

# 1	Introduction

In this paper, we discuss

-	Configuration of SBFD and DC

-	CSI-RS measurements for L3 mobility in SBFD.

In this paper, a SBFD aware UE means a UE which is capable of operating in a cell configured with SBFD feature, i.e., the cell/the gNB transmits DL and receive UL simultaneously in SBFD slots and symbols within a carrier. The UE can be aware of SBFD configurations so that the UE knows which slots/symbols are SBFD capable, which are also referred to as SBFD slots/symbols. This doesn’t mean that the UE needs to support full duplex operation.

# 2	Discussion

## 2.1	RAN1 LS reply on configuration of SBFD and DC

There is an incoming LS from RAN1 (RAN4 in cc) on configuration of SBFD and DC. RAN1 has not included this, but it would be a RAN2 decision whether to do it or not. The LS implies that there could be a need for a change to the UE specification if it is included:

RAN1 would like to thank RAN2 for the LS on simultaneous configuration of SBFD and DC in R2-2503191.

For the question on whether SBFD and DC can be configured simultaneously, and whether there is any issue with such configuration for an SBFD-aware UE, RAN1 made the following agreement.

Agreement

- From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier.

- Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly.

Even if RAN1 indicates that it is feasible for a UE to be configured with SBFD and NR-DC simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, there is RAN4 impact. However, the UE RAN4 spec is not in the SBFD WI scope. In addition, the August meeting is the last meeting for completing the WI. Introducing additional spec changes in RAN4 may delay completion of the WI. Therefore, it is more secure for RAN4 to make decision. From RAN2 perspective, RAN2 can confirm that this issue does not block the completion of the WI, since the spec changes in RAN2 is expected to be limited. Such spec changes in RAN2 can be triggered upon request from RAN4 if RAN4 concludes to address the issue in this release.

RAN2 to conclude that

  - Whether SBFD and DC can be supported is up to RAN4 decision

  - The issue doesn’t block the completion of the WI in RAN2

  - Further spec changes in RAN2 can be triggered based on RAN4 decision.

## 2.2	CSI-RS measurements for L3 mobility

In RRC, L3 mobility measurements related RRC IEs can be observed as the below

It can be observed that

- Measurement results for each cell comprises both cell results and beam results

- Cell results may be based on the best beam or averaged among best beams which are above the threshold

- Measurement results may be based on either SSB or CSI-RS.

If SSB is configured as the measurement RS, there is no impact on L3 mobility measurements due to SBFD operation.

If SSB is configured as the measurement RS, there is no impact on L3 mobility measurements due to SBFD operation.

Regarding CSI-RS based L3 mobility, two possible options for the way forward

Option 1 – the UE is provided with the SBFD configuration of neighbor cells. The UE may provide cell measurement results and beam measurement results for SBFD symbols and non SBFD symbols separately.

The potential spec impact for Option 1 would require cross-work group impacts (impacts to RAN1, RAN2 and RAN4). Serving cell/gNB needs to signal SBFD T/F configuration of each neighbor cell to UEs via dedicated RRC signaling. The UE measures each neighbor cell considering its SBFD T/F configuration.

Option 2 – the UE is not aware of the SBFD T/F configuration of neighbor cells.

The UE measures each neighbor cell without knowing SBFD T/F configuration of neighbor cells, i.e., as if the cell is not configured with SBFD.

In this case, the SBFD aware UE would behave as if the UE were a legacy UE. The UE provides L3 measurement report considering measurement samples in both SBFD symbols and non SBFD symbols.

Comparing the two options, we think option 2 is sufficient, given there is no TU left for other working groups to handle this issue if RAN2 would go for Option 1.

In addition, RAN2 has discussed this issue at RAN2#130, and made the below working assumption

- Working assumption: RAN2 confirms that the separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported, no RRC spec impact is expected.

The above working assumption is aligned with Option 2. We therefore suggest RAN2 to directly confirm the above working assumption and conclude the issue.

RAN2 confirms that the separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported, no RRC spec impact is expected.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	If SSB is configured as the measurement RS, there is no impact on L3 mobility measurements due to SBFD operation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to conclude that

a.	Whether SBFD and DC can be supported is up to RAN4 decision

b.	The issue doesn’t block the completion of the WI in RAN2

c.	Further spec changes in RAN2 can be triggered based on RAN4 decision.

Proposal 2	RAN2 confirms that the separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported, no RRC spec impact is expected.

# 4	References

- [1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Shanghai, China, June 17-20, 2024, RAN#104.

[2] R1-2501560, Reply LS on CSI-RS measurement with SBFD operation, MediaTek, Athens, Greece, February 17th – 21st, 2025, RAN1#120.