3GPP TSG-RAN WG2 Meeting #131	R2-2505689

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:		8.8.4

Source:		Lenovo

Title:		Some remaining issues for MBS and ETWS broadcast in NR NTN

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding broadcast services in NTN:

RAN2#129bis Agreements: - RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts) - If UE knows it’s not in any intended service areas of any MBS services the UE is interested in, the UE may not need to acquire MCCH - If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies. - UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. FFS whether and how the association between intended service area and frequencies is acquired. - The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE. RAN2#130 Agreements - Do not introduce service area information specific for FSAI in SIB21. - Do not introduce service area per neighbour cell in MCCH - RAN2 assumes that the ISA can be valid for all the frequencies providing the MBS session - The new SIBxx containing MBS ISA(s) can be cell or area specific, which is up to NW implementation (No specs change).

In this contribution we discuss some remaining issues.

- Discussion

As per RAN2 discussions throughout the latest meetings, some enhancements for MBS and ETWS broadcast services in NR NTN were agreed in principle while other details are still FFS. As the Rel-19 function freezing is approaching we list some remaining issues to be resolved as well as other enhancements in potential.

## Timing of start MCCH monitoring

In legacy MBS, UE monitors MCCH to receive the MBS configuration for broadcast session. And when an MCCH change notification is received, UE monitors MCCH to update the MBS configuration for broadcast session. MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary; A notification mechanism is used to announce the change of MCCH contents due to broadcast session start, modification or stop and due to neighboring cell information modification. It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification.

In the previous RAN2 meetings it has been agreed to introduce Intended Service Area (ISA) for MBS broadcast service in NTN, and if UE knows it’s not in any intended service areas of any MBS services the UE is interested in, the UE may not need to acquire MCCH. Based on this understanding, the exact timing of stop MCCH monitoring can be UE implementation based on the ISA information. Meanwhile the timing of start MCCH monitoring may have some further details irrelevant to the ISA.

In NTN UE needs to ensure a valid UE location information and a valid ephemeris data (in SIB19) for uplink synchronization. UE performing re-acquiring SIB19 in NTN may possibly miss the change notification. In such case, enhancements can be considered to accelerate UE in awareness of MCCH change and re-acquire MCCH configuration. To update MCCH configuration when UE possibly misses a MCCH content change notification in NTN, the following options can be considered:

- Automatic update: In an NTN cell UE re-acquires MCCH configuration in SIB20 after re-acquiring SIB19 complete, e.g., if the corresponding process covers an MCCH modification period boundary.

- Conditional update: In an NTN cell UE re-acquires MCCH configuration in SIB20 when an MCCH change notification or MCCH configuration is received in the re-acquired SIB19 (while the MCCH change notification in SIB19 does not trigger SIB19 update).

Proposal 1: RAN2 to consider enhancement for updating MCCH configuration when UE may possibly miss a MCCH content change notification due to SIB19 acquiring, including:

- Option 1: UE re-acquires MCCH configuration in SIB20 after re-acquiring SIB19 complete.

- Option 2: MCCH change notification or MCCH configuration can be received in re-acquired SIB19.

MBS broadcast service continuity for RRC_CONNECTED

The service continuity guaranteeing for RRC_IDLE/INACTIVE has been discussed and agreed in previous RAN2 meetings, while the service continuity for RRC_CONNECTED remains untouched.

For CONNECTED MBS broadcast service continuity in TN, the UE in RRC_CONNECTED state may send MBS Interest Indication to the gNB, including the list of MBS frequencies and interested services, as well as frequency priorities. The gNB may use this indication when providing handover configuration to allow the UE to receive the MBS services the UE is interested in. Due to the same assumption that an MBS broadcast service area is mapped to one or more TN cells, UE does not need to provide its current or expected MBS broadcast service area information in the MBS Interest Indication.

When considering that an NTN cell coverage could be larger than an MBS broadcast service area, UE’s current/expected MBS broadcast service area information is necessary at gNB for further handover decisions e.g., to judge whether UE is staying/moving into or moving out of an MBS broadcast service area and configure an appropriate target cell.

Proposal 2: UE may report its current broadcast service area information or area ID to gNB for service continuity in RRC_CONNECTED.

Applicability of warningAreaCoordinates for ETWS broadcast in NTN

In previous releases before Rel-19 the PWS mechanisms are designed for TN, and UE can simply judge whether it is in an emergency broadcast service area by its serving cell and the corresponding PWS broadcast configuration it receives. This mechanism is workable based on the assumption that a PWS broadcast service area is larger than a TN cell coverage so that the service area can be mapped to one or more TN cells.

However, the diameter of an NTN cell could be much larger than an emergency broadcast service area. An NTN cell coverage may also across country border on one side of which an emergency broadcast service is not allowed or authorized. It is observed that, for ETWS there is no IE to describe a geographical area in the contents of legacy NR SIB6 and SIB7, while for CMAS as an optional IE warningAreaCoordinatesSegment was introduced in SIB8 to describe the alert area coordinates of a CMAS message.

Regarding the area indication for ETWS in NR NTN, there were discussions in RAN2#129, a straight-forward way is to reuse the IE warningAreaCoordinatesSegment in SIB8 for CMAS, with its formats following ATIS-0700041:

ATIS-0700041 5.2 Warning Area Coordinates Information Element This information element conveys the description of Warning Area Geometries specified by the Alert Originator to the mobile device to perform geo-fencing on the alert. These elements are coded as a finite bit string. In the figures, the bit string is described by octets from top downward, and in the octet from left to right. Number encoding strings start with the most significant bit. The total number of all circles and polygons is limited to 10. The total number of coordinates of all circles and polygons is limited to 100. 5.2.1 WAC Tag-Length-Value (TLV) At the most basic level the WAC IE is made up of one or more TLVs indicating specific parameters and the values of the parameters. Each WAC TLV is illustrated in Table 7. Tag (4 bits): Indicates the type of parameter Length (10 bits): Indicates the length of the entire WAC TLV in octets Reserved (2 bits): Shall be set to 0 by the network and ignored by the device Value (variable number of octets, up to 1022): The value of the parameter 5.2.2 WAC TLV characteristics The following attributes apply to the WAC TLV in the WAC IE: • WAC may contain multiple TLVs • There may be more than one WAC TLV with the same tag unless otherwise specified • There is no ordering significance to WAC TLVs in a WAC IE • If a WAC tag is not recognized by a device the WAC TLV shall be ignored • WAC TLVs shall be padded with binary 0s to octet boundaries …

By reviewing the ATIS-0700041, the following observations are given:

Observation 1: The IE warningAreaCoordinatesSegment for CMAS can describe circles and polygons.

Observation 2: The IE warningAreaCoordinatesSegment has a total number limit 10 for all circles and polygons, and a total number limit 100 for coordinates of all circles and polygons.

Observation 3: When describing circles, in the IE warningAreaCoordinatesSegment the granularity of radius is 1km and the value range is 64km.

Meanwhile for comparation, the granularity of radius introduced in NR NTN is 10km and the value range is 2560km.

Proposal 3: For ETWS broadcast service area indication, RAN2 can discuss whether the format of warningAreaCoordinatesSegment for CMAS is sufficient for NTN deployment.

- Conclusion

In this contribution we discuss some remaining issues of MBS broadcast service in NTN. It is proposed that:

Proposal 1: RAN2 to consider enhancement for updating MCCH configuration when UE may possibly miss a MCCH content change notification due to SIB19 acquiring, including:

- Option 1: UE re-acquires MCCH configuration in SIB20 after re-acquiring SIB19 complete.

- Option 2: MCCH change notification or MCCH configuration can be received in re-acquired SIB19.

Proposal 2: UE may report its current broadcast service area information or area ID to gNB for service continuity in RRC_CONNECTED.

Proposal 3: For ETWS broadcast service area indication, RAN2 can discuss whether the format of warningAreaCoordinatesSegment for CMAS is sufficient for NTN deployment.