3GPP TSG-RAN WG2 Meeting #R2-131		        R2-2505731      Bangalore, India, Aug 25-29,2025

Agenda Item:	8.6.3

Source: 	Rakuten

Title:  	Remaining issues of L1 event triggered measurement reporting

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

The WID objective includes Event based L1 measurements for LTM.

In this contribution, we discuss the feasibility of configuring Event based L1 measurements and using them for LTM candidate cell addition/release. Further, we discuss redundancy of L3 and L1 measurements for LTM.

# Discussion

## Open issue from RAN2#126bis

## Scheduling of L1 measurement report MAC CE

The below agreement was made in RAN2 #126bis.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

While it was agreed that legacy SR procedure for resource allocation is the baseline, requesting resources using BSR to send event-triggered L1 measurements MAC CE was not agreed. We believe there is a need to include some information of MR MAC CE size, otherwise if MR MAC CE size is variable and large, there is no way for the network to assign a grant to guarantee max size.

Requesting resource allocation using the legacy BSR procedure to send the event-triggered L1 measurements MAC CE is supported.

## Open issue from RAN2#130

## Measurement of activated semi-persistent resources

UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. Postpone target cell case to the next meeting.

It was decided to postpone the issue of how to handle the already activated semi-persistent resources at the target cell, after LTM cell switch.

We think that the target cell case should follow the rest of the candidate cells and the UE should deactivate the SP CSI-RS resources of the target cell.

UE stops to measure the activated semi-persistent CSI-RS resources of the target cell, after LTM cell switch.

## Coexistence of LTM event triggered reporting and mTRP operation at serving cells

Proposal 6a: Support the coexistence of LTM event triggered reporting and mTRP operation at serving cells. Proposal 6b: When LTM event-triggered reporting and mTRP operation coexist in the serving cells, the network configures which of the indicated TCI states of the serving cell is to be used for LTM event-triggered measurements evaluation. Will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

We support configuring both mTRP and event triggered measurement reporting simultaneously. And when they are configured simultaneously, UE evaluates the event for both the beams i.e associated with the respective TRPs and report the L1 event if either of the current beams satisfy the event. This is useful since the “best beam” can change dynamically, after the evaluation has started.

When LTM event-triggered reporting and mTRP operation coexists in the serving cell, beams associated with both the TRPs are considered current beams.

When LTM event-triggered reporting and mTRP operation coexists in the serving cell, UE evaluates the configured event for both the current beams and report the event if either of the current beams satisfy the event.

When beams of both TRPs are used for evaluation of the measurement event, the UE needs to let the network know which beam has been reported. Hence when mTRP is used, the UE shall indicate in MR MAC CE, which TCI state is associated with the reported RSRP value for the serving/current beam.

When LTM event-triggered reporting and mTRP operation coexist in the serving cell, the UE shall indicate in MR MAC CE, which TCI state is associated with the reported RSRP value for the serving/current beam.

# Conclusion

We have the following proposals:

Proposal 1	Requesting resource allocation using the legacy BSR procedure to send the event-triggered L1 measurements MAC CE is supported.

Proposal 2	UE stops to measure the activated semi-persistent CSI-RS resources of the target cell, after LTM cell switch.

Proposal 3	When LTM event-triggered reporting and mTRP operation coexists in the serving cell, beams associated with both the TRPs are considered current beams.

Proposal 3	When LTM event-triggered reporting and mTRP operation coexists in the serving cell, beams associated with both the TRPs are considered current beams.

Proposal 5	When LTM event-triggered reporting and mTRP operation coexist in the serving cell, the UE shall indicate in MR MAC CE, which TCI state is associated with the reported RSRP value for the serving/current beam

# References

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

RAN2 Chairman Notes R2_127bis_ChairNotes

RAN2 Chairman Notes R2_128_ChairNotes