3GPP TSG-RAN WG2 Meeting #131	R2- 2505756

Bengaluru, India, 25-29 August, 2025

Agenda item:	8.7.4

Source: 	Huawei, HiSilicon

Title: 	Remaining issues on LCP and DSR enhancements

Document for:	Discussion and Decision

# 1. Introduction

For scheduling enhancements, RAN2 has made a lot of progress but some remaining open issues remain, e.g. on UE capabilities related to DSR, and the possible TP for DSR cancellation in DC. In this contribution, we further discuss these remaining open issues of scheduling enhancement as provided in the UE capability and MAC.

# 2.	Discussion

## 2.1	UE capability for DSR enhancement (UE-cap 1)

In the open issue list of the running CR of UE capability [1], there is one open issue related to DSR enhancement:

Open issue 1: the pre-requisite of enhanced delay status report This was discussed in RAN2#129bis meeting (as shown below) and an FFS was captured: “FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.” - LGE, QCM thinks we do not need to couple R18 and R19 DSR. Vivo thinks reasonable UE will implement R18 if implements R19, but does not want to couple in specs. - Huawei does not think it is possible to support R19 without R18

In our view, there is no additional complexity for UE supporting Rel-19 DSR enhancement to also support Rel-18 DSR, considering that the mechanism and procedure of Rel-19 DSR enhancement is based on Rel-18 DSR, e.g., DSR triggering and procedure are the same for Rel-18 and Rel-19. The only additions are on the reporting of multiple buffer status for different reporting thresholds and the possibility of taking into account of non-delay reporting data ahead of delay reporting data. Furthermore, it provides more flexibility from the network point of view to configure either Rel-19 DSR enhancement with triggering threshold separated from the reporting threshold(s), or configure R18 DSR only which combines both triggering and reporting threshold, to reduce RRC signalling overhead in case multiple thresholds are not needed for a certain application.

Proposal 1:	(UE-Cap-1) UE supporting enhancedDelayStatusReport-r19 supports delayStatusReport-r18 as pre-requisite.

## 2.2	DSR cancellation in Dual Connectivity [MAC-6]

The DSR cancellation in Dual Connectivity scenario related issues were discussed in last RAN2#130 meeting, and RAN2 agreed the following:

- (MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR. - (MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU. - (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

Based on the above agreement 2 and agreement 3, it can be seen that the common understanding of DSR cancellation in DC case is: if there is transmission of MAC PDU in one MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR. This means that if one MAC entity sends the delay-critical data, then the other MAC entity can cancel the pending DSR and doesn't need to request UL grant for the delay-critical data associated with the pending DSR.

However, considering that the 38.321 specification is normally written and specifies the behaviour from a MAC entity perspective, not from a UE perspective. Therefore, one MAC entity cannot cancel the pending DSR according to an event happening in another MAC entity, meaning that the pending DSR cannot be cancelled if a MAC PDU including delay-critical data is sent in another MAC entity. Therefore, the DSR cancellation in DC scenario should be specified clearly.

In addition, during MAC running CR review [2], companies don't support cross CG coordination for DSR cancellation in DC scenario, which may depend on indication from other MAC entity as below:

From our understanding, to avoid the inter-MAC interaction, a condition “if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted” can be introduced for the MAC entity to cancel the pending DSR. With such condition, it can be clearly specified that if a MAC entity finds that there are no more delay-critical data associated with a pending DSR, and even though the MAC entity didn't perform any delay-critical data transmission, which means that the other MAC entity may already send the delay-critical data, then the MAC entity can cancel the pending DSR. This condition clearly specified the previous RAN2 agreement “(MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.”. Without adding such text in the procedure text and note, the current MAC spec may not be completely clear.

To make this clear in the spec, the corresponding TP shown below can be added to the procedure text or as a note:

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR, or if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

Proposal 2:	[MAC-6] In Dual Connectivity case, for completeness in the spec and aligning with the understanding from the RAN2 agreement ‘MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR’, add in procedure text or as a note that ‘a MAC entity can cancel the pending DSR if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted’.

See Appendix for a possible change to the procedure text.

## 2.3	Rel-19 DSR on BSR cancellation [MAC-2]

The newly introduced DSR MAC CE is capable of reporting delay information with finer granularity for an LCG, where the delay reporting data with remaining time above the triggering threshold can be included in the DSR MAC CE. However, this may lead to redundant report between BSR and DSR if there is only delay-reporting data in all LCHs of the UE. In a simple example, when there is new data arrived at the LCH configured with reporting threshold and all other LCHs are empty and the remaining time of the new data are within the reporting threshold, a BSR and Rel-19 DSR will be triggered. Assuming that there is an available UL resource, both the BSR and DSR MAC CE will be sent in the same MAC PDU, which contain the same data volume for the LCH. Since DSR contains not just the data volume but also the smallest remaining time, the triggered BSR should be cancelled as the DSR provides more information.

Proposal 3:	[MAC-2] RAN2 to specify that all triggered BSRs can be cancelled if amount of data to be reported in the BSR is already indicated by the DSR included in the same MAC PDU, as given in Appendix.

The corresponding TP for the above Proposal 3 is shown in Appendix.

## 2.4	DSR cancellation [MAC-1]

In the open issue list of the MAC running CR [3], there is one further open issue related to DSR cancellation:

[MAC-1] Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

In the above open issue, there are two questions to clarify: 1. whether a pending DSR should be cancelled if the UE has delay reporting SDUs but no delay critical SDUs; 2. whether a pending DSR should be cancelled if the UE has delay reporting SDUs but all delay critical SDUs have been reported.

For question 1, from our understanding, DSR MAC CE is reported when DSR is triggered, i.e., when there is delay-critical data. This means that information of delay-reporting data is reported together with information of delay-critical data only when there is delay-critical data. Therefore, there is no such case where UE has delay reporting SDUs but no delay critical SDUs.

For question 2, the current DSR trigger condition is that “1>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG;” Therefore, even if the information of a delay-reporting data is not reported, such delay-reporting data can still trigger DSR when it becomes delay-critical data, so there is no impact on DSR cancellation. Furthermore, it is our understanding that this is a conscious decision to leave the triggering and the cancellation unchanged with the introduction of the Rel-19 DSR as agreed in the RAN2#128:

The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR.

Proposal 4:	[MAC-1] No change in the cancellation condition of pending DSR (i.e., DSR cancellation is only related to delay-critical data as in Rel-18), therefore, a pending DSR should be cancelled if all delay critical SDUs have been transmitted or all delay critical SDUs have been reported (regardless of whether all the delay reporting SDUs are reported or transmitted).

# 3. Conclusion

In this contribution, we analyzed the potential aspects for DSR enhancements. The following observations and proposals were made:

UE capability for DSR enhancement [Open Issue 1 in UE cap running CR]

Proposal 1:	(UE-Cap-1) UE supporting enhancedDelayStatusReport-r19 supports delayStatusReport-r18 as pre-requisite.

DSR cancellation in Dual Connectivity [MAC-6]

Proposal 2:	[MAC-6] In Dual Connectivity case, for completeness in the spec and aligning with the understanding from the RAN2 agreement ‘MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR’, add in procedure text or as a note that ‘a MAC entity can cancel the pending DSR if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted’.

Rel-19 DSR on BSR cancellation [MAC-2]

Proposal 3:	[MAC-2] RAN2 to specify that all triggered BSRs can be cancelled if amount of data to be reported in the BSR is already indicated by the DSR included in the same MAC PDU, as given in Appendix.

DSR cancellation [MAC-1]

Proposal 4:	[MAC-1] No change in the cancellation condition of pending DSR (i.e., DSR cancellation is only related to delay-critical data as in Rel-18), therefore, a pending DSR should be cancelled if all delay critical SDUs have been transmitted or all delay critical SDUs have been reported (regardless of whether all the delay reporting SDUs are reported or transmitted).

# 4. Reference

[1]. R2-250xxxx Open issues of Rel-19 XR UE capabilities, Xiaomi, RAN2#131 Bangalore, India, 25 – 29 Augugst 2025

[2]. R2-250xxxx R19 XR MAC running CR, Qualcomm Incorporated, RAN2#131 Bangalore, India, 25 – 29 Augugst 2025

[3]. R2-250xxxx List of open issues in MAC Qualcomm Incorporated, RAN2#131 Bangalore, India, 25 – 29 Augugst 2025

# 5.	Appendix

### 5.4.9	Delay status reporting

…… Omitted……

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR, or if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

### 5.4.5	Buffer Status Reporting

…… Omitted……

All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long, Refined Long, Extended Long, Short, or Extended Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly. All triggered BSRs may be cancelled if amount of data to be reported in the BSR is indicated by the DSR included in a MAC PDU.

---------------------------------------------------------------Next Change-------------------------------------------------------------------