3GPP RAN WG2 Meeting #130	R2- 2505787

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.6.3

Source:	Ofinno

Title:	Remaining issues for L1 event triggered measurement report

Document for:	Discussion, Decision

# Introduction

In the previous RAN2 meetings, the following agreements were made:

Serving beam in truncated MR MAC CE: - For the truncated MR MAC CE, the UE determines the beam to be included in the MAC CE, based on the priority: Type 1 beam > Type 2 beam > Type 3 beam > Type 4 beam. If the UE cannot include all of the beams with the same priority, the beams are selected in the (truncated) MR MAC CE based on measured RS quality. Among all selected beams, the UE puts the best quality beam as the reference beam as the first one included in the (truncated) MR MAC CE. - Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE. SR for MR MAC CE: - NW can configure a dedicated SR configuration for MR MAC CE transmission - When MR is triggered, if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, UE will trigger RACH procedure, as in legacy.

In this contribution, we discuss open issues on L1 event triggered measurement report.

# Identified open issues:

## Serving beam information

RAN2 agreed in #129bis that when UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE. It is possible that the UL grant can accommodate more beams after accommodating the triggered beams. We think sending a truncated L1 MR serves 2 distinct purposes, one being to indicate to the network that there are more triggered beams available and is pending to be transmitted, and the second being to allow the network to make a quick LTM cell switch/ early UE synchronization decision without having to wait for the complete MR i.e., the measurement for the remainder of the N beams.

There are some cases where the network can afford to wait until the complete MR is received from UE. For example, if the serving beam signal is good to maintain the radio link long enough for UE to transmit the complete MR and later receive the LTM cell switch command MAC CE or to receive PDCCH order triggering early UL sync. However, when the serving beam is weak, the network cannot afford to wait till it receives the complete MR. There is no consensus in RAN2 on how to include the serving beam information in truncated MR MAC CE. At RAN2#130, RAN2 made a working assumption that If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. Further the working assumption adds a restriction to the size of minimum grant.

Many companies think that the working assumption provides is aligned with normal MAC CE and is simple. It can be noted that a candidate beam information in a (truncated) MR MAC CE requires 2 octets while a serving/ current beam information requires 1 octet. However, considering the working assumption where the UE always includes a candidate beam information and a serving beam information, the size of a truncated MR MAC CE is always an even number of octets, if the network configures the UE to report the serving/ current beam information e.g., first octet containing report config ID, second and third octet for candidate beam information and fourth octet for serving/ current beam information.

Observation 1: a candidate beam information in a (truncated) MR MAC CE requires 2 octets while a serving/ current beam information requires 1 octet.

UE sends a truncated MR MAC CE if an available UL resource is not sufficient to accommodate a normal MR MAC CE. The size of UL resource available for transmitting truncated MR MAC CE may either be odd number of octets of even number of octets. If the UL resource available for transmitting the truncated MR MAC CE is an even number of octets, then the working assumption efficiently utilizes the UL resources. However, if the UL resources available for transmitting the truncated MR MAC CE is an odd number of octets, then the working assumption would result in inefficient utilization of UL resources/ wastage of UL resources as at least 1 octet in the UL resource is not used by the truncated MR MAC CE.

Observation 2: If the odd number of octets are available for a UL resource to transmit a truncated L1 MR MAC CE, implementing the working assumption leads to wastage of UL resources.

The resource utilization can be optimized if the UE can intelligently select the beam information to be added in the truncated MR MAC CE based on size of UL resource available. For example, after including the information of a beam that triggered the MR, if the remaining UL resource is sufficient to accommodate a candidate beam information (i.e., 2 bytes available after adding triggered beam information), the UE includes the candidate beam information. Alternatively, if the UL resource is sufficient to accommodate only the serving/ current beam information (i.e., one byte available after adding triggered beam information), the UE includes the current beam information. Otherwise, if the UL resource is not sufficient to accommodate the serving/ current beam information, the truncated L1 MR MAC CE only includes the triggered beam information.

Proposal 1: After including triggered beam information in a truncated L1 MR MAC CE,

- if the remaining available UL resource can accommodate a candidate beam information, include a candidate beam information in the truncated L1 MR MAC CE;

- else if the remining available UL resource can accommodate only the current beam information, include the current beam information in the truncated L1 MR MAC CE;

- else the truncated MR MAC CE only includes the triggered beam information.

The MAC PDU format in NR is optimized for parallel processing where the receiver (e.g., MAC entity at the network) can demultiplex multiple MAC sub-PDUs and process them in parallel. To achieve this, the receiver needs to know the exact size of the MAC sub-PDU as quickly as possible. It is not mandatory for UE to transmit the serving beam information in truncated L1 MR MAC CE. When transmitting a truncated L1 MR MAC CE, the network cannot efficiently identify whether the truncated L1 MR MAC CE consists of the serving beam or not. If the beam type indication is used and the last octet is allocated for serving beam information, the network can identify whether the truncated L1 MR MAC CE consists of the serving beam or not only while decoding the last octet of the MAC CE. Thus, it is beneficial for network to identify, as quickly as possible, whether the truncated L1 MR MAC CE includes the serving beam information or not.

Proposal 2: In truncated L1 MR MAC CE, a field indicates whether serving beam information is included or not.

## Truncated L1 MR MAC CE

According to previous RAN2 agreements and the current TS 38.321 running CR, if UL resources are sufficient to accommodate a MR MAC CE and a subheader of the MR MAC CE, the UE sends a MR MAC CE to the network. If the UL resources are not sufficient to accommodate the MR MAC CE and the subheader of the MR MAC CE, but is sufficient to accommodate a truncated MR MACE CE and a subheader of the truncated MR MAC CE, the UE sends a truncated MR MAC CE to the network. If the UL resources are not sufficient to accommodate even the truncated MR MAC CE and the subheader of the truncated MR MAC CE, then the UE triggers a SR. The UE transmits the SR using dedicated SR configured by the network. If there is not dedicated SR, configured by the network, for MR MAC CE, then the UE initiates a RACH procedure.

Based on the size of the UL resources, the UE may transmit a truncated MR MAC CE. The truncated MR MAC CE serves as an indication to the network, that there are more beams for which the measurement information is not included in the truncated MR MAC CE. The network may give more UL resources to the UE for transmitting the measurement information of the remaining beams. However, it is possible that the UE does not receive a UL grant after sending the truncated MR MAC CE. The current running CR does not handle this scenario. The UE simply waits till either more UL grants are received, or some other procedure triggers SR (e.g., BSR triggers an SR).

Observation 3: If not UL grant is received after transmitting a truncated MR MAC CE, there is not procedure to request for more UL grants.

We think that RAN2 needs to introduce a procedure when the UE can request for UL resources if the UE does not receive any UL grants following sending a truncated L1 MR MAC CE. In one solution, the UE may trigger SR or RACH if the UE does not receive UL grants within certain time after sending the truncated MR MAC CE.

Proposal 3: For truncated MR MAC CE, introduce a timer with the following behavior:

- Start the timer on transmitting the truncated MR MAC CE

- Stop the timer on receiving UL grant for subsequent transmission of MR MAC CE

- On expiry of the timer, trigger SR. If SR is not configured, initiate RACH.

According to RAN2 agreements and TS 38.321 running CR, a UE may subsequently send a normal MR MAC CE after initially sending a truncated MR MAC CE. The normal MR MAC CE may comprise the remaining beams that are not included in the truncated MR MAC CE. It is possible that the normal MR MAC CE, in this case, includes only information about type 4 beams i.e., the candidate beams that has neither fulfilled entering condition, nor fulfilled leaving condition of an LTM event. Type 3 beams were introduced to allow the network to have information of other beams that have satisfied an LTM event in the past but is not associated to the bam that triggered the current MR MAC CE. The benefit of this is to ease the network implementation from remembering the complete history of the beams reporting in MR MAC CE.

Receiving a MR MAC CE with only type 4 beams may cause some ambiguity to the network. It is possible that the network may not comprehend what this MR MAC CE is for. The network may not identify that the MR MAC CE is associated with a previous truncated MR MAC CE.

In another example, the triggered beam may not be transmitted in a truncated MR MAC CE and before receiving the UL grant for transmitting the subsequent MR MAC CE, according to the current running CR, the UE may cancel the triggered MR. In this case it is possible that when the network receives the next MR MAC CE for the same report configuration, the network wrongly assumes that the MR MAC CE is associated with the previous MR MAC CE and not a new MR MAC CE.

To avoid these problems, we think the network should be informed whether a MR MAC CE is a new MR MAC CE with new beam information, or a subsequent MR MAC CE with remaining beam information that is not included in an initial truncated MR MAC CE.

Proposal 4: In an MR MAC CE transmitted after transmitting a truncated MR MAC CE, indicate whether the MR MAC CE is a new MR MAC CE or continuation of the truncated MR MAC CE or not.

## Event-triggered MR handling during beam failure

Figure 1 shows an existing beam failure recovery procedure. When a UE detects a beam failure on a cell (e.g., PCell, SCell), the indicated TCI state (e.g., TCI state 1) of the cell which represents the serving beam of the cell is likely to have a low radio link quality. This can result in frequent triggering of event-triggered MR. The ongoing beam failure recovery procedure is already trying to recover (or indicate) the potential blockage, and there is no need for event-triggered measurement reporting during the ongoing beam failure recovery. Especially, in the case of no carrier aggregation (e.g., only PCell is configured), when there is a beam failure on the PCell, the uplink channels are also likely to fail (the reason for resetting the uplink channels after beam failure recovery), so the MR MAC CE transmissions, for the trigger event-triggered MR, are likely to fail. Therefore, in our view, upon beam failure detection, the UE should cancel, if any, triggered L1 MR and reset TTT.

Proposal 5: On detecting beam failure on SpCell, cancel the triggered L1 MR and reset TTT.

Figure 1

According to current 38.213, 28 symbols after a beam failure recovery is completed, the UE applies the candidate reference signal of the serving cell identified for the beam failure recovery to PDCCH/PDSCH/A-CSI-RS receptions until application of a new indicated TCI state (e.g., between t2 and t5 in Figure 1). However, according to current TS 38.321 running CR, the UE considers the beam that corresponds to the RS configured in the indicated TCI State or the RS QCLed with the RS configured in the indicated TCI State in the serving cell, as the current beam. Therefore, since the indicated TCI state for the cell is still TCI state 1 between t2 and t5 though it is not used, the conditions of event-triggered L1 measurement reporting may still be satisfied frequently even after successful completion of a beam failure recovery, and thus the UE triggers unnecessary event-triggered MR and unnecessary MR MAC CE reporting.

In our view, these unnecessary triggering and reporting of MR MAC CE should be avoided. One solution can be to restrict evaluation of the conditions for event-triggered measurement reporting upon beam failure detection or upon beam failure recovery completion and until a new TCI state is indicated (e.g., no trigger-event condition evaluation between t1/t2 and t5). Alternatively, another solution can be to evaluate the conditions event-triggered measurement reporting, after the beam failure recovery completion and till receiving new indicated TCI state, based on the candidate reference signal identified for the beam failure recovery instead of the indicated TCI state prior to the beam failure detection/recovery.

Proposal 6: Upon detecting beam failure on SpCell, the UE stops performing L1 measurement evaluation.

Proposal 7: After beam failure detection on the SpCell, the UE starts performing L1 measurement evaluation after:

Alt1: a new TCI state is indicated for the SpCell.

Alt2: completion of beam failure recovery and L1 measurement evaluation is performed based on the candidate RS of the SpCell identified for the beam failure recovery.

## Serving beam selection during mTRP

In an mTRP scenario, there may be two indicated TCI states for the serving cell (e.g., one per TRP). In the email discussion #117 post RAN2#130 copied above, companies expressed their views related to which one of the two TCI states is used for evaluating event triggered MR. There were several options discussed. Most companies prefer option a) where UE choses the best beam of the two serving/current beams for event evaluation. Supporting this option may require changes to the MR MAC CE format. Further, agreeing to option a) may cause delayed reporting because the “best beam” can change dynamically. It also creates ambiguity in inter-cell mTRP scenarios, where the best beam may belong to a non-serving cell. Option a) may increase UE complexity, since the UE would need to determine, for each MR MAC CE, previous best beam. This could require additional measurements or monitoring, which is unnecessary under the current design. For example, the UE may need to measure both beams simultaneously which adds measurement overhead or need to compare current measurements of one beam using historic measurements from another beam.

A few companies expressed their support for option b) where UE has the freedom to choose one of the two serving/current beams based on UE implementation. Although the impact is lesser than that of option a) agreeing to option b) also results in changes to the MR MAC CE format, similar to that required if option a) is agreed.

In our view, option c) where the network indicated which of the two beams shall be used, is much simpler and has least impact to specification. Further the measurement report is configured by the network based on any information that is required by the network in order to take mobility related decision. We think option c) serves this purpose more than the other options as the control of selecting the serving/current beam is given to the network and not left to the UE selection.

Proposal 8: When two indicated TCI states are indicated for a serving cell, the network indicates which of these two TCI states is used for evaluating event-triggered MR.

Current MR MAC CE has no ID information for the serving beam. In case of mTRP operation, though our preference is option c) listed above, which does have any impact on the MR MAC CE format as there is no ambiguity regarding the RSRP value reported in the MR MAC CE, if option a) or option b) listed above is adopted, the NW does not know what the reported RSRP value corresponds to (e.g., the first TCI state or the second TCI state).

For option a) or option b), we think the UE has to indicate whether the first or second TCI state was used for the event evaluation. The last octet in MR MAC CE comprises one reserved (R) bit which can be used to indicate one of the two TCI states in mTRP operation.

Proposal 9: MR MAC CE shall indicate the TCI state associated with the reported RSRP value for the serving/current beam.

# Conclusions

The contribution discussed aspects related to open issues for L1 event triggered MR. The following observations and proposals are made:

Observation 1: a candidate beam information in a (truncated) MR MAC CE requires 2 octets while a serving/ current beam information requires 1 octet.

Observation 2: If the odd number of octets are available for a UL resource to transmit a truncated L1 MR MAC CE, implementing the working assumption leads to wastage of UL resources.

Proposal 1: After including triggered beam information in a truncated L1 MR MAC CE,

- if the remaining available UL resource can accommodate a candidate beam information, include a candidate beam information in the truncated L1 MR MAC CE;

- else if the remining available UL resource can accommodate only the current beam information, include the current beam information in the truncated L1 MR MAC CE;

- else the truncated MR MAC CE only includes the triggered beam information.

Proposal 2: In truncated L1 MR MAC CE, a field indicates whether serving beam information is included or not.

Proposal 3: For truncated MR MAC CE, introduce a timer with the following behavior:

- Start the timer on transmitting the truncated MR MAC CE

- Stop the timer on receiving UL grant for subsequent transmission of MR MAC CE

- On expiry of the timer, trigger SR. If SR is not configured, initiate RACH.

Proposal 4: In an MR MAC CE transmitted after transmitting a truncated MR MAC CE, indicate whether the MR MAC CE is a new MR MAC CE or continuation of the truncated MR MAC CE or not.

Proposal 5: On detecting beam failure on SpCell, cancel the triggered L1 MR and reset TTT.

Proposal 6: Upon detecting beam failure on SpCell, the UE stops performing L1 measurement evaluation.

Proposal 7: After beam failure detection on the SpCell, the UE starts performing L1 measurement evaluation after:

Alt1: a new TCI state is indicated for the SpCell.

Alt2: completion of beam failure recovery and L1 measurement evaluation is performed based on the candidate RS of the SpCell identified for the beam failure recovery.

Proposal 8: When two indicated TCI states are indicated for a serving cell, the network indicates which of these two TCI states is used for evaluating event-triggered MR.

Proposal 9: MR MAC CE shall indicate the TCI state associated with the reported RSRP value for the serving/current beam.