3GPP TSG-RAN2 Meeting #131	R2-2505793

# Bangalore, India, 25 - 29 August 2025

Agenda Item:	8.2.3

Source:	Ericsson

Title:	UL multiple access for Ambient IoT

Document for:	Discussion/Decision

# 1	Introduction

A new work item for A-IoT has been agreed in RP#106 [1]. The following objectives are defined for RAN2 scope:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

We focus on the open issues on RACH summarized in the email discussion [POST129bis][017][AIoT] 38.391 Running CR (Huawei) in this paper.

# 2	Discussion

RAN2 has made below progress on random access related aspects in RAN2#130

Agreements

- For Msg1 resource selection procedure capture as guidance the countdown behaviour in the MAC specification (use TP in R2-2503952).  Capture a NOTE that other implementation are allowed.   X, Y will be signalled by paging message

- The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages.  R2D trigger message is not sent in CFRA procedure.   Come back if RAN1/4 sees any issues.  Send LS to RAN1/RAN4

- FFS  R2D byte alignment dependent on TBS size discussion

[POST130][036][AIoT] LS to RAN1/RAN4 (Vivo)

Intended outcome: LS to RAN1/RAN4

Deadline: Short

Agreements on RA 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. Agreements on NACK reception: - After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify. Agreements on RN16/AS ID maintainance: - Confirm a device is not expected to maintain both AS ID and RN16. After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader. This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission. How to capture device behavior is FFS

## 2.1	Frequency Index Indication In MSG2

In the last meeting there is an FFS

Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible.  FFS Discuss further whether to include it.

We believe, it is beneficial to include frequency index along with RN16 in Msg2 due to following reasons

- This reduces collision rate as two or more devices can select same AS ID even these devices transmit their Msg1s in indifferent frequency resources.

- In case frequency index is included in subPDUs, the device does not go through all subPDUs, and this might save processing cost. For instance, if there are 4 carriers associated with 4 frequency indicates, then device will know a-priory which subPDU it needs to check. For instance, if a device transmits its Msg1 in 3rd carrier, then in Msg2, it will jump to 3rd subPDU among group of 4 subPDUs to check for acknowledgment (assuming subPDUs are bitmapped in increasing sort order based on frequency indices)

- Although Rel-19 devices expected to be very simplistic (mostly passive), however the capabilities we RE looking in Rel-20 are expected by higher than currently we have. This will allow network to support more load in the same framework, and therefore we may expect higher collision rate. Therefore, we believe having frequency index inclusion also future proof from higher loading scenarios we may foresee in upcoming releases.

- Last, the possible solution on indicating frequency index is not complex. Having a frequency index as part of subHeader of a subPDU or subPDU bitmapped to frequency carriers are doable solutions without introducing complexity.

Therefore, it is recommended that based on above following reasons:

To confirm that frequency index along with RN16 is included in MSG2.

## 2.2 Number indication of echoed random IDs

In the MAC running CR, the below issue is raised.

| Issue 2-6: number indication of echoed random IDs in Msg2 | Whether to indicate the number of echoed random IDs included in Msg2. - RAN2 agreed that A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices, but there is no discussion on whether/how to indicate the number of echoed random IDs. The Rapp understands this can be considered as signaling design/stage3 issue which should be quite straightforward. - Status in running CR: the CR is implemented assuming no explicit number indication, and no issue is identified. | Companies are invited to input views for Q#5 |
| --- | --- | --- |

As the running CR implemented, msg2 may include multiple entries, and each entry includes random ID, AS ID indication and AS ID if assigned. So the question is whether the device need to know the number of the entry when decoding msg2. The CR is implemented without an explicit number indication assuming the device just decodes the entry one by one till the end of the message. The same logic can also apply to NACK Feedback message.

We believe there is no need to indicate number of echoed random IDs. The device decodes the message’s CRC first and then it can check its random ID entry subject to solutions, e.g., by checking entry one by one or if random IDs are mapped to frequency shifts (possibly allowing device to skip some entries mapping to frequency shifts which device did not utilize in transmitting MSG1). This shows that device already has decoded the MSG2 before knowing number of random ID entries, and thus, we find it meaningless to include it.

No need to indicate number of echoed random IDs in MSG2.

## 2.3 Trigger Message Byte Alignment

In the MAC running CR, the below issue is raised.

| Issue 2-3: R2D trigger message byte alignment | The R2D trigger message should be byte aligned or not. - Relevant agreements: - The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message. - FFS R2D byte alignment dependent on TBS size discussion - Status in running CR: not captured yet. | Companies are invited to input views for Q#11 |
| --- | --- | --- |

There is no strong preference. However, the processing and ease of handling may be slightly better if transmissions are byte aligned especially some device vendors may have only byte aligned implementation. Additionally, byte aligned trigger allow reserved bits which can be utilized for future scenarios indicating more information which we do not have the opportunity in this release due to lack of time.

- Byte aligned trigger message allows better processing and ease of handling at device side.

Confirms R2D trigger message is byte aligned.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Byte aligned trigger message allows better processing and ease of handling at device side.

Proposal 1	To confirm that frequency index along with RN16 is included in MSG2.

Proposal 2	No need to indicate number of echoed random IDs in MSG2.

Proposal 3	Confirms R2D trigger message is byte aligned.

# 4	References

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.