3GPP TSG-RAN WG2 Meeting #131	R2-2505841

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	6.1.3.1

Source:	Samsung, ASUSTeK, CATT, Ericsson, Nokia, Huawei, Apple, Xiaomi, ZTE Corporation, vivo

Title:	Discussion on eventD1/D2 and condEventD1/D2/T1

WID/SID:	NR_NTN_solutions, NR_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In Rel-17/18, eventD1/D2 are introduced to support location-based measurement, condEventD1/D2/T1 are introduced to support location/time-based CHO. In Rel-17 NTN CHO, condEvent/D1/D2/T1 is always configured with an RRM event. In Rel-18 NTN CHO, condEvent/D1/D2/T1 can be configured without RRM event.

In the last meeting, we have clarified some issues regarding eventD1/D2/T1.

RAN2 understands that only for (cond)EventD1/D2/T1, layer 3 filtering is not applicable (no spec change)

Come back in the next meeting to continue the discussion on whether/how RRM measurements results for neighbour cells are performed for eventD1/D2.

But there are different views on the issue whether/how RRM measurements results for neighbour cells are performed for eventD1/D2. In this paper, we try to conclude the issue.

# Discussion

In Rel-17, there were some discussion and specification change regarding the issue for eventD1, and similar specification change was made for eventD2 in Rel-18. See Appendix 5.1 a summary of the discussion in RAN2#118. Similar issue for eventD2 was discussed in RAN2#126 but no change was pursued. Based on the past discussion, we have the following observations and proposal, and there should be no need to repeat the discussion for eventD1/D2.

Observation 1: The conclusion from RAN2#118 is that UE does not need to report measurement results for neighbor cells for the measId configured with eventD1 since eventD1 is not configured to associate with specific neighbour cells. The same is applied for eventD2.

Observation 2: The working assumption in RAN2#115 that piggybacking RRM measurement report when location-based event triggers a report was not agreed/confirmed. There is no corresponding UE behaviour specified for eventD1, so neither for eventD2.

Proposal 1: RAN2 clarifies for measId configured with eventD1/D2 for neighbor cells, UE does not need to perform RRM measurement or report result for neighbor cells in the report of the measId.

In the current procedure of CHO evaluation, as highlighted in Appendix 5.2, UE consider the cell which has a physical cell identity matching the value configured in condRRCReconfig to be applicable cell, and evaluates whether the event condEventD1/D2/T1 is fulfilled for the applicable cell. The applicable cell becomes triggered cell if the corresponding event(s) is fulfilled, and then considered as the selected cell (i.e., target cell) for CHO execution.

Normally with RRM event UE performs RRM measurement according to measObject configuration associated to the condRRCReconfig and UE considers the detected cell which has the PCI same as the configured PCI in condRRCReconfig as the applicable cell for CHO. However, for condEventD1/D2/T1 only without RRM event, UE performs time/distance measurement when evaluating condEventD1/D2/T1, and it is not clear whether/how UE performs cell measurement when CHO is not configured with RRM trigger event. So the problem is for CHO with condEventD1/D2/T1 only, whether/how UE performs cell measurement for the applicable cell, and how UE detects the applicable cell, i.e., the cell that has the same PCI as configured in reconfigurationWithSync.

Observation 3: For CHO with condEventD1/D2/T1 only, it is not clear 1) whether/how UE performs cell measurement based on SSB/CSI-RS when no RRM event is configured and 2) how to find the applicable cell, i.e., the cell that has the same PCI as configured in reconfigurationWithSync.

Similar to eventD1/D2, for CHO for the measID configured with condEventD1/D2/T1, UE does not need to perform RRM measurement for the applicable cell. This is for the measId configured with condEventD1/D2/T1 in two cases: 1) CHO with a first measId for RRM event and a second measId for condEventD1/D2/T1, 2) CHO without RRM event and with only condEventD1/D2/T1.

Regarding how to detect the applicable cell, the current procedure does not explicitly specify (even for legacy CHO). One interpretation can be UE may find the applicable cell according to the associated measObject since the associated measObject can be configured with frequency or PCI. UE can consider the cell that is detected on the associated measObject which has a PCI matching the indicated PCI in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig as applicable cell, so that UE can perform CHO to that cell upon the execution condition of condEventD1/D2/T1 is triggered.

A second understanding can be that UE does not detect/search the applicable cell when evaluating condEventD1/D2/T1, similar to EventD1/D2. UE considers the cell indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig as applicable cell. UE can search the cell based on the target cell RRCReconfiguration upon CHO execution. This seems natural if UE does not need to do cell measurement when evaluating CHO, but may have some drawback like blind HO.

It seems both ways can work from UE perspective. For CHO with only condEventD1/D2/T1, the current procedure can work allowing both. If the associated measObject is configured with some information that can be utilized, e.g., ssbFrequency, gap, it is possible for UE to find the applicable cell when evaluating condEventD1/D2/T1, which can speed up the CHO execution. If detecting applicable cell is not possible for the UE, UE can search the cell upon CHO execution in the worst case. We may not restrict to one case or the other.

Proposal 2: For CHO, RAN2 clarifies UE does not need to perform RRM measurements for a measId configured with condEventD1/D2/T1 when evaluating condEventD1/D2/T1. This does not preclude the UE from detecting based on the associated measObject the applicable cell which has a physical cell identity matching the value in reconfigurationWithSync when evaluating condEventD1/D2/T1 for CHO with only condEventD1/D2/T1.

Regarding the specification impact, the concern on the current procedure (see highlighted part in Appendix 5.3) is that 1) UE shall follow the cell measurement procedure, 2) it is not clear how to detect applicable cell for CHO. Considering there are different views, we can further discuss the specification impact of Proposal 1 and 2 (e.g., no change, a NOTE for clarification, or change on procedural text).

Proposal 3: FFS the specification impact of Proposal 1 and 2 (e.g., no change, a NOTE for clarification, or change on procedural text).

# Conclusion

In this contribution we discussed the following proposals.

Observation 1: The conclusion from RAN2#118 is that UE does not need to report measurement results for neighbor cells for the measId configured with eventD1 since eventD1 is not configured to associate with specific neighbour cells. The same is applied for eventD2.

Observation 2: The working assumption in RAN2#115 that piggybacking RRM measurement report when location-based event triggers a report was not agreed/confirmed. There is no corresponding UE behaviour specified for eventD1, so neither for eventD2.

Proposal 1: RAN2 clarifies for measId configured with eventD1/D2 for neighbor cells, UE does not need to perform RRM measurement or report result for neighbor cells in the report of the measId.

Observation 3: For CHO with condEventD1/D2/T1 only, it is not clear 1) whether/how UE performs cell measurement based on SSB/CSI-RS when no RRM event is configured and 2) how to find the applicable cell, i.e., the cell that has the same PCI as configured in reconfigurationWithSync.

Proposal 2: For CHO, RAN2 clarifies UE does not need to perform RRM measurements for a measId configured with condEventD1/D2/T1 when evaluating condEventD1/D2/T1. This does not preclude the UE from detecting based on the associated measObject the applicable cell which has a physical cell identity matching the value in reconfigurationWithSync when evaluating condEventD1/D2/T1 for CHO with only condEventD1/D2/T1.

Proposal 3: FFS the specification impact of Proposal 1 and 2 (e.g., no change, a NOTE for clarification, or change on procedural text).

# Reference

[1] R2-2206152 Report from Break-out session on R17 NTN, REDCAP and CE, Vice Chairman (ZTE Corporation)

[2] R2-2206191	Rapporteur report of [offline-101] RRC CR

[3] R2-2206209	Rapporteur report of [offline-101] RRC CR – second round

[4] R2-2206508	Rapporteur report of [offline-101] RRC CR – third round

[5] R2-2206502	Correction for NR NTN WI	Ericsson	CR	Rel-17	38.331	17.0.0	3088	1	F	NR_NTN_solutions-Core

# Appendix

## RAN2#118 discussion on eventD1

- RAN2#115 working assumption never confirmed

Working Assumptions:

- Specify that measurement reports can be configured to be piggybacked with location report when location based event triggers it

- RAN2#118 R2-2206191	Rapporteur report of [offline-101] RRC CR

Couple of RILs were raised in context of D1 report [RIL]: L011 [Delegate]: LGE (SungHoon) [WI]: NTN [Class]: 2 [Status]: ToDo [TDoc]: None [Proposed Conclusion]: [Description]: A cell triggering event D1 is not included in the measurement report [Proposed Change]: In the current formulation, MeasurementReport triggered by event D1 does not include the cell meeting event D1 and its cell. So we propose to add the procedure text to include the cell meeting event D1. There are a couple of ways to enable this, and we think it is most straightforward to include the cell in cellsTriggeredList, as similar to other event cases. [Comments]: In the event D1, there is no cell that triggers the event so it is unclear how cells could be added based on the triggering. A related RIL, acknowledges this and proposes to add PCI in the D1: [RIL]: H801 [Delegate]: Huawei (Lili) [WI]: NTN [Class]: 1 [Status]: ToDo [TDoc]: Yes [Proposed Conclusion]: v167 [Description]: For event D1, there is a reference location of neighbour cell, but the UE does not know which neighbour cell it corresponds to. In fixed cell scenarios, there is no problem. However in moving cell scenarios, the UE needs to predict the trajectory of the reference location based on the ephemeris of the neighbour cell. So UE should know which cell the reference location belons to. [Proposed Change]: Add a PCI in the configuration of event D1 and modify the field description accordingly. We will submit a Tdoc addressing this issue. However, it is unclear what is the use of the PCI here. Network knows which location it has configured as ”target cell location” and the event has measID associated. Thus, when report is sent, network knows which event triggered it. Note that it is not actually mandated that the reference location2 is associetd to any actual cell. It is just a location coordinate. Secondly, it should be further elaborated what does the UE do with the information of the PCI.

- RAN2#118 R2-2206209	Rapporteur report of [offline-101] RRC CR – second round

Previous round, the following comments were provided on behalf of current specification not needing any changes on this: Current specification works. UE triggers measurement report for event D1 based on distance. So, UE can’t decide which cell can be included in the cellsTriggeredList. And adding the PCI to indicate the cell associated to reference location is not needed. For moving cell, NW can update the reference location in event D1 Agree, we believe there is no need to associate the reference location with any particular cell/PCI. When the location-based event will trigger, the UE will report measurements, where cell ID can be found. Then, there has been arguments that UE would need to determine PCI related to the D1 event but there has not been any which discussion or conclusion. D1 is an coordinate on Earth and does not have to be specific to any cell. As there is no cell associated the L011 seems redundant. Purpose of the D1 is as follows: - Event triggered-based UE location reporting are configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED That means it is not meant to track cells but UE location. The location based event is also primarily for fixed cells and handling moving cells-even for idle mode- is not discussed in Rel-17 There is also the following comment: Firstly, we think RAN2 should first discuss whether to confirm the following working assumption at RAN2#115e. - Specify that measurement reports can be configured to be piggybacked with location report when location based event triggers it If the working assumption is not confirmed, then we agree with Ericsson that we don’t see any issue here. However, if the working assumption is confirmed, then to piggyback RSRP/RSRQ, PCI information might be needed so that UE knows for which cell it needs to include RSRP/RSRQ. However, independent of whether this is agreed or not, there is still no need to associated PCI to the event. If UE detects a cell it uses the PSS/SSS to determine the PCI before measuring anyway.

- RAN2#118 R2-2206508	Rapporteur report of [offline-101] RRC CR – third round

The WA to include also RSRP results is not confirmed so it is not discussed. If network wants RSRP results network would configure UE to report corresponding RSRP results based on RSRP events A1, A2 and so on.

- RAN2#118 Chair note

R2-2206508 [offline-101] RRC CR – third round Ericsson discussion Rel-17 NR_NTN_solutions-Core Proposal 1 RAN2 to continue reviewing whether current specification has an issue with the approach of not confirming the WA, hence ”have UE to report only location when UE has MO associated only with eventD1” Proposal 2 RAN2 to agree to reject RILs 801, L011, H800 - HW wonders whether we can capture that RAN2 understands that the reference location of TN cell should not be broadcast - Samsung, Xiaomi, Intel, Apple think we should leave this to NW implementation - LG thinks: - Absence of neighbor cell measurement results at all in MR triggered by event D1 is sub-optimal, but we share the view that there is no big problem with that, since network can configure other measurement reporting configurations as per its need. In this sense, I am fine with not agreeing to proposal in L011. - What the intend to address now is whether the current spec (v17.0.0 has the problem that UE is required to include cells in the cellTriggeredList when constructing the MR triggered by event D1 as described in phase 3 discussion over email and below, i.e. whether the current spec is error-free for event D1 or not. There seems no technical/direct input so far on whether the problem is correct or not. There might be no serious issue in the end, but without checking it, we cannot be confident if the current MR procedure is error-free for event D1. - LG thinks there are easy solutions for this, e.g. add ”if available” - Agreed - RAN2 understands that Event D1 is not optimized for moving cells scenario - Discuss in the CR updating phase how to address the issue raised by LG Agreements: - RAN2 agrees to Propreject RILs 801, L011, H800

- Based on the above, in RRC CR R2-2206502 eventD1 is added below to exclude event D1 for the neighbour cell measurement reporting. In Rel-18, eventD2 is added in addition to eventD1.

4> for measurement events other than eventA1, eventA2, or eventD1: 5> if useAllowedCellList is set to true: 6> consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId; 5> else: 6> consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId … 1> if there is at least one applicable neighbouring cell to report: 2> if the reportType is set to eventTriggered or periodical: 3> if the measurement report concerns the candidate L2 U2N Relay UE: 4> set the sl-MeasResultCandRelay to include the best candidate L2 U2N Relay UEs up to maxReportCells in accordance with the following: 5> if the reportType is set to eventTriggered: 6> include the L2 U2N Relay UEs included in the relaysTriggeredList as defined within the VarMeasReportList for this measId; 5> else: 6> include the applicable L2 U2N Relay UEs for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset; 3> else: 4> set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following: 5> if the reportType is set to eventTriggered and eventId is not set to eventD1: 6> include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId; 5> else: 6> include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

## CHO procedure

##### 5.3.5.13.4	Conditional reconfiguration evaluation

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	if the RRCReconfiguration within condRRCReconfig includes the masterCellGroup including the reconfigurationWithSync:

3>	if the associated condExecutionCondPSCell is configured:

4>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell; and

4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the nr-SCG within the received condRRCReconfig to be applicable cell;

3>	else:

4>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell;

…

2>	for each measId included in the measIdList within VarMeasConfig indicated in the condExecutionCond, condExecutionCondSCG, or condExecutionCondPSCell of the condReconfigId:

3>	if condExecutionCond, condExecutionCondSCG, and subsequentCondReconfig are included for the condReconfigId:

4>	ignore the measId(s) in the condExecutionCond of the condReconfigId;

3>	if the condTriggerConfig is not configured with nesEvent:

4>	if the condEventId is associated with condEventT1, and if the entry condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

4> if the condEventId is associated with condEventD1 or condEventD2, and if the entry conditions applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

4>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be fulfilled;

4>	if the measId for this event associated with the condReconfigId has been modified; or

4>	if the condEventId is associated with condEventT1, and if the leaving condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

4>if the condEventId is associated with condEventD1 or condEventD2, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

4>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be not fulfilled;

## Concerned current procedure

### 5.5.3	Performing measurements

#### 5.5.3.1	General

...

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	if the reportType for the associated reportConfig is periodical, eventTriggered; or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG VarMeasConfig and is indicated in the condExecutionCond or in the condExecutionCondPSCell associated to a condReconfigId in the MCG VarConditionalReconfig (for CHO, CPA, MN-initiated inter-SN CPC, or subsequent CPAC in NR-DC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the SCG VarConditionalReconfig (for intra-SN CPC or subsequent CPAC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCondSCG associated to a condReconfigId in the MCG VarConditionalReconfig (for SN-initiated inter-SN CPC or subsequent CPAC in NR-DC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the triggerConditionSN associated to a condReconfigurationId in VarConditionalReconfiguration as specified in TS 36.331 [10] (for SN-initiated inter-SN CPC in EN-DC):

3>	if a measurement gap configuration is setup, or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or

4>	if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP:

5>	if the measObject is associated to NR and the rsType is set to csi-rs:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

…

##### 5.3.5.13.4	Conditional reconfiguration evaluation

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	if the RRCReconfiguration within condRRCReconfig includes the masterCellGroup including the reconfigurationWithSync:

3>	if the associated condExecutionCondPSCell is configured:

4>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell; and

4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the nr-SCG within the received condRRCReconfig to be applicable cell;

3>	else:

4>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell;

…