3GPP TSG-RAN2 Meeting #131	R2-2505858

Bengaluru, India, 25 - 29 August 2025	Revision of R2-2504290

Agenda Item:	8.4.4 Procedures for LP-WUS in RRC_CONNECTED

Source: 	Ericsson

Title:  	LP-WUS in Connected

Document for:	Discussion and Decision

# Introduction

In this contribution the open issues for LP-WUS in Connected are discussed further.

For the RAN1 and RAN2 agreements in previous meetings the reader is referred to the Annex.

# Discussion

## Dual DRX group (open issue 1)

During email discussion #213 it was discussed whether the UE only monitors LP-WUS in FR1, when both FR1 and FR2 are outside Active Time (AT). The drx-InactivityTimer for FR2 is smaller than the drx-InactivityTimer for FR1, which means that it is not a rare event that FR1 is inside AT while FR2 is outside AT. The UE is only required to monitor LP-WUS during AT, i.e. it would not be possible to wake-up FR2 when FR1 is active. This is a significant degradation in the usefulness of LP-WUS with dual DRX group. During email discussion #213 it is proposed that the UE only monitors LP-WUS when both FR1 and FR2 are outside AT. Such a UE implementation should be allowed when the UE is not able to monitor LP-WUS on FR1 while FR1 is active and FR2 is inactive. However it is beneficial if a UE implementation that is able to monitor LP-WUS when FR1 is active and FR2 is inactive, i.e. enable LP-WUS to only wake-up FR2. Therefore it is proposed:

The UE can indicate via UE capability signalling that it is able to monitor LP-WUS when FR1 is in Active Time and FR2 is outside Active Time.

When LP-WUS is configured with option 1-1 then each DRX group has its own drx-OnDurationTimer with its own parameter value. When LP-WUS is configured with option 1-2 then it should be possible to configure a separate lpwus-PDCCHMonitoringTimer parameter and value for the secondary DRX group:

Introduce lpwus-PDCCHMonitoringTimer for the secondary DRX group that can be configured when LP-WUS with lpwus-PDCCHMonitoringTimer and secondary DRX is configured.

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

…

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup-r16 }                   OPTIONAL    -- Need M

]],

…

tar-Config-r18                      SetupRelease { TAR-Config-r18  }                                OPTIONAL    -- Need M

]],

drx-ConfigSecondaryGroup-r19        SetupRelease { DRX-ConfigSecondaryGroup-r19 }                   OPTIONAL    -- Cond Lpwus

]]

}

| Conditional Presence | Explanation |
| --- | --- |
| Lpwus | This field is optionally present, Need R, if LP-WUS with lpwus-PDCCHMonitoringTimer and secondary DRX are configured. It is absent otherwise. |

DRX-ConfigSecondaryGroup-r16 ::=   SEQUENCE {

drx-onDurationTimer-r16            CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer-r16            ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

DRX-ConfigSecondaryGroup-r19 ::=   SEQUENCE {

lpwus-PDCCH-MonitoringTimer-r19      TBD

}

## LP-WUS collisions (open issue 2)

In case option 1-1 is configured it is clear that when the UE is not able to monitor the LP-WUS Occasion (LO) then the UE shall start the drx-OnDurationTimer.

In case option 1-2 is configured, this is less obvious. In case the configured LO periodicity is very short, then it can be ok when the UE is not able to monitor one (or more) LO’s, because there is another LO very quickly again. RAN1 did not reach agreement on this topic [2]:

LP-WUS monitoring

Agreement:

As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS.

Agreement:

As the reply to RAN2 LS in R1-2503616, RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19. RAN1 understanding is that the terminology of LR and MR operations are for discussion purpose and will not be specified

- LR operation is the UE operation for LP-WUS monitoring

- MR operation is the UE operation for all other NR signals/channels transmissions/receptions in connected mode

Conclusion

From RAN1 perspective, for the case of potential collision (if any) in Option 1-2, when the UE is not able to monitor all the LP-WUS MO(s) in a LP-WUS periodicity,

- It is up to RAN2 to further discuss and finalize the specification support, if any.

In case option 1-2 is configured, the following options were discussed during email #213:

- Option 1: UE starts the lpwus-PDCCHMonitoringTimer when there is collision. (Same operation as Option 1-1)

- Option 2: UE doesnot start the lpwus-PDCCHMonitoringTimer when there is collision.

- Option 3: NW can configure UE whether to start the lpwus-PDCCHMonitoringTimer or not when there is collision.

Companies have different views whether to select option 1 or option 2. The rapporteur suggested to go for option 3 as a compromise.

The NW configures the LP-WUS periodicity with option 1-2 to meet certain latency requirements. The NW expects that the UE is reachable within an LO periodicity. Option 1 and 2 are the two extremes how to handle collisions with option 1-2. In case the UE does not start the timer during a collision, then there is a risk that the UE is unreachable for some time when there are consecutive collisions. To find a better trade-off it is proposed:

The UE can be configured to start lpwus-PDCCHMonitoringTimer when there is a collision or when there are N consecutive collisions (N > 1).

RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19 [2]. This is a rather strong statement, i.e. perhaps some UE implementation can execute certain MR and LR tasks simultaneously (e.g. MR intra-frequency measurements and LR serving cell measurements).

For DCP it is captured in 38.300:

The UE does not monitor DCP on occasions occurring during active-time, measurement gaps, BWP switching, or when it monitors response for a CFRA preamble transmission for beam failure recovery (see clause 9.2.6), in which case it monitors the PDCCH during the next on-duration.

RAN1 indicated that the UE is not able to monitor LP-WUS during Active Time (AT), measurement gap and RAR window for BFR [2]:

As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS.

It is already specified in RAN1 specification that the UE is not required to monitor LP-WUS during Active Time. But there may not be an LP-WUS Occasion between the end of the Active Time and the next on-duration, i.e. it is good to describe that use case explicitly in addition to the general RAN1 agreement that the UE does not monitor LP-WUS during Active Time.

There is a benefit to specify the use cases where the UE may not be able to use LP-WUS to guarantee a minimum quality of the UE implementation. In case these use cases are not explicitly mentioned then the expected UE behavior remains unclear.

Based on the discussion above it proposed for option 1-1 and collisions, that a UE implementation is not prevented that can operate MR and LR simultaneously for certain use cases:

With option 1-1 when the UE is not able to monitor LP-WUS during measurement gap, BWP switching, or CFRA preamble transmission for beam failure recovery (see clause 9.2.6), then the UE monitors PDCCH during the next on-duration. And when there is no LP-WUS occasion between the end of active time and the next on-duration, then the UE monitors PDCCH during the next on-duration.

Similar requirement can be defined for option 1-2 in case there are N consecutive collisions:

With option 1-2 when the UE is not able to monitor N consecutive LP-WUS Occasions during measurement gap, BWP switching, or CFRA preamble transmission for beam failure recovery (see clause 9.2.6), then the UE starts lpwus-PDCCHMonitoringTimer. When there are more than N collisions after the end of active time then the UE starts lpwus-PDCCHMonitoringTimer.

## UAI preferred time offset (RRC-5, MAX-X3)

There is an open issue whether the UE is allowed to send an “empty” UAI message, i.e. a UAI message including UEAssistanceInformation-v19 but excluding lpwus-OffsetPreference-r19:

UEAssistanceInformation-v19xx-IEs ::= SEQUENCE {

lpwus-OffsetPreference-r19               LPWUS-OffsetPreference-r19             OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                                      OPTIONAL

}

LPWUS-OffsetPreference-r19 ::=           SEQUENCE {

timeOffset-r19              ENUMERATED {ms5, ms13, ms37}                        OPTIONAL

}

An UAI message, i.e. UE preference, is sent after the feature is configured or when there is a change in the preference and the prohibit timer is not running.

PS: it has not been explicitly described in the procedure text, where a change also includes “no preference”.

First a recap of what has been discussed and agreed for UAI signalling for UE power saving (e.g. preferred DRX parameters, maximum CC’s, etc):

- Initially it was agreed that the UE does not signal “no preference” (RAN2#108):

      - Explicit signalling for ‘no preference’ is not supported for any of the power saving parameters (cDRX, number of SCells, aggregated BW or number of MIMO layers)

- But after further discussions it was agreed (RAN2#109-e bis):

1	Delta signalling applies at a ‘feature’ level, where the ‘features’ for power saving are: drx-Preference, maxBW-Preference, maxCC-Preference, maxMIMO-LayerPreference, minSchedulingOffsetPreference and releasePreference.  No further grouping is considered.

2	When reporting a ‘feature’, the all parameters that the UE has a preference for are included. Parameters that are not included are interpreted as the UE having no preference for those parameters.

3	An empty ‘feature’ IE can be signalled to indicate that the UE has no preference for all parameters in the ‘feature’ (i.e. similar to overheating).

FYI: UAI signalling for UE power saving:

UEAssistanceInformation-v1610-IEs ::= SEQUENCE {

idc-Assistance-r16                  IDC-Assistance-r16                  OPTIONAL,

drx-Preference-r16                  DRX-Preference-r16                  OPTIONAL,

maxBW-Preference-r16                MaxBW-Preference-r16                OPTIONAL,

maxCC-Preference-r16                MaxCC-Preference-r16                OPTIONAL,

maxMIMO-LayerPreference-r16         MaxMIMO-LayerPreference-r16         OPTIONAL,

minSchedulingOffsetPreference-r16   MinSchedulingOffsetPreference-r16   OPTIONAL,

releasePreference-r16               ReleasePreference-r16               OPTIONAL,

sl-UE-AssistanceInformationNR-r16   SL-UE-AssistanceInformationNR-r16   OPTIONAL,

referenceTimeInfoPreference-r16     BOOLEAN                             OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1700-IEs   OPTIONAL

}

DRX-Preference-r16 ::=              SEQUENCE {

preferredDRX-InactivityTimer-r16    ENUMERATED {ms0, … ms2560} OPTIONAL,

preferredDRX-LongCycle-r16          ENUMERATED {ms10, … ms10240} OPTIONAL,

preferredDRX-ShortCycle-r16         ENUMERATED {ms2, … ms640} OPTIONAL,

preferredDRX-ShortCycleTimer-r16    INTEGER (1..16)    OPTIONAL

}

MaxBW-Preference-r16 ::=            SEQUENCE {

reducedMaxBW-FR1-r16                ReducedMaxBW-FRx-r16                     OPTIONAL,

reducedMaxBW-FR2-r16                ReducedMaxBW-FRx-r16                     OPTIONAL

}

This means that:

- If the UE wants to change a drx-Preference, and the UE has sent a maxBW-Preference earlier, then the UE is not required to include the maxBW-Preference again in the UAI message, i.e. this information is stored in the gNB.

- But the UE is required to send a “full” drx-Preference, i.e. when a DRX parameter is omitted this means that the UE does not have a preference for that DRX parameter. Omission of a DRX parameter does not mean that there is no change for that parameter.

For some use cases it is essential or useful that the UE sends “no preference”. For example for overheating the UE should indicate when it is no longer overheated, and for IDC when there is no longer a problem. We think that for time offset preference there is no strong motivation to send “no preference”. But we are fine to follow the general UAI principle:

When the UE sends an “empty” UAI message for the preferred LP-WUS time offset then the UE does not have a preference for the LP-WUS time offset.

## Short DRX and option 1-2

With option 1-1 the LP-WUS Occasion is coupled to the configured long DRX cycle and the UE does not monitor DCP during short DRX cycle (even though it is technically feasible to monitor DCP during short DRX cycle).

With option 1-2 the LP-WUS Occasions are decoupled from the configured short/long DRX cycle. It is best and most logical to maintain this decoupling w.r.t. the short DRX cycle:

With option 1-2 the LP-WUS monitoring is decoupled from the configured short/long DRX cycle, i.e. the UE monitors LP-WUS when it is in short and long DRX.

## Fallback to PDCCH monitoring

With LP-WUS in connected mode the UE monitors the LP-WUS when configured. The NW is expected to reconfigure the UE when it roams in and out of LP-WUS coverage, based on existing measurement/CSI reporting of the UE.

RAN1 did not reach consensus on UE autonomous fallback to PDCCH monitoring:

R1-2504833	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Conclusion

There is no consensus in RAN1 whether to specify UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS in RRC CONNECTED mode.

Using existing RRM/CSI measurement reporting the gNB can configure or de-configure LP-WUS via RRCReconfiguration, e.g. when the UE is inside/outside LP-WUS coverage. When LP-WUS is configured the gNB uses LP-WUS to wake-up the UE irrespective whether the UE is inside or outside LP-WUS coverage. To avoid ping-pong, i.e. frequent reconfigurations, the gNB can decide to configure LP-WUS after a certain time period when the UE has reported good coverage and immediately de-configure LP-WUS when the quality drops below a certain threshold. This can be left to gNB implementation.

It is important that LP-WUS is reliable and robust, i.e. risk for the error case that the UE is using LP-WUS but the NW is not should be avoided since the UE becomes unreachable in the DL. The opposite, i.e., when the NW is using LP-WUS but the UE directly monitors PDCCH, does not cause the UE to become unreachable in the DL but wastes resources from unnecessary LP-WUS transmission.

For example, when the UE is outside LP-WUS coverage and cannot receive LP-WUS then the UE should monitor PDCCH directly. It is possible that the UE suddenly loses LP-WUS coverage and there is delay in the RRM/CSI measurement reporting, then the UE may become temporarily unreachable until LP-WUS is de-configured. Typically, RRM and CSI reporting is not configured periodically to avoid high signalling load, instead, event based (RRM) or aperiodic (CSI) reporting are more commonly configured. Independent on how the reporting is configured, the UE performs RRM/RLM/BFD measurements much more often than the reporting, and a local decision by the UE on current measurements can take much quicker effect.

There is a risk that the UE suddenly may lose LP-WUS coverage before the NW based on measurement/ CSI reporting of the UE is able to de-configure LP-WUS. To reduce this risk an LR exit condition can be configured, similar when LP-WUS is configured in SIB for UEs in idle/inactive. However in connected mode the UE is not required to measure LP-SS, i.e. an exit condition based on LP-SS measurements is not possible. But the exit condition can be based on RRM/RLM/BFD measurements, and RLM/BFD measurements are preferred because the UE is also required to measure RLM/BFD outside Active Time at least every C-DRX cycle, i.e. exit condition is also detected outside Active Time when there is no traffic:

The LP-WUS configuration can include an exit condition based on RLM/BFD measurements. When the exit condition is fulfilled the UE stops monitoring LP-WUS and monitors PDCCH directly.

## Paging PDCCH

In connected mode the UE does not receive Paging PDSCH, but the UE is required to monitor Paging PDCCH, e.g. to receive ETWS/CMAS warning messages. If LP-WUS is configured in SIB, then the UE in connected mode can use the idle/inactive LP-WUS to monitor Paging PDCCH:

If LP-WUS in idle/inactive mode is configured in SIB then the UE in connected mode may use the LP-WUS configuration in SIB to monitor Paging PDCCH e.g. SI-change and ETWS/CMAS indication in connected mode.

# Summary

RAN2 is kindly asked to discuss LP-WUS in connected mode:

No table of figures entries found.

Proposal 1	The UE can indicate via UE capability signalling that it is able to monitor LP-WUS when FR1 is in Active Time and FR2 is outside Active Time.

Proposal 2	Introduce lpwus-PDCCHMonitoringTimer for the secondary DRX group that can be configured when LP-WUS with lpwus-PDCCHMonitoringTimer and secondary DRX is configured.

Proposal 3	The UE can be configured to start lpwus-PDCCHMonitoringTimer when there is a collision or when there are N consecutive collisions (N > 1).

Proposal 4	With option 1-1 when the UE is not able to monitor LP-WUS during measurement gap, BWP switching, or CFRA preamble transmission for beam failure recovery (see clause 9.2.6), then the UE monitors PDCCH during the next on-duration. And when there is no LP-WUS occasion between the end of active time and the next on-duration, then the UE monitors PDCCH during the next on-duration.

Proposal 5	With option 1-2 when the UE is not able to monitor N consecutive LP-WUS Occasions during measurement gap, BWP switching, or CFRA preamble transmission for beam failure recovery (see clause 9.2.6), then the UE starts lpwus-PDCCHMonitoringTimer. When there are more than N collisions after the end of active time then the UE starts lpwus-PDCCHMonitoringTimer.

Proposal 6	When the UE sends an “empty” UAI message for the preferred LP-WUS time offset then the UE does not have a preference for the LP-WUS time offset.

Proposal 7	With option 1-2 the LP-WUS monitoring is decoupled from the configured short/long DRX cycle, i.e. the UE monitors LP-WUS when it is in short and long DRX.

Proposal 8	The LP-WUS configuration can include an exit condition based on RLM/BFD measurements. When the exit condition is fulfilled the UE stops monitoring LP-WUS and monitors PDCCH directly.

Proposal 9	If LP-WUS in idle/inactive mode is configured in SIB then the UE in connected mode may use the LP-WUS configuration in SIB to monitor Paging PDCCH e.g. SI-change and ETWS/CMAS indication in connected mode.

# References

- R2-2503187, LS on LP-WUS in RRC_CONNECTED, RAN2 (Interdigital), LS out, To: RAN1, RAN2#129-bis

- R1-2504888, Reply LS on LP-WUS in RRC_CONNECTED, RAN1, LS out, To: RAN2, RAN1#121

# Appendix: RAN1 and RAN2 agreements

## RAN1 agreements (selection)

R1-2504940	Final summary on LP-WUS operation in CONNECTED mode     	Moderator (NTT DOCOMO)

R1-2504713	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

For LP-WUS MOs in connected mode for Option 1-1/1-2, the time offset1/3 in previous agreement are configured relative to the start of SFN0 where the offset is a symbol-level offset

Agreement

As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS.

R1-2504805	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, when Rel-17 unified TCI framework is NOT configured or UE does NOT support Rel-17 unified TCI framework

- Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS

R1-2504833	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Conclusion

There is no consensus in RAN1 whether to specify UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS in RRC CONNECTED mode

Agreement

As the reply to RAN2 LS in R1-2503616, RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19. RAN1 understanding is that the terminology of LR and MR operations are for discussion purpose and will not be specified

- LR operation is the UE operation for LP-WUS monitoring

- MR operation is the UE operation for all other NR signals/channels transmissions/receptions in connected mode

Conclusion

From RAN1 perspective, for the case of potential collision (if any) in Option 1-2, when the UE is not able to monitor all the LP-WUS MO(s) in a LP-WUS periodicity,

- It is up to RAN2 to further discuss and finalize the specification support, if any.

Agreement

For the UE capability report on the minimum time gap between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, the same candidate values {V1, V2, V3} are supported for different receiver types

- V1=5ms

- V2=13ms

- V3=37ms

Agreement

For the UAI of preferred time offset for LP-WUS monitoring in RRC connected mode,

- The candidate values are same as those for the UE capability of minimum time gap, i.e., {V1=5, V2=13, V3=37}ms

- The reported UAI value is equal to or longer than the minimum time gap reported by UE capability for a UE

Note: There is no change to the RAN1 agreed definition for UAI

R1-2504886	FL summary #4 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

R1-2504887	Draft reply LS on LP-WUS in RRC_CONNECTED	Moderator (NTT DOCOMO)

Final reply LS on LP-WUS in RRC_CONNECTED is endorsed in R1-2504888

Agreement

Nominal MO duration (X1, in unit of OFDM symbols) and actual LP-WUS duration (X2, in unit of OFDM symbols) are configured for LP-WUS in connected mode. (Alt C)

- A LP-WUS MO spans the nominal MO duration (i.e., the LP-WUS MO duration is the same as the nominal MO duration.)

- If the number of available OFDM symbols within the nominal MO duration is no less than the actual LP-WUS duration, UE monitors LP-WUS on the first X2 available symbols within the LP-WUS MO.

- Otherwise, UE does not monitor LP-WUS in this MO (i.e., the MO is dropped).

- Note: Any symbols that are not defined as unavailable are available symbols for LP-WUS.

- Further discuss possible introduction of UE capability to restrict configuration of LP-WUS MO and LP-WUS duration. For example,

  - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration

  - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO

Agreement

For UE to determine whether a symbol is available for LP-WUS in connected mode, at least the following is supported:

- A unit level bitmap with a periodicity 10, 20, or 40 units and a 14-bit or 28-bit symbol-level bitmap that covers 1 or 2 slots can be configured, where each unit is 1 or 2 slots for 14-bit or 28-bit symbol-level bitmap, respectively, with a maximum periodicity of 40ms.

  - ‘1’ in unit-level bitmap means the symbol level bitmap is applied to determine which symbols are unavailable in the unit for LP-WUS (‘0’ means unavailable).

  - ‘0’ in unit-level bitmap means: all the symbols in the unit are unavailable for LP-WUS

- If the slot-level bitmap is not configured, UE assumes all 1’s for the bitmap.

- If the symbol-level bitmap is not configured, UE assumes all 1’s for the bitmap.

Agreement

For the handling of at least PCell/PSCell SSB for connected mode, the SSB symbols are considered as unavailable for LP-WUS

Agreement

The UL symbols/slots configured in tdd-UL-DL-configurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as unavailable for LP-WUS in connected mode

## RAN2 agreements (selection)

RRC-3, MAC-1, on whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group

Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

RRC-5, whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2

R2-2503615	Discussion on LP-WUS WUR in RRC_Connected, vivo, Disc

- ?? It is allowed to report an empty UAI on time offset for LP-WUS monitoring for both option 1-1 and option 1-2, which means UE has no preference on the time offset between LP-WUS and PDCCH monitoring.

Chair: other issues can also be discussed in the CB sesion, if time allows

R2-2503811	Remaining issues of LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

Check whether we need to capture in MAC that UE is not expected to monitor LP-WUS if not in Cell DTX active period