3GPP TSG-RAN WG2 #131	R2-2505908

# Bangalore, India, 24th – 29th August 2025

Agenda Item:	 8.1.2.3

Source:	Ericsson

Title:	RAN1 Agreements impacting RAN2 and Addressing Open Issues

Document for:	Discussion

# 1	Introduction

In this paper, we discuss the RAN1 agreements which have RAN2 impacts and address the open issues.

# 2 	Discussion

## 2.1 LPP Open Issue 20

## RAN1 Agreements

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope. Agreement (RAN#120) - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications.

As agreed in RAN1; UE (PRU or non-PRU UE) can generate label for data collection and provide to NW. Further, UE may be configured to transmit SRS and also be requested to provide its location (e.g: if the UE is a PRU UE). For data collection for case 3b; LMF would require a synchronized reception of gNB SRS measurements (part A) and UE location when gNB performed the measurements (part B). Similarly gNB would require a synchronized reception of true RTOA measurement (part B) from LMF when gNB is performing the UL-SRS measurement (part A). As per above RAN1 agreement, the

- The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted).

i.e for case 3b, the LMF should be able to request the time stamp of Part B when the label is generated to be provided to LMF and also UE may provide for how long the part B is valid for.

Since, this involves LMF and UE interaction for part B information collection w.r.t timestamp and validity duration of the label; We see the below RAN2 impacts as per the above RAN1 agreements. In NRPPa, there is already timestamp when gNB provides the measurement to LMF for case3b.

#### –	CommonIEsRequestLocationInformation

The CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message Type.

-- ASN1START

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType		LocationInformationType,

triggeredReporting			TriggeredReportingCriteria	OPTIONAL,	-- Cond ECID

periodicalReporting			PeriodicalReportingCriteria OPTIONAL,	-- Need ON

additionalInformation		AdditionalInformation		OPTIONAL,	-- Need ON

qos							QoS							OPTIONAL,	-- Need ON

environment					Environment					OPTIONAL,	-- Need ON

locationCoordinateTypes		LocationCoordinateTypes		OPTIONAL,	-- Need ON

velocityTypes				VelocityTypes				OPTIONAL,	-- Need ON

...,

[[

messageSizeLimitNB-r14	MessageSizeLimitNB-r14		OPTIONAL	-- Need ON

]],

[[

segmentationInfo-r14	SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

scheduledLocationTime-r17

ScheduledLocationTime-r17	OPTIONAL,	-- Need ON

targetIntegrityRisk-r17

TargetIntegrityRisk-r17		OPTIONAL	-- Need ON

]],

[[

periodicalReportingExt-r18

PeriodicalReportingCriteriaExt-r18

OPTIONAL	-- Need ON

]]

}

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...,

locationEstimateAndMeasurementsRequired-r18,

locationEstimateAndTimeStampRequired-r19

}

Further, when UE/PRU reports the location, it should be able to inform the validity duration of the reported location. This is specifically valid for PRUs that are stationary and thus it can provide a duration until when it expects to be stationary.

#### –	CommonIEsProvideLocationInformation

The CommonIEsProvideLocationInformation carries common IEs for a Provide Location Information LPP message Type.

-- ASN1START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate			LocationCoordinates		OPTIONAL,

velocityEstimate			Velocity				OPTIONAL,

locationError				LocationError			OPTIONAL,

...,

[[	earlyFixReport-r12		EarlyFixReport-r12		OPTIONAL

]],

[[	locationSource-r13		LocationSource-r13		OPTIONAL,

locationTimestamp-r13	UTCTime					OPTIONAL

]],

[[

segmentationInfo-r14	SegmentationInfo-r14	OPTIONAL		-- Cond Segmentation

]],

[[

integrityInfo-r17		IntegrityInfo-r17		OPTIONAL

]],

[[

locationValidityDuration-r19	UTC						OPTIONAL

]]

}

As per RAN1 agreements the measurement and the associated label where the measurement was taken should be synchronized. If different entities have performed the measurement (part A) and location estimate (part B) then the time stamp of both part A and part B are required.

The validity duration of part B (label) can be reported if known to the LMF.

(LPP-20) LMF can request the UE/PRU to provide the time stamp of the location estimate.

( LPP-20) UE/PRU provides a validity duration for the location estimate.

## 2.2 	Consistency between training and Inference, AI/ML UE Applicability Reporting and UE capability

##### In this section we handle below two open issues:

##### Open issue LPP-19 (new): Applicability of BM related agreements& Open issue LPP-18 (new) Consistency between training and Inference

It was discussed in last meeting [R2-2503403] that UE should be able to request using on-demand TRP IDs of the TRP which it has trained for. However, rather than TRP ID granular level, we believe associate ID is enough. If UE has trained in one cell, it should be able to de deduce the location estimates even if the same set of TRPs used in training were not provided in AD.

The UE applicability report from UE for AI/ML should consist of where UE is able to perform the inference, i.e. which cell ID, TRP ID, PRS ID or associate ID is applicable to perform the inference. These are intrinsic UE capabilities associated with the AI/ML Model, since UE has trained the model with such PRS and area characteristics, it has the capability to perform the inference when such conditions are met. It has been agreed that UE reports whether AI/ML applicability condition is met or not using LPP provide capability message. That is, the UE change of applicable conditions is reported in a UE capability message.

Currently, LPP UE capabilities are stored in the AMF and retrieved directly by LMF which saves latency as LMF does not need to retrieve the same from UE. In order to determine if AI/ML method should be selected or not, the associated attributes such as associated ID where UE has trained, area (cell ID, TRP ID) can also be stored as part of UE capabilities. This would speed up any subsequent positioning procedure for the same UE and would avoid any mismatch between inference and training phase.

To save latency and avoid the risk of inconsistency between training and inference phase, the associated ID where UE has trained can be stored by NW. This can be easily done if these attributes are reported as part of UE capabilities (or applicable condition as part of applicability report where UE capability message is reused for this purpose as already agreed). These attributes can be stored in AMF as part of UE capabilities and can be retrieved by LMF to determine whether AI/ML method would be feasible or not.

(LPP-19, LPP-18) Associate ID, Cell ID (NCGI) where UE can perform inference are signalled as part of UE applicability report

(LPP-19, LPP-18) NW stores applicability condition information (associate ID, cell ID) and reuse for subsequent positioning procedure similar as UE capability.

-- ASN1START

#### CommonIEsProvideCapabilities

The CommonIEsProvideCapabilities carries common IEs for a Provide Capabilities LPP message Type.

-- ASN1START

CommonIEsProvideCapabilities ::= SEQUENCE {

...,

[[

segmentationInfo-r14			SegmentationInfo-r14			OPTIONAL,	-- Cond Segmentation

lpp-message-segmentation-r14	BIT STRING { serverToTarget	(0),

targetToServer	(1) }	OPTIONAL

]],

[[

remoteUE-Indication-r18						BOOLEAN						OPTIONAL,	-- Cond NR

locationEstimateAndMeasurementReporting-r18	ENUMERATED { supported }	OPTIONAL

]]

[[

nr-DL-AIML-SupportedAssociateIDList-r19		SEQUENCE { SIZE(1..maxAssociateID))

NR-DL-AIML-SupportedAssociateID-r19 OPTIONAL,

]]

}

NR-DL-AIML-SupportedAssociateID-r19 ::= SEQUENCE	{

associatedID						INTEGER (1..255)

cellID								NCGI

}

-- ASN1STOP

## 2.3 Associated ID

##### Open issue LPP-21 (new): "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19)

Issue description:

The following issues have been discovered during LPP running CR review:

- Must the "Associated ID" for the TRP coordinates be associated with a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated with a cell) being handled? And how to implement RAN1 agreement that UE does not expect to receive different values of associated ID for TRPs belonging to the same cell?

- Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs?

- Is the value range of 0..255 for the "Associated ID" sufficient? In particular if the "Associated ID" should be "configured per cell"?

- Are the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both?

- If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16), must a UE support also implicit location info (nr-TRP-ImplicitLocationInfo-r19) (and vice versa)?

Our Answers to above questions as below:

Answer 1: The associate ID is not supported for PRS only TP at least in Rel-20. In our understanding, RAN1 has not worked on designing associate ID without cell association so we should leave it for Rel-20.

Answer 2: This should be left to NW implementation. Up to NW to decide on what granular level the associate ID should be updated. For now, we should stick to RAN1 definition.

Answer 3: Yes, that is RAN1 view and should be ok.

Answer 4: Mutually Exclusive. This is clear from RAN1 agreements that AIML works also without AD#7

Answer 5: Yes.

(LPP-21) Associate ID for PRS only TP is not considered in Rel-20. The scope of Associate ID (to what granular level the associate ID needs to be updated) is left to NW implementation. Associate ID value range is 0 to 255. Explicit and Implicit TRP coordinates are mutually inclusive. In terms of UE capability, the UE supporting case1 AI/ML must support implicit location info.

# Conclusion

In the previous sections we made the following observations:

Observation 1	As per RAN1 agreements the measurement and the associated label where the measurement was taken should be synchronized. If different entities have performed the measurement (part A) and location estimate (part B) then the time stamp of both part A and part B are required.

Observation 2	The validity duration of part B (label) can be reported if known to the LMF.

Observation 3	To save latency and also avoid the risk of inconsistency, the associated ID where UE has trained can be stored by NW. This can be easily done if these attributes are reported as part of UE capabilities (or applicable condition as part of applicability report where UE capability message is reused for this purpose as already agreed). These attributes can be stored in AMF as part of UE capabilities and can be retrieved by LMF to determine whether AI/ML method would be feasible or not.

Based on the discussion in the previous sections we propose the following:

Proposal 1	(LPP-20) LMF can request the UE/PRU to provide the time stamp of the location estimate.

Proposal 2	( LPP-20) UE/PRU provides a validity duration for the location estimate.

Proposal 3	(LPP-19, LPP-18) Associate ID, Cell ID (NCGI) where UE can perform inference are signalled as part of UE applicability report

Proposal 4	(LPP-19, LPP-18) NW stores applicability condition information (associate ID, cell ID) and reuse for subsequent positioning procedure similar as UE capability.

Proposal 5	(LPP-21) Associate ID for PRS only TP is not considered in Rel-20. The scope of Associate ID (to what granular level the associate ID needs to be updated) is left to NW implementation. Associate ID value range is 0 to 255. Explicit and Implicit TRP coordinates are mutually inclusive. In terms of UE capability, the UE supporting case1 AI/ML must support implicit location info.

# References

TS 37.355, LPP