3GPP TSG RAN WG2 Meeting #131	R2-2505950

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.2.2

Source:	CMCC

Title:	Discussion on A-IoT paging

Document for:	Discussion

Introduction

In RAN2#130 meeting [1], the following agreements have been made:

Agreements on parallel service request 1 Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in stage 2 specification. 2 The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in stage 3 specification. 3 Send LS to RAN3 to notify them of agreements 1 and 2 4 Parallel service request for overlapping reader scenario can be addressed in Rel-20 Agreements on paging 1.For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case. 2.For CFRA, the device always responds to paging regardless of transaction ID (if we put a transaction ID) (i.e. as long as it is addressed to the corresponding device). 3.To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field. FFS if more than one R bit is required. 4.Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message. 5.Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

Then in this contribution, we would like to provide our views on remaining issues in A-IoT paging.

Discussion

## 2.1 General analysis

### 2.1.1 Transaction ID

#### 2.1.1.1 Size of transaction ID

In RAN2#129 meeting, the following agreements have been made [2]:

3.The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID 4.1 bit solution is excluded. FFS the size. Aim to have a reasonable size.

As for the generation of transaction ID, it can be left to A-IoT reader’s implementation. With the consideration that the transaction ID is only used in A-IoT radio interface by RAN2 and is totally transparent to CN, so we think that RAN2 can determine the size of transaction ID by ourselves. In Re-19, since only gNB-reader is in scope and multi-reader scenario can be avoided by implementation as agreed in RAN2#130 meeting, the size of transaction ID does not need to be too long. While, further considering the scenario where the intermediate UE performs as an A-IoT reader in R20, we think 4-bit transaction ID makes sense.

Proposal 1: The way to generate “transaction ID” can be left to A-IoT reader’s implementation.

Proposal 2: RAN2 is asked to agree to a 4-bit transaction ID.

#### 2.1.1.2 Presence/absence in A-IoT Paging message that triggers CFA

In RAN2#130 meeting, it has been agreed that [1]:

For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case.

For transaction ID, we think that it may be absent for A-IoT paging message which triggers CFA, since one A-IoT device should always respond to the A-IoT paging which only targets to itself, regardless of the transaction ID, that is, an A-IoT device does not need to check the transaction ID field when processing A-IoT Paging message that triggers CFA procedure. And whether transaction ID field exists in an A-IoT paging message or not can be indicated by the RA Type field as captured in MAC running CR. For command case, we have not observed any differences.

Proposal 3: RAN2 confirm that Transaction ID field is always absent in A-IoT Paging message which triggers CFA.

### 2.1.2 Paging identifier

#### 2.1.2.1 Visibility of paging identifier to A-IoT MAC layer

In RAN2#129 meeting, it has been agreed that [2]:

2.The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer

Some companies think that the paging identifier may need to be visible to A-IoT MAC layer in gNB-reader so that sub-grouping scheme can be performed. From our perspective, we think that it is an optimization scheme for the A-IoT procedure and totally transparent to A-IoT devices. From our perspective, we do not find much benefits of this scheme.

Apart from this, since AS ID is be used for the purpose of D2R scheduling and R2D reception, it seems that the identifier transmitted in NGAP, should be visible to at least the A-IoT MAC layer in gNB-reader, so that the gNB-reader can map it to the corresponding AS ID or vice versa. In RAN3, we have noticed that RAN NGAP Device ID has been agreed to identify one A-IoT device, so the AS ID can be associated to the RAN NGAP Device ID.

Based on above analysis, we think that, from the perspective of specification impact, there is no necessity for the paging identifier to be visible to the A-IoT MAC layer.

Observation 1: Sub-grouping performed by the reader is an optimization scheme for the A-IoT procedure and is totally transparent to A-IoT device.

Observation 2: In RAN3, RAN NGAP Device ID has been agreed to identify one A-IoT device.

Proposal 4: From the perspective of specification impact, there is no necessity for the paging identifier to be visible to the A-IoT MAC layer.

#### 2.1.2.2 Length indication of paging identifier

In RAN2#129bis meeting, the following agreements have been made [3]:

A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small.

RAN2 sent LS to CT4 and SA2 in R2-2503197 asking for their feedback on the above agreement, for RAN2 to determine the field for paging ID length. CT4 replied with LS and CR in C4-252466 (LS on paging ID). SA2 reply LS is in S2-2505793. According to C4-252466, we have found that when the paging ID serves as filter information, its length is the most flexible, ranging from 8 to 256 bits, as illustrated in Figure 1. Therefore, a 8-bit filed is needed to indicate the length of paging identifier.

Figure 1: Structure of Filtering Information [4]

Proposal 5: A 8-bit filed is needed in A-IoT Paging message to indicate the length of paging identifier.

### 2.1.3 Security parameter

In SA3#122 meeting, LS S3-252392 is approved and sent to RAN2 indicating “SA3 is assuming the ability to include an additional parameter, a 128bit random number in the paging request message. There may be other security parameters required. Therefore, SA3 would like to request RAN2 to confirm the feasibility of including additional security parameters and their maximum size”.

From RAN2 perspective, we think it is feasible to include 128-bit random number and additional security parameters (if needed) in A-IoT Paging message and reply to SA3. Besides, we think that we can also include a supplementary note in our reply to SA3 that "RAN2 expects that the length of the additional security parameters to be included is as short as possible to avoid potential negative impacts, such as segmentation of A-IoT Paging message."

Proposal 6: RAN2 confirms it is feasible to include 128-bit random number and additional security parameters (if needed) in A-IoT Paging message and reply to SA3.

Proposal 7: Reply to SA3 that "RAN2 expects that the length of the additional security parameters to be included is as short as possible to avoid potential negative impacts, such as segmentation of A-IoT Paging message."

### 2.1.4 R2D TBS

According to RAN1 LS on Ambient IoT R2D control information, the following agreement is made by RAN1 for R2D reception [5]:

Agreement R2D postamble is specified with 4 ON chips corresponding to M value of the PRDCH - R2D postamble is added immediately after the PRDCH - R2D postamble has always 4 ON chips - Note: For M=24, 2 ON chips at the end of OFDM symbol for CP handling are in addition to R2D postamble, and are not part of the R2D postamble - R2D padding duration is determined after R2D postamble insertion TBS information for R2D is supported via higher layer R2D control signalling. - Send LS to RAN2 asking to include R2D TBS information (excluding CRC length) in higher layer signaling, at least for messages with variable size. Note: Exact method for determining the end of PRDCH at the device is not specified.

As for the A-IoT Paging message, it is clear that it is with variable size because the length of the “Paging ID” field is variable, and some fields are absent in the A-IoT Paging message that triggers CFA. Therefore, one “R2D TBS” field should be contained in the A-IoT Paging message. As for the length of “R2D TBS” field, it relates to the possible length of R2D messages. However, we do not think it is necessary to analyze all possible lengths of R2D messages just to try to reduce the length of “R2D TBS” field somewhat. Therefore, we think a 7-bit “R2D TBS” field, which indicates the value range from 0 to 127 bytes, is suitable.

Observation 3: The length of “R2D TBS” field, it relates to the possible length of R2D messages.

Proposal 8: A 7-bit “R2D TBS” field, which indicates the value range from 0 to 127 bytes, is contained in the A-IoT Paging message.

### 2.1.5 Forward compatibility

In RAN2#130 meeting, it has been agreed that [1]:

3.To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field. FFS if more than one R bit is required. 4.Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message.

The remaining issue is that whether one reserved bit is enough to ensure the forward compatibility of the A-IoT Paging message. From our point of view, one bit is enough to support more than one paging identifiers to be contained in one A-IoT Paging message by indicating there are more Paging ID(s) within the message.

In addition, we think that this method of supporting forward compatibility by reserving one bit can also be applied to other R2D messages which require consideration of forward compatibility, if any.

Proposal 9: 1-bit R field is contained in A-IoT Paging message for forward compatibility.

Proposal 10: The method of supporting forward compatibility by reserving one bit can also be applied to other R2D messages for forward compatibility, if any.

### 2.1.6 Padding bit(s)

Considering that the indication of R2D TB size is in bytes, and the length of the “Paging ID” field (especially for filter information) in the A-IoT Paging message is highly flexible, we think that padding bit(s) may need to be added in some cases. To reduce the additional overhead, i.e., to avoid indicating the length of padding bit(s), we suggest to put the padding bit(s) at the end of the A-IoT Paging message, so that the A-IoT device can parse the A-IoT Paging message field by field starting from the beginning.

Observation 4: Considering that the indication of R2D TB size is in bytes, and the length of the “Paging ID” field (especially for filter information) in the A-IoT Paging message is highly flexible, padding bit(s) may need to be added in some cases.

Proposal 11: RAN2 is asked to confirm that padding bit(s) may be contained at the end of the A-IoT Paging message, since the length of Paging ID is variable.

## 2.2 Subsequent A-IoT paging

In RAN2, it has been discussed and agreed that one or more subsequent A-IoT Paging messages can be sent by gNB-reader to trigger the A-IoT device(s) which fails to complete the procedure initiated by initial A-IoT Paging message. In fact, with the consideration of the unavailability of A-IoT devices caused by their insufficient energy or fading in PRDCH, the subsequent A-IoT paging message sent by gNB-reader may be treated as initial A-IoT paging message by some A-IoT devices. To this end, we think that there should be no difference between the subsequent A-IoT paging message and initial A-IoT paging message for at least the fields other than the “Number of Access Occasions” and “D2R Scheduling Info” field. As for the “Number of Access Occasions” and/or “D2R Scheduling Info” field, we think that some of the parameters within it can differ between the initial A-IoT Paging message and subsequent A-IoT Paging message(s), so as to support flexible adjustment by the gNB-reader based on resource conditions and/or the estimated number of A-IoT devices to be inventoried.

Observation 5: With the consideration of the unavailability of A-IoT devices caused by their insufficient energy or fading in PRDCH, the subsequent A-IoT Paging message sent by gNB-reader may be treated as initial A-IoT Paging message by some A-IoT devices.

Proposal 12: In the subsequent A-IoT Paging message, “Number of Access Occasions” and/or “D2R Scheduling Info” field can differ from that in the initial A-IoT Paging message, while other fields keep the same.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on A-IoT paging as following:

Observation 1: Sub-grouping performed by the reader is an optimization scheme for the A-IoT procedure and is totally transparent to A-IoT device.

Observation 2: In RAN3, RAN NGAP Device ID has been agreed to identify one A-IoT device.

Observation 3: The length of “R2D TBS” field, it relates to the possible length of R2D messages.

Observation 4: Considering that the indication of R2D TB size is in bytes, and the length of the “Paging ID” field (especially for filter information) in the A-IoT Paging message is highly flexible, padding bit(s) may need to be added in some cases.

Observation 5: With the consideration of the unavailability of A-IoT devices caused by their insufficient energy or fading in PRDCH, the subsequent A-IoT Paging message sent by gNB-reader may be treated as initial A-IoT Paging message by some A-IoT devices.

Proposal 1: The way to generate “transaction ID” can be left to A-IoT reader’s implementation.

Proposal 2: RAN2 is asked to agree to a 4-bit transaction ID.

Proposal 3: RAN2 confirm that Transaction ID field is always absent in A-IoT Paging message which triggers CFA.

Proposal 4: From the perspective of specification impact, there is no necessity for the paging identifier to be visible to the A-IoT MAC layer.

Proposal 5: A 8-bit filed is needed in A-IoT Paging message to indicate the length of paging identifier.

Proposal 6: RAN2 confirms it is feasible to include 128-bit random number and additional security parameters (if needed) in A-IoT Paging message and reply to SA3.

Proposal 7: Reply to SA3 that "RAN2 expects that the length of the additional security parameters to be included is as short as possible to avoid potential negative impacts, such as segmentation of A-IoT Paging message."

Proposal 8: A 7-bit “R2D TBS” field, which indicates the value range from 0 to 127 bytes, is contained in the A-IoT Paging message.

Proposal 9: 1-bit R field is contained in A-IoT Paging message for forward compatibility.

Proposal 10: The method of supporting forward compatibility by reserving one bit can also be applied to other R2D messages for forward compatibility, if any.

Proposal 11: RAN2 is asked to confirm that padding bit(s) may be contained at the end of the A-IoT Paging message, since the length of Paging ID is variable.

Proposal 12: In the subsequent A-IoT Paging message, “Number of Access Occasions” and/or “D2R Scheduling Info” field can differ from that in the initial A-IoT Paging message, while other fields keep the same.

Reference

- “RAN2 chair’s notes”, RAN2#130, Malta, RAN2 Chair (InterDigital).

- “RAN2 chair’s notes”, RAN2#129, Greece, Athens, RAN2 Chair (InterDigital).

- “RAN2 chair’s notes”, RAN2#129bis, Wuhan, China, RAN2 Chair (InterDigital)..

- “Definition of AIoT Device Permanent Identifier”, TS 23.003 CR.

- “LS on Ambient IoT R2D control information”, RAN2#130, Malta, RAN WG1.