3GPP TSG-WG2 Meeting #131                                        R2-2505953

Bangalore, India  Aug 25th – 29th , 2025

Agenda item:	8.8.2

Title:	Discussion on DL coverage enhancements due to extended SSB periodicity

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In previous RAN2 130 meetings, the following statements on DL coverage enhancements have been agreed with some remaining issues:

- Agreements:

- 1.	the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue)

- 2.	To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

- 3.	We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

- 4.	No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

- Working Assumption:

- 1.	We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options:

- Alt 1: UE provide the closest N reference locations/neighbor cells to network.

- Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

- We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

In this contribution, we will discuss the following open issues for SMTC enhancements and further provide our views on Link level coverage enhancements:

- Open issue 1: Configuration of 6 SMTC offsets (and periodicities)

- Open issue 2: Clarification on the size of the lists

- Open issue 3: Multiple periodicities for inter-frequency neighbour cells

- Open issue 4: SMTC enhancements in connected mode

- Discussion

2.1 SMTC enhancements

2.1.1 Open issue 1: Configuration of 6 SMTC offsets (and periodicities)

Figure 1 content information of SIBs

According to the spec [4], the content information of SIBs is as follows:

–	ServingCellConfigCommonSIB in SIB1

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMONSIB-START

ServingCellConfigCommonSIB ::=      SEQUENCE {

downlinkConfigCommon                DownlinkConfigCommonSIB,

uplinkConfigCommon                  UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

supplementaryUplink                 UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                           OPTIONAL, -- Need S

ssb-PositionsInBurst                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

}

–	SIB19

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

/*Omitting unrelated*/

[[

movingReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL,       -- Need R

ntnCovEnh-r18                            NTN-CovEnh-r18                                  OPTIONAL,       -- Need R

satSwitchWithReSync-r18                  SatSwitchWithReSync-r18                         OPTIONAL        -- Need R

]]

}

}

...,

Observation 1: The SSB configuration of the serving cell is contained in SIB1, and the reference location information of the serving cell is contained in SIB19. UE can obtain the above serving cell measurement configuration through SIB messages.

The conclusion in [7] is as follows:

[Proposals for easy agreement]

SMTC5 list can be configured independently. Delta signalling with SMTC4 list can be supported to avoid unnecessary overhead (5/5)

The maximum number of 6 configured SMTCs include the SMTC of the serving cell (5/5)

Existing mechanisms are sufficient to associated the ISA with relevant neighbour frequencies to perform MBS frequency prioritization (3/4).

There are maximum 6 intra-frequency neighbour cells and the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue) based on the RAN2#130 agreement. In case that there are 6 intra-frequency neighbour cells, if the first SMTC corresponds to the serving cell, the left 5 SMTCs are not enough for 6 intra-frequency neighbour cells.

Observation 2: In case that there are 6 intra-frequency neighbour cells, if the first SMTC for idle/inactive UE is only used for serving cell, the left 5 SMTCs are not enough for 6 intra-frequency neighbour cells.

According to the spec [4], the definition of IE smtc in SIB2 (the first SMTC) is as follows:

smtc

Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequnecy cells. If the field is broadcast by an NTN cell, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell  and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference.

There is no limits in the definition of IE smtc that the first (Rel-15) SMTC corresponds always to the serving cell. From our perspective, it can be configured to a specific neighbour cell for performing intra-frequency measurement on the neighboring cell.

Proposal 1: RAN2 clarify whether IE smtc in SIB2 can be configured for neighboring cell measurement.

Based on the RAN2#129bis agreement, legacy UE will not be barred from DL CE cell, so NW need to configure SMTC for both legacy UE and R19 DL CE capable UE. From the perspective of minimizing the impact on legacy UE and providing more flexibility, it is preferable to introduce new SMTC information element (SMTC5) with backwards compatible manner. In addition, it is agreed to introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications) in RAN2#130 meeting.

Proposal 2: RAN2 introduce new SMTC information element (smtc5list-r19) in SIB2 for R19 DL CE capable UE. The legacy UEs use smtc4list-r17 and the R19 DL CE capable UEs use smtc5list-r19 ignoring smtc4list-r17.

Proposal 3: New SMTC information element(smtc5list-r19) is introduced to include PCI list, periodicity and offsetList. And referenceLocationList-r19 will be added in SIB2 for location-based SMTC selection. The text proposals are as follows:

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

/*Omitting unrelated*/

...,

[[

t-ReselectionNR-SF                  SpeedStateScaleFactors                          OPTIONAL        -- Need R

]],

[[

smtc2-LP-r16                        SSB-MTC2-LP-r16                                 OPTIONAL,        -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                    OPTIONAL         -- Cond SharedSpectrum

]],

[[

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                    OPTIONAL         -- Cond SharedSpectrum2

]],

[[

smtc4list-r17                       SSB-MTC4List-r17                                OPTIONAL         -- Need R

]],

[[smtc5list-r19                       SSB-MTC5List-r19                                OPTIONAL         -- Need R

]],

[[

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760              OPTIONAL,       -- Need R

frequencyBandListSUL-v1760          MultiFrequencyBandListNR-SIB-v1760              OPTIONAL        -- Need R

]],

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18         OPTIONAL        -- Need S

]],

[[

referenceLocationList-r19           ReferenceLocationList-r19                       OPTIONAL         -- Need R

]]

},

...,

ReferenceLocationList-r19 ::= SEQUENCE (SIZE(1..56)) OF ReferenceLocation      OPTIONAL        -- Need R

-- TAG-SIB2-STOP

-- ASN1STOP

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

...

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

SSB-MTC5List-r19 ::=   SEQUENCE (SIZE (1..5)) OF SSB-MTC5-r19

SSB-MTC5List-r19 ::= SEQUENCE {

pci-List-r19               SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId      OPTIONAL,  -- Need M,

periodicity                ENUMERATED { sf5, sf10, sf20, sf40, sf80, sf160},

offsetList                 SEQUENCE (SIZE (1..maxNrofOffset)) OF INTEGER (0..159)

}

2.1.2 Open issue 2: Clarification on the size of the lists

The size of smtc5list-r19

According to the spec [4], the existing SMTC configuration is as follows:

–	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                     INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

...

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

Observation 3: The IE smtc1 in IE MeasObjectNR can configure at most one periodicity and one Offset, other SMTCs can reuse the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration.

Based on the RAN2#130 agreement, the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue). So the maximum sequence size of smtc5list-r19 is 5 for supporting 6 SMTCs per frequency for idle/inactive UEs.

Proposal 4: The size of smtc5list-r19 is 5 for supporting 6 SMTCs per frequency for idle/inactive UEs.

The size of ReferenceLocationList-r19

According to the spec[4], the IE ReferenceLocation is as follows:

–	ReferenceLocation

The IE ReferenceLocation contains location information used as a reference location. The value of the field is same as Ellipsoid-Point defined in TS37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

ReferenceLocation information element

-- ASN1START

-- TAG-REFERENCELOCATION-START

ReferenceLocation-r17 ::= OCTET STRING

-- TAG-REFERENCELOCATION-STOP

-- ASN1STOP

According to the spec [5], the IE Ellipsoid-Point is as follows:

–	Ellipsoid-Point

The IE Ellipsoid-Point is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

Observation 4: The IE ReferenceLocation in current specification is proper for indicating the reference location of neighbour cells.

Proposal 5: The IE ReferenceLocation in current specification may be reused in ReferenceLocationList-r19.

Figure 2 different intra-frequency neighbour cell with different SMTC

Since the SSBs of surrounding neighbour cells may be transmitted at different times, this situation maybe exist, where UE uses different SMTC to measure different neighboring cell, as shown in Figure 2.

For location-based SMTC selection, the idle/inactive UE selects up to 4 SMTCs from 6 SMTCs based on the distances from UE’s location to each reference location of intra-frequency neighbour cells, and perform RRM measurements on each neighbour cell which corresponds to one of the selected SMTCs. In order to filter SMTC based on reference location, each SMTC needs to correspond one-to-one with one reference location.

Observation 5: To choose up to 4 SMTCs from 6 SMTCs, association between each SMTC and reference location in ReferenceLocationList-r19 need to be created.

Proposal 6: The size of ReferenceLocationList-r19 is 6 for supporting 6 neighboring cells. And the number of ReferenceLocation in ReferenceLocationList-r19 may be the same as the number of SMTCs configured for intra-frequency neighbours cells.

Proposal 7: RAN2 clarify whether the Rel-15 SMTC parameter (the first SMTC) is subject to location-based SMTC selection.

2.1.3 Open issue 3: Multiple periodicities for inter-frequency neighbour cells

Figure 3 scattered active inter-frequency neighbour cells

According to the spec[4], the SMTC information in SIB4 is as follows:

SIB4 ::=                            SEQUENCE {

interFreqCarrierFreqList            InterFreqCarrierFreqList,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

interFreqCarrierFreqList-v1610      InterFreqCarrierFreqList-v1610              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1700      InterFreqCarrierFreqList-v1700              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1720      InterFreqCarrierFreqList-v1720              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1730      InterFreqCarrierFreqList-v1730              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1760      InterFreqCarrierFreqList-v1760              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1800      InterFreqCarrierFreqList-v1800              OPTIONAL   -- Need R

]]

}

InterFreqCarrierFreqList ::=        SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

...,

InterFreqCarrierFreqList-v1720 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1720

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                                     OPTIONAL,   -- Need S

...,

InterFreqCarrierFreqInfo-v1720 ::=  SEQUENCE {

smtc4list-r17                       SSB-MTC4List-r17                                            OPTIONAL     -- Need R

}

smtc Measurement timing configuration for inter-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms in this frequency. If the field is broadcast by an NTN cell, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference.

smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

Observation 6: In case that inter-frequency neighbour cells are activated in a scattered manner, the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough. And SMTCs for inter-frequency measurement has the same IE structure as the ones for intra-frequency measurement.

Proposal 8: Multiple SMTC periodicies are needed in frequency layers different from the serving cell frequency (e.g., inter-frequency neighbours). And SMTC for inter-frequency neighbour cells should be configured the same as for intra-frequency neighbour cells, and the number of SMTC and assistance information should also be expanded. When the frequency priority of different inter-frequency neighbour cells is the same, UE can select SMTCs based on the assistance information.

2.1.4 Open issue 4: SMTC enhancements in connected mode

The working assumption of RAN2#130 meeting is as follows: We introduce a mechanism to assist the NW to configure the SMTCs in connected mode:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

For Alt 1, UE can determine the closest N reference locations/neighbor cells based on newly-introduced ReferenceLocationList-r19 in SIB2. Similar with location-based SMTC selection for the idle/inactive UE, NW can selects up to 4 SMTCs for UE based on the reported information, and UE performs RRM measurements on each neighbour cell using the distributed SMTCs correspondingly.

For Alt 2, there are several things that need to be clarified:

- (1) Where does UE select SMTC from? from SMTCs in SIB2 or from MO configuration in RRC signaling?

(2) Does UE report the selected SMTCs to NW before performing measurements or after performing measurements?

(3) What steps does NW execute after receiving the selected SMTC?

Observation 7: From the comparison above, Alt 1 (neighbour cell reference location based SMTC selection) is feasible and easier for UE than Alt 2.

Proposal 9: RAN2 adopt Alt 1 as baseline.

2.2 Link level coverage enhancements

As agreements of RAN2#129bis meetings, RAN2 will wait for RAN1 conclusion on link level enhancements.

- Agreements:

- We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

In RAN1#120bis meeting, link level enhancement for NR-NTN downlink coverage enhancements  had been discussed,  the following agreements have been achieved:

Agreement: When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination. Agreement: For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition. Agreement: For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS. Agreement: For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots: - The same resource allocation is assumed for all repetitions - The supported repetition factors are: 2 and 4 - The network configures a single value between 2 and 4 at a given time - The RV cycling is used for each repetition - If N=4: Starting RV; RV applicable to the nth repetition/transmission. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3.

In previous RAN1#120 meetings, related agreements are as follow:

Agreement: Support only inter-slot repetition for Type0 PDCCH CSS. Agreement: For Msg4 PDSCH repetition support, RAN1 to consider: - Option 1: UE specific repetition indication via DCI - Option 2: Msg4 repetition is configured by SIB1 - Option 3: Msg4 PDSCH repetition is implicitly determined by SIB1 PDSCH repetition Agreement: At least for enabling PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, RAN1 to consider the following options - Option 1: Using the spare 1 bit in MIB - Option 2: Using reserved bit(s) in PBCH payload - Option 3: Using codepoint(s) in PBCH payload - Option 4: UE blind decoding without signaling from the network during initial access

The repetition scenarios discussed in RAN1 are focused on the following three aspects: PDCCH CSS type-0 repetition, SIB1 PDSCH repetition and Msg4 PDSCH repetition, respectively using cell level or UE level resources.

For above repetition scenarios, repetition trigger condition (e.g. threshold for signal quality) and Specific thresholds may be discussed by RAN1 since the values of the threshold need to be simulated and evaluated by RAN1.

Observation 8: For above repetition scenarios, repetition trigger condition (e.g. threshold for signal quality) and  Specific thresholds may be discussed by RAN1 since the values of the threshold need to be simulated and evaluated by RAN1.

- For Msg4 PDSCH repetition

For Msg4 PDSCH repetition, the network should know whether UE is capable of Msg4 PDSCH repetition before performing repetition. Similar to PUCCH enhancements for Msg4 HARQ-ACK in R18, Msg3 can be used to indicate the capability/request for Msg4 PDSCH repetition.

Proposal 10: RAN2 assume Msg3 can be used to indicate the capability/request for Msg4 PDSCH repetition.

To enable Msg3 based capability/request indication, LCID range by 64 need to be extended. According to the spec [6], codepoints 8 to 63 are reserved. These reserved codepoints can be used to indicate UE capability/request for Msg4 PDSCH repetition.

6.2	Formats and parameters

6.2.1	MAC subheader for DL-SCH and UL-SCH

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 to 63 | (216 + 328) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

Observation 9: These reserved codepoints 8 to 63 can be used to indicate UE capability/request for Msg4 PDSCH repetition.

From our perspective, if the downlink transmission power is set higher than that in the simulation results in TR38.830,the Msg4 PDSCH channel may not necessarily be better than PUCCH with HARQ-ACK Msg4 channel. Hence, we proposed as follows:

- Proposal 11:  RAN2 discuss two options for supporting Msg4 PDSCH repetition:

- Option1: Adding the following codepoints in Msg3 to indicate UE capability/request for Msg4 PDSCH repetition:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE

- Option 2: Adding the  following codepoints in Msg3 to indicate UE capability/request for Msg4 PDSCH repetition and sending two MAC CE simultaneously when Msg4 HARQ-ACK & Msg4 PDSCH repetition are supported:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- Conclusion

Observation 1: The SSB configuration of the serving cell is contained in SIB1, and the reference location information of the serving cell is contained in SIB19. UE can obtain the above serving cell measurement configuration through SIB messages.

Observation 2: In case that there are 6 intra-frequency neighbour cells, if the first SMTC for idle/inactive UE is only used for serving cell, the left 5 SMTCs are not enough for 6 intra-frequency neighbour cells.

Proposal 1: RAN2 clarify whether IE smtc in SIB2 can be configured for neighboring cell measurement.

Proposal 2: RAN2 introduce new SMTC information element (smtc5list-r19) in SIB2 for R19 DL CE capable UE. The legacy UEs use smtc4list-r17 and the R19 DL CE capable UEs use smtc5list-r19 ignoring smtc4list-r17.

Proposal 3: New SMTC information element(smtc5list-r19) is introduced to include PCI list, periodicity and offsetList. And referenceLocationList-r19 will be added in SIB2 for location-based SMTC selection. The text proposals are as above.

Observation 3: The IE smtc1 in IE MeasObjectNR can configure at most one periodicity and one Offset, other SMTCs can reuse the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration.

Proposal 4: The size of smtc5list-r19 is 5 for supporting 6 SMTCs per frequency for idle/inactive UEs.

Observation 4: The IE ReferenceLocation in current specification is proper for indicating the reference location of neighbour cells.

Proposal 5: The IE ReferenceLocation in current specification may be reused in ReferenceLocationList-r19.

Observation 5: To choose up to 4 SMTCs from 6 SMTCs, association between each SMTC and reference location in ReferenceLocationList-r19 need to be created.

Proposal 6: The size of ReferenceLocationList-r19 is 6 for supporting 6 neighboring cells. And the number of ReferenceLocation in ReferenceLocationList-r19 may be the same as the number of SMTCs configured for intra-frequency neighbours cells.

Proposal 7: RAN2 clarify whether the Rel-15 SMTC parameter (the first SMTC) is subject to location-based SMTC selection.

Observation 6: In case that inter-frequency neighbour cells are activated in a scattered manner, the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough. And SMTCs for inter-frequency measurement has the same IE structure as the ones for intra-frequency measurement.

Proposal 8: Multiple SMTC periodicies are needed in frequency layers different from the serving cell frequency (e.g., inter-frequency neighbours). And SMTC for inter-frequency neighbour cells should be configured the same as for intra-frequency neighbour cells, and the number of SMTC and assistance information should also be expanded. When the frequency priority of different inter-frequency neighbour cells is the same, UE can select SMTCs based on the assistance information.

Observation 7: From the comparison above, Alt 1 (neighbour cell reference location based SMTC selection) is feasible and easier for UE than Alt 2.

Proposal 9: RAN2 adopt Alt 1 as baseline.

Observation 8: For above repetition scenarios, repetition trigger condition (e.g. threshold for signal quality) and  Specific thresholds may be discussed by RAN1 since the values of the threshold need to be simulated and evaluated by RAN1.

Proposal 10: RAN2 assume Msg3 can be used to indicate the capability/request for Msg4 PDSCH repetition.

Observation 9: These reserved codepoints 8 to 63 can be used to indicate UE capability/request for Msg4 PDSCH repetition.

- Proposal 11:  RAN2 discuss two options for supporting Msg4 PDSCH repetition:

- Option1: Adding the following codepoints in Msg3 to indicate UE capability/request for Msg4 PDSCH repetition:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE

- Option 2: Adding the  following codepoints in Msg3 to indicate UE capability/request for Msg4 PDSCH repetition and sending two MAC CE simultaneously when Msg4 HARQ-ACK & Msg4 PDSCH repetition are supported:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- References

- Chairman's Notes RAN2#130

- Chairman's Notes RAN2#129bis

- Chairman's Notes RAN2#129

- TS 38.331, Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification (Release 18)

- TS 37.355, Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP) (Release 18)

- TS 38.321, Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification (Release 18)

- Remaining RRC open issues for NR NTN Rel-19, Ericsson