3GPP TSG-RAN2 Meeting #131			R2-2505970

Bangalore, India, Aug. 25th-29th, 2025

Agenda item:	8.7.5

Title:	Remaining open issues on scheduling enhancement for XR

Source:	CMCC

Document for:	Discussion

Introduction

In the last RAN2 meeting [1], RAN2 has reached following agreements on LCP and DSR.

Agreements for scheduling enhancements - [MAC-01] The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization. - (MAC-02) When PSI-based discarding is activated for a DRB, priority adjustment is applied only if there is at least one high importance packet in the buffer has a remaining time smaller than the threshold. (MAC spec is already aligned with this agreement). - (MAC-13) When pdu-SetDiscard is configured, PDU sets should be treated as a whole in the LCP procedure with adjusted priority. - Confirm the working assumption “No Bj enhancement is introduced” as an agreement. Agreements for DSR enhancements - (PDCP-1, RLC-5) Both PDCP and RLC consider Control PDU and/or retransmitted data into the shortest configured reporting threshold. - (PDCP-1, RLC-5) The value of the remaining time field in the enhanced DSR MAC CE is set to 0, if there are only control PDUs and/or retransmitted data to be reported for the shortest configured reporting threshold of the LCG. - (PDCP-1, RLC-5) An understanding is that there will be no DSR with no data indication (i.e. indicating only volume of C-PDU and/or retransmissions for any LCG) - (RRC-7) If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold. - New LCID is used for R19 DSR - (MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR. - (MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU. - (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

This contribution focuses on the remaining issues of scheduling enhancement, and propose several solutions for them.

Discussion

  - Definition of delay-critical PDCP SDU

In the post-meeting email discussion, some company argues that:

The definition of a PDCP SDU associated with a DSR may be revised. Currently, the definition relies on the concept of delay-critical PDCP SDU to determine whether a pending DSR should be cancelled (i.e., when no delay-critical PDCP SDU remains). However, the cancellation condition for Multiple Entry DSR may also be based on the absence of delay-reporting PDCP SDU (i.e., no delay-reporting PDCP SDU remains or all the related information is reported). A specific scenario illustrates the issue: Suppose the largest configured threshold in dsr-ReportingThresList is greater than remainingTimeThreshold. In such a case, there may be data whose remaining time falls into the range above remainingTimeThreshold but below the largest configured reporting threshold. If the current definition (which is based on delay-critical PDCP SDU) is applied—and there is no data with a remaining time below remainingTimeThreshold—the Multiple Entry DSR will be cancelled, even though relevant delay-reporting PDCP SDU still exists.

With regarding to the intention of introducing DSR in RAN, the delay-critical PDCP SDU should refer to a PDCP SDU whose remaining time to be discarded is no larger than a threshold configured for the logical channel. It also means that any PDCP SDU who remaining time is no shorter than the threshold is not-delay critical and therefore should not be report solely in the DSR. Therefore, the definition of delay-critical and delay-reporting PDCP SDU is not changed and it’s reasonable not to report delay-reporting PDCP SDU without delay-critical PDCP SDU.

Proposal 1a: There is no impact on the definition of delay-critical and delay-reporting PDCP SDU.

Besides, it is acceptable to cancel the DSR for the LCG when the remaining time of the buffered data is larger than the DSR trigger threshold but still shorter than the largest reporting threshold, especially in Multi Entry DSR.

The multiple reporting thresholds are intended as a supplementary mechanism for scheduling optimization; therefore, when an LCH does not contain any delay-critical PDCP SDU, its scheduling priority remains unchanged. In such cases, reporting delay information for non-delay-critical PDCP SDUs provides limited benefit and may result in unnecessary signalling overhead.

Proposal 1b: RAN2 confirms that if there is no delay-critical PDCP SDU for an LCG, any delay-reporting data should not be reported in DSR as well.

  - BSR cancellation caused by DSR

In the post-meeting email discussion, some company argues that:

The newly introduced DSR MAC CE is capable of reporting delay information with finer granularity for an LCG, where the delay reporting data with remaining time above the triggering threshold can be included in the DSR MAC CE. Since DSR contains not just the data volume but also the smallest remaining time, the triggered BSR should be cancelled as the DSR provides more information. The changes can be done as follow: ### 5.4.5 Buffer Status Reporting …… Omitted…… All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long, Refined Long, Extended Long, Short, or Extended Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly. All triggered BSRs may be cancelled if amount of data to be reported in the BSR is indicated by the DSR included in a MAC PDU.

From our perspective, the BSR and DSR related procedures should remain decoupled. In the case of the existence of pending BSR, cancelling the pending BSR due to a triggered DSR may in fact lead to additional UE-side operations, e.g., removing the BSR that has already been assembled in the MAC PDU.

Existing mechanisms for BSR handling are already very mature, and simultaneous reporting of BSR and DSR does not introduce functional issues. DSR only affects the priority in LCP, but BSR is still responsible of UE buffer management in gNB.

If the BSR is cancelled merely because the DSR has reported the whole data volume, this could increase the RAN’s reliance on DSR while offering only marginal benefits in terms of overhead reduction. Furthermore, such scenarios are expected to be infrequent, and introducing a complex optimization solely for these rare cases is not justified.

Proposal 2: DSR shall not trigger any BSR cancellation.

  - RLC visibility on delay-reporting data

In the RLC CR review, some company argues that:

Whether the delay-reporting or non-delay-reporting information can be invisible to RLC? As we commented in the RLC running CR, it is benifical to not distinguish delay-reporting RLC SDU and non-delay-reporting RLC SDU in RLC spec. RLC can rely on current delay reporting indication from PDCP to evaluate RLC data volume. For example, if dsr-ReportNonDelayCriticalData is configured, PDCP indicates both delay-reporting PDCP SDUs and non-delay reporting PDCP PDUs as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold. Otherwise, PDCP only indicates the delay-reporting PDCP SDUs as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold. We see that the delay-reporting or non-delay-reporting information can be invisible to RLC, and all the description related to non-delay reporting RLC SDU can be removed. [Rapp] A new open issue is added, i.e. RLC-13, to check companies’ views.

From a functional layering perspective, RLC is responsible for segmentation/reassembly and ARQ, but does not directly utilize delay-reporting or non-delay-reporting information for transmission prioritization.

The distinction can be handled entirely in PDCP, which already determines the data volume for delay reporting, e.g., via dsr-ReportNonDelayCriticalData configuration.

If RLC only receives the aggregated data volume corresponding to the configured i-th dsr-ReportingThreshold, regardless of SDU type, the RLC specification can be simplified by removing descriptions related to non-delay-reporting RLC SDUs. This avoids unnecessary cross-layer dependency, while PDCP retains full flexibility to decide the scope of delay reporting.

Proposal 3: (RLC-13) Delay-reporting and non-delay-reporting data can be invisible to RLC entity.

  - PDCP SN GAP report

There is a new identified open issue for PDCP:

PDCP-2 Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layers.

We acknowledge the open issue raised regarding R19 AM RLC stopReTxDiscardedSDU, where an PDCP SDU may be partially submitted (in form of SDU segments) to lower layers before becoming obsolete due to a PDCP discard indication. In such cases, the remaining segments will never be delivered, resulting in the complete SDU being discarded at the receiving side after t-RxDiscard expiry.

With the current PDCP SN gap report triggering conditions, such cases are not covered, since the “discarded PDCP SDU(s)” in the third condition refers to fully discarded SDUs, and no report is triggered. This may lead to unnecessary reordering delays at the receiving PDCP entity.

Considering the time constraint, instead of modifying the PDCP SN gap report triggering conditions, we propose that when the transmitting RLC entity receives a discard indication from PDCP and detects that at least one segment of the affected SDU has already been submitted to lower layers, it should indicate PDCP that there may be an SN gap. This can be done via an internal indication between RLC and PDCP by implantation to avoid specification impact

Upon receiving such an internal indication, PDCP can trigger an SN gap report immediately, thereby allowing the peer PDCP entity to avoid unnecessary reordering delays

Proposal 4: (PDCP-2) RLC entity can indicate PDCP entity whether any segment of a PDCP SDU is submitted to lower layer after receiving discard indication from PDCP to trigger an SN gap report, e.g., by implementation.

  - UE capability on R18 and R19 DSR

A UE supporting Rel-19 DSR capability is also expected to support Rel-18 DSR, since Rel-19 DSR is kind of an enhancement of Rel-18 DSR.

The Rel-18 DSR can be regarded as a Rel-19 DSR configuration without any reporting thresholds, and thus does not introduce additional complexity at the UE side. Consequently, supporting the legacy Rel-18 DSR is not considered challenging for UEs that already implement the Rel-19 DSR functionality.

Proposal 5: UE support R19 DSR shall also support R18 DSR reporting.

Conclusion

Based on the above, we propose that:

Delay-critical and delay-reporting data issues:

Proposal 1a: There is no impact on the definition of delay-critical and delay-reporting PDCP SDU.

Proposal 1b: RAN2 confirms that if there is no delay-critical PDCP SDU for an LCG, any delay-reporting data should not be reported in DSR as well.

DSR issues:

Proposal 2: DSR shall not trigger any BSR cancellation.

Proposal 3: (RLC-13) Delay-reporting and non-delay-reporting data can be invisible to RLC entity.

New SN gap report trigger:

Proposal 4: (PDCP-2) RLC entity can indicate PDCP entity whether any segment of a PDCP SDU is submitted to lower layer after receiving discard indication from PDCP to trigger an SN gap report, e.g., by implementation.

UE capability:

Proposal 5: UE support R19 DSR shall also support R18 DSR reporting.

- References

[1]	R2_130 R19 XR and LTE Broadcast notes (Dawid) 2025-05-23_EOM_rev1