3GPP TSG-RAN WG2 Meeting #131	R2-2505999

Bengaluru, India, August 25th – 29rd, 2025

Source:        ZTE Corporation

Title:           Consideration on the Remaining Asn.1 Issues

Agenda item:   8.12.3

Document for:  Discussion and Decision

- Introduction

According to the post email discussion [Post130][218][MIMO_Ph5] Running CR for 38.331 (Ericsson), there are still some remaining ASN.1issues for the UEIBM/CSI as follows.

| - Topic | - Issue | - Remarks |
| --- | --- | --- |
| UEI BMR | FFS whether UEI BM parameters should be moved to CSI-MeasConfig. | Since now most of the parameters are stable this could be discussed based on company contributions. |
| CSI-CRI | Editor’s note: FFS on how to define additionalOneSlotOffsetDoppler as a list. | This could be handled during a round of updates for 38.331 based on companies input. |
| CSI-CJTC | Parameter referenceAntennaPort is indicated in RAN1 list as follows: up to RAN2 regarding {1,..,x}, where x is x in ‘xTyR’ when SRS usage set to antenna switching. | This could be discussed based on company contributions. |

In this paper we would like to share our views on these remaining issues.

- Discussion

In this chapter, we will discuss the above remaining ASN.1 issues one by one.

2.1 UEIBR

In this sub-chapter, we’d like to discuss whether UEI BM parameters should be moved to CSI-MeasConfig first, then to clarify how to indicate the cell for the indicated TCI state, current beam/new beam, first PUCCH and the PUSCH.

Whether UEIBR parameters should be moved to CSI-MeasConfig

In the current running CR, the UEIBR signalling is under the CSI-ReportConfig as follows:

Fig1: UEIBM Signaling in the CSI-ReportConfig

With this structure, it means the network has to include the mandatory field when configure the UEIBM. In the current CSI-ReportConfig, the following parameters are mandatory present and at least 16 bits would be needed to indicate these fields

Table 1: Bits for the Mandatory Prameters

| Index | Mandatory Parameters | Minimum Bits |
| --- | --- | --- |
| 1 | reportConfigType CHOICE { periodic SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH SEQUENCE { reportSlotConfig ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320}, reportSlotOffsetList SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32), p0alpha P0-PUSCH-AlphaSetId }, aperiodic SEQUENCE { reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32) } }, | To indicate aperiodic (with least bits) 2bits (for choice structure +4 (for the size of list) +5 for one Integer |
| 2 | timeRestrictionForChannelMeasurements ENUMERATED {configured, notConfigured}, timeRestrictionForInterferenceMeasurements ENUMERATED {configured, notConfigured}, | 2 |
| 3 | groupBasedBeamReporting CHOICE { enabled NULL, disabled SEQUENCE { nrofReportedRS ENUMERATED {n1, n2, n3, n4} OPTIONAL -- Need S } }, | 1 |
| 4 | subbandSize ENUMERATED {value1, value2}, | 2 |

Besides, additional 27bits would be also needed to indicate the absence of the optional fields, which means that the gNB has to include unnecessary 43bits for the UEIBM report configuration, which maybe even larger than the UEIBM configuration itself.

Observation 1: The gNB must include at least 43 unnecessary bits (16 bits for mandatory fields and 27 bits for optional fields) in the UEIBM report configuration when UEIBM parameters are included under CSI-ReportConfig.

If we move the UEIBM configuration parameters to the CSI-MeasConfig, the signaling would be quite simple as below, and the network doesn’t need to send the unnecessary fields.

Fig2: UEIBM Signaling in the CSI-MeasConfig

Furthermore, there is no need to further clarify which parameters (especially the mandatory present parameters in the legacy CSI-ReportConfig) are not used for the UEIBM.

Proposal 1: Relocate the UEIBM parameters to the CSI-MeasConfig.

Besides the above issue, another issue is about the cell indication for the cross-cell scenario. The related agreements in RAN1 are copied as follows for reference:

Agreement #1 On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied. Agreement #2 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. Agreement#3 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. Agreement#4 - The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability Agreement #5 On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate one from {SpCell, PUCCH-Scell} as the cell of the configured PUCCH.

For the discussion convenience, we can note the cells for the indicated TCI state, current beam/new beam, first PUCCH and the PUSCH as Cell A/B/C/D respectively:

- Cell A: The cell on which the indicated TCI state used to determine the current beam RS

- Cell B: The cell of the current beam and the new beams

- Cell C: The cell of the PUCCH

- Cell D: The cell of the PUSCH

Based on the above agreements, Cell A/B/C/D can be totally different cells, and according to the current Asn.1 signaling in the running CR, they are signaling as below:

Table 2 Asn.1 Signaling for each Cell

| Cell | Asn.1 Signaling |
| --- | --- |
| Cell A: Indicatd TCI state of the current Cell | CSI-ReportConfig->csi-ReportUE-IBR-r19-> tci-ServCellIndex-r19 |
| Cell B: The cell of the current beam and the new beams | CSI-ReportConfig->Carrier |
| Cell C: The cell of the PUCCH | CSI-ReportConfig->pucch-Resource-r19-> ul-BWP-Id-r19/pucch-Cell-r19 |
| Cell D: The cell of the PUSCH | CSI-ReportConfig->reportTransmissionMode-r19 modeB-r19-> ul-BWP-Id-r19/servCellIndex-r19 |

For the cell A/B/C, the current signaling is OK, but for the cell D, according to the above agreements #2, the Cell D should be the Cell in which the CSI report is configured.

Agreement #2 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured.

Thus the cell D has been implicitly indicated by the corresponding ServingCellConfig of the CSI-ReportConfig (e.g. ServingCellConfig-> CSI-MeasConfig->CSI-ReportConfig).

Proposal 2: Remove the “servCellIndex-r19” from the CSI-ReportConfig->reportTransmissionMode-r19->modeB-r19 in the current draft running CR.

Furthermore, in the current signaling, the eventCountWindow-r19 is an optional filed, but for the UEIBM report, it should be mandatory present, this can be further discussed in the ASN.1 review.

eventCountWindow-r19                     SEQUENCE {

eventInstanceCount-r19			      INTEGER (2..16),

eventDetectionTimeWindow-r19           ENUMERATED {ms4, ms5, ms8, ms10, ms16, ms20, ms40, ms80, ms160, ms320, ms640, ms1280}

}                                                                      												  OPTIONAL,    -- Need R

Proposal 3: RAN2 to confirm whether the eventCountWindow-r19 should be mandatory for the UEIBM.

2.2 CSI-CRI

For the CSI-CRI topic, the remaining issue is on how to define additionalOneSlotOffsetDoppler as a list.

| CSI-CRI | Editor’s note: FFS on how to define additionalOneSlotOffsetDoppler as a list. | This could be handled during a round of updates for 38.331 based on companies input. |
| --- | --- | --- |

The parameter in the corresponding RAN1 table and the running CR is as follows:

| NZP-CSI-RS-ResourceSet | additionalOneSlotOffset Dopp | For Rel-19 Type-II based on Rel-18 Type-II Doppler: 1-slot offset (per NZP-CSI-RS-Resource Group) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet | {enabled} |
| --- | --- | --- | --- |

[[ kdopp-r19 SEQUENCE { numberOfResourceGroups-r19 ENUMERATED {n4, n8, n12}, numberOfResourcesPerGroup-r19 ENUMERATED {n2, n3, n4} } OPTIONAL, -- Need R additionalOneSlotOffsetDoppler-r19 CHOICE { resourceGroup4 BIT STRING(SIZE(4)), resourceGroup8 BIT STRING(SIZE(8)), resourceGroup12 BIT STRING(SIZE(12)) } OPTIONAL -- Need R Editor’s note: FFS on how to define additionalOneSlotOffsetDoppler as a list. ]]

However, RAN1’s intention is not to define this parameter per Group, instead this parameter is for all of the groups. Thus there is no need to define this parameter as a list.

Proposal 4: No need to define additionalOneSlotOffsetDoppler-r19 as a list, instead it should be define as ENUMERATED {enabled} and applied for all groups.

2.3 CSI-CJTC

For the CSI-CJTC topic, the remaining issue is about the parameter referenceAntennaPort:

| CSI-CJTC | Parameter referenceAntennaPort is indicated in RAN1 list as follows: up to RAN2 regarding {1,..,x}, where x is x in ‘xTyR’ when SRS usage set to antenna switching. | This could be discussed based on company contributions. |
| --- | --- | --- |

In RAN1’s parameter list, this parameter is defined as follows:

| CSI-ReportConfig | referenceAntennaPort | The UE antenna port for receiving the CSI-RS configured for phase offset measurement that is same as the UE antenna port(s) for transmitting the configured ports from the associated SRS resource | up to RAN2 regarding {1,..,x}, where x is x in ‘xTyR’ when SRS usage set to antenna switching |
| --- | --- | --- | --- |

Obviously, this parameter is also related to the parameter associatedSRSResourceSet:

| CSI-ReportConfig | associatedSRSResourceSet | This parameter is used to indicate the SRS resource (along with resource set) associated with the CSI-RS resource used for PO measurement. Q=1 SRS resource is supported | FFS |
| --- | --- | --- | --- |

For each SRS resource, the network configure the number of ports as follows:

SRS-Resource ::= SEQUENCE { srs-ResourceId SRS-ResourceId, nrofSRS-Ports ENUMERATED {port1, ports2, ports4}, nrofSRS-Ports-n8-r18 ENUMERATED {ports8, ports8tdm} OPTIONAL, -- Need R /************omit the unrelated part********************/

Upon receiving the SRS, the network can determine which antenna port should be used to receive the CSI-RS for phase offset measurement, thus the “x” should be no larger than the nrofSRS-Ports of the associated SRS-Resource. From the signaling aspect, the maximum value of x should be 4.

Proposal 5: Define referenceAntennaPort as INTEGER (1..8), with the following field description:

referenceAntennaPort The UE antenna port for receiving the CSI-RS configured for phase offset measurement that is same as the UE antenna port(s) for transmitting the configured ports from the associated SRS resource [FFS: associatedSRSResourceSet ], this parameter should be no larger than the nrofSRS-Ports of the [FFS: associatedSRSResourceSet ]

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: The gNB must include at least 43 unnecessary bits (16 bits for mandatory fields and 27 bits for optional fields) in the UEIBM report configuration when UEIBM parameters are included under CSI-ReportConfig.

Proposal 1: Relocate the UEIBM parameters to the CSI-MeasConfig.

Proposal 2: Remove the “servCellIndex-r19” from the CSI-ReportConfig->reportTransmissionMode-r19->modeB-r19 in the current draft running CR.

Proposal 3: RAN2 to confirm whether the eventCountWindow-r19 should be mandatory for the UEIBM.

Proposal 4: No need to define additionalOneSlotOffsetDoppler-r19 as a list, instead it should be define as ENUMERATED {enabled} and applied for all groups.

Proposal 5: Define referenceAntennaPort as INTEGER (1..8), with the following field description:

referenceAntennaPort The UE antenna port for receiving the CSI-RS configured for phase offset measurement that is same as the UE antenna port(s) for transmitting the configured ports from the associated SRS resource [FFS: associatedSRSResourceSet ], this parameter should be no larger than the nrofSRS-Ports of the [FFS: associatedSRSResourceSet ]