3GPP TSG-RAN WG2 Meeting #131	 R2-2506008

Bangalore, India, August 25-29, 2025

Agenda item:		8.5.3	On-demand SIB1

Title:	Discussion on whether to support SUL in OD-SIB1 cell.

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During RAN2 #130, the following agreements were made [1]:

Agreements on OD-SIB1

- Not to include access related information of NES into WUS configuration.

- Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

- TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

- Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

- If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

- Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

During the post #130 email discussion [107], there is one open issue of whether to support SUL, which was not concluded (actually it was not asked to discuss explicitly) [2]. In this contribution, we discuss that remaining issue on SUL in OD-SIB1 cell and provide our view.

# 2. Report of [POST130][107][NES] (Ericsson); Q3: Can the EN be removed or is further discussion needed?

2.1	Support of SUL in OD-SIB1 cell.

During RAN2#130, there was discussion on the issue of whether to support SUL in OD-SIB1 cell. This was marked as revisit [1].

R2-2503391	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Revisit it next meeting.

The uplink coverage is generally weaker than the downlink due to the limited UE transmit power. To address this, Supplementary Uplink (SUL) was introduced in lower frequency bands, improving uplink coverages, especially for cell-edge users. For NES cells supporting on-demand SIB1, there could be need of the SUL support, as the NES cell may be capacity cell on higher frequency band. For example, an operator deploys an FR1 mid-band carrier (e.g., NUL at 3.5 GHz) with an associated SUL carrier in sub-1 GHz (e.g., 700 MHz) to extend UL coverage.

However, if a NES cell supports on-demand SIB1 and the SUL, but configures the on-demand SIB1 request configuration only for NUL, a UE located outside the NUL coverage would be unable to request SIB1 due to poor UL link budget, leading to cell access delay (e.g. waiting for SIB1 broadcast via a request from other UEs or triggered by the network). The UE may consider the cell as barred due to lack of the essential SI in the cell.

To avoid cell access delay for UEs located outside NUL coverage, both NUL and SUL shall be configured in SIB1 when a SUL-capable cell supports on-demand SIB1. This allows the UE to select the appropriate uplink carrier for initiating the random access preamble transmission based on measured uplink signal quality.

# Proposal 1: In SUL-capable cells supporting on-demand SIB1, the gNB can configure the on-demand SIB1 request configuration for both NUL and SUL to enable uplink carrier selection for SIB1 request based on signal quality.

# 3. Conclusion

In this contribution, we have made the following proposal.

# Proposal 1: In SUL-capable cells supporting on-demand SIB1, the gNB can configure the on-demand SIB1 request configuration for both NUL and SUL to enable uplink carrier selection for SIB1 request based on signal quality.

# 4. References

[1] R2-2504671, Report from session on R18 SL, R18/19 MOB and R19 NES, Vice Chairman (Samsung)

[2] R2-250xxxx, Report of [POST130][107][NES] (Ericsson)