3GPP TSG-RAN-WG2 Meeting #131	R2-2506023

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item	:		8.6.3

Source	: 	Sharp

Title	: 	Discussion on issues for supporting L1 event triggered measurement reporting

Document for	:	Discussion and Decision

Introduction

In this paper, we discuss the issue related to L1 event triggered measurement reporting.

Discussion

(MAC-8) Coexistence with mTRP

In the RAN2#129bis meeting and the e-mail discussion ([POST130][117][MOB] Nokia)[1], the issues related to the coexistence between mTRP and event triggered L1 MR are discussed.

Support or Not

Supporting coexistence between mTRP and event triggered L1 MR has benefits (e.g. for coverage, reliability and date rate) and only a few issues need to be discussed, such as how to select/report the current beam. Therefore, RAN2 should support coexistence between mTRP and event triggered measurement reporting for LTM.

Proposal 1. (MAC-8) RAN2 support coexistence between mTRP and event triggered L1 measurement reporting

Current beam selection

When the UE is configured with mTRP, the UE has two serving beams. In this case, there are multiple options[1] to support coexistence between mTPR and event triggered MR.

In our view, the option (a) [UE should use the best beam of the two “current beams” for event evaluation] is the simple and straightforward solution. In addition, for the option (a), when the network receives the measurement report triggered by the event 2/3/5, the network can know both of current beams have an RSRP lower than a threshold.

Proposal 2. (MAC-8) When the UE is configured with mTRP and event triggered L1 MR, the UE should use the best beam of the two “current beams” for event evaluation and reporting.

Association between the serving beam and the reported RSRP

The next issue is whether the indication for which beam/TCI the RSRP value is reported in the MR MAC CE is needed or not. In our understanding, the network can trigger the LTM cell switch and early sync without such an indication. Therefore, we propose that the MR does not need to include a such indication.

Proposal 3. (MAC-8) Indication for which beam/TCI the RSRP value is reported in the MR MAC CE is not needed.

(MAC-24) Truncated MR MAC CE

In the last meeting, RAN2 discussed and made the following assumption related to the minimum size of truncated MR MAC CE.

[MAC-24] It is still working assumption on the details for truncated L1 MR MAC CE

Editor’s note: It is working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

According to this working assumption, the UE cannot send the Truncated MR MAC CE when the UL resource can include only one beam and current beam is configured to be sent. If the UE is configured to report the RSRP for the current beam, the minimum size of the truncated MR MAC CE is 2 beams (i.e. triggered beam and current beam). The network requires information on the current beam for deciding LTM cell switch or early synchronization; therefore, it configures the UE to report the current beam. Accordingly, reporting only the triggered beam information is not beneficial to the network. In accordance with this, even if the UE has only the resource for one beam, the UE should not send the truncated MR MAC CE and the UE send the SR (if configured) or perform the random access.

Proposal 4. (MAC-24) If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

MR MAC CE

In the MAC CE for event triggered L1 MR in the current MAC CR[2], the RSRI fields are separated by the R bit and bits for the RSRI fields are not contiguous. Since this design may be difficult to read, we suggest modifying it by changing the position of the R bits so that the RSRI is represented by contiguous bits.

Proposal 5. Modify the MAC CE as shown in Figure 1 to represent the RSRI with consecutive bits.

Conclusion

Proposal 1. (MAC-8) RAN2 support coexistence between mTRP and event triggered L1 measurement reporting

Proposal 2. (MAC-8) When the UE is configured with mTRP and event triggered L1 MR, the UE should use the best beam of the two “current beams” for event evaluation and reporting.

Proposal 3. (MAC-8) Indication for which beam/TCI the RSRP value is reported in the MR MAC CE is not needed.

Proposal 4. (MAC-24) If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

Proposal 5. Modify the MAC CE as shown in Figure 1 to represent the RSRI with consecutive bits.

Reference

[1] R2-25xxxxx Report from [POST130][117][MOB] (Nokia)

[2] R2-250xxxx_Running MAC CR for Mob Ph4-after RAN2#130