3GPP TSG-RAN WG2 Meeting #131				   		        R2-2506080

Bengaluru, India, 25 - 29 Aug, 2025

Title: 	Discussion on LCM for Positioning use case of UE-sided model

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2.3

Document for:	Discussion/Decision

- Introduction

This Tdoc is to address remaining open issues for Positioning use case of UE-sided model. Some of open issues are indicated from [1].

- Discussion

LPP-6a: Number of PRUs in NR-PRU-DL-Info

- Background:

Issue description:

Currently, the IE NR-PRU-DL-Info provides PRU measurements for a single PRU. However, the issue was raised at RAN2#130 whether more PRUs are needed to be included for training purposes.

If more PRUs are needed, a LMF can always provide multiple Provide Assistance Data messages. However, the Request Assistance Data message may need to include an indication of e.g., number of PRUs requested (if needed).

Proposed resolution:

Companies to provide contributions to the following meeting on whether more PRUs are needed to be included for training purposes, and if so, the corresponding signalling details.

- Solutions:

For the AI/ML positioning case 1, it is UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning. Based on the legacy positing, the UE performs positioning by measuring the downlink positioning reference signal (DL-PRS).  As for case 1, we also think the model input should be the related reference signal, and we do not see the need of using the other PRUs measurements for case 1. Therefore, PRUs measurements are not needed for training purpose.

Proposal 1: (LPP-6a) PRUs measurements are not needed for training purpose.

LPP-14: Target device error causes

- Background:

Issue description:

The IE NR-AI-ML-Positioning-Error is used by the location server or target device to provide NR AI/ML positioning error reasons to the target device or location server, respectively.

RAN2#128: - As a baseline, if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered. RAN2#129 - The content of error cause is discussed while drafting stage3 CRs.

Status in running CR:

The IE NR-DL-AI-ML-TargetDeviceErrorCauses currently includes the error causes from DL-TDOA.

Captured as "Editor's Note" in IE NR-DL-AI-ML-TargetDeviceErrorCauses.

Proposed resolution:

Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-TargetDeviceErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.)

Status Update: Resolved at RAN2#130.

Agreement from RAN2#130:

Introduce ‘DL AIML positioning not available’ as new target device error cause for AI/ML positioning case 1, to indicate UE cannot perform positioning method (e.g. model not available and performance monitoring outcome not available).

Above error cause is added to the running CR _v02 [3].

Company contributions (if any) can propose additional details.

- Solutions:

In the last meeting, RAN2 agreed to introduce “DL AIML positioning not available” as new target device error, which means the UE cannot perform positioning method. Besides, we think the UE can provide the power state to the LMF when it  enters low power state similar to NW-side data collection. This is because the UE may switch off the AI/ML positioning when it enters low power state. Therefore, we think the RAN2 should introduce “low power state” as new target device error.

Proposal 2: (LPP-14) It is proposed RAN2 to introduce “low power state” as new target device error.

LPP-18 (new): Consistency between training and inference

- Background:

Issue description:

At RAN2#130, contribution R2-2503403 "Discussion on consistency between training and inference for AI POS", vivo et al. was discussed which resulted in the following agreement:

To ensure the consistency between training and inference, the UE should be able to request assistance data associated with a specific group of TRPs. FFS the request associated information in on demand prs request.

Proposed resolution:

Companies to provide contributions to the following meeting on details for "the request associated information in on demand prs request".

- Solutions:

Currently, the NR-On-Demand-DL-PRS-Information IE includes following filed:

–	NR-On-Demand-DL-PRS-Information

The IE NR-On-Demand-DL-PRS-Information defines the requested on-demand DL-PRS.

-- ASN1START

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-On-Demand-DL-PRS-PerFreqLayer-r17

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...,

p128-v1760, p256-v1760, p512-v1760}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17		ENUMERATED {n2, n4, n6, n12, ..., n1-v1800}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReqTRPlist-r17	DL-PRS-QCL-InformationReqTRPlist-r17	OPTIONAL,

...

}

DL-PRS-QCL-InformationReqTRPlist-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

DL-PRS-QCL-InformationReqPerTRP-r17

DL-PRS-QCL-InformationReqPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17						INTEGER (0..255),

nr-PhysCellID-r17					NR-PhysCellID-r16							OPTIONAL,

nr-CellGlobalID-r17					NCGI-r15									OPTIONAL,

nr-ARFCN-r17						ARFCN-ValueNR-r15							OPTIONAL,

dl-prs-QCL-InformationReqSet-r17	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-QCL-InfoReq-r17,

...

}

DL-PRS-QCL-InfoReq-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r17			NR-DL-PRS-ResourceSetID-r16,

dl-prs-QCL-InformationReq-r17		CHOICE {

dl-prs-QCL-InfoRecPerResourceSet-r17	DL-PRS-QCL-Info-r16,

dl-prs-QCL-Info-requested-r17			NULL

},

...,

[[

dl-prs-QCL-InfoRecPerResource-r17	SEQUENCE  (SIZE (1..nrMaxResourcesPerSet-r16)) OF

DL-PRS-QCL-Info-r16			OPTIONAL

]]

}

-- ASN1STOP

The meaning of each filed as following description:

- dl-prs-FrequencyRangeReq: Specifies the requested frequency range for on-demand DL-PRS.

- dl-prs-ResourceSetPeriodicityReq: Specifies the requested periodicity of the DL-PRS resource set (in slots).

- dl-prs-ResourceBandwidthReq: Specifies the requested number of PRBs (Physical Resource Blocks) allocated for the DL-PRS resource.

- dl-prs-ResourceRepetitionFactorReq: Specifies the requested DL-PRS resource repetition factor.

- dl-prs-NumSymbolsReq: Specifies the requested number of symbols per DL-PRS resource within a slot.

- dl-prs-CombSizeN-Req: Specifies the requested spacing of resource elements within each symbol in the DL-PRS resource.

- dl-prs-QCL-InformationReqTRPlist: Specifies the recommended or requested QCL (Quasi-Co-Location) indication with other DL reference signals.

- The "NR-On-Demand-DL-PRS-Request" information element also includes the "dl-prs-StartTime-and-Duration" field, which specifies the desired start time and duration for the requested DL-PRS.

In our understanding, these IEs can be used to ensure the consistency between training and inference, and then the motivation of introducing additional information in NR-On-Demand-DL-PRS-Information for AI/ML positioning should be clarified first, before discussing solutions.

Observation 1: (LPP-18) The motivation of introducing additional information for consistency between training and inference should be clarified.

LPP-19 (new): Applicability of BM related agreements

- Background:

Issue description:

As mentioned in [POST130][023][AI PHY] 38.305  CR (CATT), since the RAN2 intention is to have the same design for BM case and positioning case as much as possible, whether the agreements made for BM applicability reporting can also be applicable to positioning case 1.

Whether the following agreements made for BM applicability reporting are also applicable to positioning case 1 and whether there are any associated LPP impacts:

-	UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

-	Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.

Proposed resolution:

Companies to provide contributions to the following meeting.

- Solutions:

For the AI/ML positioning case 1, we think the model availability should be decided in device due to the model at UE side. As for the reporting of applicability/inapplicability, we don’t think it should be reported explicitly due to the fact that the UE can send NR-On-Demand-DL-PRS-Information to the NW to request the preference configuration, which can implicitly mean the applicability/inapplicability of the configurations.

Observation 2: (LPP-18) The UE can send NR-On-Demand-DL-PRS-Information to the NW to request the preference configuration, which can implicitly mean the applicability/inapplicability of the configurations. So the need of UE reporting of the applicability/inapplicability of configurations to LMF is unclear.

- Conclusion

Based on the above analysis, we have the following observations and proposals:

Observation 1: (LPP-18) The motivation of introducing additional information for consistency between training and inference should be clarified.

Observation 2: (LPP-18) The UE can send NR-On-Demand-DL-PRS-Information to the NW to request the preference configuration, which can implicitly mean the applicability/inapplicability of the configurations. So the need of UE reporting of the applicability/inapplicability of configurations to LMF is unclear.

Proposal 1: (LPP-6a) PRUs measurements are not needed for training purpose.

Proposal 2: (LPP-14) It is proposed RAN2 to introduce “low power state” as new target device error.

- References

[1] [POST130][025][AI PHY] 37.355 CR (Qualcomm)