3GPP TSG-RAN WG2 #131	R2-2506095

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.5.4

Source:	Ericsson

Title:	Adaptation of common signal/channel transmissions for NES

Document for:	Discussion

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised in [4]. In this paper, we address the third objective given in the revised WID as:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we discuss the following aspects:

- adaptation of SSB signals,

- adaptation of PRACH occasions and related signaling, and

- adaptation of paging.

# 2	Discussion

### 2.1 Adaptation of SSB signals

RAN2 made the following agreements regarding this objective in the last meeting:

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Table 1: RAN1 parameters related to SSB adaptation

| addl-ssb-Periodicity | New | Additional SSB burst periodicity for the Scell. | same as ssb-periodicity X≤2 additional SSB burst periodicities from SSB burst periodicities in ssb-periodicity | per secondary serving cell |
| --- | --- | --- | --- | --- |
| ssbPeriodicityIndication-RNTI | New | The RNTI value for scrambling CRC of DCI format 2_9 for SSB burst periodicity switching for Scell(s). | RNTI-Value | per cell group |
| SearchSpace | Existing | The search space for monitoring DCI format 2_9 for SSB burst periodicity switching. | Existing searchSpace configuration with addition of dci-Format2-9 | Per serving cell on which DCI 2_9 is monitored (SearchSpace) |
| sizeDCI-2-9-r18 | Existing | The size of DCI format 2_9 for SSB burst periodicity switching(see TS 38.212, subclause 7.3.1.3.10). | same as for existing sizeDCI-2-9-r18 | Per serving cell on which DCI 2_9 is monitored |
| positionInDCI-ssbPeriodicityIndicationForScell | New | The starting bit position of an information block of DCI format 2_9 for SSB burst periodicity switching of this serving cell (see TS 38.212, subclause 7.3.1.3.10). | 0..maxDCI-2-9-Size-1-r18 | per secondary serving cell |
| addl-ssb-Offset | New | Indicate SFN offset from the SFN which satisfies (SFN index *10) modulo (addl-SSB periodicity) = 0 | INTEGER (0..15) Note: The network configures this field according to the field addl-ssb-Periodicity such that the indicated system frame does not exceed the additional SSB periodicity. | per addl-SSB-periodicity |
| addl-ssb-halfFrameIndex | New | Indicate whether SSB according to the addl-SSB-periodicity is in the first half or the second half of the frame. | [0, 1] | per addl-SSB-periodicity |

In Appendix A, we provide a TP regarding how to capture RAN1 parameters related to SSB adaptation in the 38.331 CR

Adopt the TP in Appendix A in the 38.331 CR.

### 2.2 Adaptation of random-access occasions

In this section we discuss the remaining aspects concerning the configuration of adaptive PRACH resources and related signalling. In the last RAN2 meeting (RAN2#130), the following open issue was captured in the meeting minutes:

“RAN2 will select one of the following options. (CE here means MSG1 repetition).

Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.

Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):(s), the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17).”

Network implementation is restricted in Option 1c. Even though it is possible for the network to avoid mixing CE feature and non-CE features in the same RACH-ConfigCommon, one should consider that networks tend to limit the number RA resources in general due to various reasons including radio resource efficiency and energy saving. Therefore, we prefer Option 1b and hence a clarification in the MAC spec.

Option-1b is captured.

### 2.3 Adaptation of paging

In this section we discuss the remaining aspects concerning the signalling overhead associated with N and Ns parameters, and the aspects concerning the co-existence of PEI and paging adaptations.

#### 2.3.1 Implementation of firstPDCCH-MonitoringOccasionOfPO-r19 IE

In RAN2#130 meeting, following agreements were made concerning the firstPDCCH-MonitoringOccasionOfPO parameter:

Agreements on stage-3 details

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

During the follow-up email discussion on the running CR for 38.331, several issues concerning the firstPDCCH-MonitoringOccasionOfPO-r19 IE in 38.331 have been identified. We address these open issues below.

The first open issue (OPPO005-Question 1) is whether firstPDCCH-MonitoringOccasionOfPO-r19 IE should be configured in both DownlinkConfigCommonSIB and PDCCH-ConfigCommon or only in DownlinkConfigCommonSIB. One argument is that configuration in PDCCH-ConfigCommon is unnecessary, as Rel-19 paging adaptations are not supposed to be utilized by UEs in RRC_CONNECTED. Yet, another argument is that configuration in PDCCH-ConfigCommon may be required to support RedCap UEs depending on the configured BWP.

Rel-19 paging adaptations are not supposed to be utilized by UEs in RRC_CONNECTED.

No issues have been identified regarding RedCap UEs utilizing Rel-19 paging adaptations.

Therefore, we propose the following:

FirstPDCCH-MonitoringOccasionOfPO-r19 IE can be configured in both DownlinkConfigCommonSIB and PDCCH-ConfigCommon.

The second open issue concerns the details of the ASN.1 implementation of firstPDCCH-MonitoringOccasionOfPO IE. One question related to this issue (OPPO005-Question 2, Samsung 003, LGE003, Apple003, Q13b) is whether it should be possible to configure the Rel-19 UEs with the legacy values in the firstPDCCH-MonitoringOccasionOfPO IE, which are not affected by the newly introduced N = T/32 values in Rel-19. Yet another related question is whether to carry the whole legacy chain of SCS-N labels and append “-SCSXKHZoneThirtySecondT” to the existing names or whether to introduce only “SCSXKHZoneThirtySecondT” in the firstPDCCH-MonitoringOccasionOfPO-r19 IE. We discuss these options below. Note that the corresponding ASN.1 implementations of firstPDCCH-MonitoringOccasionOfPO IE is given in Appendix B.

One can observe that in Option 1 maxPO-perPF-r19 is used for SCS-N combinations affected by a new value N = T/32, and maxPO-perPF is used for SCS-N combinations that are not affected by a new value N = T/32 (i.e., these configurations are the same as in legacy).

Differently, in Option 2 maxPO-perPF-r19 is used for all SCS-N combinations. We prefer Option 1 over Option 2 for the following reasons: Based on the agreements from RAN2#125 and RAN2#126 paging adaptations are achieved by “Extending the values of N to have increased interval between and compensating decrease in number of PFs by increasing the number of POs per PF.” Consequently, the paging adaptation is not intended to be used for increasing the number of POs per PF in case when N is configured according to the legacy (i.e., as in Option 2)). This would lead to a higher density of non-condensed POs than in legacy, contradicting the goal of paging adaptations aimed at network energy savings.

Increasing the number of POs per PF (i.e., using maxPO-perPF-r19) in case when N is configured with the legacy value range leads to a higher density of spread POs with respect to legacy.

maxPO-perPF-r19 is not used for SCS-N combinations in firstPDCCH-MonitoringOccasionOfPO IE that are not affected by the new value N = T/32.

Additionally, in Option 3 we present a simplified version of the ASN.1 implementation of the firstPDCCH-MonitoringOccasionOfPO-r19 IE.  Option 3 has simpler and more readable ASN.1 structure with significantly smaller signalling overhead compared to Option 1 and Option 2. The simplification used in Option 3 is based on the observation that some of the configurations based on legacy N and Ns values are not beneficial in terms of network energy savings.

The SCS-N combinations from firstPDCCH-MonitoringOccasionOfPO IE that are not affected by a new value of N can be omitted in the firstPDCCH-MonitoringOccasionOfPO-r19 IE since they result in configurations that are not beneficial with respect to network energy savings.

Discuss the options above regarding how to implement the firstPDCCH-MonitoringOccasionOfPO-r19 IE.

The third issue (Q13a) concerns the maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19. To answer this question, we evaluate the message size when Ns for Rel-19 UEs is set to 8 and compare the outcome with the message size of legacy configuration as baseline.

In the following example, for the sake of evaluation, we assume a SIB1 configuration with hardly anything but paging configuration in all scenarios.

- Legacy baseline scenario: T = 128, N = T/16, Ns = 4 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs), and four instances of firstPDCCH-MonitoringOccasionOfPO parameter is configured for SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 1 .

Figure 1: Legacy baseline scenario (T = 128, N = T/16, Ns = 4, and SCS =15 kHz).

- Option i) scenario: T = 128, N = T/16, for legacy UEs Ns = 4 and for Rel-19 UEs Ns = 8 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs for legacy UEs and 8 POs for Rel-19 UEs). We consider two cases with SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO) and SCS = 480 kHz (i.e., corresponds to the case sCS480KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 2.

Figure 2:  Option i) scenario (T = 128, N = T/16, Ns for legacy = 4, Ns for Rel-19 UEs = 8 and SCS =15 kHz).

In Table 2, we provide the message sizes for the scenarios above. We assumed that the firstPDCCH-MonitoringOccasionOfPO-r19 parameters for Rel-19 UEs are defined in the same way as in legacy (i.e., the range of firstPDCCH-MonitoringOccasionOfPO-r19 parameters for Rel-19 UEs is given in symbols).

Table 2: Message sizes for different paging configurations (symbol level configuration of firstPDCCH-MonitoringOccasionOfPO-r19 parameter for Rel-19 UEs)

|  | Message size [octets] |
| --- | --- |
| Legacy T = 128, N = T/16, Ns = 4, SCS = 15 kHz (baseline) | 28 |
| Option i) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 15 kHz | 41 |
| Option i) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 480 kHz | 50 |

According to the results above in Table 2, option i) with SCS = 15 kHz and option i) with SCS = 480 kHz have 13 and 22 octets larger message size than the legacy (baseline) configuration, respectively. This accounts for approximately 46% and 79% increase in message size. One can observe that option b) introduces significant signalling overhead even for the case where Ns for Rel-19 UEs is set to 8, according to the latest RAN2 agreements.

Option i) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even for the case where Ns for Rel-19 UEs is set to 8.

Based on the results in Table 2  above, we think that it would be beneficial to reduce the impact of option i) on the message size. This can be simply achieved by defining the range of the firstPDCCH-MonitoringOccasionOfPO-r19 parameter for Rel-19 UEs at slot level instead of symbol level as currently specified for legacy UEs.

Note that we are not proposing to change the meaning of firstPDCCH-MonitoringOccasionOfPO-r19, but its resolution only (i.e., the unit of firstPDCCH-MonitoringOccasionOfPO=r19 parameter would still be monitoring occasions).  This would allow us to define the range of the firstPDCCH-MonitoringOccasionOfPO-r19 parameters for Rel-19 UEs to be the same as of the firstPDCCH-MonitoringOccasionOfPO parameters for legacy UEs but with fewer bits.

In Table 3, we summarize the evaluation for message sizes for two scenarios with option i) in which firstPDCCH-MonitoringOccasionOfPO-r19 parameters for Rel-19 UEs are defined at the slot level.

Table 3: Message size for different paging configuration scenarios (slot level configuration of firstPDCCH-MonitoringOccasionOfPO-r19 parameter for Rel-19 UEs)

|  | Message size [octets] |
| --- | --- |
| Option i) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 15 kHz | 37 |
| Option i) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 480 kHz | 46 |

Based on the results in Table 3 above, the configuration of the firstPDCCH-MonitoringOccasionOfPO-r19 parameter for Rel-19 UEs at slot level results in 4 octets smaller message size than the configuration of the firstPDCCH-MonitoringOccasionOfPO-r19 parameter for Rel-19 UEs at symbol level, which corresponds to approximately 31% reduction of message size.

The configuration of firstPDCCH-MonitoringOccasionOfPO-r19 parameter at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO-r19 parameter at symbol level.

For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPO-r19 is configured at slot level.

The final open issue (Samsung004) concerning the implementation of paging adaptations is whether to group pagingAdaptation-NS-r19, pagingAdaptationNAndPagingFrameOffset-r19 and firstPDCCH-MonitoringOccasionOfPO-19 together in one IE, and whether pagingAdaptation-NS-r19, and pagingAdaptationNAndPagingFrameOffset-19 should be mandatory inside this new IE. Considering that the grouping would improve the readability of the ASN.1 implementation and that ns and nAndPagingFrameOffset are mandatory in the legacy configuration, we propose to adopt this change.

Group all parameters for paging adaptation in one IE pagingAdaptation-Config-r19. Define pagingAdaptation-NS-r19, and pagingAdaptationNAndPagingFrameOffset-19 as mandatory (i.e., same as in legacy).

Taking into consideration the above discussion and proposals on the open issues concerning the firstPDCCH-MonitoringOccasionOfPO-r19 IE, we propose the TP below.

Adopt the TP below in the running 38.331 CR.

----------------------------------------------------------------TP STARTS----------------------------------------------------------------

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINKCONFIGCOMMONSIB-START

pagingAdaptation-Config-r19 ::=             SEQUENCE {

pagingAdaptation-NS-r19                 ENUMERATED {eight, four, two, one}

pagingAdaptationNAndPagingFrameOffset-r19               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15),

oneThirtySecondT                     INTEGER(0..31)

}

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..9),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..19),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..39),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..79),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..159),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT- SCS15KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..319),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT- SCS30KHZoneThirtySecondT                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..639),

sCS480KHZquarterT-SCS120KHZoneSixteenthT- SCS60KHZoneThirtySecondT                                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..1279),

sCS480KHZoneEighthT-SCS120KHZoneThirtySecondT        								SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..2559),

sCS480KHZoneSixteenthT                   											SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..5119),

sCS480KHZoneThirtySecondT  															SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..10239)

}       														  OPTIONAL  -- Need R

} 																	  OPTIONAL  -- Need R

-- TAG-DOWNLINKCONFIGCOMMONSIB-STOP

-- ASN1STOP

DownlinkConfigCommonSIB field descriptions

pagingAdaptation-Config The paging adaptation related configuration.

pagingAdaptation-Config field descriptions

pagingAdaptation-NS Number of paging occasions per paging frame for paging adaptation.

firstPDCCH-MonitoringOccasionOfPO Points out the first PDCCH monitoring occasion for paging of each PO of the PF for paging adaptation, see TS 38.304 [20].

pagingAdaptationNAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]) for paging adaptation. A value of oneThirtySecondT corresponds to T/32, a value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 2 or 3 (as specified in TS 38.213 [13]): - for ssb-periodicityServingCell of 5 or 10 ms, N can be set to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 20 ms, N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 40 ms, N can be set to one of {quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 80 ms, N can be set to one of {oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 160 ms, N can be set to {oneSixteenthT, oneThirtySecondT} If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 1 (as specified in TS 38.213 [13]), N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} If pagingSearchSpace is not set to zero, N can be configured to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT}

----------------------------------------------------------------TP ENDS------------------------------------------------------------------

### 2.3.3 Rel-19 PEI configuration parameters

RAN2 made the following agreements in the RAN2#129bis meeting:

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

According to the agreements from RAN2#129bis meeting, Rel-19 PEI independently addresses the additional Rel-19 POs. Furthermore, the separation of additional Rel-19 PEI transmission occasions from legacy ones was discussed during RAN2#130 meeting and the following agreements were made:

- A new pei-ConfigBWP IE is not introduced for Rel-19 UEs that support adaptive paging.

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum number of PEI occasion per paging frame is extended to 8.

- The maximum offset value is extended to 32 radio frames.

As highlighted in the list of agreements from RAN2#130, it was agreed to introduce the following new parameters firstPDCCH-MonitoringOccasionOfPEI-O-R19, pei-FrameOffset-r19 and maxPEI-perPF-r19. The open issue raised in the follow-up discussion is whether to introduce the following parameters:

- po-NumPerPEI-r19

- payloadSizeDCI-2-7-r19

- subgroupsNumPerPO-r19

- subgroupsNumForUEID-r19

- nrofCandidates-PEI-R19

Considering the agreements from RAN2#129bis meeting according to which Rel-19 PEI independently addresses the additional Rel-19 POs, we think that the above parameters are needed. Furthermore, we think that the value ranges of the above parameters should be aligned with those of the Rel-17 PEI counterparts.

When it comes to the parameter lastUsedCellOnly-r17 we don’t think there is need for a new counterpart, and the Rel-17 parameter applies to both legacy and Rel-19 configurations.

For Rel-19 PEI configuration, introduce the following parameters and, use the same value ranges as for legacy PEI:
-po-NumPerPEI-r19, 
-payloadSizeDCI-2-7-r19, 
-subgroupsNumPerPO-r19,
-subgroupsNumForUEID-r19,
-nrofCandidates-PEI-R19.

While it was agreed in RAN2#130 meeting to introduce new firstPDCCH-MonitoringOccasionOfPEI-O-R19 parameter, the value range of this parameter is still an open issue. We think that the appropriate range for firstPDCCH-MonitoringOccasionOfPEI-O-R19 parameter should be at the slot level, i.e., in line with ongoing discussions and our reasoning for condensed POs and firstPDCCH-MonitoringOccasionOfPO-r19 parameter.

firstPDCCH-MonitoringOccasionOfPEI-O-r19 is configured at slot level with the same value range as for legacy PEI.

Adopt the TP below in the running 38.331 CR.

----------------------------------------------------------------TP STARTS----------------------------------------------------------------

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..9),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1.. maxPEI-perPF-r17)) OF INTEGER (0..19),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1.. maxPEI-perPF-r17)) OF INTEGER (0..39),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1.. maxPEI-perPF-r17)) OF INTEGER (0..79),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1.. maxPEI-perPF-r17)) OF INTEGER (0..159),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT- SCS15KHZoneThirtySecondT    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..319),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT- SCS30KHZoneThirtySecondT                    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..639),

sCS480KHZquarterT-SCS120KHZoneSixteenthT- SCS60KHZoneThirtySecondT                                    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..1279),

sCS480KHZoneEighthT-SCS120KHZoneThirtySecondT        								SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..2559),

sCS480KHZoneSixteenthT                   											SEQUENCE (SIZE (1.. maxPEI-perPF-r17)) OF INTEGER (0..5119),

sCS480KHZoneThirtySecondT  															SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..10239)

}       													OPTIONAL     -- Cond InitialBWP-Paging

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of slots, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP for paging adaptation, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O for the slot-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O for the slot-level offset.

# 3	Conclusion

In this contribution we discuss the objective on the adaptation of common signal/channel transmissions. Based on the discussion in the previous sections we made the following observations:

Observation 1	Rel-19 paging adaptations are not supposed to be utilized by UEs in RRC_CONNECTED.

Observation 2	No issues have been identified regarding RedCap UEs utilizing Rel-19 paging adaptations.

Observation 3	Increasing the number of POs per PF (i.e., using maxPO-perPF-r19) in case when N is configured with the legacy value range leads to a higher density of spread POs with respect to legacy.

Observation 4	The SCS-N combinations from firstPDCCH-MonitoringOccasionOfPO IE that are not affected by a new value of N can be omitted in the firstPDCCH-MonitoringOccasionOfPO-r19 IE since they result in configurations that are not beneficial with respect to network energy savings.

Observation 5	Option i) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even for the case where Ns for Rel-19 UEs is set to 8.

Observation 6	The configuration of firstPDCCH-MonitoringOccasionOfPO-r19 parameter at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO-r19 parameter at symbol level.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Adopt the TP in Appendix A in the 38.331 CR.

Proposal 2	Option-1b is captured.

Proposal 3	FirstPDCCH-MonitoringOccasionOfPO-r19 IE can be configured in both DownlinkConfigCommonSIB and PDCCH-ConfigCommon.

Proposal 4	maxPO-perPF-r19 is not used for SCS-N combinations in firstPDCCH-MonitoringOccasionOfPO IE that are not affected by the new value N = T/32.

Proposal 5	Discuss the options above regarding how to implement the firstPDCCH-MonitoringOccasionOfPO-r19 IE.

Proposal 6	For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPO-r19 is configured at slot level.

Proposal 7	Group all parameters for paging adaptation in one IE pagingAdaptation-Config-r19. Define pagingAdaptation-NS-r19, and pagingAdaptationNAndPagingFrameOffset-19 as mandatory (i.e., same as in legacy).

Proposal 8	Adopt the TP below in the running 38.331 CR.

Proposal 9	For Rel-19 PEI configuration, introduce the following parameters and, use the same value ranges as for legacy PEI: -po-NumPerPEI-r19,  -payloadSizeDCI-2-7-r19,  -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -nrofCandidates-PEI-R19.

Proposal 10	firstPDCCH-MonitoringOccasionOfPEI-O-r19 is configured at slot level with the same value range as for legacy PEI.

Proposal 11	Adopt the TP below in the running 38.331 CR.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

3GPP TS 38.304 V18.1.0 User Equipment (UE) procedures in Idle mode and in RRC Inactive state

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

R2-2502150, “Comments to 38.331 CR for NES”, Ericsson, 2025 April 7-11, Wuhan, China.

R2-2502173, “Running RRC CR for enhancements for network energy efficiency”, Ericsson, 2025 April 7-11, Wuhan, China.

R2-2503607, “Running RRC CR for enhancements for network energy efficiency”, Ericsson, 2025 May 19-23, St. Julian’s, Malta.

# Appendix A

----------------------------------------------------------------TP STARTS----------------------------------------------------------------

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

<cut>

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Need S

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17       SetupRelease {PLMN-IdentityInfoList}                            OPTIONAL,   -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17        SetupRelease {NPN-IdentityInfoList-r16}                         OPTIONAL    -- Cond SCellSIB20-Opt

]],

[[

od-SSB-ConfigToAddModList-r19                     SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-Config-r19    OPTIONAL,   -- Need N

od-SSB-ConfigToReleaseList-r19                    SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-ConfigId-r19    OPTIONAL,   -- Need N

adap-SSB-Config-r19                               Adap-SSB-Config-r19                           OPTIONAL   -- Need N

]]

Adap-SSB-Config-r19 ::=       SEQUENCE {

adap-SSB-BurstPeriodicityList-r19 ::=	   SEQUENCE (SIZE (1..2)) OF Adap-SSB-BurstPeriodicity-r19       OPTIONAL   -- Need N

adap-PosInDCI-ssbPeriodicityIndicationForScell            INTEGER (1..maxDCI-2-9-Size-r18)       OPTIONAL   -- Need N

}

Adap-SSB-BurstPeriodicity-r19 ::=      SEQUENCE {

adap-ssb-Periodicity-r19           ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL,      -- Need S

adap-ssb-Offset                    INTEGER (1..maxDCI-2-9-Size-r18)                     OPTIONAL,      -- Need N

adap-ssb-halfFrameIndex            ENUMERATED { firsthalf, secondhalf }                OPTIONAL,      -- Need N

adap-ssb-smtc                      SSB-MTC                                                     OPTIONAL    -- Need S

}

}

SCellConfig field descriptions

adap-PosInDCI-ssbPeriodicityIndicationForScell The starting bit position of an information block of DCI format 2_9 for SSB burst periodicity switching of this serving cell (see TS 38.212 [17], subclause 7.3.1.3.10).

adap-ssb-halfFrameIndex Indicate whether SSB according to the adap-ssb-Periodicity is in the first half or the second half of the frame. The network configures this field according to adap-ssb-Periodicity such that the indicated system frame does not exceed the corresponding adaptive SSB periodicity.

adap-ssb-Periodicity Additional SSB burst periodicity for the Scell. If the field is absent, the UE applies the value ms5.

adap-ssb-Offset Indicate SFN offset from the SFN which satisfies (SFN index *10) modulo (adap-ssb-Periodicity) = 0

----------------------------------------------------------------TP ENDS-------------------------------------------------------------------

----------------------------------------------------------------TP STARTS----------------------------------------------------------------

#### –	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI                        RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUCCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUSCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

sp-CSI-RNTI                         RNTI-Value                                                      OPTIONAL,   -- Need R

cs-RNTI                             SetupRelease { RNTI-Value }                                     OPTIONAL,   -- Need M

...,

[[

mcs-C-RNTI                          RNTI-Value                                                      OPTIONAL,   -- Need R

p-UE-FR1                            P-Max                                                           OPTIONAL    -- Cond MCG-Only

]],

[[

xScale                              ENUMERATED {dB0, dB6, spare2, spare1}                           OPTIONAL    -- Cond SCG-Only

]],

[[

pdcch-BlindDetection                SetupRelease { PDCCH-BlindDetection }                           OPTIONAL    -- Need M

]],

<cut>

[[

ncr-RNTI-r18                      RNTI-Value                                                OPTIONAL,   -- Cond NCR

cellDTRX-DCI-config-r18           SetupRelease { CellDTRX-DCI-config-r18 }                  OPTIONAL,   -- Need M

twoQCL-TypeD-ForMultiDCI-r18      ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18           ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableType2HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18           ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableType3HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18           ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableDiffPUCCH-Resource-r18                                       ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableDiffCB-Size-r18                                              ENUMERATED {enabled}     OPTIONAL    -- Need R

]],

[[

adap-SSBPeriodicityIndication-RNTI-r19           RNTI-Value                                    OPTIONAL,   -- Need N

adap-SSB-SizeDCI-2-9-r19                         INTEGER (1..maxDCI-2-9-Size-r18)       OPTIONAL,   -- Need N

]]

}

<cut>

CellDTRX-DCI-config-r18 ::=         SEQUENCE {

cellDTRX-RNTI-r18                   RNTI-Value,

sizeDCI-2-9-r18                     INTEGER (1..maxDCI-2-9-Size-r18)

}

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

PhysicalCellGroupConfig field descriptions

adap-SSBPeriodicityIndication-RNTI The RNTI value for scrambling CRC of DCI format 2_9 for SSB burst periodicity switching for Scell(s).

adap-SSB-SizeDCI-2-9 The size of DCI format 2_9 for SSB burst periodicity switching(see TS 38.212, subclause 7.3.1.3.10).

----------------------------------------------------------------TP ENDS-------------------------------------------------------------------

----------------------------------------------------------------TP STARTS----------------------------------------------------------------

#### –	PDCCH-Config

The IE PDCCH-Config is used to configure UE specific PDCCH parameters or MBS multicast PDCCH parameters such as control resource sets (CORESET), search spaces and additional parameters for acquiring the PDCCH. If this IE is used for the scheduled SCell in case of cross carrier scheduling, the fields other than searchSpacesToAddModList and searchSpacesToReleaseList are absent. If the IE is used for a dormant BWP, the fields other than controlResourceSetToAddModList and controlResourceSetToReleaseList are absent. If this IE is used for MBS CFR, the field downlinkPreemptiom,tpc-PUSCH, tpc-SRS, uplinkCancellation, monitoringCapabilityConfig, and searchSpaceSwitchConfig are absent.

PDCCH-Config information element

-- ASN1START

-- TAG-PDCCH-CONFIG-START

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet                      OPTIONAL,   -- Need N

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId                    OPTIONAL,   -- Need N

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace                            OPTIONAL,   -- Need N

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId                          OPTIONAL,   -- Need N

downlinkPreemption                  SetupRelease { DownlinkPreemption }                              OPTIONAL,   -- Need M

tpc-PUSCH                           SetupRelease { PUSCH-TPC-CommandConfig }                         OPTIONAL,   -- Need M

tpc-PUCCH                           SetupRelease { PUCCH-TPC-CommandConfig }                         OPTIONAL,   -- Need M

tpc-SRS                             SetupRelease { SRS-TPC-CommandConfig}                            OPTIONAL,   -- Need M

...,

[[

controlResourceSetToAddModListSizeExt-v1610 SEQUENCE (SIZE (1..2)) OF ControlResourceSet             OPTIONAL,   -- Need N

controlResourceSetToReleaseListSizeExt-r16 SEQUENCE (SIZE (1..5)) OF ControlResourceSetId-r16        OPTIONAL,   -- Need N

searchSpacesToAddModListExt-r16     SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-r16                     OPTIONAL,   -- Need N

uplinkCancellation-r16              SetupRelease { UplinkCancellation-r16 }                          OPTIONAL,   -- Need M

monitoringCapabilityConfig-r16      ENUMERATED { r15monitoringcapability,r16monitoringcapability }   OPTIONAL,   -- Need M

searchSpaceSwitchConfig-r16         SearchSpaceSwitchConfig-r16                                      OPTIONAL    -- Need R

]],

[[

searchSpacesToAddModListExt-v1700   SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1700                   OPTIONAL,   -- Need N

monitoringCapabilityConfig-v1710    ENUMERATED { r17monitoringcapability }                           OPTIONAL,   -- Need M

searchSpaceSwitchConfig-r17         SearchSpaceSwitchConfig-r17                                      OPTIONAL,   -- Need R

pdcch-SkippingDurationList-r17      SEQUENCE(SIZE (1..3)) OF SCS-SpecificDuration-r17                OPTIONAL    -- Need R

]],

[[

pdcch-MonitoringResumptionAfterNack-r18 ENUMERATED {true}                                            OPTIONAL,   -- Need R

searchSpacesToAddModListExt-v1800   SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1800                   OPTIONAL    -- Need N

]],

[[

searchSpacesToAddModListExt-v1900   SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1900                   OPTIONAL    -- Need N

]]

}

<cut>

-- TAG-PDCCH-CONFIG-STOP

-- ASN1STOP

----------------------------------------------------------------TP ENDS-------------------------------------------------------------------

----------------------------------------------------------------TP STARTS----------------------------------------------------------------

#### –	PDCCH-ConfigCommon

The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as in dedicated signalling.

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=              SEQUENCE {

controlResourceSetZero              ControlResourceSetZero                                  OPTIONAL,   -- Cond InitialBWP-Only

commonControlResourceSet            ControlResourceSet                                      OPTIONAL,   -- Need R

searchSpaceZero                     SearchSpaceZero                                         OPTIONAL,   -- Cond InitialBWP-Only

commonSearchSpaceList               SEQUENCE (SIZE(1..4)) OF SearchSpace                    OPTIONAL,   -- Need R

searchSpaceSIB1                     SearchSpaceId                                           OPTIONAL,   -- Need S

searchSpaceOtherSystemInformation   SearchSpaceId                                           OPTIONAL,   -- Need S

pagingSearchSpace                   SearchSpaceId                                           OPTIONAL,   -- Need S

ra-SearchSpace                      SearchSpaceId                                           OPTIONAL,   -- Need S

...,

<cut>

[[

applyIndicatedTCI-State-r18            ENUMERATED {first, second, both, none}                OPTIONAL,   -- Cond FollowUTCI

commonSearchSpaceListExt-r18           SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1800         OPTIONAL,   -- Need R

searchSpaceMulticastMCCH-r18           SearchSpaceId                                         OPTIONAL,   -- Need R

searchSpaceMulticastMTCH-r18           SearchSpaceId                                         OPTIONAL    -- Need S

]],

[[

commonSearchSpaceListExt-r19           SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1900         OPTIONAL,   -- Need R

pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneThirtySecondT                                                SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneThirtySecondT                                                SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

}                                                                                           OPTIONAL,    -- Cond OtherBWP

pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19  CHOICE {

sCS15KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneThirtySecondT                                                 SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneThirtySecondT                                                SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneThirtySecondT                                                SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS)

}                                                                                           OPTIONAL     -- Cond InitialBWP-Paging

]]

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

----------------------------------------------------------------TP ENDS-------------------------------------------------------------------

----------------------------------------------------------------TP STARTS----------------------------------------------------------------

#### –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled SCell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent (regardless of their presence conditions). For a scheduled SpCell in the case of the cross carrier scheduling, if the search space is linked to another search space in the scheduling SCell, all the optional fields of this search space in the scheduled SpCell are absent (regardless of their presence conditions) except for nrofCandidates.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup4

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need S

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

dci-Formats-MT-r16                   ENUMERATED {formats2-5}                                OPTIONAL,    -- Need R

dci-FormatsSL-r16                    ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1,

formats3-0-And-3-1}                        OPTIONAL,    -- Need R

dci-FormatsExt-r16                   ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2}

OPTIONAL     -- Need R

]],

[[

dci-FormatsNCR-r18                   ENUMERATED {formats2-8}                                OPTIONAL,    -- Need R

dci-FormatsSL-PRS-r18                ENUMERATED {formats3-2, formats3-0-And-3-2, formats3-0-And-3-1-And-3-2,

formats3-1-And-3-2}                        OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

<cut>

SearchSpaceExt-v1800 ::=            SEQUENCE {

searchSpaceType-r18                 CHOICE {

common-r18                          SEQUENCE {

dci-Format2-9-r18                   SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

},

ue-Specific-r18                     SEQUENCE {

dci-FormatsMC-r18                   ENUMERATED {formats0-3, formats1-3, formats0-3-And-1-3} OPTIONAL,   -- Need R

...

}

}                                                                                                   OPTIONAL    -- Need R

}

SearchSpaceExt-v1900 ::=            SEQUENCE {

searchSpaceType-r19                 SEQUENCE {

common-r19                          SEQUENCE {

dci-Format2-9-r19                   SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

}

}                                                                                                   OPTIONAL    -- Need R

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

----------------------------------------------------------------TP ENDS-------------------------------------------------------------------

# Appendix B

Different Options for ASN.1 implementation of firstPDCCH-MonitoringOccasionOfPO-r19 IE.

Option 1:

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..FFS),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..FFS),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..FFS),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..FFS),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..FFS),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT- SCS15KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT- SCS30KHZoneThirtySecondT                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZquarterT-SCS120KHZoneSixteenthT- SCS60KHZoneThirtySecondT                                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

sCS480KHZoneEighthT-SCS120KHZoneThirtySecondT        								SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneSixteenthT                   											SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..FFS)

sCS480KHZoneThirtySecondT  															SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..FFS)

}

Option 2:

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT- SCS15KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT- SCS30KHZoneThirtySecondT                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZquarterT-SCS120KHZoneSixteenthT- SCS60KHZoneThirtySecondT                                    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

sCS480KHZoneEighthT-SCS120KHZoneThirtySecondT        								SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneSixteenthT                   											SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

sCS480KHZoneThirtySecondT  															SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..FFS)

}

Option 3:

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneThirtySecondT  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS30KHZoneThirtySecondT  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS60KHZoneThirtySecondT  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS120KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS),

sCS480KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS)

}