3GPP TSG-RAN WG2 Meeting #131	R2-2506134

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.3.2.2

Source:	Nokia

Title:	Functionality management for measurement event prediction

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# 1	Introduction

It was agreed in RAN2#129bis that:

Agreements - The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

The following agreements were made in RAN2#129bis:

Agreements

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

- For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

The following agreements were made in RAN2#130:

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

In the contribution, we present our final remarks related to indirect event prediction and highlight the need for specifying the prediction reporting criteria configuration in the WI phase. For direct prediction, we discuss the need for providing further metadata associated with the models that produced the predictions. Last, we present our views on monitoring for the measurement event prediction use-case and the associated KPIs that can be considered.

# 2	Discussion

## 2.1 	Indirect Measurement Event Prediction

Figure 2.1-1 Indirect measurement event prediction, sub-case 2

The required accuracy of measurement predictions depends on how the measurements and/or predictions are used. In temporal domain case B, the assumption may be that the predicted measurements can replace actual measurements, e.g., that they may be used to evaluate measurement event conditions and that the network may even decide on handover execution based on them. This requires high accuracy, comparable to the accuracy of the actual measurements.

On the other hand, if the predicted measurements are used, for example, to optimize early handover preparation decisions (to reduce the handover execution duration) or measurement configuration, required accuracy may be lower. This is one possible use case utilizing temporal domain case A predictions.

Observation 1: The required accuracy of measurement predictions depends on how the measurements and/or predictions are used.

Observation 2: Temporal domain case B measurement prediction may require high accuracy, comparable to the accuracy of the actual measurements.

Observation 3: In temporal domain case A measurement prediction optimizing, e.g., early handover preparation decisions, the required accuracy may be relaxed.

In 3GPP TR 38.744 draft [1] the following observations have been captured based on the evaluation results:

For both FR2 intra-frequency temporal domain case A and FR1 intra-frequency temporal domain case B predictions, the following observations are made:

- Higher UE speed correlates with decreased prediction accuracy;

- Longer PW length correlates with decreased prediction accuracy;

So, if the predictions are accurate enough to utilize them in measurement event condition evaluation may depend on how far in the future the measurements are predicted.

Figure 2.1-1 Temporal Domain Case B Predictive Filter Sub-Case 1 with unfiltered L1 measurements as input

Furthermore, the “state machine” of evaluating a measurement event may include both measurements and predictions, for example when predicting if an A3 condition will hold for the duration of the TTT after the A3 entry condition has been fulfilled, as shown in the example in Figure 2.1-1. It may be necessary for the network to configure how either measurements or predictions may be used by the UE to evaluate measurement reporting criteria, and if measurements and/or predictions should be reported. Furthermore, depending on the scenario, the predictions may be accurate enough for the reporting condition evaluation only in part of the PW.

Observation 4: The predictions may be accurate enough for the reporting condition evaluation only in some parts of the PW.

Proposal 1: RAN2 to specify the prediction reporting criteria configuration in the WI phase, including how the expected accuracy of the predictions should be considered.

## Direct Measurement Event Prediction

When the network configures the UE to perform direct measurement event prediction, the UE sends a prediction report to the network whenever the configured event is predicted to occur. The prediction report should contain information on what type of event it is (A1-A6, B1-B2, handover, HOF, RLF, etc.) and what kind of metadata is related to this event. For example, it is not enough that the network knows a handover is predicted to happen for a specific UE, but it should also know the source and target cells for that handover. The network may also configure the UEs with several events to predict, which raises the need to identify which even the prediction report belongs to.

Therefore, when the network configures the UEs with measurement event prediction, it should also configure them with corresponding prediction IDs to identify what kind of event and what kind of metadata is associated with that event. Then when the UE sends the prediction report, it sends the correct prediction ID along with it.

Observation 5: The UE needs to identify, in the prediction report, the type of event and the metadata associated with it.

Proposal 2: RAN2 to specify how to identify the type of event and its metadata in each prediction report.

In our simulation evaluations, we have observed that in scenarios where UEs have a random-walk kind of movement, it is very challenging for ML models to learn patterns in the input data, which are mainly measurement time series. ML models can more easily learn to correlate the measurement time series with the desired output (measurement events in this case) when the movements of at least some UEs follow relatively consistent patterns (e.g. walking along a street lane, highways, or railways).

Observation 6: ML models learn better when at least some UEs have relatively similar movement patterns, e.g. walking along a street lane.

We have also observed that network-sided models tend to have access to more data than UE-sided models during training. Moreover, when a network-sided model is trained, the model can learn cell- or site-specific patterns that enhance its performance compared to generic models that have no cell- or site-specific information. Therefore, we propose that RAN2 to specify how the network can assist UE-sided models with cell- or site-specific assistance information that have the potential to enhance UE-sided models’ performance.

Observation 7: NW-sided models have access to more training data and to cell- or site-specific information.

Proposal 3: Discuss specification impact on network assistance for UE-sided models with cell- or site-specific information.

## Monitoring KPIs

In case the UE-sided model experiences model or concept drift, it is necessary for it to inform the network that the quality of its predictions is degrading, so that the network can choose to not respond to wrong or uncertain predictions. This can be achieved by either the UE signalling its own KPIs that is has calculated (precision, recall, F1 score) or by the UE signalling both the predictions and the ground truth information to the network, where the network can monitor the performance of the model as part of its implementation.

In the first option, the UE calculates the monitoring KPIs in a monitoring window and signals the results to the network either periodically or when the KPIs fall below a threshold. In the second option, the UE sends the event predictions to the network, and the network logs the event occurrences and can calculate the performance KPIs based on that.

Proposal 4: RAN2 to consider two options to perform monitoring for measurement event prediction: (1) The UE signals its own monitoring KPIs to the network; (2) The UE signals the predictions, and their ground truth labels, and the network does the monitoring as part of its implementation.

When it comes to measurement event prediction, we observe that false positives have a higher cost than false negatives. In case of the latter, i.e. when a UE predicts that an event will not occur but it actually occurs, the UE can fall back to the legacy procedure of using current measurements to e.g. trigger an A3 event entry and wait for the TTT. In contrast, when a UE predicts that an event will occur but it doesn’t occur (false positive), the network will have wasted whatever preparations it has performed in anticipation of the event. To account for such an observation in the monitoring KPIs, we propose to use the precision or the false positive rate as a UE-sided monitoring KPI.

Observation 8: A false positive measurement event prediction results in the network wasting the preparations it has done in anticipation of the event.

Observation 9: In case of a false negative measurement event prediction, the UE/NW can fall back to the legacy behaviour of measurement reporting, which poses virtually no concern.

Observation 10: In measurement event prediction, false positives have a higher cost than false negatives.

Proposal 5: Use the precision or false positive rate as a UE-sided monitoring KPI in measurement event prediction.

# 3	Conclusion

This document has made the following observations:

Observation 1: The required accuracy of measurement predictions depends on how the measurements and/or predictions are used.

Observation 2: Temporal domain case B measurement prediction may require high accuracy, comparable to the accuracy of the actual measurements.

Observation 3: In temporal domain case A measurement prediction optimizing, e.g., early handover preparation decisions, the required accuracy may be relaxed.

Observation 4: The predictions may be accurate enough for the reporting condition evaluation only in some parts of the PW.

Observation 5: The UE needs to identify, in the prediction report, the type of event and the metadata associated with it.

Observation 6: ML models learn better when at least some UEs have relatively similar movement patterns, e.g. walking along a street lane.

Observation 7: NW-sided models have access to more training data and to cell- or site-specific information.

Observation 8: A false positive measurement event prediction results in the network wasting the preparations it has done in anticipation of the event.

Observation 9: In case of a false negative measurement event prediction, the UE/NW can fall back to the legacy behaviour of measurement reporting, which poses virtually no concern.

Observation 10: In measurement event prediction, false positives have a higher cost than false negatives.

And proposed the following:

Proposal 1: RAN2 to specify the prediction reporting criteria configuration in the WI phase, including how the expected accuracy of the predictions should be considered.

Proposal 2: RAN2 to specify how to identify the type of event and its metadata in each prediction report.

Proposal 3: Discuss specification impact on network assistance for UE-sided models with cell- or site-specific information.

Proposal 4: RAN2 to consider two options to perform monitoring for measurement event prediction: (1) The UE signals its own monitoring KPIs to the network; (2) The UE signals the predictions, and their ground truth labels, and the network does the monitoring as part of its implementation.

Proposal 5: Use the precision or false positive rate as a UE-sided monitoring KPI in measurement event prediction.