3GPP TSG-RAN WG2 Meeting #131	R2-2506158

Bengaluru, India Aug 25th – 29th, 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on inter-CU LTM

Document for:        	Discussion and decision

Agenda Item:         	8.6.2	Inter-CU LTM

- Introduction

In RAN2-130 meeting [1], RAN2 discussed the detail of inter-CU LTM procedure, and the following was agreed.

Agreements on stage-3 details

15.	(RRC-6) In inter-MN LTM failure (LTM cell switch failure) the UE fall back to the previous source PCell configuration before selecting a cell (same as legacy procedure).

Agreements on inter-CU LTM

1.	RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

2.	If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

In this contribution, we provide our understanding for inter-CU LTM procedure, such as the procedure related open issue of the security key update upon LTM fast recovery.

- Discussion

In the last meeting, RAN2 agreed that if the selected cell has the same R19 ID as the target cell after the inter-CU MCG LTM failure, the UE performs LTM fast recovery to the selected cell, and in inter-MN LTM failure the UE falls back to the previous source PCell configuration before selecting a cell. Therefore, the UE performs the security key update based on the NH (or the security key) used in the previous source PCell using the NCC value in the LTM cell switch command and the selected cell’s PCI and frequency.

From these agreements, in the reconfiguration with sync failure case, if the UE selects the same target cell that previously failed, the UE derives the same security key as the one used at the previous LTM cell switch since it uses the same NCC value, PCI, and frequency of the target cell. This scenario raises issue about keystream reuse. In Rel-18 LTM fast recovery, RAN2 also discussed the issue of keystream reuse. In our understanding, the keystream reuse in Rel-18 LTM occurred since the same COUNT is used for two different target cells. Therefore, in Rel-18 LTM, the UE does not revert the COUNT to the previous target cell to avoid this keystream reuse. However, in Rel-19 LTM, the UE applies the same security key when the same target cell is selected during the cell selection for LTM fast recovery. If a different cell from the previous target cell is selected, the UE derives a new security key using the different PCI and frequency.

In SCPAC, RAN2 discussed the keystream reuse, which refers to using the same sk-counter to the cells belonging to the same SN. In this case, the issue is that the same security key already used during the previous PSCell change, is applied again to the same SN. However, in Rel-19, the UE applies the security key that previously failed to access the target cell. Therefore, from NW perspective, the UE continues to access the same target cell with the same security key.

Based on these reasons, we think that the UE can perform the LTM fast recovery when selecting the previously failed cell after reconfiguration with sync failure. Therefore, as a simple solution, we propose that the UE can perform LTM fast recovery on all cells with the same Rel-19 ID as the previous target cell.

Proposal: The UE can perform LTM fast recovery on all cells with the same Rel-19 ID as the previous target cell.

- Summary and proposal

This paper discussed the procedure for LTM failure recovery procedure of inter-CU LTM. In summary, the following was proposed:

Proposal: The UE can perform LTM fast recovery on all cells with the same Rel-19 ID as the previous target cell.

- References

[1] R2-2505002, “RAN2#130 Meeting Report,” ETSI MCC.