3GPP TSG-RAN WG2 Meeting #131	R2-2506191

Bengaluru, India, 25th – 29th August 2025

Source:	NTT DOCOMO, INC.

Title:	On remaining issue on XR UL rate control

Document for:	Discussion

Agenda Item:	8.7.6

- Introduction

This contribution is to provide our views with the remaining issues on XR UL rate control.

- Discussion

  - Identification of QoS flow

The following open issue hasn’t completely resolved although there were some agreements about this in RAN2#130 meeting. [1]

[MAC-04] Which type of ID should be used to identify a QoS flow, e.g. DRB ID + QFI ID or some other identifier

- (MAC-04) We rule out explicit signalling of PDU session ID + QFI - (MAC-04) We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping

If a QFI and a DRB ID is used to indicate a QoS Flow in explicit signalling, the number of bits in MAC CE which indicates a QoS flow is 11 bits (QFI: 6bits, DRB ID: 5bits), which is too much overhead. Therefore, We should adopt implicit signalling to minimize overhead.

In implicit signaling, it’s the most efficient to indicate QoS flow in 2 step procedures:

(1) Pre-configuration via RRC; Introduce an RRC parameter to configure a list of QoS flows, represented by a combination of a QFI and a DRB ID. Although indicating either a combination of a QFI and a DRB ID or of a QFI and a PDU session ID will work, a DRB ID is slightly preferable since the DRB ID is shorter. This list is referred to as RateControlQoSFlowList in TP below.

CellGroupConfig ::=                        SEQUENCE {

---unchanged sections excluded---

[[

rateControlQoSFlowList-r19    SetupRelease {RateControlQoSFlowList-r19}	OPTIONAL,   -- Need M

]]

}

RateControlQoSFlowList-r19 ::= SEQUENCE (SIZE(1..maxNrofRateControlQoSflow-r19)) OF RateControlQoSFlow-r19

RateControlQoSFlow-r19 ::= SEQUENCE {

drb-ID               DRB-Identity,

qfi                        QFI

}

RateControlQoSFlowList A list of QoS flows to be indicated in UL Rate Control MAC CE, as specified in TS 38.321 [3].

(2) Bitmap-based signaling in the MAC CE; The MAC CE should include an 16 bits bitmap corresponding to the RateControlQoSFlowList, to indicate 16 QoS flows. Each bit (0 or 1) in the bitmap represents the presence or absence of a specific QoS flow.

Fig: Potential format for UL Rate Control MAC CE - QFi: QFi field set to 1 indicates the presence of Bit Rate field of i+1th QoS flow indicated by RateControlQoSFlowList preconfigured by RRC in procedure (1). QFi field set to 0 indicates the absence of Bit Rate field of i+1th QoS flow indicated by RateControlQoSFlowList preconfigured by RRC in procedure (1). - Bit Rate： This field indicates Bit Rate Index of the indicated QoS flow. Bit Rate Index corresponds to bit rate in Table 6.1.3.x-1.

This approach allows multiple QoS flows to be indicated simultaneously in the MAC CE with minimal overhead. By using a bitmap, each QoS flow can be represented with just one bit, eliminating the need to repeatedly transmit a full ID such as a combination of a QFI and a DRB ID.

Proposal 1.	(MAC-04) We should adopt the implicit signaling, i.e., a list of combinations of a QFI and a DRB ID is configured by RRC reconfiguration in advance, and then a bitmap in the MAC CE which corresponds to the list preconfigured in RRC, indicates one or multiple QoS flows.

  - Logical channel priority of the UL Rate Control MAC CE

The following issue has been presented by a company in the email discussion: [POST130][505][XR] MAC running CR and open issues (Qualcomm).

(offino) Logical channel priority of the UL Rate Control MAC CE. We understand this has been captured as the same priority as the legacy Recommended bit rate MAC CE in the current running CR. However, it seems we haven’t had agreement for this, and the characteristic of this UL Rate Control MAC CE is different from the legacy Recommended bit rate query MAC CE, probably we can discuss this to see companies’ view.

For DL-SCH, the UL Rate Control MAC CE indicated is used to limit bit rates in uplink when the network congestion occurs or is predicted to occur. Similarly, the existing Recommended Bit Rate MAC CE is to provide the UE with information about the bit rates which the gNB recommends to mitigate the network congestion which is recognized that the UL Rate Control MAC CE has the same feature as the Recommended Bit Rate MAC CE in terms of urgency from the network point of view.

On the other hand, for UL-SCH, the UL Rate Control MAC CE is used to indicate the desired bit rate to the gNB without taking into account the congestion in the network, similarly to Recommended Bit Rate MAC CE.

Then we would like to discuss the priority between the Recommended Bit Rate Query MAC CE and the UL Rate Control MAC CE. For example, if it is clarified that a UE supporting the UL Rate Control MAC CE does not transmit the Recommended Bit Rate Query MAC CE anymore, then assigning the same priority to both MAC CEs would be reasonable.

On the other hand, if there are UEs that support the transmission of both MAC CEs simultaneously, our view is that it is preferable from operation point of view to prioritize obtaining per-service level requested bit rate information. Therefore, rather than leaving the priority decision to UE implementation, it is desirable to assign the UL Rate Control MAC CE one level higher than the Recommended Bit Rate Query MAC CE, and one level lower than data from any Logical Channel.

Proposal 2.	If UEs do not simultaneously support transmission of the UL Rate Control MAC CE and the Recommended Bit Rate Query MAC CE, the priority of the two MAC CEs is set to the same level.

Proposal 3.	If UEs can simultaneously support transmission of the UL Rate Control MAC CE and the Recommended Bit Rate Query MAC CE, the priority of the UL Rate Control MAC CE is set to one level higher than the Recommended Bit Rate Query MAC CE and one level lower than data from any Logical Channel.

- Summary and proposal

Proposal 1.	(MAC-04) We should adopt the implicit signaling, i.e., a list of combinations of a QFI and a DRB ID is configured by RRC reconfiguration in advance, and then a bitmap in the MAC CE which corresponds to the list preconfigured in RRC, indicates one or multiple QoS flows.

Proposal 2.	If UEs do not simultaneously support transmission of the UL Rate Control MAC CE and the Recommended Bit Rate Query MAC CE, the priority of the two MAC CEs is set to the same level.

Proposal 3.	If UEs can simultaneously support transmission of the UL Rate Control MAC CE and the Recommended Bit Rate Query MAC CE, the priority of the UL Rate Control MAC CE is set to one level higher than the Recommended Bit Rate Query MAC CE and one level lower than data from any Logical Channel.

- References

[1]　R2-2503361, “List of open issues in MAC,” Qualcomm Incorporated, RAN2#130.