3GPP TSG-RAN WG2 Meeting #131bis	R2-2506877

Prague, Czech Republic, 13-17 Oct 2025

Agenda Item:	8.13.3

Source:	NEC

Title:	Remaining issues for Multi-hop Relay

Document for:	Discussion, Decision

# Introduction

This paper discusses the solutions for a remaining issue for Multi-hop Relay.

# Discussion

In the RAN2 # 130 meeting, it was agreed to extend the T300, T301 and T319 timers for multi-hop U2N relay operation such that the UE and gNB will multiply the signalled timer value by the hop count.

During the discussion in RAN2#131, it was agreed to exclude the option of defining new multihop timers for T300/T301/T319. It was also agreed that whether to multiply the Rel-17 timers or the legacy Uu-only timers can be confirmed in ASN.1 review.

In the agreed CR (R2-2506574) for TS38.331, our understanding is that the Rel-17 timers were simply multiplied by the hop count value as shown below.

UE-TimersAndConstantsRemoteUE field descriptions

t300-RemoteUE Indicates the timer value of T300 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t300 applies to L2 U2N Remote UE. The effective T300 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components,, is obtained by multiplying the base T300 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively.

t301-RemoteUE Indicates the timer value of T301 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t301 applies to L2 U2N Remote UE. The effective T301 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components, is obtained by multiplying the base T301 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively.

t319-RemoteUE Indicates the timer value of T319 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t319 applies to L2 U2N Remote UE. The effective T319 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components,, is obtained by multiplying the base T319 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively.

We think the hop count based timers may be too long for the Remote UE when there are multiple hops. Taking T300 as an example, T300 is a connection setup timer started by the Remote UE when it sends a connection request (RRCSetupRequest) to the network. The timer ensures that the network responds within a reasonable time, stopping when the Remote UE receives an RRCSetup or RRCReject message. If the timer expires without a response, the Remote UE considers the connection setup to have failed and may initiate a retry or transition to the RRC-IDLE state. As RAN2 has agreed to take only the UE in connected state to act as the Relay UE, Relay UE should be able to forward the received connection request (RRCSetupRequest) to the next hop without outstanding delay, and this should be true at every sidelink based hops. The longest delay over the chain may be the Uu hop. Hence, simply multiplying the timer T300 by the hop count value may be too long for the Remote UE, which may downgrade its access performance. In case of no response for its first connection request message, the Remote UE needs to wait for a long time before sending the next connection request message. Our proposal is to allow the Remote UE to choose based on its implementation to whether apply simply the legacy t300-RemoteUE or t300-RemoteUE*hopCount. This way may be a compromise between he UE implementation and UE access performance.

Proposal-1: For multi-hop scenarios involving two or more Relay UEs, allow the Remote UE to choose to whether apply the legacy t300-RemoteUE or t300-RemoteUE*hopCount based on UE implementation.

Proposal-2: Agree the corresponding change in the Annex.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: For multi-hop scenarios involving two or more Relay UEs, allow the Remote UE to choose to whether apply the legacy t300-RemoteUE or t300-RemoteUE*hopCount based on UE implementation.

Proposal-2: Agree the corresponding change in the Annex.

# Annex, TP

#### –	UE-TimersAndConstantsRemoteUE

The IE UE-TimersAndConstantsRemoteUE contains timers and constants used by the L2 U2N Remote UE in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE.

UE-TimersAndConstantsRemoteUE information element

-- ASN1START

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-START

UE-TimersAndConstantsRemoteUE-r17 ::= SEQUENCE {

t300-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t301-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t319-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

...

}

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-STOP

-- ASN1STOP

UE-TimersAndConstantsRemoteUE field descriptions

t300-RemoteUE Indicates the timer value of T300 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t300 applies to L2 U2N Remote UE. The effective T300 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components,, is obtained by multiplying the base T300 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. For multi-hop scenarios, the L2 U2N Remote UE chooses either t300-RemoteUE or t300-RemoteUE*hopCount based on its implementation.

t301-RemoteUE Indicates the timer value of T301 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t301 applies to L2 U2N Remote UE. The effective T301 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components, is obtained by multiplying the base T301 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. For multi-hop scenarios, the L2 U2N Remote UE chooses either t301-RemoteUE or t301-RemoteUE*hopCount based on its implementation.

t319-RemoteUE Indicates the timer value of T319 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t319 applies to L2 U2N Remote UE. The effective T319 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components,, is obtained by multiplying the base T319 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. For multi-hop scenarios, the L2 U2N Remote UE chooses either t319-RemoteUE or t319-RemoteUE*hopCount based on its implementation.