3GPP TSG-RAN WG2 Meeting #131bis	R2-2507138

# Prague, Czech Republic, 13 – 17 Oct. 2025

Agenda item:	10.2

Source:	MediaTek Inc.

Title:	Consideration of 6G NTN

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

# Introduction

According to the objectives in 6GR SI [1], in 6GR we aim at a harmonized 6G radio design for TN and NTN.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

To achieve a harmonized design for TN and NTN in 6GR NTN study, we have the following rule of thumb: (1) adopt common designs if possible, and (2) study functional blocks requiring NTN-specific designs, due to e.g., physical characteristics of NTN and TN-NTN mobility.

This contribution provides our recommendations on the technology study direction to fulfil the objectives of 6GR, especially for NTN-specific features. We start from reviewing NTN-specific features introduced in 5G, and then discuss what’s new in 6G.

# Discussion

## Adopting 5G NTN-specific features in 6G

The study of 5G NTN including NR-NTN and IoT-NTN started from Rel-16, and many NTN-specific features have been specified throughout Rel-17 to Rel-19. The discussion and evaluation of these features took significant efforts, even if they have not been widely deployed in the field. Therefore, these NTN-specific features should be considered for 6G NTN to avoid duplicate discussions addressing the same issues.

Proposal 1:	RAN2 should study the 5G-NTN specific features and evaluate which features should be included for 6G-NTN.

Major NTN-specific features specified in 5G include:

- UP-related features

      - HARQ feedback disabling for DL/UL

- Mobility-related features

      - SMTC timing adjustment for connected mode and idle mode

      - RACH-less HO/CHO

      - Time-/Location-based mobility, including CHO combined with A4 event in connected mode and measurement initiation in idle mode

      - TN/NTN information, including TN coverage information broadcasted by NTN cells and NTN information provided in TN cells

      - Satellite switch with resync (for transparent payload in a quasi-earth fixed cell)

- Deployment-related features

      - Discontinuous coverage

      - Store-and-forward

In the following, we evaluate each feature and see whether it is feasible for 6G-NTN or not.

1. UP-related features

HARQ feedback disabling for DL/UL

To eliminate or reduce the HARQ process stalling, HARQ feedback disabling for DL and UL was introduced in Rel-17 NR-NTN and Rel-18 IoT-NTN. Although the problem may also be resolved by increasing the number of HARQ processes, HARQ feedback disabling is beneficial for long propagation delay scenario, e.g., GSO.  These features are also useful for real-time service to disable unnecessary re-transmission or to perform fast blind re-transmission. So, this feature should be considered for 6G-NTN.

Observation 1:  HARQ feedback disabling is useful for 6G-NTN.

2. Mobility-related features

SMTC timing adjustment

No matter which reference signal is utilized for measurement (which is under RAN1 discussion) in 6GR, the concept of measurement window and timing (i.e., SMTC) would be re-utilized for 6GR to facilitate the measurement on the neighbor cells.  For TN, the timing of SMTC is fixed, but for NTN, it needs adjustment due to the satellite movement.  In NR-NTN, the timing of SMTC for connected mode is adjusted by NW; instead, the timing of SMTC for idle mode is adjusted by UE itself.  There would be no difference for SMTC timing adjustment between NR-NTN and 6G-NTN.  However, gapless concept is also considered by many companies to reduce the interruption time of data transmission.  In this case, NW-based SMTC timing adjustment may be not needed.  As a result, SMTC-timing adjustment could be further studied after gapless discussion.

Observation 2:  The SMTC timing adjustment could be further studied after measurement gap discussion.

RACH-less HO/CHO

In 5G NTN, the TA could be derived by UE based on UE location information and satellite ephemeris information. Thus, RACH-less HO could be utilized to reduce the latency of HO and to reduce the RA resource congestion, especially for quasi-earth fixed cell.  In addition, RACH-less is also supported for the time-based CHO in NR-NTN.  However, the GNSS-free is proposed by many companies.  RACH-less may be not appliable for 6G-NTN if GNSS-free is adopted. The main idea for RACH-less HO is to reduce the access overhead in terms of latency and RA resource. Therefore, the low overhead access methods, e.g., RACH-less HO/CHO, should be studied for 6G-NTN HO.

Observation 3:  Low overhead access methods, e.g., RACH-less HO/CHO, should be studied for 6G-NTN.

Time-/Location-based mobility

For connected mode, time-based and location-based CHO combined with A4 event are utilized to reduce the HO command congestion and RA congestion.  In addition, since the signal strength variation between the edge and the center of an NTN cell is small, it is hard for network to determine handover triggering based solely on the measurement results. Therefore, time-based and location-based CHO with A4 event allow UE to determine the timing of HO. Besides, location-based events (i.e., event D1/D2) can also assist NW to determine HO. As a result, time-based/location-based mechanism for mobility are still useful for 6G-NTN.

For idle mode, time-based/location-based measurement initiation can bring the power saving benefit for UE in terms of cell reselection. They should be re-utilized in 6G-NTN.

Observation 4:  Time-based/Location-based mobility mechanisms are useful for 6G-NTN.

Assistant information for TN-NTN mobility

Regarding mobility between TN and NTN, in 5-NTN, the assistant information in the system information or RRC release message, e.g., satellite information in a TN cell and TN coverage information in an NTN cell, to assist UE performing cell reselection or redirection can bring the power saving gain for UE.  Therefore, providing the assistant information for cell reselection or re-direction between TN and NTN should be re-utilized in 6G-NTN.

Observation 5: 	Cell-reselection or re-direction assistant information for TN-NTN mobility is useful for 6G-NTN, e.g., TN coverage information in an NTN cell and NTN-related information in a TN cell.

Satellite switch with re-sync

For the transparent mode in a quasi-earth fixed cell, the satellite switch with re-sync is introduced in Rel-18 to avoid the HO overhead. In addition, based on network capability and UE capability, the satellite switch with re-sync supports soft switch and hard switch.  Since it can avoid the HO overhead, it should be supported for 6G NTN including hard and soft switch at least for transparent mode in a quasi-earth fixed cell.

Observation 6:  Satellite switch with re-sync (including hard switch and soft switch) is useful for 6G-NTN.

3. Deployment-related features

Discontinuous coverage

In NR-NTN, coverage continuous is assumed.  But for IoT-NTN, discontinuous coverage is considered due to the sparse satellite deployment for IoT service.  However, for LEO deployment, to provide continuous coverage needs thousands of satellites and it may take long time to deploy/launch these satellites.  Thus, it is hard to provide continuous coverage, especially for the initial deployment phase so discontinuous coverage should be considered in 6G NTN.  Meanwhile, 6GR also needs to support IoT service and satellite may be deployed for IoT service only.  As a result, to cover the initial deployment phase and IoT service due to the sparse satellite deployment, discontinuous coverage should be considered in 6G-NTN.

Observation 7:  For initial satellite deployment and provision of IoT service, discontinuous coverage should be considered for 6G-NTN.

Store-and-forward

Store-and-forward mechanism is introduced for IoT NTN to serve the area where the feeder link is unavailable.  It can provide non-real time service, especially IoT service in the area where there is no feeder link.  For example, it is useful for the service area in ocean or desert.  Even though the inter-satellite link can resolve the issue of satellite without feeder link, it cannot guarantee that the inter-satellite link is always available, especially for sparse satellite deployment or low capability satellite for IoT service. Thus, store and forward should be considered for 6G-NTN.

Observation 8:  Store-and-forward should be considered for 6G-NTN.

Proposal 2:	Among 5G-NTN features, at least the following shall be considered for 6G-NTN:

- HARQ feedback disabling for DL/UL

- Low overhead access (e.g., RACH-less like), in particular for mobility

- Time-/Location-based mobility mechanism

- Assistant information for TN-NTN mobility

- Satellite switch with resync

- Discontinuous coverage

- Store and forward

## 6G NTN study

### New NTN deployment scenarios and requirements

Based on the 6G deployment scenarios and requirements studied in RAN plenary [2], there may be some new deployment scenarios and requirements for 6G-NTN in addition to 5G-NTN scenarios.  For example, case 2 of quasi-earth fixed cell, multi-orbit, multi-beam cell, and TN-NTN integration are proposed and discussed in RAN plenary.  RAN2 can wait for the outcome or the decision from RAN plenary and then start to study these new scenarios and requirements.

Proposal 3:  RAN2 should start to study the new 6G NTN deployment scenarios/requirement after approved by RAN plenary.

### New NTN-specific features

Since a lot of NTN-specific features are already discussed and specified in 5G-NTN (IoT-NTN and NR-NTN), a new proposed feature for 6G-NTN should be more carefully evaluated to avoid multiple options/solutions to resolve the same issue.  Optimization of the 5G-NTN specific features or new features for 6G-NTN should be further evaluated by analysis or based on the test result of 5G-NTN in the field.

Proposal 4:  Any 6G feature specific to NTN should be motivated with analysis.

# Conclusion

Based on the above analysis, we have the following observations and proposals.

Observations:

Observation 1:  HARQ feedback disabling is useful for 6G-NTN.

Observation 2:  The SMTC timing adjustment could be further studied after measurement gap discussion.

Observation 3:  Low overhead access methods, e.g., RACH-less HO/CHO, should be studied for 6G-NTN.

Observation 4:  Time-based/Location-based mobility mechanisms are useful for 6G-NTN.

Observation 5: 	Cell-reselection or re-direction assistant information for TN-NTN mobility is useful for 6G-NTN, e.g., TN coverage information in an NTN cell and NTN-related information in a TN cell.

Observation 6:  Satellite switch with re-sync (including hard switch and soft switch) is useful for 6G-NTN.

Observation 7:  For initial satellite deployment and provision of IoT service, discontinuous coverage should be considered for 6G-NTN.

Observation 8:  Store-and-forward should be considered for 6G-NTN.

Proposals:

Proposal 1:	RAN2 should study the 5G-NTN specific features and evaluate which features should be included for 6G-NTN

Proposal 2:	Among 5G-NTN features, at least the following shall be considered for 6G-NTN:

- HARQ feedback disabling for DL/UL

- Low overhead access (e.g., RACH-less like), in particular for mobility

- Time-/Location-based mobility mechanism

- Assistant information for TN-NTN mobility

- Satellite switch with resync

- Discontinuous coverage

- Store and forward

Proposal 3:  RAN2 should start to study the new 6G NTN deployment scenarios/requirement after approved by RAN plenary.

Proposal 4:  Any 6G feature specific to NTN should be motivated with analysis.

# References

- RP-252912	New SID: Study on 6G Radio, NTT DOCOMO (Moderator), RAN#109, Sep. 2025.

- RP-252882	TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, RAN#109, Sep. 2025.