3GPP TSG-RAN WG2 Meeting #131bis			R2-2507205

Prague, Czech Republic, October 13-17, 2025

Source:	NTT DOCOMO, INC.

Title:	Views on 6G general aspects

Agenda item:	10.2

Document for:	Approval

- Introduction

Revised SID Study on 6G Radio has been approved in RAN#109 [1]. This contribution provides general aspects regarding to 6G RAN2 study.

- Discussion

  - 6G RAN architecture and migration requirements.

According to latest approved pCR for 38.914 on Requirements for architecture and migration for 6G [2] in RAN plenary#109, the following requirements should be taken into consideration into RAN2 study.

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The 6G RAN architecture shall allow non-public networks.

Proposal1: RAN2 study should be based on the above 6G RAN architecture and migration requirements captured in TR 38.914.

Regarding additional migration options (other than standalone, MRSS, and inter-RAT mobility between NR-6G), RAN will start the study in March 2026 and will make a decision by September 2026 whether expand WG SI scope to cover additional migration option(s) as stated in [1]. RAN will also task relevant RAN WGs for any specific technical analysis. Therefore, RAN2 should wait for liaison from RAN regarding additional migration options study.

Observation1: RAN will start the study in March 2026 and will make a decision by September 2026 whether expand WG SI scope to cover additional migration option(s). RAN will also task relevant RAN WGs for any specific technical analysis.

Proposal2: RAN2 should wait for liaison from RAN regarding additional migration options study.

  - UE capability framework

Excessive UE capability reporting signaling is considered as an issue in today’s MNO networks. In the latest RAN Plenary #109, as noted in [3], Proposal 2 (highlighted in yellow) suggests that 6GR aims to specify a set of mandatory baseline functionalities for all UEs, with details to be further studied, by taking into account diverse device types for scalable and forward compatible design, and the requirements, as necessary, as in the 6G SID. This approach is expected to help reduce unnecessary signaling related to UE capability reporting.

Additionally, although UE radio capability signaling optimization (RACS) was standardized in Release 16, it has not yet been commercialized by MNOs. Therefore, we believe that a simplified version (easy implementable) of RACS should be considered for inclusion in the initial phase of 6G (Day 1) to address the UE capability signaling efficiency.

Observartion2: Excessive UE capability reporting signaling is considered as an issue in today’s MNO networks.

Observation3: RAN plenary is studying that 6GR aims to specify a set of mandatory baseline functionalities for all UEs, with details to be further studied, by taking into account diverse device types for scalable and forward compatible design, and the requirements, as necessary, as in the 6G SID.

Prposal3: RAN2 should design a new UE capability framework to minimize excessive signaling related to UE capability reporting.

Proposal4: RAN2 to study a simplified (easy implementable) version of RACS to address the UE capability signaling efficiency.

Another lesson learnt from NR is optionality of UE capabilities. Most of NR UE capabilities are optional, which leads to the chicken-egg problem: the NW cannot achieve expected gain unless a capability is supported by majority of UEs, while UEs would not support the capability unless many NWs support the feature.

Moreover, from operator perspective, it is difficult to estimate cost to introduce a feature since it is not clear which capability is important to enable the basic of the feature and which capability is for enhancement on the feature.

Therefore, we believe that it is important to establish a 6G UE capability framework which encourages to visualize basic and enhancing capabilities. Here, the term 'basic capability' refers not so much to a capability commonly held by all UEs, but rather to a UE capability that should be commonly deployed when utilizing a specific feature.

Observation4: The optional nature of most NR UE capabilities led to limited deployment and made cost estimation difficult for operators.

Proposal5: RAN2 should design a UE capability framework to associate capabilities to a feature and visualize basic capabilities and enhancing capabilities for the feature.

# 3	Conclusion

In this contribution, the follows are proposed.

- 6G RAN architecture and migration requirements

Proposal1: RAN2 study should be based on the 6G RAN architecture and migration requirements captured in TR 38.914.

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The 6G RAN architecture shall allow non-public networks.

Observation1: RAN will start the study in March 2026 and will make a decision by September 2026 whether expand WG SI scope to cover additional migration option(s). RAN will also task relevant RAN WGs for any specific technical analysis.

Proposal2: RAN2 should wait for liaison from RAN regarding additional migration options study.

- UE capability framework

Observartion2: Excessive UE capability reporting signaling is considered as an issue in today’s MNO networks.

Observation3: RAN plenary is studying that 6GR aims to specify a set of mandatory baseline functionalities for all UEs, with details to be further studied, by taking into account diverse device types for scalable and forward compatible design, and the requirements, as necessary, as in the 6G SID.

Prposal3: RAN2 should design a new UE capability framework to minimize excessive signaling related to UE capability reporting.

Proposal4: RAN2 to study a simplified (easy implementable) version of RACS to address the UE capability signaling efficiency.

Observation4: The optional nature of most NR UE capabilities led to limited deployment and made cost estimation difficult for operators.

Proposal5: RAN2 should design a UE capability framework to associate capabilities to a feature and visualize basic capabilities and enhancing capabilities for the feature.

# 4	References

[1]	RP-252912  “Revised SID: Study on 6G Radio”  NTT DOCOMO, CMCC, AT&T, Vodafone

[2]	RP-252870  “pCR for 38.914 on Requirements for architecture and migration for 6G”  NTT DOCOMO

[3]	RP-252873   “Summary of 6G Diverse Device Types (including Minimum UE Bandwidth)”  Apple