3GPP TSG RAN WG2 Meeting #131bis	 R2-2507317

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	10.3.3

Source: 	ITL

Title: 	Discussion on common user plane and control plane for 6GR

Document for:	Discussion and decision

Introduction

In RAN#108, a new Study Item (SI) on 6G Radio (6GR) was approved [1]. This contribution discusses energy efficiency and radio bearer aspects in the context of the common user plane and control plane of the 6GR design, with the intention of facilitating further study in RAN2.

Discussion

Energy efficiency

As part of energy efficiency considerations, the SID defines the following objectives:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In NR, various features were introduced to improve energy efficiency. On the UE side, techniques such as DRX and BWP help devices avoid unnecessary monitoring and processing. On the network side, removal of CRS and flexible scheduling options reduce overall power consumption. While effective, most of these mechanisms were primarily optimized for either the UE or the network perspective, without fully considering the trade-offs between the two.

Observation 1: In NR, energy saving schemes have mainly been designed from the perspective of either the network or the UE individually

For 6GR, it will be important to design energy saving features that benefit both sides simultaneously, ensuring a balanced approach that considers network efficiency, UE consumption, and overall system performance.

As an example, in a multi-carrier serving cell scenario, certain carriers may be configured with DRX/DTX to support both network and UE energy saving. From the UE perspective, this reduces control channel monitoring, while from the network perspective, reference signal transmission can be minimized, resulting in mutual energy gains.

Proposal 1: Study 6GR energy saving schemes that jointly address the perspectives of both UE and network

Multi-carrier serving cell & BWP (Bandwidth part)

In 5G NR, the concept of Bandwidth Part was introduced to address the challenge of supporting wide system bandwidths (up to 400 MHz) while maintaining device energy efficiency. Instead of requiring the UE to continuously monitor and process the full carrier bandwidth, NR allows the operation on a smaller portion of spectrum through BWP configuration.

A BWP is defined as a contiguous set of physical resource blocks within the overall carrier bandwidth.

UE Operation:
Each UE can be configured with multiple BWPs, but at a given time, only one downlink BWP and one uplink BWP are active.

- Initial BWP: Used at RRC connection setup or cell access.

- Active BWP: The currently used BWP for data transmission/reception.

- Switching: The network may instruct the UE to switch to another BWP depending on traffic demand, service requirement, or energy saving considerations.

Benefits:

- Enables UE power saving by operating on a narrower BWP under low traffic.

- Provides flexibility to switch to wider BWPs for higher throughput services (e.g., eMBB).

- Supports QoS differentiation by associating different BWPs with different service needs.

In 6G, the concept of a Multi-Carrier Serving Cell (MCSC) aims to treat multiple carriers as a single Serving Cell from a control and management perspective. While resources are spread across multiple frequency bands, the UE and the network interact with them through unified Serving Cell procedures.

- In 5G NR, BWP allows the UE to monitor and operate on a portion of the carrier bandwidth to save energy.

- A UE can be configured with multiple BWPs, but only one DL BWP + one UL BWP can be active at a time per serving cell.

- When multiple carriers are aggregated (CA), each carrier (serving cell) manages its own BWPs independently.

Proposal 2: Study the multi-carrier serving cell (MCSC) concept in the context of 6GR, considering the following aspects:

- Definition and scope of MCSC as an extension of serving cell concepts in NR

- Impact of MCSC on BWP and HARQ operation, including entity, process management, and timer operation

In the 6G MCSC environment, BWP operation should be studied in RAN2, since it directly impacts UE power saving, signaling, protocol procedures, and overall system efficiency. Compared to 5G NR, the following issues can be identified:

1. Unified vs. Per-Carrier BWP Control

- In 5G: each carrier has independent BWP operation and switching.

- In 6G MCSC: multiple carriers are grouped as one serving cell → question arises whether BWP should be managed per-carrier or jointly across carriers.

- Issue: Increased signaling complexity if per-carrier management remains; loss of flexibility if only unified management is enforced.

2. HARQ / DRX Alignment

- In 5G: BWP switching is linked to cell-specific DRX/HARQ timers.

- In MCSC: unified HARQ/DRX operation may conflict with carrier-specific BWP configurations.

- Issue: Potential mismatch between per-carrier scheduling needs and unified timers

3. Cross-Carrier Scheduling and Control Monitoring

- In 5G: PDCCH monitoring is tied to a specific BWP per serving cell.

- In MCSC: one control channel may need to schedule multiple carriers with potentially different active BWPs.

- Issue: UE complexity and energy consumption when monitoring control channels across multiple carriers.

4. Energy Saving vs. Performance Trade-off

- In 5G: narrow BWPs help reduce UE energy but limit throughput.

- In MCSC: simultaneous wideband operation across multiple carriers may dilute energy saving benefits unless joint BWP scaling is supported.

- Issue: How to design BWP scaling rules that balance performance and energy efficiency in multi-carrier environments.

Observation 2: New protocol support is needed in RAN2 to manage BWP switching and signaling across multiple carriers under a single serving cell

Proposal 3: RAN2 is invited to study enhancements of BWP operation for 6GR in the MCSC environment, including:

- Signaling and protocol aspects for unified or coordinated BWP management across carriers

- Mechanisms to ensure efficient HARQ/DRX alignment in multi-carrier serving cell operation

- Procedures for control channel monitoring and cross-carrier scheduling under BWP operation

- Solutions that balance UE energy saving and network performance when operating with multiple carriers

In NR, HARQ operation has been specified on a per-serving cell basis, with each serving cell maintaining its own HARQ entity and processes and associated timers. This design has proven effective in single-carrier serving cell environments, particularly for Carrier Aggregation (CA) operation, where independent HARQ management per serving cell provides robustness and flexibility.

Figure 1. Example of NR HARQ Entity for serving cell

In Figure 1, illustrates the structure of HARQ entities at the MAC layer in NR. Each serving cell (e.g., cell #X, #Y, #Z) is associated with its own independent HARQ entity, and within each HARQ entity multiple HARQ processes are maintained (e.g., Process 0 to Process 5). This representation highlights that HARQ operation is managed per serving cell, ensuring that retransmission processes, feedback timing, and timers are independently controlled even when multiple serving cells belong to the same cell group.

Figure 2. Example of Multi-carrier serving cell

Looking forward to 6G radio (6GR), the framework is expected to support tighter integration of multiple serving cells and carriers, possibly with unified scheduling and advanced aggregation concepts. The Figure 2 illustrates the difference between carrier-to-cell mapping in 5G NR and the envisioned multi-carrier serving cell (MCSC) concept in 6GR. In 5G NR, each carrier is associated with an individual serving cell (e.g., Carrier #A with Cell #X, Carrier #B with Cell #Y, Carrier #C with Cell #Z). In contrast, the MCSC approach, where multiple carriers (e.g., Carrier #A, Carrier #B, Carrier #C) can be integrated under a single serving cell (Cell #X). This unification allows for simplified cell management, reduced signaling overhead, and a more flexible HARQ operation framework across multiple carriers within one serving cell.

Together, these two illustrations show why the current per-serving cell HARQ entity, while effective in NR for CA scenarios, may need to be revisited for 6GR. With MCSC unifying multiple carriers under a single serving cell, HARQ process instantiation and timer operation could evolve from a strictly per-serving cell basis to group-based or unified structures, better aligned with the 6GR framework.

Observation 3: The NR approach of maintaining a separate HARQ entity per serving cell is not well suited for the 6GR MCSC environment, where multiple carriers are unified under a single serving cell

Therefore, in 6GR MCSC environments, a clear definition of how HARQ entities and processes should be instantiated and operated is required to ensure consistent timer alignment, process indexing, and feedback handling across carriers.

- How HARQ process indexing and entity instantiation should be adapted for MCSC.

- Methods for aligning HARQ timers across carriers when the serving cell boundary is unified.

- Mechanisms for efficient feedback signaling (ACK/NACK) across multiple carriers under one HARQ entity.

Several potential approaches can be considered for HARQ configuration and operation in multi-carrier serving cell environments:

- Option A: Maintain per-serving cell HARQ operation (NR approach)
– Each serving cell retains independent HARQ processes and timers.

- Option B: Group-based HARQ operation across serving cells
– Serving cells within a group share HARQ configuration and may pool HARQ processes.

- Option C: Unified HARQ operation across multiple serving cells
– Multiple serving cells are treated as a single HARQ entity with unified process indexing and timers.

The outcome of this study will provide a baseline for defining HARQ operation that is both efficient and robust in future 6GR deployments supporting MCSC.

Proposal 4: RAN2 is invited to study HARQ operation in the 6GR MCSC environment, considering the following aspects:

- Definition of HARQ entity instantiation and process management in MCSC

- Alignment of HARQ timers across multiple carriers within a single serving cell

- Efficient design of HARQ ACK/NACK feedback signaling in unified HARQ operation

Radio bearer for AI/ML and sensing data

In NR, radio bearers are defined as logical channels to carry either control plane signaling (SRBs) or user plane data (DRBs). The current bearer framework is primarily designed for eMBB, URLLC, and mMTC traffic with IP-based QoS flows.

With 6G radio (6GR), new service categories such as AI-native traffic, sensing, XR/metaverse applications, and industrial deterministic services are expected to generate data with very different characteristics. These include bursty model-update traffic, high-rate sensing streams, multi-stream XR flows, and deterministic periodic transmissions. The existing NR radio bearer structure may not be sufficient to efficiently support these diverse traffic types.

Figure 3. NR Radio Bearer vs. Enhanced Radio Bearer Framework for 6GR

In NR, SRBs carry control plane signaling and DRBs carry user plane data, with QoS flows mapped to DRBs. Bearer management is tied to individual serving cells, which is effective for current 5G services but limited for new service categories. In 6GR, new bearer categories such as AI/ML bearer, sensing bearer, and flexible DRB could be considered alongside extended SRBs.

Several issues arise when applying the current NR radio bearer design to 6GR services:

- QoS granularity limitations: Current DRB–QoS flow mapping is not flexible enough for fine-grained differentiation of AI, sensing traffic

- Signaling overhead: Establishment and release of bearers can cause excessive RRC signaling for dynamic and bursty traffic

- Cross-layer mismatch: Bearer QoS attributes are not tightly coupled with MAC/PHY parameters needed for sensing or deterministic traffic

Observation 4: In NR, DRBs can carry multiple QoS flows with attributes mainly designed for IP-based traffic.

Observation 5: For 6G traffic such as AI/ML and sensing, additional attributes like deterministic latency and synchronization are needed.

To address the above challenges, enhancements to the radio bearer framework in 6GR can be considered:

- New bearer types: Define dedicated bearer categories for AI/ML traffic and sensing data, enabling tailored QoS, reliability, and scheduling support

- Lightweight bearer management: Introduce dynamic, temporary, or semi-persistent bearer mechanisms to efficiently handle bursty or short-lived traffic

Proposal 5: RAN2 is invited to study radio bearer operation for 6GR, considering the need to accommodate diverse traffic types such as AI/ML and sensing and considering the following aspects:

- Extending QoS attributes for new service requirements

- Evaluating new bearer categories (AI/ML, sensing, flexible DRB)

- Investigating lightweight bearer management procedures

Conclusion

In this contribution, We discussed the energy efficiency and radio bearer aspects. According to the discussion in Section 2, we have the following observations and proposals:

Observation 1: In NR, energy saving schemes have mainly been designed from the perspective of either the network or the UE individually

Proposal 1: Study 6GR energy saving schemes that jointly address the perspectives of both UE and network

Proposal 2: Study the multi-carrier serving cell (MCSC) concept in the context of 6GR, considering the following aspects:

- Definition and scope of MCSC as an extension of serving cell concepts in NR

- Impact of MCSC on HARQ operation, including entity instantiation, process management, and timer alignment

Observation 2: New protocol support is needed in RAN2 to manage BWP switching and signaling across multiple carriers under a single serving cell

Proposal 3: RAN2 is invited to study enhancements of BWP operation for 6GR in the MCSC environment, including:

- Signaling and protocol aspects for unified or coordinated BWP management across carriers

- Mechanisms to ensure efficient HARQ/DRX alignment in multi-carrier serving cell operation

- Procedures for control channel monitoring and cross-carrier scheduling under BWP operation

- Solutions that balance UE energy saving and network performance when operating with multiple carriers

Observation 3: The NR approach of maintaining a separate HARQ entity per serving cell is not well suited for the 6GR MCSC environment, where multiple carriers are unified under a single serving cell

Proposal 4: RAN2 is invited to study HARQ operation in the 6GR MCSC environment, considering the following aspects:

- Definition of HARQ entity instantiation and process management in MCSC

- Alignment of HARQ timers across multiple carriers within a single serving cell

- Efficient design of HARQ ACK/NACK feedback signaling in unified HARQ operation

Observation 4: In NR, DRBs can carry multiple QoS flows with attributes mainly designed for IP-based traffic.

Observation 5: For 6G traffic such as AI/ML and sensing, additional attributes like deterministic latency and synchronization are needed.

Proposal 5: RAN2 is invited to study radio bearer operation for 6GR, considering the need to accommodate diverse traffic types such as AI/ML and sensing and considering the following aspects:

- Extending QoS attributes for new service requirements

- Evaluating new bearer categories (AI/ML, sensing, flexible DRB)

- Investigating lightweight bearer management procedures

Reference

RP-252912 revised SID: Study on 6G radio