3GPP TSG-RAN WG2 Meeting #131bis	R2-2507427

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	8.13.2

Source:	Ericsson

Title:	Discussion on RIL E044 and E029

Document for:	Discussion, Decision

# 1 	Introduction

In this contribution, we discuss the RIL issue E044 and E029.

# 2 	Discussion

## 2.1 	RIL issue E044

The RIL issue E044 is described in the following

=================================================================================

NOTE 4:	The L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on any information available in the discovery message including the RRC state information. The RRC state information in the discovery message RRC container reflects the state of the UE that sends the discovery message.

The capturing of the above note is not aligned with RAN2 agreements

- It needs to capture that RRC state may be considered by remote UE in multi-hop relay scenario.

- RAN2 didn’t agree that remote UE can consider any other information in discovery, other than RRC state.

=================================================================================

Based on the above description, this RIL issue needs to be corrected.

The note doesn’t capture that the note is only applicable to multi-hop L2 U2N Relay communication.

For the bullet 2), we think it is unclear or infeasible for the remote UE to consider any information other than RRC state, since some information in the discovery message e.g., accumulated QoS etc are upper layer information, which may be not available to the AS layer.

The note is unclear if the remote UE is allowed to consider any information in discovery other than the RRC state.

Therefore, we suggest corrections to the note for the aspect in bullet 1) as the below.

Include the condition in the note 4 that the note is only applicable to multi-hop L2 U2N Relay communication.

Therefore, we suggest corrections to the note for the aspect in bullet 2) as the below.

Update the note 4 that the L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on the RRC state information in the discovery message.

we have also prepared the TP captured in clause 4.

Adopt the TP for RIL E044 captured in section 4.

## 2.2 	RIL issue E029

Description]:

##### 5.8.9.9.X	Reception of the UuMessageTransferSidelink by the L2 Intermediate U2N Relay UE

Upon receiving the UuMessageTransferSidelink message from the connected L2 U2N Parent Relay UE, the L2 Intermediate U2N Relay UE shall:

1>	if sl-PagingDelivery contains the ue-Identity of the child UEs:

2>		consider the paging message of the child UE is acquired;

1>	if sl-SystemInformationDelivery requested by the child UEs and/or sl-SIB1-Delivery is included:

2>		consider the SIB requested by the child UE is acquired;

Bullet 2) highlighted is not correct, since SIB1 may or may not be requested by child Ues,

[Proposed Change]:

in this case. We shall reformulate “2>		consider the SIB requested by the child UE is acquired;”

2>	consider the SIBs valid for child UEs is acquired;

[Comments]:

[Rapporteur]: Agree to clarify the it with slightly different wording as suggested above . Have changed the status from “ToDo” to “PropAgree”.

2>		consider the SIBs (other than SIB1) requested by the child UE is acquired

For the RIL issue E029, we don’t think the rapporteur suggested change makes sense. The reason is that

Regarding SIB1 forwarding, it has been captured in clause 16.12.5.5 of the stage 2 spec as in the following

For SIB1 forwarding, for L2 U2N Remote UE, both request-based delivery (i.e., SIB1 request by the U2N Remote UE) and unsolicited forwarding are supported by L2 U2N Relay UE, of which the usage is left to L2 U2N Relay UE implementation. If SIB1 changes, for L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, the L2 U2N Relay UE always forwards SIB1.

It means that SIB1 can be forwarded by the U2N relay UE to the remote UE without the request message from the remote UE, i.e., unsolicited forwarding.

SIB1 can be forwarded by the U2N relay UE to the remote UE without the request message from the remote UE, i.e., unsolicited forwarding by the U2N relay UE.

In this case, the suggested change by the rapporteur would rule out the unsolicited forwarding scenario for SIB1.

The rapporteur suggested change would rule out the unsolicited forwarding scenario for SIB1.

Therefore, we think the correction to the RIL issue E029 should be

2>	consider the SIBs valid for child UEs is acquired;

For RIL E029, agree the below correction

- “2> consider the SIB requested by the child UE is acquired;” is corrected as “2>	consider the SIBs valid for child UEs is acquired”.

we have also prepared the TP captured in clause 5.

Adopt the TP for RIL E029 captured in section 5.

# 3 	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include the condition in the note 4 that the note is only applicable to multi-hop L2 U2N Relay communication.

Proposal 2	Update the note 4 that the L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on the RRC state information in the discovery message.

Proposal 3	Adopt the TP for RIL E044 captured in clause 4.

Proposal 4	For RIL E029, agree the below correction

“2> consider the SIB requested by the child UE is acquired;” is corrected as “2>	consider the SIBs valid for child UEs is acquired”.

Proposal 5	Adopt the TP for RIL E029 captured in clause 5.

# 4	Text proposal on RIL E044

NOTE 4:	In case of multi-hop L2 U2N Relay communication, the L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on the RRC state information in the discovery message. The RRC State information in the discovery message RRC container reflects the state of the UE that sends the discovery message.

# 5	Text proposal on RIL E029

##### 5.8.9.9.X	Reception of the UuMessageTransferSidelink by the L2 Intermediate U2N Relay UE

Upon receiving the UuMessageTransferSidelink message from the connected L2 U2N Parent Relay UE, the L2 Intermediate U2N Relay UE shall:

1>	if sl-PagingDelivery contains the ue-Identity of the child UEs:

2>		consider the paging message of the child UE is acquired;

1>	if sl-SystemInformationDelivery requested by the child UEs and/or sl-SIB1-Delivery is included:

2>		consider the SIB valid for the child UE is acquired;