3GPP TSG-RAN WG2 Meeting #131bis	R2-2507611

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on remaining RIL [V211, C001, V215, S900 and S901] for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we discuss the RILs marked as ToDo in the ASN.1 review for IoT NTN, including V211, C001, V215, S900 and S901.

# 2	Discussion

## 2.1	V211

# V211 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. V211; IoTNTN; 1; Indicate the regenerative operation mode to NAS; Yes, R2-250xxxx; vivo (Stephen); v003; ToDo. [Description]: As per the current spec, when t-ModeSwitching is not configured for the transition from S&F to regenerative, upon the satellite transitions from S&F to regenerative, no mode indication of regenerative mode is indicated to NAS. The NAS may always consider the cell is in S&F mode. It is incorrect. [Proposed Change]: We propose that the RRC of an S&F capable UE shall indicate the cell is operating in regenerative mode if sf-OperationMode is not present. Alternatively, t-ModeSwitching shall always be configured by the network when the cell is operating in S&F. [Comments]: Rapporteur’s comments: If there is no mode switching from SF mode to normal mode in this cell, this issue doesn’t exist. If there is mode switching from SF to normal in this cell, since both the sf indication and sf-ModeSwitching will be forwarded, NAS will know when the mode switches. We believe it is difficult to mandate broadcasting sf-ModeSwitching in the spec and it can be up NW implementation. This issue is left open for now and proponent can submit contribution to discuss in the next meeting.

In RAN2-131 meeting, the LS from CT1 on usage of SF-mode switching timer information for NAS layer, CT1 responded that this parameter is not needed. This also means that whenever the AS layer knows the mode transition and if it is informed to NAS layer it will be sufficient for NAS layer for further resuming pending operation. In our view the current mode of operation needs to be known to NAS to decide on its action for NAS signalling transmission. This is also applicable for application higher layers.

Observation 1: NAS layer needs to know the current operating mode of the cell to decide on its NAS signalling transmission.

Proposal 1: AS layer informs NAS on SF mode transition from SF to Normal and vice versa.

## 2.2	C001

# C001 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. C001; IoTNTN; 2; sf-OperationModeNeigh; Da Wang; V002; ToDo. [Description]: As we comment during the email discussion, we think the intention of agreement is to add S&F mode and nomal mode indication in SIB33, which doesn’t have the intention to include “barred” and “notBarred” in SIB33. Because currently the barred/notBarred information is not provide from neighbour cell to serving cell. [Proposed Change]: NeighSatelliteInfo-v19xy ::= SEQUENCE { sf-OperationModeNeigh-r19 ENUMERATED {barred, notBarreds&f, normal} OPTIONAL, -- Need OP t-ModeSwitchingNeigh-r19 TimeOffsetUTC-r17 OPTIONAL -- Need OR } [Comments]: Rapporteur’s comments: This was discussed and companies have different understanding on the intention of the agreement. So we need to stick to the agreement itself for now: - The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite. Besides, this is aligned with the NOTE added in 36.304, that a UE may deprioritize or choose not to access a SF cell based on this indication. Companies can submit contributions to this issue.

In RAN2-131, it was agreed to introduce SF mode and associated timer parameter in the neighbour cells. For this purpose the same parameter which was used in serving cell can be introduced in the neighbour cell information. In the serving cell system information, the presence of the parameter implicitly indicates the cell is operating in SF mode. When the cell switches to normal mode this parameter will be absent and the cell will behave like a legacy normal cell and all the UE can access the cell.

Observation 2: For serving cell, the presence of SF-operation-mode parameter implicitly indicates whether cell operates in SF mode or not.

It is possible to extend the same parameter in the neighbour cell as such. When neighbour cell is not operating in SF mode this optional parameter can be absent indicating it is operating in normal mode. However, this parameter also comes with additional information that whether SF mode operation is barred or not barred. If these two values are supported in neighbour cell then UE behaviour on how the UE should handle if this parameter is set to barred also need to be handled as part of cell reselection. To simplify the UE behaviour this parameter SF-mode can be changed to SF-mode with single value ENUMERATED {true} and it is included only when SF mode is not barred in the neighbouring cell. In all other cases this parameter can be absent.

Proposal 2: Consider the following options for neighbour cell SF mode indication.

- Option 1: The SF-operating-mode parameter in neighbour cell is provided as

NeighSatelliteInfo-v19xy ::=	SEQUENCE {

sf-OperationModeNeigh-r19		ENUMERATED {TRUE}	OPTIONAL,	-- Need OP

t-ModeSwitchingNeigh-r19		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

- Option 2:

NeighSatelliteInfo-v19xy ::=	SEQUENCE {

t-ModeSwitchingNeigh-r19		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

If this parameter is present, it indicates neighbour cell operate in SF mode. In this option we can also save some bits for neighbour cell

## 2.3	V215

# V215 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. V215; IoTNTN; 2; Removal of p0-UE-PUSCH-r19; Yes, R2-250xxxx; vivo (Stephen); v008; ToDo. [Description]: The field p0-UE-PUSCH-r19 is totally not used in CB-Msg3 PUSCH power control, as per the P0 formula from MAC - the CB-MSG3_RECEIVED_TARGET_POWER is set to cb-Msg3-InitialReceivedTargetPower + (CB_MSG3_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep; Note that the p0-UE-PUSCH for PUR is used as a power offset to clacluate the initial received target power for PUR based on preambleInitialReceivedTargetPower. As we had introduce cb-Msg3-InitialReceivedTargetPower-r19, there is not need to use p0-UE-PUSCH and preambleInitialReceivedTargetPower for CB-Msg3 PUSCH. [Proposed Change]: Removal of p0-UE-PUSCH-r19 from both CB-Msg3-ConfigSIB and cb-Msg3-ConfigSIB-NB [Comments]: Rapporteur’s comments: I have some sympathy with this proposal. Since the proponent is going to prepare a contribution, we can discuss during the next meeting.

In the agreed Rel-19 RRC CR for IoT NTN (R2-2506573), the parameter p0-UE-PUSCH-r19 was introduced with value range of -8 dB to +7 dB, which is a copy of the p0-UE-PUSCH-r16 parameter defined for PUR. This reflects RAN2’s intention to reuse the PUR power control mechanism for CB-Msg3. For PUR, the p0-UE-PUSCH-r16 parameter represents a UE-specific power offset on top of the cell-level target UL power level (P_0) which is range from -126dBm to +24 dBm. The parameter is included in the pur-Config and can be signalled to the UE via the RRCConnectionRelease message to configure the UE-specific power offset for PUR transmissions.

However, for CB-Msg3-EDT, the newly introduced p0-UE-PUSCH-r19 parameter is included in CB-Msg3-PUSCH-Config-r19 which is signalled to UE via SIB2(-NB) in a broadcast manner. Since this configuration is cell-wide and not UE-specific, it cannot be used to indicate the UE-specific power offset for CB-Msg3 transmissions. In this context, defining a new parameter p0-UE-PUSCH-r19 for CB-Msg3 appears unnecessary.

CB-Msg3-PUSCH-Config-r19 ::= SEQUENCE { numRUs-r19 BIT STRING (SIZE(2)), prb-AllocationInfoSet-r19 SEQUENCE (SIZE(1..48)) OF BIT STRING (SIZE(10)), mcs-r19 BIT STRING (SIZE(4)), numRepetitions-r19 BIT STRING (SIZE(3)), p0-UE-PUSCH-r19 INTEGER (-8..7), alpha-r19 Alpha-r12 } PUR-PUSCH-Config-r16 ::= SEQUENCE { … pur-PUSCH-FreqHopping-r16 BOOLEAN, p0-UE-PUSCH-r16 INTEGER (-8..7), alpha-r16 Alpha-r12, pusch-CyclicShift-r16 ENUMERATED {n0, n6}, pusch-NB-MaxTBS-r16 BOOLEAN, locationCE-ModeB-r16 INTEGER (0..5) OPTIONAL -- Cond SubPRB } p0-UE-PUSCH Parameter: P0_UE_PUSCH,c (3). See TS 36.213 [23], clause 5.1.1.1, unit dB.

Observation 3: It is not possible to configure a UE-specific power offset for CB-Msg3-EDT transmission using the newly introduced parameter p0-UE-PUSCH-r19.

Furthermore, RAN2 agreed the CB-MSG3_RECEIVED_TARGET_POWER is set to

cb-Msg3-InitialReceivedTargetPower + (CB_MSG3_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep

where UE power ramping is supported based on the target power defined by cb-Msg3-InitialReceivedTargetPower. In this mechanism, a UE-specific power offset is also unnecessary.

Observation 4: The UE-specific power offset defined by parameter p0-UE-PUSCH-r19 is not used in the CB-Msg3 power ramping calculation.

Based on above observations, we understand the parameter p0-UE-PUSCH-r19 should be removed from both CB-Msg3-ConfigSIB and cb-Msg3-ConfigSIB-NB.

Proposal 3: The parameter p0-UE-PUSCH-r19 should be removed from both CB-Msg3-ConfigSIB and cb-Msg3-ConfigSIB-NB.

## 2.4	S900

# S900 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. S900; IoTNTN; X; Accumulate SI decoding across SI windows for PWS SIB; Yes, R2-250xxxx; Samsung (Jonas); vXXX; ToDo. [Description]: Currently, for NB-IoT, the UE can accumulate SI decodings across SI windows in case the UE fails to decode during an SI window: 1> if the SI message was not possible to decode from the accumulated SI message transmissions by the end of the SI-window, continue reception and accumulation of SI message transmissions on DL-SCH in the next SI-window occasion for the concerned SI message; When this feature was introduced in Release 13, it was never considered that an IoT device shall acquire PWS, and since most SIBs use SI notification procedure if the content changes, this is not an issue for non-PWS SIBs. But for PWS, different SIB content i.e via the segments will not trigger the SI notification procedures. The issue however, is that accumulating across SI windows is a useful feature, in particular for PWS, to increase coverage of PWS notifications for PWS message that are not segmented. So we equally think that there is a risk in completely turning it off in case no segmentation is being performed. [Proposed Change]: Allow the network to configure that the UE shall not accumulate SI decodings across SI windows for PWS SIBs. [Comments]: Rapporteur’s comments: Discuss based on proponent’s contribution.

According to the legacy specification the NB-IoT UE is allowed, but not required to accumulate SI messages across windows:

Within the SI-window, the corresponding SI message can be transmitted a number of times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS.The UE acquires the detailed time/frequency domain scheduling information and other information, e.g. used transport format for the SI messages from schedulingInfoList field in SystemInformationBlockType1-NB. The UE is not required to accumulate several SI messages in parallel but may need to accumulate a SI message across multiple SI windows, depending on coverage condition.

Observation 5: The NB-IoT UE is allowed, but not required, to accumulate SI messages across SI windows.

This comes with the inherent risk of attempting to accumulate messages that do not have the same content, especially when considering that some SI message content can change without triggering the SI modification procedure. In the legacy specification, change of ETWS and CMAS information change is also exempt.

Change of system information (other than for ETWS, CMAS, EAB, UAC, and satellite assistance information parameters except for discontinuous coverage scenarios and for NB-IoT, other than for AB parameters and satellite assistance information parameters except for discontinuous coverage scenarios) only occurs at specific radio frames, i.e. the concept of a modification period is used. … E-UTRAN may not update systemInfoValueTag upon change of some system information e.g. ETWS information, CMAS information, RLOS indication (i.e., rlos-Enabled), regularly changing parameters like time information (SystemInformationBlockType8, SystemInformationBlockType16, hyperSFN-MSB in SystemInformationBlockType1-NB), EAB and AB parameters, UAC parameters, positioning system information blocks, or satellite assistance information. Similarly, E-UTRAN may not include the systemInfoModification within the Paging message upon change of some system information.

Observation 6: change of the ETWS and CMAS SI message content does not trigger the SI modification procedure.

Furthermore, the legacy specification does not prevent the UE from attempting to accumulate the PWS/CMAS SI messages across SI windows. All the UE is required to do is to check the message Identifier and Serial Number when it has received a new SI message.

Since the legacy specification already allows SI message accumulation despite the risk of the UE attempting to accumulate messages with different content, we think it is best to follow the legacy procedure also for PWS.

Proposal 4: Allow the NB-IoT UE implementation to attempt to accumulate PWS (ETWS/CMAS) SI messages across SI windows. No specification impact is expected.

## 2.4	S901

# S901 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. S901; IoTNTN; 2; Clarification on TA report; Yes, R2-250xxxx; Samsung (Jonas); vXXX; ToDo. [Description]: TA report is according to the RRC procedures applicable for CB-Msg3-EDT (although the field description seems to indicate that it is not applicable to CB-MSg3-EDT). The CB-Msg3-EDT being applicable to CP solution is likely not an issue as the MAC procedures will not include the TA report if the CB-Msg3 TBS allocation is too small. However, for UP solution, the TA report has higher priority compared to DCCH, meaning that the MAC CE may use unnecessarily occupy data. This is because the ta-Report applies to all types of access procedures. Also, the arguing against this because this was not addressed for PUR is not valid, as the usefulness of PUR in NTN is questionable. CB-Msg3-EDT is introduced specifically for NTN, so it is in our interest to introduce protocols and procedures that make these procedures as efficient as possible. [Proposed Change]: TA report is not triggered for the CB-Msg3-EDT procedures or a ta-Report configuration specifically for CB-Msg3-EDT is introduced. For not triggering the TA report we propose the following: 1> if UE supports timing advance reporting and ta-Report is included in SystemInformationBlockType2 and the UE is not performing CB-Msg3-EDT in accordance with 5.3.3.3b: 2> instruct the associated MAC entity to trigger Timing Advance reporting; [Comments]: Rapporteur’s comments: Discuss based on companies’ contributions on whether there is an issue to support TA report during CB-Msg3-EDT.

Due to the large coverage area of an NTN cell, Timing Advance (TA) reporting was introduced to differentiate the TA for the UE located in cell center and the UE in cell edge. This enables NW to adjust the UE-specific Koffset, thereby reducing the scheduling latency for UEs located in cell center. The size of TA report MAC CE is 2 bytes which may be included in Msg3 and subsequent UL messages in legacy RACH procedure. However, to our understanding, the TA report is kind of optimization. The system can work well even without the TA report since NW can always configure a cell-specific Koffset for all UEs. Consequently, this feature can be enabled or disabled via the flag ta-Report in SIB2.

For CB-Msg3-EDT, it is designed to carry small amounts of UL data via Msg3, with the UE’s payload size limited to the TBS threshold defined in CB-Msg3-TBS. In most of the scenarios, the CB-Msg3-EDT procedure will be terminated after UE receives CB-Msg4. Including a 2-byte TA report in CB-Msg3 may not bring any benefit as no further UL transmissions occur after CB-Msg4. Instead, it will cost unnecessary two-bytes overhead in CB-Msg3, making it inefficient for this use case.

Furthermore, even if the UE is not allowed to report TA in Msg3 itself, the NW implementation can still trigger the UE to report the TA later, e.g., via RRC message such as RRCConnectionSetup by newly configuring the offsetThresholdTA parameter.

TS36.321 5.4.9: Timing Advance reporting shall be triggered if any of the following events occur: - if triggered by upper layers; - upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell; - if the variation between current information about Timing Advance and the last reported information about Timing Advance is equal to or larger than offsetThresholdTA, if configured.

Proposal 5: TA report shall not be included in Msg3 during the CB-Msg3-EDT procedure.

# 3	Conclusion

This document has made the following observations and proposal:

RIL V211:

Observation 1: NAS layer needs to know the current operating mode of the cell to decide on its NAS signalling transmission.

Proposal 1: AS layer informs NAS on SF mode transition from SF to Normal and vice versa.

RIL C001:

Observation 2: For serving cell, the presence of SF-operation-mode parameter implicitly indicates whether cell operates in SF mode or not.

Proposal 2: Consider the following options for neighbour cell SF mode indication.

- Option 1: The SF-operating-mode parameter in neighbour cell is provided as

NeighSatelliteInfo-v19xy ::=	SEQUENCE {

sf-OperationModeNeigh-r19		ENUMERATED {TRUE}	OPTIONAL,	-- Need OP

t-ModeSwitchingNeigh-r19		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

- Option 2:

NeighSatelliteInfo-v19xy ::=	SEQUENCE {

t-ModeSwitchingNeigh-r19		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

If this parameter is present, it indicates neighbour cell operate in SF mode. In this option we can also save some bits for neighbour cell

RIL V215:

Observation 3: It is not possible to configure a UE-specific power offset for CB-Msg3-EDT transmission using the newly introduced parameter p0-UE-PUSCH-r19.

Observation 4: The UE-specific power offset defined by parameter p0-UE-PUSCH-r19 is not used in the CB-Msg3 power ramping calculation.

Proposal 3: The parameter p0-UE-PUSCH-r19 should be removed from both CB-Msg3-ConfigSIB and cb-Msg3-ConfigSIB-NB.

RIL S900:

Observation 5: The NB-IoT UE is allowed, but not required, to accumulate SI messages across SI windows.

Observation 6: change of the ETWS and CMAS SI message content does not trigger the SI modification procedure.

Proposal 4: Allow the NB-IoT UE implementation to attempt to accumulate PWS (ETWS/CMAS) SI messages across SI windows. No specification impact is expected.

RIL S901:

Proposal 5: TA report shall not be included in Msg3 during the CB-Msg3-EDT procedure.