3GPP TSG-RAN WG2 Meeting #132 	 R2-2508041

Dallas, USA, November 17th – 21st, 2025

Agenda item:	8.8.3

Source:	vivo

Title:	Remaining Issues on CB-Msg4 Reception

Document for:	Decision

# Introduction

In this contribution, we would like to provide our consideration of the remaining issues on CB-Msg4 Reception.

# Discussion

## 2.1 CB-Msg4 reception with two MPDCCH narrowbands

In RAN1’s reply LS R2-2508014 [1], it can be known that RAN1 is fine to support two MPDCCH narrowband resources. But the corresponding UE behavior for two MPDCCH narrowband monitoring is expected to be captured in the RAN2 spec, as RAN1 specs have been frozen for 2 meetings.

RAN1 response for support MPDCCH narrowband configuration for CB-Msg3-EDT: - From RAN1 perspective, introducing 2 MPDCCH narrowbands requires additional RAN1 standard effort. RAN1 would like to inform RAN2 that there are no concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

In the eMTC TN scenario, each of the two MPDCCH narrowbands is associated with distinct PRACH resources (i.e., unique preamble sequences). Per  TS 36.213 sub-clause 6.2, if two UEs select different PRACH resources, the network may transmit RAR corresponding to each UE via separate MPDCCH narrowbands. This mechanism ensures that RARs will not be concentrated in a single MPDCCH narrowband, avoiding potential congestion.

Table 6.2-E: Narrowband () for MPDCCH RAR.

| Mapped Preamble Index |  |
| --- | --- |
| mod( Preamble Index, 2 )=0 | First narrowband configured by high layer parameter mpdcch-NarrowbandsToMonitor-r13 |
| mod( Preamble Index, 2 )=1 | Second narrowband configured by high layer parameter mpdcch-NarrowbandsToMonitor-r13 |

For the CB-Msg3-EDT procedure, we see the value of supporting two MPDCCH narrowband resources, considering that RAN1 is also okay with this. The next question is how to finalize the required solutions. Given that there is currently no preamble sequence used, if scattered MPDCCH monitoring is required, RAN2 can only rely on the time domain resource index or frequency domain resource index to distribute UEs across two different MPDCCH narrowbands. Unfortunately, the MAC spec allows UEs to select both time and frequency resources completely randomly for each CB-Msg3 replica. This leads to a critical challenge: it is impossible to scatter UEs evenly, as we cannot define a feasible rule to achieve this load-balanced scattering while ensuring both UEs and the network have a consistent understanding of the target MPDCCH narrowband for CB-Msg4 monitoring.

To address this issue, we propose a straightforward solution: UEs shall transmit all CB-Msg3 replicas within a transmission window using the same frequency resource. From the network side, once any one of the replicas is successfully decoded, the network can, based on the frequency resource index, determine the specific MPDCCH narrowband to use for CB-Msg4 delivery to the corresponding UE. Conversely, the UE can determine the target MPDCCH narrowband for CB-Msg4 reception based on the index of the frequency resource it selected for CB-Msg3 transmission. For example, if mod(Frequency Index, 2)=0, the first narrowband configured is used, and if mod(Frequency Index, 2)=1, the second narrowband configured is used.

Observation 1: In the current MAC specification, UEs are allowed to randomly select both time-domain and frequency-domain PUSCH resources for CB-Msg3 transmission. As a result, it is not feasible to evenly distribute UEs across the two MPDCCH narrowbands based on CB-Msg3 resources.

Proposal 1: RAN2 confirms to support 2 MPDCCH narrowbands for CB-Msg3-EDT procedure.

Proposal 2: RAN2 to agree that UE randomly selects a frequency domain PUSCH resource for all selected time domain resources within the selected CB-Msg3 transmission window.

Proposal 3: Frequency index is used to determine the target narrowband resources for CB-Msg4 reception when two narrowband resources are provided. E.g., if mod(Frequency Index, 2)=0, the first narrowband configured is used, and if mod(Frequency Index, 2)=1, the second narrowband configured is used.

The corresponding MAC TP for Proposal 1 is given below for reference.

### 5.1a.2 CB-Msg3 transmission Before the CB-Msg3 transmission, the MAC entity shall select cb-Msg3-NumOfReplicas UL grants for CB-Msg3 transmission as follows: - select the next upcoming CB-Msg3 transmission window provided by the cb-Msg3-TxWindow associated with the selected enhanced coverage level; - randomly select cb-Msg3-NumOfReplicas PUSCH resources in the time domain within the selected CB-Msg3 transmission window from the CB-Msg3 resources provided by cb-Msg3-TimeResource associated with the selected enhanced coverage level; - randomly select a frequency domain PUSCH resource for all selected time domain resource provided by prb-AllocationInfoSet (npusch-SubCarrierSetList for NB-IoT) associated with the selected enhanced coverage level.

### 5.1a.3 CB-Msg4 reception After the CB-Msg3 transmission, the MAC entity shall: - start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT; - monitor the PDCCH while CB-Msg3ResponseTimer is running; ---------------------------------------<the unrelated part is omitted>------------------------------ - if the C-RNTI field is absent in the corresponding CMR and no corresponding MAC SDU is present in CB-Msg4: - indicate the successful completion of CB-Msg3-EDT procedure to the upper layers. NOTE: The UE can receive CB-Msg4 after the CB-Msg3ResponseTimer expired if the corresponding PDCCH is received before the CB-Msg3ResponseTimer expired. NOTE: For BL UEs and UEs in CE mode A, MPDCCH for CB-Msg4 is determined by mpdcch-Narrowband if only one narrowband is configured, otherwise, it is determined by Table 5-1a-1. Table 5.1a-1: Narrowband for MPDCCH CB-Msg4. Mapped Frequency Index; Narrowband. mod(Frequency Index, 2 )=0; First narrowband configured by high layer parameter mpdcch-Narrowband. mod(Frequency Index, 2 )=1; Second narrowband configured by high layer parameter mpdcch-Narrowband.

## 2.2 SCS indication

In NB-IoT TN scenario, the uplink SCS, for example, 3.75 kHz or 15 kHz, is indicated via the MSB of the UL grant included in the Random Access Response. This SCS configuration determines the UL SCS to be used for subsequent UL transmissions, such as those in the CONNECTED state following the RACH procedure.

Nr-bit =15, and the content of these 15 bits starting with the MSB and ending with the LSB are as follows: - Uplink subcarrier spacing is '0'=3.75 kHz or '1'=15 kHz – 1 bit - Subcarrier indication field as determined in Clause 16.5.1.1 – 6 bits - Scheduling delay field () as determined in Clause 16.5.1 with k0 = 12 for IDelay = 0 , where NB-IoT DL subframe n is the last subframe in which the NPDSCH associated with the Narrowband Random Access Response Grant is transmitted – 2 bits - Msg3 repetition number as determined in Clause 16.5.1.1 – 3 bits - MCS index indicating TBS, modulation, and number of RUs for Msg3 – 3 bits

For the CB-Msg3-EDT procedure, the UE may transition to the CONNECTED state for subsequent data transmission. However, the current CB-Msg3-EDT framework does not support adjusting the UL SCS post CB-Msg3 transmission. In our understanding, the UE has to adhere to the UL SCS used for CB-Msg3 PUSCH when operating in the CONNECTED state, and the network lacks a mechanism to modify this SCS when the subsequent PUSCH transmission requirements and data characteristics differ from those of CB-Msg3. As a result, the flexibility inherent in the legacy RACH procedure and EDT is lost in the CB-Msg3-EDT.

To address this issue and restore the required flexibility, we propose introducing a 1-bit field in the subheader of the CMR. This field will enable the network to flexibly indicate an alternative UL SCS for the UE to use in the CONNECTED state, For example, the format of the subheader is given below,

E/T/R/F/H/TA/C MAC subheader

Observation 2: In the legacy RACH and EDT procedure, the network can flexibly indicate the UL SCS at the per-UE level for subsequent UL transmissions—for example, PUSCH transmissions in the CONNECTED state. However, this flexibility is not available in the CB-Msg3-EDT. The current PDU format lacks an indication for the network to signal the UL SCS to a UE.

Proposal 4: RAN2 to introduce a 1-bit field in the subheader of CMR to indicate uplink SCS.

# 3 Conclusion

In this contribution, we have discussed the remaining issues regarding CB-Msg4 reception. And we make the following observations and proposals,

CB-Msg4 reception with two MPDCCH narrowbands:

Observation 1: In the current MAC specification, UEs are allowed to randomly select both time-domain and frequency-domain PUSCH resources for CB-Msg3 transmission. As a result, it is not feasible to evenly distribute UEs across the two MPDCCH narrowbands based on CB-Msg3 resources.

Proposal 1: RAN2 confirms to support 2 MPDCCH narrowbands for CB-Msg3-EDT procedure.

Proposal 2: RAN2 to agree that UE randomly selects a frequency domain PUSCH resource for all selected time domain resources within the selected CB-Msg3 transmission window.

Proposal 3: Frequency index is used to determine the target narrowband resources for CB-Msg4 reception when two narrowband resources are provided. E.g., if mod(Frequency Index, 2)=0, the first narrowband configured is used, and if mod(Frequency Index, 2)=1, the second narrowband configured is used.

SCS indication

Observation 2: In the legacy RACH and EDT procedure, the network can flexibly indicate the UL SCS at the per-UE level for subsequent UL transmissions—for example, PUSCH transmissions in the CONNECTED state. However, this flexibility is not available in the CB-Msg3-EDT. The current PDU format lacks an indication for the network to signal the UL SCS to a UE.

Proposal 4: RAN2 to introduce a 1-bit field in the subheader of CMR to indicate uplink SCS.

# 4 Reference

[1] R2-2508014, Reply LS on power ramping and RRC configuration for CB-Msg3-EDT, MediaTek.