3GPP TSG RAN WG2 Meeting #132         	                                   R2-2508076

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.2.1

Source:	Xiaomi

Title:	UE Capability pain points and considerations

Document for:	Discussion and Decision

# Introduction

The following agreements were agreed in RAN2 #131bis meeting:

Agreements on UE capabilities for next meeting

- For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

- Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

- Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this contribution, we focus on the pain points of 5G UE capability framework, and further discuss the key use case/scenarios for dynamic capability reporting. In the end, we also discussed IODT issues raised in RAN2 #131bis meeting and provide our views on how to progress them.

# Discussion

## 5G UE Capability Pain Points

As captured in the SID [1], simplified signaling framework for UE capability needs to be studied in 6G:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In the below section, we analyse the key pain points of 5G UE capability signalling and framework.

### Aspects with RAN1/4 Dependency

### Pain Point 1: Significant Capability Signalling Size

As the technology evolved and features introduced in different releases, the size of UE capability message increases significantly. In 5GNR, several techniques have been studied, including RRC message compression (not supported in 5GNR), UE Radio Capability Signalling Optimisation (RACS), UL RRC segmentation, filtering, etc. However, the above optimization can only provide a way-out for reporting massive capability signalling to network based on multiple approaches, but none of the above approaches can fundamentally reduce the reported signalling message size. We further analyse the root causes of the increased capability signalling size.

### Root Cause 1: Complex BC signalling structure

At the early stage of NR capability discussion, separation between baseband and RF capabilities were discussed. FeatureSetCombination was proposed to simply the signalling structure. Though it is introduced, there’s not many BC capabilities use this design in 5G.

Table 1. Band Combination Signalling Structure

| BandCombination | Band parameters (including frequency band indicator, CA BW class, etc) |
| --- | --- |
|  | FeatureSetCombination -> FeatureSetDL/FeatureSetUL -> FSPC |
|  | CA parameters -> EUTRA/NR/NRDC |
|  | MR-DC parameters |

Based on the basic 6G baseband and RF design, it would be good to better utilize FeatureSetCombination for simplification and other signalling structures for optimization can also be further considered.

Observation 1: Per BC features in 5G didn’t leverage the design of FeatureSetCombination well for signalling simplification.

### Root Cause 2: Complex RF requirements

We further observe that there’s a significant number of capabilities introduced along with complex RAN4 requirements.

For example, maximum output power requirement is the most important requirement for UE Tx. Almost all of other UE Tx requirements are defined based on UE’s MOP. The current approach is that if UE supports several power classes, the Tx requirements are separated to several sets with different power classes. Recalling that, when PC3 was introduced in 4G, PC3 is almost used in the whole 4G stage. Although PC2 was introduced in the 4G later stage, it was limited to mostly TDD bands and was not widely supported by the market. Therefore, the power class definition and the requirements based on power class methodology didn’t bring much trouble for 4G. However, during 5G era, much more power classes are defined compared to 4G, which leads to the fact that the power class-based requirements bring a lot of standardization work to differentiate UE supporting of different requirements (e.g., ACLR, MPR, AMPR and MSD).

Observation 2: Due to complex RF requirement (e.g., UE Tx requirement based on power class), massive capability signallings were introduced in 5G to differentiate UE support of different feature requirements.

### Root Cause 3: Redundant Reporting

Based on our observation from current UE capability signalling, redundant reporting is mainly contributed from the following aspects:

Case 1: Same BC capabilities reported in BandCombination and BandCombination-UplinkTxSwitch

Due to some different capabilities between band combination and band combination with uplink Tx switch, band combination list and some capabilities per BC need to be reported redundantly even with the same value.

BandCombinationList ::=             SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombinationList-UplinkTxSwitch-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-r16

BandCombination ::=                 SEQUENCE {

bandList                            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination               FeatureSetCombinationId,

ca-ParametersEUTRA                  CA-ParametersEUTRA                          OPTIONAL,

ca-ParametersNR                     CA-ParametersNR                             OPTIONAL,

mrdc-Parameters                     MRDC-Parameters                             OPTIONAL,

supportedBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL,

powerClass-v1530                    ENUMERATED {pc2}                            OPTIONAL

}

BandCombination-UplinkTxSwitch-r16 ::= SEQUENCE {

bandCombination-r16                 BandCombination,

bandCombination-v1540               BandCombination-v1540                      OPTIONAL,

bandCombination-v1560               BandCombination-v1560                      OPTIONAL,

bandCombination-v1570               BandCombination-v1570                      OPTIONAL,

bandCombination-v1580               BandCombination-v1580                      OPTIONAL,

bandCombination-v1590               BandCombination-v1590                      OPTIONAL,

bandCombination-v1610               BandCombination-v1610                      OPTIONAL,

supportedBandPairListNR-r16         SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-r16,

uplinkTxSwitching-OptionSupport-r16 ENUMERATED {switchedUL, dualUL, both}      OPTIONAL,

uplinkTxSwitching-PowerBoosting-r16 ENUMERATED {supported}                     OPTIONAL,

...,

[[

-- R4 16-5 UL-MIMO coherence capability for dynamic Tx switching between 3CC 1Tx-2Tx switching

uplinkTxSwitching-PUSCH-TransCoherence-r16     ENUMERATED {nonCoherent, fullCoherent}   OPTIONAL

]]

}

Case 2: Capabilities reported consistently across all/some bands

Due to FRx difference, there are several examples (around 60 capabilities) that UE reports the same capability value consistently for multiple bands. For example:

| bfd-Relaxation-r17 Indicates whether the UE supports BFD relaxation criteria and requirement as specified in TS 38.133 [5]. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. UE indicating support of this feature shall also indicate support of maxNumberCSI-RS-BFD, maxNumberSSB-BFD and maxNumberCSI-RS-SSB-CBD. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| cg-SDT-r17 Indicates whether the UE supports transmission of data and/or signalling over allowed radio bearers in RRC_INACTIVE state via configured grant type 1 (i.e. CG-SDT), as specified in TS 38.331 [9]. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. UE supports multiple CG-SDT configurations when a UE indicates the support of this feature and activeConfiguredGrant-r16; otherwise UE only supports one CG-SDT configuration. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| condHandover-r16 Indicates whether the UE supports conditional handover including execution condition, candidate cell configuration and maximum 8 candidate cells. Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Though we understand the initial intention is for different UE capability supported in TDD and/or FDD band, FR1 and/or FR2 bands, defining such capability as per band capability significantly increases the signalling overhead.

Case 3: Duplicate reporting due to finer granularity

UE capability parameters in 5GNR have hierarchical structure, where ‘Per’ indicates the level of the associated parameter is included, including ‘Per frequency range (FR1/FR2)’, ‘Per band’, ‘Per band combination’, ‘Per feature set (per band per band combination)’, ‘Per feature set per CC’, ‘Per band pair per band combination’, ‘Per UE’, etc.

We understand that the key motivation of defining multiple levels of granularity is to provide UE flexibility based on different UE implementation for deployment in different frequency/band/carriers, meanwhile, we have to admit that it also increases signalling overhead due to report same value repeatedly in each finer granularity.

However, we think it would be impractical to remove finer granularity for signalling overhead reduction, as different UE implementation may have different restrictions in different scenarios. On the other hand, we do think RAN1/2/4 need to take responsibility to carefully discuss and define proper granularity for each feature.

For band combination, there was some discussion during 4G whether UE can report the maximum capability for a group of band combination without indicating capability for subset band combination (except fallback mechanism). During the discussion, the key question is how to interpret subset band combination capability within the super band combination. However, the conclusion in 4G is that such aspect needs to be discussed case by case and there’s no way-out except defining finer granularity.

Observation 3: Finer granularity may be still needed for flexibility of different UE implementations. RAN WGs need to carefully decide the proper granularity case by case to avoid over-reporting finer granularity.

In the end, we also observe that RAN4 started some discussion on capability optimization [6]:

# Topic #8: Spectrum aggregation - Agreements - Below spectrum aggregation topics can be discussed as starting point. - RF switch-time performance - Need for intra-band support of non-contiguous UL CA and RB allocations - Study benefits-drawbacks of using CA vs. using wider single carrier BW - Study how to account for simultaneous TX/RX between the bands properly from the beginning - Discuss band combination simplification in spectrum agenda

Therefore, we believe a close collaboration between RAN2 and RAN4 is needed to improve redundant capability reporting for BC.

### Root Cause 4: Massive Optional Features

It is observed that massive optional features have been introduced in late 5G, which increases the signaling message size significantly. Though we understand it is unavoidable to define optional features (especially should not mandate UE implementing certain features), avoiding defining optional capabilities for multiple options of same functionality needs to be seriously considered in 6G.

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

Observation 4: Avoid adopting multiple options for the same functionality by introducing optional capability can help to reduce unnecessary capability signalling.

### Root Cause 5: Unnecessary introduction of capabilities

There are also many capabilities unnecessarily introduced with different purposes. For example:

- Some capabilities are only for testing purpose, e.g., RF related capabilities including Tx diversity, dual PA, etc. In our understanding, those testing purpose can be declared by UE without signalling, similar as BS.

- Some capabilities are introduced only for network knowledge of UE’s distribution. For example, RAN1 FG 65-1-1 was introduced to allow the NW to collect the statistic about the percentage of the UEs that support this feature. This is not the right approach of using UE capability.

- There are also some cases that features are not properly defined and can be merged, with the respect to UE implementation flexibility.

In 6G, all RAN WGs need to take the responsibility to avoid introducing unnecessary capability signalling.

Observation 5: There are many unnecessary capability signallings introduced which cause the increased number of capability signalling size.

### Pain Point 2: Lack support of asymmetric UL/DL capability

In 5G, FeatureSet was defined in a symmetric UL/DL approach, where UE needs to report both UL and DL capabilities simultaneously. Consdiering spectrum aggregation is now being widely studied, there might be more and more cases that introducing asymetric UL/DL or multiple-to-one mapping between UL/DL.

-- ASN1START

-- TAG-FEATURESETCOMBINATION-START

FeatureSetCombination ::=       SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand

FeatureSetsPerBand ::=          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet

FeatureSet ::=                  CHOICE {

eutra                           SEQUENCE {

downlinkSetEUTRA                FeatureSetEUTRA-DownlinkId,

uplinkSetEUTRA                  FeatureSetEUTRA-UplinkId

},

nr                              SEQUENCE {

downlinkSetNR                   FeatureSetDownlinkId,

uplinkSetNR                     FeatureSetUplinkId

}

}

-- TAG-FEATURESETCOMBINATION-STOP

-- ASN1STOP

However, we understand that most of capabilities defined per FS comes from RAN1/4. It would be good to check with RAN1/4 whether it is possible to 1) consider one-to-multiple mapping between UL/DL; 2) decouple DL/UL capability signalling for BB/RF capabilities in 6G.

Observation 6: In 5G, UE needs to report coupled UL and DL FeatureSet, which provides less flexibility when introducing asymmetric UL/DL capability. This may become a bottleneck if 6G considers more flexible spectrum aggregation for UL/DL.

### Pain Point 3: Fail to Consider Forward Compatibility

Band Combination

The most recent practice of lack of considering forward compatibility is on LBCA. In the end, RAN2 agreed to introduce an alternative FeatureSetCombiantionId for LBCA.

- Introduce an alternative FeatureSetCombinationId for a UE supporting low band carrier switching. - We will focus on NR standalone case for the LBCA capability signaling.

Power Class

Another example is on power class. For 5G, power classes for different features are reported to NW and the signaling defined in TS 38.331 [5] is summarized in following table.

Table 2. Power Class Capability

| Parameter | Description | Applicability |
| --- | --- | --- |
| ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 | UE power class | Per band, UL MIMO also uses this capability TxD also uses this capability |
| powerClass, powerClass-v1610 | Indicates power class the UE supports when operating according to this band combination. | Per band combination |
| ue-PowerClassPerBandPerBC-r17, ue-PowerClassPerBandPerBC-v1820 | Indicates the UE power class per band per band combination. | Per band per band combination |

The problem of the above power class signal structure was raised many times in previous RAN2/4 discussions. The issues can be summarized roughly to the following several aspects,

- powerclass defined in per band combination limits the CA MOP when power class per band is different. Although this is solved by higherPowerLimit-r17 signaling, it can be fixed in the beginning of 6G without introduce extra signaling.

- 1 layer and two layers UL MIMO have the same power class in current signaling. Although there’s a ULFPTx mode which allows different PA capabilities for different Tx chain, but it seems a rather complicated solution with RAN1 TPMI involved.

- Transparent TxD power class reuses ue-PowerClass but the implementation seems similar with ULFPTx fullpowerMode1. It’s also confusing if the ue-PowerClass can be different for non-TxD mode and TxD mode.

Channel Bandwidth

As for channel bandwidth, it took multiple releases to expand channel bandwidth signalling design for new bands and features, especially some special handling for 90mhz channel bandwidth, less than 5MHz, etc.

Example on 90mhz channel bandwidth: To determine whether the UE supports a channel bandwidth of 90 MHz for the band combination with other bandwidth combination set than BCS5, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, and supportedBandwidthCombinationSetIntraENDC-v1790. To determine whether the UE supports a channel bandwidth of 90 MHz for the band combination with BCS5, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, supportedAggBW-FR1- r17, and supportedBandwidthCombinationSetIntraENDC-v1790. FeatureSetUplinkPerCC ::= SEQUENCE { supportedSubcarrierSpacingUL SubcarrierSpacing, supportedBandwidthUL SupportedBandwidth, channelBW-90mhz ENUMERATED {supported} OPTIONAL, mimo-CB-PUSCH SEQUENCE { maxNumberMIMO-LayersCB-PUSCH MIMO-LayersUL OPTIONAL, maxNumberSRS-ResourcePerSet INTEGER (1..2) } OPTIONAL, maxNumberMIMO-LayersNonCB-PUSCH MIMO-LayersUL OPTIONAL, supportedModulationOrderUL ModulationOrder OPTIONAL } channelBWs-UL CHOICE { fr1 SEQUENCE { scs-15kHz BIT STRING (SIZE (10)) OPTIONAL, scs-30kHz BIT STRING (SIZE (10)) OPTIONAL, scs-60kHz BIT STRING (SIZE (10)) OPTIONAL }, fr2 SEQUENCE { scs-60kHz BIT STRING (SIZE (3)) OPTIONAL, scs-120kHz BIT STRING (SIZE (3)) OPTIONAL }

For 6G, it is important to consider to define forward-compatible signalling framework for those basic features, including:

- Band combination

- Power class

- Channel bandwidth

Observation 7: Lack of forward compatible signaling design, especially for BC, power class, channel BW, etc increased signalling complexity and readability in 5G.

Based on the above analysis, for capability signalling reduction, there’s a high dependency with RAN1/4 input. To facilitate RAN2 study on capability signalling optimization, it is suggested to ask RAN1/4 to consider above pain points in their basic system design as early as possible.

### Aspects without RAN1/4 Dependency

Despite the aspects of capability signalling size optimization with RAN1/4 dependency, there are also other aspects that can highly reduce the reported capability message size, which can be solved by RAN2 first.

### Pain point 4: Duplicate retrieval framework with/without RACS

As shown in Figure 1, we summarize UE capability management and retrieval functions supported in 5GNR. In Rel-15, based on the network configuration, UE reports its full UE capabilities supported (on the network requested frequency band(s)) as a container to gNB. Then, gNB further propagates the received radio capability container to CN for long-term storage. Based on the release supported by the gNB, gNB uses the relevant capabilities based on its own release. Later on, a new gNB can receive the corresponding UE capability from CN after UE is moved to the new gNB. With the introduction of RACS in Rel-16, RACS based UE capability retrieval framework was introduced to exchange RACS ID during UE’s mobility (avoid exchanging massive signallings of UE capability).

For 6G, if RACS was supported in Day 1 as mandatory feature, it seems duplicated to support both retrieval framework with/without the support of RACS.

Figure 1. UE Capability Management and Retrieval

Furthermore, it is observed that UE radio capabilities for paging were not supported in 5GNR Day 1, as the basic paging function are assumed mandatorily supported without signalling. In later releases, when paging features, such as PEI, LP-WUS, on-demand SIB1, were introduced, those capability(ies) related to paging are forwarded via Xn interface across gNBs as separate IEs, instead of RRC container. This leads to some drawbacks for new paging features, for example, if a UE initially accesses to an old release gNB then moves to a new release gNB, due to lack of radio capability for paging, the UE cannot be paged by the new gNB using the newly supported feature. Even RAN2 introduced radio capability for paging as a container in Rel-19, gNBs with legacy releases can only support such new paging feature by upgrading itself by network implementation. Furthermore, UE may still need to report the same capability for paging in the container for inter-node message, as well as over the air interface (e.g., which is now under-discussion in Rel-19 LP-WUS), which further causing duplicated capability reporting over the air interface. Additionally, capability for paging doesn’t need to be exchanged between gNBs, if those capabilities are included in capability container to CN from Day 1.

Observation 8: Incomplete UE capability management/retrieval solution in Day 1 may bring drawbacks to later releases, e.g., duplicate reporting over air interface, duplicate UE capability retrieval framework, forward-compatibility issue, etc.

### Pain Point 5: Impractical RACS ID

In 5G, RACS has been introduced as a capability ID, which may be exchanged in NAS signalling over the air and in network signalling, instead of the UE capability structure. RACS ID in NAS signalling can represent a pre-provisioned set of radio capabilities for one or more RATs from the same vendor in order to reduce signalling overhead, which can either be assigned by the manufacturer or by the serving PLMN.

However, due to different requirements from different markets/regions, etc, massive number of UE capability sets are now available in the field. This makes it too complicated to set a proper RACS ID representing multiple combinations of UE features across different UEs, let alone engineering effort for testing.

For 6G, finer granularity of RACS ID can be considered to represent a standardized capability set for flexibility. This can not only help UE vendors to easily meet requirements from different regions/operators, but also reduce capability reporting overhead. Once finer granular RACS ID is supported, UE capability retrieval framework based on finer RACS ID can be further studied.

Based on RAN2 study outcome, SA2/CT can be further involved for normative phase.

Observation 9: Manufacturing or serving PLMN defined RACS ID is difficult to satisfy different UE implementation and different requirements from different regions, which lacks of flexibility for inter-vendor collaboration.

### Pain Point 6: Mismatch between UE and NW supported features

Another aspect that causes massive size of UE capability reporting is due to UE’s unawareness of network supported features. Though reporting full UE capabilities and stored in CN can reduce UE frequently reporting its capability to network, UE still sometimes cannot report all of its supported capabilities. For example, when capability reporting size is larger than 9kB * network supported segment number, UE will only report part of its capability that suits the size. The selection of reported capability is up to UE implementation. This means even network supports certain feature(s), if UE doesn’t report it due to the limited UL message size, the feature still cannot be enabled.

On the other hand, due to lack of network supports, network may only provide the configuration(s) to the UE reported capability corresponding to network supported feature(s). This means, even UE can report all of its supported feature in UL message, UE may report unnecessary capabilities, as the network doesn’t support such feature.

Therefore, to make UE capability being fully used and help network enabling the cared feature(s) for both UE and network, it is essential for UE to understand what feature(s) does the network supports (e.g., via broadcasting supported features, capability filtering, etc). Based on this, UE can further report its capability according to network supported/interested capabilities.

Observation 10: UE’s unawareness of network supported feature(s) may cause mismatch between UE and NW supported feature: 1) NW may not receive capabilities of its cared feature due to limited UL message size; 2) UE reports unnecessary capabilities of features that are not supported by network.

### Pain Points 7: Capability without corresponding network configuration

As captured in TS 38.306 [2], while configuring and scheduling the UE, network needs to respect and consider UE capability, i.e., the configuration from network cannot exceed UE capability.

The network needs to respect the signalled UE radio access capability parameters when configuring the UE and when scheduling the UE.

The intention of UE capability is to avoid the mismatch between UE supported feature(s) and network configuration(s). However, during 5G practice, we observed there are some cases that no configuration defined corresponding to the UE capability.

One example in RAN2 #131 meeting TEI19 [UAI-cellDTRX], an optional capability with signaling was introduced to indicate whether the UE supports providing its preference of a cell group on DRX parameters for power saving and its preference on cell DTX/DRX related parameters for PCell in RRC_CONNECTED. However, there’s neither clear network behavior defined nor corresponding configuration for the reported DRX parameters. It is a clear example on the misusage of the UE capability signalling.

Observation 11: During 5GNR practice, there are some cases that some capabilities don’t have any corresponding configuration defined.

From 6GR Day 1, it is important to define UE capabilities only if there’s a corresponding configuration provided by the network, and find an efficient way to associate capability and configuration together.

Currently, most of capabilities in TS 38.306 [2] and UE capability ASN.1 are organized based on protocol layers, e.g., PHY parameters, MAC parameters, RLC parameters, etc. There are also some sections organized based on the feature, e.g., IAB parameters, SON parameters, (e)RedCap parameters, Aerial UE parameters, etc. It would be good to make it clearer on what capabilities should be supported in order to support a feature for easy readability.

As observed from 5GNR, device-type specific capabilities (e.g., reduced optional capabilities) based on protocol-based capability signalling structure introduces huge complexity in UE capability specification, as shown in Table 3:

Table 3. Reduced Capability in 5GNR

| Approach | Description | Example | Comment |
| --- | --- | --- | --- |
| Approach 1 | Introduce single bit optional capability signalling to represent mandatory without capability signalling features for certain device type. | supportOfRedCap-r17, supportOfERedCap-r18. | Efficient and straightforward for UE to understand what mandatory features need to be supported by single bit signalling. |
| Approach 2 | For each mandatory without capability signaling feature (specified in TR 38.822) that is considered as optional for (e)RedCap, IAB-MT and NCR-MT, new optional capability with signalling is introduced. | longSN-RedCap-r19 for (e)RedCap longSN-NCR-r18 for NCR-MT | Multiple duplicated capabilities with the same meaning need to be introduced. |
| Approach 3 | For those mandatory capabilities that can be considered as a feature group, e.g., CA, MR-DC, etc, a general description was captured in TS 38.306, neither mentioning the specific capabilities nor introducing new optional capabilities. | TS 38.306 Section 4.2.15.1 for IAB-MT; Section 4.2.21/22.1 for (e)RedCap; Section 4.2.23.1 for NCR-MT. | It is not clear and accurate on which specific capabilities are not supported by the UE |
| Approach 4 | For (conditional) mandatory features with capability signalling, adding ‘It is optional for xxx UEs’ in each description field of features that are not supported by (e)RedCap, IAB-MT or NCR-MT | pdsch-256QAM-FR1 field description: It is optional for (e)RedCap UEs, IAB-MT, and NCR-MT, and mandatory with capability signalling for other UEs. | Spread across the whole specification. This further makes reader difficult to understand the full picture of what features can be supported by the reduced capability device type. |
| Approach 5 | For optional capability with signalling where reduced capability device type has constraints, limitations for reduced capability devices are added in the field description of the corresponding optional capabilities. | bwp-DiffNumerology field description: For the UE which is a (e)RedCap UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of the CORESET#0 (if configured) and SSB for PCell. For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). |  |

Observation 12: Some feature-/device type-based UE capability signaling was only organized per protocol layer, which makes it difficult to understand what need to be support for a device type/feature.

As discussed in our companion contribution [7], RRC signalling can be separated into one common module(s) for basic communication functions and a number of feature-/functionality-specific modules that have no dependency with those specified for other modules (e.g., no cross-module parameter referencing).

A similar concept can also be adopted in UE capability framework, i.e., consider device type-/feature-based UE capability reporting, comprising a set of mandatory capabilities for each device type/feature and other optional capabilities.

This can not only serve the purpose of easy readability, but also allow UE to report UE capability in modular based on network interested/supported feature(s). Based on this, feature-based/device type-based UE capability reporting can also be easily associated to common configuration and feature-/functionality- specific RRC module(s).

Based on above discussion, we summarized the following pain points of 5G UE capability framework and proposals on how to further address them.

Proposal 1: RAN2 to study solutions for the following pain points of 5G UE capability framework:

- Pain point 1: Significant capability signalling size

  - Root cause 1: complex BC signalling structure;

  - Root cause 2: complex RF requirement;

  - Root cause 3: redundant reporting

    - BC capabilities for BC and BC-UplinkTxSwitch;

    - Capabilities reported consistently across all/some bands;

    - Capabilities with finer granularity repeatedly reported the same value across band/BC;

  - Root cause 4: massive optional features

  - Root cause 5: unnecessary introduction of capabilities

- Pain point 2: Asymmetric UL/DL capability cannot be supported in 1:1 DL/UL mapping FeatureSet framework;

- Pain point 3: Non-forward compatible signaling structure design, e.g., band combination, power class, etc;

- Pain point 4: Duplicate retrieval framework with/without RACS;

- Pain point 5: Impractical RACS ID;

- Pain point 6: Mismatch between UE and NW supported features;

- Pain point 7: Capability without corresponding network configuration.

Proposal 2: For pain point 1-4, RAN2 sends LS to RAN1/RAN4 and ask RAN1/4 to consider pain point 1-3 and root causes in 6G system design requirement for capability signaling size reduction. RAN2 to discuss capability signalling structure simplification in later phase of SI based on feedback from RAN1/4.

Proposal 3: RAN2 can first work on the following aspects (pain point 4-7 without RAN1/4 dependency) to reduce UE capability signalling size and complexity:

- For pain point 5, study proper granularity of RACS ID.

- For pain point 4/5, study single UE capability retrieval framework considering the adoption of RACS. Collaboration with SA/CT after RAN2 study;

- For pain point 6, study methods of UE capability reporting based on UE’s awareness of network supported feature(s), e.g., network broadcasting its supported feature, capability filtering, etc.

- For pain point 7, study feature-based/device type-based UE capability reporting, with a clear mapping/association to configuration(s) provided by network.

## Dynamic Capability

In 5GNR, UE RRC capability reporting procedure can only be initiated by network when network needs (additional) UE radio access capability information. Moreover, whenever UE wants to update its capability, UE either needs to go to RRC_IDLE and perform TAU or DETACH from the network for capability reporting.

Based on the practice of 5GNR, it has been observed that network-initiated AS capability reporting is not sufficient in 5GNR. During RAN2 #131bis meeting, it was agreed to identify use cases for dynamic capability change in connected mode.

- Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

In below sections, we show two key use cases that need the support of dynamic capability in connected mode.

Use Case 1: MU-SIM

In 5G, the following temporary capability restrictions are supported in MU-SIM and report to network via UAI:

Table 4. MU-SIM temporary restricted capabilities

| MUSIM Related | musim-MIMO-Layers-DL/UL | maxNumberMIMO-LayersNonCB-PUSCH maxNumberMIMO-LayersCB-PUSCH maxNumberMIMO-LayersPDSCH |
| --- | --- | --- |
|  | musim-SupportedBandwidth-DL/UL | supportedBandwidthDL supportedBandwidthUL |
|  | musim-AvoidedBandsList | supportedBandList supportedBandCombinationList |

A MUSIM device in RRC_CONNECTED state in Network A may indicate its preference on temporary UE capability restriction or removal of restriction with Network A when the MUSIM device needs transmission or reception in Network B (e.g., including start/stop connection to Network B). Though UE can report its temporary restriction to network, but it is still up to network implementation how to configure UE. MUSIM device may decide to enter RRC_IDLE if it does not receive response message from network, so that the update of UE capability from MUSIM device can be reported. This approach not only increases the latency, but also downgrade user experience.

Use Case 2: Foldable Phone

Moreover, as shown in Figure 2, shipment volume of foldable phone rapidly increases during the past 5 years in China [3].

Figure 2. Foldable Phone Shipment (China Market)

Figure 3. Usage of unfoldable Phone in Unfolded Status

When the foldable phone is folded/unfolded, the form factor of foldable phone can be changed, which further leads to the change of UE capability, e.g., MIMO layer, the number of supported ports, etc. There might be some concerns on signalling overhead from the network point of view, especially when handling frequent UE capability reporting. However, based on the survey [4], the top 5 scenarios of using unfolded screen (Figure 3) show that the foldable phone will keep unfolded for a long time based on the user behaviour, i.e., user will not frequently fold/unfold the phone. This further proves that the foldable phone will not change its capability frequently.

Observation 13: Changeable mobile phone form factors and MUSIM increase the need for UE to update its UE capability to network, where clear network behaviour is strongly expected in 6GR.

There are two cases of how UE change its capability: 1) When UE capability is changed from high to low; 2) When UE capability is changed from low to high.

During RAN2 #131bis meeting, there was some discussion on when dynamic capability should be indicated to network, e.g., whether UE reports before or after the change of UE capability. Here, we use the maximum number of MIMO layer as an example:

Scenario 1: When UE changes its the maximum number of MIMO layer from lower capability to higher capability (e.g., maximum number of 4 MIMO layer to maximum number of 8 MIMO layer): There’s no issue from scheduling or UE behavior point of view, since network configuration for lower capability can still be used.

Scenario 2: When UE changes its the maximum number of MIMO layer from higher capability to lower: During the period when UE changes its capability till network reception of dynamic capability reporting, the scheduling can still work based on network implementation:

In uplink SRS, UE can decide the number of uplink ports for SRS transmission based on its implementation within network configuration (based on higher MIMO layer), which means network will always decode the received signalling based on UE’s higher MIMO layer. Due to the absence of receiving signals from other ports, network understands there’s a change of transmission port at UE side without knowing the specific reason.

In downlink, network can implicitly know the change of the available number of MIMO layer based on CSI feedback (i.e., the change of RI).

In summary, for the maximum number of MIMO layer, change in advance has no impact to network scheduling, i.e., UE can change its capability before reporting the change.

For other capabilities that may need to be reported dynamically, whether there’s any impact to network scheduling can be discussed case by case.

Observation 14: At least for maximum number of MIMO layer, change in advance has no impact to network scheduling.

On the other hand, people may also ask whether such adaptation can rely on network implementation and why dynamic capability is still needed? First of all, when UE capability is changed from high to low, though it is feasible to rely on network implementation for better scheduling, there could be other factors that leads to poor performance/failure of transmitting in certain ports, e.g., poor channel condition, etc. Therefore, though network can adjust configuration(s) based on its implementation, due to lack of knowledge of explicit cause, it may take a long time for network to find a good/proper configuration. Reporting the updated capability in real time via dynamic capability can significantly reduce the duration of UE suffering poor performance.

Observation 15: Without knowing UE capability change explicitly, network will take a long time to adjust proper configuration(s) based on its implementation, which further makes UE suffering a relative long time of poor performance.

The dynamic capability change would be mostly benefit for the case where UE’s capability is changed from low to high. This is mainly motivated from foldable phone -- When mobile phone is folded (for most of the time), due to lack of isolation, the UE can only use a maximum number of limited Tx/Rx antennas, which further limit the maximum number of MIMO layer used by UE. When the mobile is unfolded, the maximum number of Tx/Rx antennas and maximum number of MIMO layer could be increased. Without dynamic capability reporting, network will never know the ‘upgraded’ UE capability.

Observation 16: The dynamic capability is more beneficial for UE reporting its ‘upgraded’ UE capability.

In the end, another question is ‘What is the difference compared with UAI?’

In general, current UAI mechanism has two restrictions:

- UE is only allowed to report UAI if network configures. This means UE cannot report its changed capability autonomously.

- There’s no clear network behaviour defined for reported UE preference on change of capability. It is possible that network does not react based on the updated UE capability. The capability update via UAI is not that useful since the network’s action is not specified.

Hence, allowing UE-initiated AS capability update with predicable/standardized network action becomes more important in 6GR. Similar as 5G capability reporting, network needs to respect the signalled UE radio access capability parameters when configuring and scheduling the UE.

Proposal 4:  UE is allowed to trigger dynamic capability reporting in RRC_CONNECTED without network request/configuration, the network needs to respect the signalled UE dynamic radio access capability parameters when configuring the UE and when scheduling the UE, i.e., same as static capability reporting.

- For capabilities changed from low to high, UE reports the changed capability immediately after change, network is expected to provide configuration(s) associated to high capability upon receiving the change.

- For capabilities changed from high to low,

  - Maximum number of MIMO layer, change in advance has no impact to network scheduling;

  - For other dynamic capabilities, RAN2 to discuss the impact to network case by case.

Except temporary change of capabilities, UAI was also used for other UE assistance information is not tightly associated with any specific UE AS capability, e.g., indicating UE preferences on network configuration parameters and scheduling.

Such information was introduced to adjust/optimize the network configurations or scheduling strategy, but may not be as useful as intended due to the ignorance of network, which further results in the lack of motivation from the UE implementation of this type of assistance information. To make such UE assistance information reported really useful, the bottom line would also be to let network respect/follow the received information, and specify this in a high-level manner as well.

Proposal 5: UAI framework in 6GR, with the general principle that it only includes the information irrelevant to UE AS capability change and with a high-level specified network behaviour that the network should respect/follow the received UAI for network configuration.

## IODT and Interoperability

Vendor-specific capability reporting

It was raised in [8] that UE has to wait for the IODT with the “slowest” network vendor before activating a capability in operator’s network and may also encounter inter-operability issues when a new version of network equipment or network equipment of a new vendor is deployed into operator’s network. It was proposed to study solutions allowing the UE to activate a feature capability after IODT with a given set of network vendor.

In our understanding, to support such early IODT feature enabling, UE can report the tested capabilities towards a given network vendor, i.e., vendor-specific capability reporting. From UE point of view, we think it’s a good approach for fast deployment. On the other hand, there’s still some issues need to be discussed first.

For example, if UE does not know which network vendor during its capability reporting, UE may need to blindly reported all vendor IDs that has been tested. However, due to the diversity of network and UE implementation, it is possible that not all features can be tested across all vendors. This means, the tested vendor ID(s) need to be added in capability signalling with certain granularity (e.g., per UE or per capability). On one hand, this tested vendor ID will only be used temporarily for the period UE only passed IODT with certain network vendor. Once the UE passes IODT test with another vendor, this tested vendor ID becomes useless. This obviously significantly increases the signalling overhead, let alone the risk of exposing the IODT status with other network vendor(s). However, reducing signalling overhead is one of the most important objectives in 6G for capability framework. Therefore, it becomes important to seriously consider what is the granularity of reported vendor ID, does UE always need to report it and how to avoid signalling overhead?

On the other hand, if UE has knowledge of the network vendor ID upon its capability reporting, it is possible for UE to report its supported capability based on current network vendor (e.g., Vendor A). However, during UE handover, if the target NG-RAN node is from another network vendor (e.g., Vendor B), since capabilities supported under Vendor A may not be applicable to Vendor B, UE may need to report its capability supported under Vendor B again during handover. How to retrieve/report vendor-specific capability during handover also needs to be addressed.

Proposal 6: On vendor-specific capability reporting, RAN2 to first address the below concerns:

- What granularity should vendor ID be added on? How to avoid the signalling overhead caused by having vendor ID(s) per capability reporting? Should UE always include vendor ID in capability reporting?

- How to retrieve/report vendor-specific capability during handover between different network vendors?

Interoperability

It was proposed in [9] that there are some problematic UEs that causing the network has to adopt conservative settings or disable the new features entirely for all of the UEs. It was suggested to let UE report its UE/chipset information to RAN over the air interface.

However, we observe that Masked IMEISV, which is defined in TS 23.003 [x] can already serve the purpose informing RAN about UE’s equipment ID and software version. The only delta is that IMEISV is optional field over the NG interface.

If network vendor indeed believes such information is essential, it is suggested to consider a network-based solution, e.g., mandatory signaling of IMEISV over NGAP, instead of introducing any air interface impact.

Proposal 7: Consider network-based solution to share IMEISV to RAN, e.g., checking with RAN3 the possibility of defining mandatory signaling over NGAP for IMEISV.

# Conclusion

5G Pain Points and How to progress

Observation 1: Per BC features in 5G didn’t leverage the design of FeatureSetCombination well for signalling simplification.

Observation 2: Due to complex RF requirement (e.g., UE Tx requirement based on power class), massive capability signallings were introduced in 5G to differentiate UE support of different feature requirements.

Observation 3: Finer granularity may be still needed for flexibility of different UE implementations. RAN WGs need to carefully decide the proper granularity case by case to avoid over-reporting finer granularity.

Observation 4: Avoid adopting multiple options for the same functionality by introducing optional capability can help to reduce unnecessary capability signalling.

Observation 5: There are many unnecessary capability signallings introduced which cause the increased number of capability signalling size.

Observation 6: In 5G, UE needs to report coupled UL and DL FeatureSet, which provides less flexibility when introducing asymmetric UL/DL capability. This may become a bottleneck if 6G considers more flexible spectrum aggregation for UL/DL.

Observation 7: Lack of forward compatible signaling design, especially for BC, power class, channel BW, etc increased signalling complexity and readability in 5G.

Observation 8: Incomplete UE capability management/retrieval solution in Day 1 may bring drawbacks to later releases, e.g., duplicate reporting over air interface, duplicate UE capability retrieval framework, forward-compatibility issue, etc.

Observation 9: Manufacturing or serving PLMN defined RACS ID is difficult to satisfy different UE implementation and different requirements from different regions, which lacks of flexibility for inter-vendor collaboration.

Observation 10: UE’s unawareness of network supported feature(s) may cause mismatch between UE and NW supported feature: 1) NW may not receive capabilities of its cared feature due to limited UL message size; 2) UE reports unnecessary capabilities of features that are not supported by network.

Observation 11: During 5GNR practice, there are some cases that some capabilities don’t have any corresponding configuration defined.

Observation 12: Some feature-/device type-based UE capability signaling was only organized per protocol layer, which makes it difficult to understand what need to be support for a device type/feature.

Proposal 1: RAN2 to study solutions for the following pain points of 5G UE capability framework:

- Pain point 1: Significant capability signalling size

  - Root cause 1: complex BC signalling structure;

  - Root cause 2: complex RF requirement;

  - Root cause 3: redundant reporting

    - BC capabilities for BC and BC-UplinkTxSwitch;

    - Capabilities reported consistently across all/some bands;

    - Capabilities with finer granularity repeatedly reported the same value across band/BC;

  - Root cause 4: massive optional features

  - Root cause 5: unnecessary introduction of capabilities

- Pain point 2: Asymmetric UL/DL capability cannot be supported in 1:1 DL/UL mapping FeatureSet framework;

- Pain point 3: Non-forward compatible signaling structure design, e.g., band combination, power class, etc;

- Pain point 4: Duplicate retrieval framework with/without RACS;

- Pain point 5: Impractical RACS ID;

- Pain point 6: Mismatch between UE and NW supported features;

- Pain point 7: Capability without corresponding network configuration.

Proposal 2: For pain point 1-4, RAN2 sends LS to RAN1/RAN4 and ask RAN1/4 to consider pain point 1-3 and root causes in 6G system design requirement for capability signaling size reduction. RAN2 to discuss capability signalling structure simplification in later phase of SI based on feedback from RAN1/4.

Proposal 3: RAN2 can first work on the following aspects (pain point 4-7 without RAN1/4 dependency) to reduce UE capability signalling size and complexity:

- For pain point 5, study proper granularity of RACS ID.

- For pain point 4/5, study single UE capability retrieval framework considering the adoption of RACS. Collaboration with SA/CT after RAN2 study;

- For pain point 6, study methods of UE capability reporting based on UE’s awareness of network supported feature(s), e.g., network broadcasting its supported feature, capability filtering, etc.

- For pain point 7, study feature-based/device type-based UE capability reporting, with a clear mapping/association to configuration(s) provided by network.

Dynamic capability

Observation 13: Changeable mobile phone form factors and MUSIM increase the need for UE to update its UE capability to network, where clear network behaviour is strongly expected in 6GR.

Observation 14: At least for maximum number of MIMO layer, change in advance has no impact to network scheduling.

Observation 15: Without knowing UE capability change explicitly, network will take a long time to adjust proper configuration(s) based on its implementation, which further makes UE suffering a relative long time of poor performance.

Observation 16: The dynamic capability is more beneficial for UE reporting its ‘upgraded’ UE capability.

Proposal 4: UE is allowed to trigger dynamic capability reporting in RRC_CONNECTED without network request/configuration, the network needs to respect the signalled UE dynamic radio access capability parameters when configuring the UE and when scheduling the UE, i.e., same as static capability reporting.

- For capabilities changed from low to high, UE reports the changed capability immediately after change, network is expected to provide configuration(s) associated to high capability upon receiving the change.

- For capabilities changed from high to low,

  - Maximum number of MIMO layer, change in advance has no impact to network scheduling;

  - For other dynamic capabilities, RAN2 to discuss the impact to network case by case.

Proposal 5: UAI framework in 6GR, with the general principle that it only includes the information irrelevant to UE AS capability change and with a high-level specified network behaviour that the network should respect/follow the received UAI for network configuration.

IODT and interoperability

Proposal 6: On vendor-specific capability reporting, RAN2 to first address the below concerns:

- What granularity should vendor ID be added on? How to avoid the signalling overhead caused by having vendor ID(s) per capability reporting? Should UE always include vendor ID in capability reporting?

- How to retrieve/report vendor-specific capability during handover between different network vendors?

Proposal 7: Consider network-based solution to share IMEISV to RAN, e.g., checking with RAN3 the possibility of defining mandatory signaling over NGAP for IMEISV.

# Reference

[1] RP-252912	Revised SID: Study on 6G Radio	NTT Docomo

[2] TS 38.306	NR; User Equipment (UE) radio access capabilities

[3] 2025 iiMedia Research

[4] 2024.2 iResearch Inc.

[5] TS 38.331	NR; Radio Resource Control (RRC)

[6] R4-2514642	WF for [116bis][102] 6G general RF and UE RF

[7] R2-2508080	Discussion on RRC (re)configuration and signalling design	Xiaomi

[8] R2-2506988	Considerations on UE capability signalling in 6G	Qualcomm

[9] R2-2507607	Consideration on the 6G UE Capability	ZTE

# Annex: Draft LS to RAN1/4

Title:	LS on 5G UE capability signaling pain points

Release:	Release 20

Work Item:	FS_6G_radio

Source:	Xiaomi [to be RAN2]

To:	RAN1, RAN4

Contact person:	Ziyi Li

liziyi5@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

6G aims to study simplified and improved radio capability signalling framework.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

Based on 5G experience, RAN2 identified the following pain points of 5G capability framework design:

- Pain point 1: Significant capability signalling size

  - Root cause 1: complex BC signalling structure;

  - Root cause 2: complex RF requirement;

  - Root cause 3: redundant reporting

    - BC capabilities for BC and BC-UplinkTxSwitch;

    - Capabilities reported consistently across all/some bands;

    - Capabilities with finer granularity repeatedly reported the same value across band/BC;

  - Root cause 4: massive optional features

  - Root cause 5: unnecessary introduction of capabilities

- Pain point 2: Asymmetric UL/DL capability cannot be supported in 1:1 DL/UL mapping FeatureSet framework;

- Pain point 3: Non-forward compatible signaling structure design, e.g., band combination, power class, etc;

- Pain point 4: Duplicate retrieval framework with/without RACS;

- Pain point 5: Impractical RACS ID;

- Pain point 6: Mismatch between UE and NW supported features;

- Pain point 7: Capability without corresponding network configuration.

It is RAN2 understanding that pain point 1-3 can be considered from 6G Day 1 system design. RAN2 respectfully asks RAN1 and RAN4 to take the above information into account in 6G system design requirement for capability signaling and framework simplification.

# Action

RAN2 respectfully asks RAN1 and RAN4 to take the above information into account in 6G system design requirement.

# Dates of the next TSG RAN WG2 meetings

TSG RAN2 Meeting #133		9 – 13 February 2026	Gothenburg, Sweden

TSG RAN2 Meeting #133bis	13 – 17 April 2026	St. Julian, Malta

# Annex: A List of examples for UE assistance information related to UE AS capability change

| Area | Information item | Direct Relevant UE capability |
| --- | --- | --- |
| Overheating Related | reducedMaxMIMOLayers-FRx) | maxNumberMIMO-LayersNonCB-PUSCH maxNumberMIMO-LayersCB-PUSCH maxNumberMIMO-LayersPDSCH |
|  | reducedMaxBW-FRx | supportedBandwidthDL supportedBandwidthUL |
|  | reducedMaxCC | ca-BandwidthClassDL-NR ca-BandwidthClassUL-NR |
| MUSIM Related | musim-MIMO-Layers-DL/UL | maxNumberMIMO-LayersNonCB-PUSCH maxNumberMIMO-LayersCB-PUSCH maxNumberMIMO-LayersPDSCH |
|  | musim-SupportedBandwidth-DL/UL | supportedBandwidthDL supportedBandwidthUL |
|  | musim-AvoidedBandsList | supportedBandList supportedBandCombinationList |
| IDC Related | IDC assistance information (idc-Assistance idc-FDM-Assistance idc-TDM-Assistance) | supportedBandList supportedBandCombinationList |
| Power Saving Related | maxMIMO-LayerPreferenceConfig | maxNumberMIMO-LayersNonCB-PUSCH maxNumberMIMO-LayersCB-PUSCH maxNumberMIMO-LayersPDSCH |
|  | maxCC-Preference | ca-BandwidthClassDL-NR ca-BandwidthClassUL-NR |
|  | maxBW-Preference maxBW-PreferenceFR2-2 | supportedBandwidthDL supportedBandwidthUL |
|  | MinSchedulingOffsetPreference/ MinSchedulingOffsetPreferenceExt | crossSlotScheduling-r16 dynamicIndicationSchedulingRestriction-r18 |
| AI/ML Related | applicabilityReportList | aiml-CSI-PredictionPerBC aiml-CSI-Prediction aiml-BM-Case1 aiml-BM-Case2 |