3GPP TSG-RAN WG2 Meeting #132	R2-2508077

Dallas, USA, Nov. 17th -21st, 2025

Agenda Item:	10.2.2

Source:	Xiaomi

Title:	Discussion on the general aspects of 6G NTN

Document for:	Discussion and Decision

Introduction

In RAN#108 meeting, RAN WG study item on “Study on 6G Radio” [1] was approved. In the SID, following NTN-related objective is mentioned [1].

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

In the last RAN2 meeting, 6G NTN was briefly discussed and following guidance was captured in the chairman notes [2].

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this contribution, we discuss the general aspects of 6G NTN and try to identify which functionalities are common with TN and which are NTN specific.

Discussion

- 2.1 Services/features for 6G NTN?

In the last RAN2meeting, following features were brought up by companies and during online discussion, it was commented that some of these may not be in RAN2 scope.

- GNSS independent NTN operation

- Extended coverage

- Flexible duplex mode support at UE level

- Support of HD-FDD at Network side

- Massive messaging capability

- Positioning, Navigation and Timing

- Enhanced network verified UE location service

- Seamless TN/NTN mobility in connected mode

- 6G NTN coexistence with IoT-NTN and NR-NTN

- ICAS (Integrated Communication And Sensing)

- Broadcast, multicast services

- HARQ feedback disabling for DL/UL

- Low overhead access (e.g., RACH-less like), in particular for mobility

- Time-/Location-based mobility mechanism

- Assistant information for TN-NTN mobility

- Satellite switch with resync

- Discontinuous coverage

- Store and forward

Among above features:

- RANP Scope:

  - Support of new service: Positioning, navigation and timing, ISAC, broadcast and multicast services are related to services to be supported by 6G, which should be discussed and decided in RAN plenary.

  - Support of device type: For discontinuous coverage and store and forward, these two are IoT NTN features and whether to support them in 6G NTN seems to relate to the discussion on whether 6G NTN is expected to support IoT devices like NB-IoT and whether 6G IoT’s services are delay tolerant, or whether IoT NTN is considered as solutions for mMTC over NTN. We think this should also be discussed in RAN plenary.

- RAN1 scope: For extended coverage and duplex mode, they are currently being discussed in RAN1 and we think RAN2 can wait for RAN1’s progress.

Proposal 1: Following NTN features/services are:

- To be discussed and decided in plenary: positioning, navigation and timing, ISAC, broadcast and multicast services, discontinuous coverage, store and forward

- To be discussed and decided in RAN1: extended coverage, duplex mode

In the following sections, we will go through other major NTN features that have been supported in 5G and discuss whether and how they should be studied in 6G.

- 2.2 Features in 5G NTN

In 5G NR, NTN features are introduced from Rel-17 and most functionalities are based on TN’s system framework with some NTN specific adaptation/enhancements. Following table gives a summary of 5G NTN specific features that have RAN2 impact so far [3] and our considerations for 6G study.

| Functionalities | 5G NTN specific features | Considerations in 6G |
| --- | --- | --- |
| Initial access | Time/frequency pre-compensation using UE GNSS and satellite ephemeris information | Common TN/NTN design is preferred, see section 2.2. (1) |
| Access control | Separate cell barred indication for NTN access | Common TN/NTN design is preferred, see section 2.2. (1) |
| SI broadcasting and acquisition | New SIB19 for broadcasting satellite assistance information | NTN specific feature, see section 2.2. (6) |
| UL/DL scheduling | Cell specific and UE differential Koffset HARQ feedback disabling, HARQ mode A/B TA reporting | NTN specific feature, see section 2.2. (7) |
| L2 procedures | RAR window/Contention Resolution Timer delayed by UE-gNB RTT DRX HARQ RTT timers extended by UE-gNB RTT RLC/PDCP timer extension | Common TN/NTN design is preferred with NTN specific value range, see section 2.2. (3) |
| Idle mobility measurement | Time/location-based neighbor cell measurement initiation UE-based SMTC adjustment | Unified framework, taking into account NTN specific characteristics, see section 2.2. (5) |
| Connected mode measurement | PDD reporting and NW-based SMTC adjustment | Unified framework, taking into account NTN specific characteristics, see section 2.2. (5) |
| Connected mode handover | Time/location-based CHO Satellite switch with resync | Unified framework, taking into account NTN specific characteristics, see section 2.2. (5) |
| Coverage enhancement | UL coverage enhancement: - PUCCH repetition for Msg4 HARQ-ACK; - NTN-specific PUSCH DMRS bundling enhancement; DL coverage enhancement: - PDCCH repetition - PDSCH repetition for Msg4/SIB1 - 160ms SSB periodicity, SMTC selection | Common TN/NTN design is preferred, see section 2.2. (4) |
| Capacity enhancement | OCC | Common TN/NTN design is preferred, see section 2.2. (4) |

Note: the details are analyzed in the following sections.

- 2.3 Supported features in 6G NTN

In this section, we will go through 5G NR NTN features and try to identify in 6G NTN whether some functionalities should be studied in the common design with TN and some NTN specific enhancements should be considered.

(1) Initial access

In NR both for TN and NTN, time and frequency synchronization are required for UEs to access the network. In 6GR, we believe that these requirements are still there, otherwise there would be strong UL/DL interference in the network. However, in 5G due to the late introduction of NTN features, time and frequency synchronization of NTN operation is based on TN signals and RACH procedures and highly relies on UE’s GNSS location information, i.e. GNSS-based time and frequency pre-compensation. This on one hand requires that UEs, even for low-end devices, must be equipped with GNSS module for NTN access and on the other hand would sometimes cause NTN communication malfunctioning, e.g., when GNSS becomes temporarily unavailable. In 6GR NTN, we should remove this kind of GNSS requirement and have common design for TN and NTN.

(2) Access control

In NR NTN, a separate cellBarredNTN indication is introduced in SIB1 to bar NTN-capable UEs and cellBarred in MIB is used to bar non-NTN-capable UEs. This is because that NTN operation requires extra UE capabilities (e.g., GNSS, SIB19 acquisition, etc.) than TN operation. In 6G, with the objective of harmonized TN/NTN design, access control should be unified for TN and NTN from day 1, e.g., to avoid separate cell barred indications in MIB and SIB1.

Proposal 2: In 6G, RAN2 should have common TN/NTN design on initial access and access control, e.g., to study GNSS-less time/frequency synchronization procedure and unified cell barring mechanism.

(3) L2/L3 procedures

In NR, the late introduction of NTN features has led to separate configurations for the same functions or parameters to accommodate NTN’s larger RTT. For example, in the RRC configuration, RLC and PDCP timers (such as t-Reassembly and discardTimer) have been extended in NR NTN using new extension to handle longer delays. This results in a more complex and less uniform specification. In MAC specification, the start time of RAR window and Contention Resolution timer in random access procedure needs to consider an additional UE-gNB RTT. In DRX procedures, UL/DL HARQ RTT timers are extended with UE-gNB RTT and to make MAC specification work, RAN2 even renames the HARQ RTT timers for NTN.

All above aspects decrease the specification’s readability although they are all targeting the same parameters or procedures. In 6G, when designing radio configurations (e.g. L2 timers/windows) and procedures, RAN2 should take both TN and NTN’s characteristics (e.g. RTT) into account from the beginning and try to avoid late extensions.

Proposal 3: In 6G, RAN2 should aim to design unified L2 procedures for TN/NTN and take into account both TN and NTN’s characteristics (e.g. RTT range) when designing values for L2 timers/windows.

(4) Coverage and capacity enhancement

In 5G, several solutions for coverage enhancements and capacity enhancements have been introduced for NTN. Coverage enhancements include mechanisms such as PUCCH repetition for Msg4 HARQ-ACK, PUSCH DMRS bundling for improved channel estimation, and downlink repetition for PDCCH, PDSCH (Msg4 and SIB1), as well as extended SSB periodicity. Capacity enhancements include inter-slot orthogonal cover codes to multiplex multiple UEs on PUSCH.

In 6G, we think that coverage and capacity are still the two things that operators care most as they are the fundamental and common design target for TN and NTN. In 6G study, we think that 3GPP should have common design from day 1 and try to avoid having late NTN-specific enhancements after TN solution is done. A unified design can reduce complexity, avoid duplication of mechanisms, and enable consistent behavior across TN and NTN, while still allowing parameter tuning for different deployments. As these two features are mostly RAN1-centric, RAN2 can first wait for RAN1’s progress.

Proposal 4: In 6G, RAN2 should have common design on coverage and capacity enhancement for TN and NTN.

(5) Measurement and Service continuity

In 5G TN, RRM-based measurements serve as the foundation for mobility and service continuity procedures, and NTN follows the same principle. Meanwhile, 5G NTN considers the following NTN specific characteristics.

- Frequent Serving Cell Changes due to Satellite Switch: Due to the high-speed movement of satellites (except GSO), the UE’s serving cell may change frequently, even when the UE itself is stationary.

- Less Pronounced Near-Far Effect: In NTN, the near-far effect is less pronounced compared to TN. Due to the long transmission distance from satellites, the signal quality experienced by the UE remains relatively consistent across the cell coverage area, including both the cell center and the cell edge.

- Variable UE-to-Satellite Distance: The distance between the UE and the satellite can fluctuate greatly, leading to variations in propagation delay. This impacts the consistency of the user experience and requires adaptive link management.

In accordance with above NTN specific characteristics, several NTN-specific solutions have been introduced in 5G NTN as follows:

- The frequent serving cell changes in NTN, particularly for non-GSO satellites, may lead to increased signaling overhead for UEs in Connected state if mobility is done via handover. In 5G, satellite switch with resynchronization has been specified for the satellite switch in quasi-earth fixed cell scenarios to bypass the L3 handover procedure for Connected state UEs,

- To address the Less Pronounced Near-Far Effect issue, 5G introduced location-based and time-based CHO for RRC connected UE, as well as location-based and time-based measurement initiation for the RRC idle/inactive UE.

- From RRM measurement’s perspective, the issue with the Variable UE-to-Satellite Distance is that one SMTC can’t cover all the satellites, so SMTC adaptation is needed. In 5G, the SMTC adaptation based on the PDD (propagation delay difference) reporting is used for the RRC connected UE, while the RRC idle/inactive UEs adjust the SMTC based on the UE’s GNSS location and satellite’s ephemeris information.

In 6G, for measurement and service continuity, we think that the fundamental procedures for TN and NTN should be based on a unified framework, i.e., the overall procedures and principles for measurement, cell reselection and handover remain consistent for TN and NTN. This kind of unified framework would simplify a lot mobility management across TN and NTN and reduce complexity by avoiding separate designs in terms of specification impact and UE/NW implementations.

However, under this unified measurement and service continuity framework, NTN specific enhancements still need to be studied to handle NTN specific characteristics for better mobility performance. In addition to the above mentioned 5G NTN characteristics, in 6G NTN, we think that multiple satellite orbital altitudes should be considered as well.

- Multiple Orbital Altitudes: Satellites operating at different orbital altitudes (e.g., LEO, MEO, GEO) offer varying coverage areas, latency profiles and service capabilities. As a result, satellites in different orbits provide cells with different service characteristics, which can impact how the UE determines the target cell.

In 5G, service continuity mechanisms do not explicitly consider the different service characteristics provided by satellites at different orbital altitudes. However, in 6G, it may be necessary to take this factor into account during cell selection/reselection and handover procedures. By taking satellite orbital altitude into account, the UE can be served by a satellite which aligns better with its service requirements, and helps to optimize user experience and power consumption.

Proposal 5: In 6G, for measurement, cell reselection and handover, RAN2 should aim at a unified framework to ensure consistent principles and procedures for TN and NTN. Under this unified framework, NTN specific characteristics should be studied, e.g.:

- Frequent serving cell changes due to satellite switch

- Less pronounced near-far effect

- Variable UE-to-satellite distance

- Multiple orbital altitudes

(6) SI broadcasting and acquisition

In 5G TN, MIB and SIB1 are essential system information for IDLE/INACTIVE state UEs. In 5G NTN, SIB19 is one more essential SIB for UE to access NTN and it contains satellite assistance information that is required for UE to perform time and frequency pre-compensation, neighbor cell RRM measurement, satellite switch with resync, etc. In 6G, we think that broadcasting satellite assistance information for NTN operation would still need to be studied as one of the NTN-specific.

Proposal 6: In 6G, broadcasting satellite assistance information should be studied as one of the NTN-specific features.

(7) UL/DL scheduling

6G NTN is expected to cover the same scenarios as what 5G NTN has done, like GEO/MEO/LEO. According to TR 38.821 [4], GEO NTN’s RTT can be as large as 541.46 ms, and LEO-600km NTN’s RTT can range from 25.77 ms to 19.53 ms. Larger RTT leads to the scheduling efficiency issue (i.e. the HARQ stalling issue), for which NR NTN has introduced HARQ feedback disabling for downlink and HARQ mode A and mode B for uplink. Varying RTT for high-speed moving LEO satellite raises the challenge for network scheduling if network always schedules based on the maximum RTT. To this end, NR NTN has introduced the concept of Koffset (including cell specific and UE differential Koffset) and TA reporting. In our understanding, these NTN specific requirements would still be valid for 6G NTN and NR NTN solutions can be taken as baseline.

Proposal 7: In 6G, mechanisms to improve scheduling latency and efficiency should be studied to handle NTN specific issues with large and varying RTT. NR NTN solutions (e.g. HARQ feedback disabling, HARQ mode A/B, cell-specific/UE-specific Koffset, TA reporting, etc.) can be taken as baseline.

Conclusion

Based on the discussion, we have following proposals:

6G NTN services/features?

Proposal 1: Following NTN features/services are:

- To be discussed and decided in plenary: positioning, navigation and timing, ISAC, broadcast and multicast services, discontinuous coverage, store and forward

- To be discussed and decided in RAN1: extended coverage, duplex mode

Common TN/NTN design

Proposal 2: In 6G, RAN2 should have common TN/NTN design on initial access and access control, e.g., to study GNSS-less time/frequency synchronization procedure and unified cell barring mechanism.

Proposal 3: In 6G, RAN2 should aim to design unified L2 procedures for TN/NTN and take into account both TN and NTN’s characteristics (e.g. RTT range) when designing values for L2 timers/windows.

Proposal 4: In 6G, RAN2 should have common design on coverage and capacity enhancement for TN and NTN.

Common TN/NTN design with potential NTN specific enhancements

Proposal 5: In 6G, for measurement, cell reselection and handover, RAN2 should aim at a unified framework to ensure consistent principles and procedures for TN and NTN. Under this unified framework, NTN specific characteristics should be studied, e.g.:

- Frequent serving cell changes due to satellite switch

- Less pronounced near-far effect

- Variable UE-to-satellite distance

- Multiple orbital altitudes

NTN-specific design

Proposal 6: In 6G, broadcasting satellite assistance information should be studied as one of the NTN-specific features.

Proposal 7: In 6G, mechanisms to improve scheduling latency and efficiency should be studied to handle NTN specific issues with large and varying RTT. NR NTN solutions (e.g. HARQ feedback disabling, HARQ mode A/B, cell-specific/UE-specific Koffset, TA reporting, etc.) can be taken as baseline.

Reference

[1] RP-251881, NTT DOCOMO (Moderator), “New SID: Study on 6G Radio”

[2] R2-250xxx, R2_131bis_ChairNotes_final

[3] 3GPP TS 38.300 V19.0.0, NR and NG-RAN Overall Description

[4] 3GPP TR 38.821 V16.2.0, Solutions for NR to support non-terrestrial networks (NTN)