3GPP TSG RAN WG2 Meeting #132         	                             R2-2508083

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.3.3.1

Source:	Xiaomi, Apple, Samsung, Qualcomm, Huawei, HiSilicon

Title:	Unnecessary standardized model transfer and delivery

Document for:	Discussion and Decision

# Introduction

During RAN2 #131bis meeting, the requirement of model transfer was proposed to be discussed by operators and RAN2 agreed to study the requirement of model transfer for 6G:

4. Study model transfer/delivery requirements/functions

Before studying the requirement of model transfer/delivery, it is essential for RAN2 to first discuss whether there’s a valid use cases and scenarios that need a standardized solution for model transfer/delivery.

In this contribution, based on scenarios to be supported for 6G AI/ML air, we analyse and discuss the necessity of a standardized solution for model transfer and delivery.

# Discussion

## Terminology and Definition

The terminology of AI/ML model transfer has been discussed during Rel-18 AI/ML air interface SI with the following understanding [1]:

AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

In Rel-19, RAN2 discussed signalling feasibility of transferring dataset/model parameter for two-sided model based on RAN1 LS [2] and further agreed ‘model parameter transfer’ for two-sided model from NW to UE-side OTT server is supported based on non-OTA approach. During [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson) [3], it is RAN2 common understanding that dataset/model parameter transfer is different from model transfer/delivery as they serve different purposes: the former one is used for UE-part two-sided model training, the later one is used for model deployment at UE-side for model inference.

Based on 6G data transfer discussion [4], model parameter transfer over non-OTA approach can be further characterized as AI/ML model training data transfer. The definition of AI/ML model transfer can further be clarified it is for model inference purpose only.

Proposal 1: Dataset/model parameter transfer for two-sided model from NW to UE-side OTT server is considered as AI/ML training data collection, instead of model transfer/delivery. Model transfer/delivery for two-sided model is model download trained UE-part two-sided model from UE-side OTT server to UE for model inference.

Based on this understanding, before studying/discussing the requirement of model transfer/delivery, it is essential for RAN2 to first discuss the use cases and scenarios that need a standardized solution for model transfer/delivery over the air interface.

During RAN1 #122bis meeting, RAN1 agreed to study several AI/ML air interface use cases, including 1) frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML, 2) low overhead DMRS with AI/ML receiver, 3) CSI compression, 4) AI-based (de)modulation, 5) AI-based DPoD/None-linearity compensation, 6) Low overhead SRS with AI/ML, 7) AI-enabled UL precoder indication, 8) Inter-Cell/M-TRP DL Tx beam prediction and management, etc [9]. During RAN1 discussion, model location for inference mainly concerns the following three scenarios as in 5G:

- NW-sided model

- UE-sided model

- Two-sided model

Observation 1: RAN1 6G studied several AI/ML based air interface use cases and considers three scenarios of model location for inference: 1) NW-sided model, 2) UE-sided model, 3) two-sided model, as in 5G.

Based on above RAN1 6G discussion, we further discuss the need of model transfer from model location for inference point of view.

## NW-sided model transfer

As discussed earlier, AI/ML model transfer mainly focuses on how to transfer AI/ML model/model parameter to UE over the air interface for inference purpose. Therefore, model transfer/delivery for NW-side model (e.g., from NW training entity to RAN) is not considered as an applicable scenario. Whether/how to do model/model parameter transfer between network nodes is out of RAN2 scope, and should also be transparent to UE.

Proposal 2: For NW-side model, whether and how to support a standardized solution on model transfer/delivery (e.g., CN -> RAN, OAM -> RAN) is within remit of RAN3/SA2/SA5. NW-side model transfer/delivery should be transparent to UE.

## UE-sided/UE-part two-sided model transfer

For UE-sided model, the necessity of a standardized 3GPP model transfer/delivery solution is still questionable. This includes both one-sided model and two-sided model when UE-part model needs to be transferred from UE-side OTT server to UE. Note that, based on the above discussion, dataset/model parameter transfer of two-sided model is considered as data transfer, instead of model transfer.

First of all, the generalization performance of AI/ML enabled use cases is always the most challenging issue to be solved. A good UE-side model should be trained by considering both NW-side additional condition and UE-side additional condition. To achieve this, RAN1/2 introduced associated ID to implicitly allow UE to train different models that are suitable for different NW-side additional condition (e.g., beam deployment, etc) based on data collected associated to different associated IDs. On the other hand, UE-side condition is not visible to MNO. The UE-sided AI/ML model development depends on the device proprietary software and hardware architecture, which are not shared with MNO and NW vendors. This makes UE-side OTT-server becomes more suitable to perform UE(vendor/chipset)-specific model training to fit different UE implementation.

Observation 2: UE-side/UE-part two-sided model needs to be trained and tailored at UE-side OTT server rather than at MNO, based on different UE implementation and device proprietary software and hardware architecture, which can be vendor-specific with proprietary AI/ML algorithms and implementations.

Figure 1. LCM of UE-side model

Since the model is trained at UE-side OTT server, the trained model will be further downloaded from UE-side OTT server and deployed at UE device. This procedure of model transfer can simply be supported by application layer, similar as downloading mobile OS/application from application server.

When there’s a need for AI/ML model download, the mobile device can first receive a notification in application layer. End user can select whether to use cellular via 3GPP network or non-3GPP network (e.g., WiFi). Whether to use cellular network is up to end-user. Even if cellular is selected by user, AI/ML model itself can be considered as one of the user traffic, i.e., AI/ML model transfer can be fully transparent and agnostic to 3GPP network.

Observation 3: UE-side/UE-part two-sided AI/ML model transfer is quite similar to mobile OS/application upgrade. Whether there’s controllability from MNO is the same as in legacy, which can be fully agnostic to 3GPP.

As discussed in [5], the main motivation of supporting 3GPP AI/ML model transfer solution for air interface is to allow MNO awareness of model transfer and avoid negative impact to 3GPP network. However, some assumptions discussed in [5] need to be further discussed and justified:

Model Size

The model size is assumed to be hundreds of megabytes in [5]. As studied and summarized in [1][10], across all RAN1/RAN2 studied use cases, the majority reported AI/ML model size is between 0.1 Mbytes and 4Mbtes, even the maximum reported AI/ML model size is less than 25Mbytes.

From TR38.843: Table 6.3.2-1: AI/ML model complexity/computation complexity used in the evaluations for AI/ML in beam management Model complexity in number of model parameters; Model complexity in number of model size; Computational complexity (FLOPs). BM-Case 1 DL Tx beam; More than 1k to 4.9M majority reported less than 1M or about 1M; 50Kbytes to 20Mbytes majority reported less than 0.1Mbytes ~ 0.6Mbytes; ~2.7K to 222M majority reported less than 1M or 10s M. BM-Case 1 DL beam pair; 72k to 4.9M majority reported less than 0.1s M ~ 1M; 0.17Mbytes to 21Mbytes majority reported less than 1Mbytes ~ 4Mbytes; 15K to 224M majority reported less than 1M ~ 4 M. BM-Case 2 DL Tx beam; 35k to 11M majority reported less than 0.1s M ~ 1M; 0.5Mbytes to 15Mbytes majority reported about 1 Mbytes; ~90K to 54M majority reported less than 0.1s M or 1s M. BM-Case 2 DL beam pair; 20k to 13M majority reported about 0.1M~1M; 0.08M to 15M majority reported about 1Mbytes; ~90K to 443M majority reported less than 0.4 M or 1s M.

From TR38.744: Table 5.2.2-1: Analysis of complexity of AIML models used for evaluations in RRM measurement prediction Model complexity in number of model parameters; Model complexity in model size; Computational complexity (FLOPs). FR1 to FR1 intra-frequency temporal domain case B; 16K to 1.51M majority reported less than 0.8M; 67Kbytes to 5.1Mbytes majority reported less than 1Mbytes; 0.12M to 23.86M majority reported less than 3M. FR1 to FR1 inter-frequency (frequency domain); 0.22K to 1.84M majority reported less than 0.33M; 2.1Kbytes to 7.3Mbytes majority reported less than 1Mbytes; 5K to 66M majority reported less than 3M. FR2 to FR2 intra-frequency temporal domain case A; 4.5k to 1.38M majority reported less than 0.7M; 69Kbytes to 10.8Mbytes majority reported less 0.74 Mbytes; 20K to 33.1M majority reported less than 10M.

It is further captured in ITU-R minimum requirements related to technical performance for IMT-2030 radio interface(s) [6], 6G is expected to support "user experienced" data rate around at least 150 or 300 Mbps. In our understanding, the limited size of AI/ML model (e.g., 25Mbytes) will not cause congestion issue within 6G network.

Table 1 5th percentile user data rate Test environment; 5th percentile DL user data rate; 5th percentile UL user data rate. Dense Urban-IC; [150 or 300 Mbps]; [50 or 75 Mbps]. [Rural-UC].

Observation 4: According to 5G AI/ML use cases captured in TR 38.843 and TR 38.744, majority reported AI/ML model size is between 0.1 Mbytes and 4Mbtes, even the maximum reported AI/ML model size is less than 25Mbytes. Thus, 5G did not identify the requirement of large volume AI/ML model transfer.

Observation 5: Transferring AI/ML model (from a few KBytes to a few Mbytes) over user plane within 6G network that supports 150-300Mbps DL user experienced data rate should not be challenging.

Model transfer periodicity/criteria

It is also assumed by [5] that the periodicity of model transfer can be daily/weekly/monthly/others and the criteria of triggering model transfer cannot be controlled by network. On one hand, the update periodicity of model is up to implementation by considering multiple factors, e.g., model generalization performance, availability of trained model, etc. As discussed in earlier section, model transfer can be trigger by end user in application layer based on system notification, based on which, network further establish/modify the PDU session to support model transfer as user traffic. There’s no need for network awareness to control/trigger model transfer/delivery.

Observation 6: Model transfer periodicity is up to implementation and the model transfer can be triggered by end user via application layer without network awareness.

Impact to service quality of other user traffic types

Another concern raised by [5] is the model transfer may impact user experience during peak hour, and a lower priority is preferred to ensure network stability. We understand the concern from operators that there’s no doubt to consider higher priority user traffic (e.g., XR, mobile AI, etc) to be guaranteed first. During 5G, SA1 studied the traffic characteristics and performance requirements for AI/ML model transfer for image recogination [7], and SA2 further defined specific 5QI to QoS characteristics mapping for intelligent transport and AI/ML application model transfer [8]. However, as we discussed in Observation#, for AI/ML model used for air interface use cases, 5G did not identify the requirement of large volume AI/ML model transfer in both RAN1 and RAN2. Without understanding the new use case(s) with a strong need to support large size AI/ML model transfer for air interface use case(s), it is not clear how RAN2 can conclude the DL traffic congestion due to AI/ML model transfer is an essential issue to deploy AI/ML in 6G. Even though there’s a need, it is expected that SA1 and SA2 can further discuss whether there’s a need and how to define separate 5QI to QoS characteristics mapping for AI/ML air interface model transfer.

5QI Value; Resource Type; Default Priority Level; Packet Delay Budget; Default Priority Level; Packet Delay Budget; Default Priority Level; Packet Delay Budget. 84; Delay-critical GBR; 24; 30 ms (NOTE 6); 10-5; 1354 bytes (NOTE 3); 2000 ms; Intelligent transport systems (see TS 22.261 [2]). 85; 21; 5 ms (NOTE 5); 10-5; 255 bytes; 2000 ms; Electricity Distribution- high voltage (see TS 22.261 [2]). V2X messages (Remote Driving. See TS 22.186 [111], NOTE 16, see TS 23.287 [121]). Split AI/ML inference - DL Split AI/ML image recognition, (see TS 22.261 [2]). 88; 25; 10 ms (NOTE 4); 10-3; 1125 bytes; 2000 ms; Interactive Service - Motion tracking data, (see TS 22.261 [2]), split AI/ML inference - UL Split AI/ML image recognition, (see TS 22.261 [2]).

Observation 7: Model transfer doesn’t need to have latency requirement, hence no need to define QoS requirement. Even if traffic differentiation between AI/ML model traffic and other services is needed, different QoS requirement can be discussed/considered in SA.

Based on above observations, there’s no need for 3GPP to develop a standardized solution for UE-side/UE-part two-sided model transfer/delivery.

Proposal 3: In 6G, UE-side/UE-part two-sided model transfer/delivery (e.g., model transfer, model content, model version, etc) is transparent to 3GPP (either via non-3GPP solutions or via UP transparently). No need to define requirement for UE-side/UE-part two-sided model transfer/delivery.

# Conclusion

Terminology

Proposal 1: Dataset/model parameter transfer for two-sided model from NW to UE-side OTT server is considered as AI/ML training data collection, instead of model transfer/delivery. Model transfer/delivery for two-sided model is model download trained UE-part two-sided model from UE-side OTT server to UE for model inference.

Observation 1: RAN1 6G studied several AI/ML based air interface use cases and considers three scenarios of model location for inference: 1) NW-sided model, 2) UE-sided model, 3) two-sided model, as in 5G.

NW-sided model

Proposal 2: For NW-side model, whether and how to support a standardized solution on model transfer/delivery (e.g., CN -> RAN, OAM -> RAN) is within remit of RAN3/SA2/SA5. NW-side model transfer/delivery should be transparent to UE.

UE-sided/UE-part two-sided model

Observation 2: UE-side/UE-part two-sided model needs to be trained and tailored at UE-side OTT server rather than at MNO, based on different UE implementation and device proprietary software and hardware architecture, which can be vendor-specific with proprietary AI/ML algorithms and implementations.

Observation 3: UE-side/UE-part two-sided AI/ML model transfer is quite similar to mobile OS/application upgrade. Whether there’s controllability from MNO is the same as in legacy, which can be fully agnostic to 3GPP.

Observation 4: According to 5G AI/ML use cases captured in TR 38.843 and TR 38.744, majority reported AI/ML model size is between 0.1 Mbytes and 4Mbtes, even the maximum reported AI/ML model size is less than 25Mbytes. Thus, 5G did not identify the requirement of large volume AI/ML model transfer.

Observation 5: Transferring AI/ML model (from a few KBytes to a few Mbytes) over user plane within 6G network that supports 150-300Mbps DL user experienced data rate should not be challenging.

Observation 6: Model transfer periodicity is up to implementation and the model transfer can be triggered by end user via application layer without network awareness.

Observation 7: Model transfer doesn’t need to have latency requirement, hence no need to define QoS requirement. Even if traffic differentiation between AI/ML model traffic and other services is needed, different QoS requirement can be discussed/considered in SA.

Proposal 3: In 6G, UE-side/UE-part two-sided model transfer/delivery (e.g., model transfer, model content, model version, etc) is transparent to 3GPP (either via non-3GPP solutions or via UP transparently). No need to define requirement for UE-side/UE-part two-sided model transfer/delivery.

# Reference

[1] TR 38.843	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2] R1-2410922	LS on signalling feasibility of dataset and parameter sharing

[3] R2-2501939	Report of [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson)

[4] R2-2508082	Consideration on 6GR data transfer and AI/ML framework	Xiaomi

[5] R2-2507583	Guidelines for 6G AI_ML for the air interface model delivery options	BT Plc, T-Mobile USA, Orange, Deutsche Telekom, Turkcell, Verizon, Vodafone, KDDI

[6] Report ITU-R M.[IMT-2030. TECH PERF REQ]

[7] TR 22.874	Study on Traffic characteristics and performance requirements for AI/ML Model Transfer

[8] TR 23.501	System architecture for the 5G System (5GS)

[9] RAN1 #122bis meeting chair note

[10] TR 38.744	Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR