3GPP TSG-RAN WG2 Meeting #132	R2-2508095

Dallas, USA,  Nov. 17th - 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on RRM measurement prediction

Agenda Item:	9.3.2

Document for:	Discussion and Decisions

- Introduction

During the RAN#109 meeting, the WID on “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was modified and the objective of the work on RRM measurement prediction for UE sided model is as below,

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]:

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported.

- L3 Cell level predication is supported

Note 1: For UE sided model, spatial domain prediction is not supported.

Note 2: For UE sided model, L3 beam level prediction is not supported.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach

- Inference configuration and inference report

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection

- For measurement event prediction:

  - Measurement event A1~A6 are in the scope in this release

Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

This paper discusses the functionality management for UE sided RRM measurement prediction, including the applicability procedure, inference configuration, inference report configuration and performance monitoring.

- UE capability

At the RAN2 #129bis meeting[2], the following were agreed for the applicability procedure for AI4Mob[2] regarding the UE capability,

Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity

According to the revised WID, following scenarios and sub-use cases as shown in Table 1 are supported for the UE sided model. It is straightforward to support the functionality granularity to be at the sub-use cases level for each sub-use case in each scenario, e.g. Frequency domain co-located and sub-use case 1.

Table 1 Scenario and Sub-use cases for UE-sided model

| Scenario | Sub-use cases |
| --- | --- |
| FR2 Temporal domain Case A | Sub-use case 1 |
|  | Sub-use case 2 |
|  | Sub-use case 3 |
| FR1 Temporal domain Case B | Sub-use case 1 |
|  | Sub-use case 2 |
|  | Sub-use case 3 |
| FR1 Frequency domain co-located | Sub-use case 1 |
|  | Sub-use case 2 |
|  | Sub-use case 3 |

Following the principle of AI4PHY as baseline, the UE capability will be exchanged by UECapabilityEnqiry and UECapabilityInformation message before the applicability checking. It was proposed by UE vendor that additional information like “predicted accuracy” could be additionally put in UECapabilityInformation[3]. One of the motivation is for the NW to decide whether and when to enable AI/ML in the UE after knowing about such improved accuracy. Moreover, other additional information like supported associated ID could also help the NW to make more appropriate decision.

However, considering the potential drawbacks for the NW side—such as increased decision-making complexity, the absence of a unified accuracy metric, and the risk of inconsistent or unverifiable UE-reported values—it is preferable that the NW explicitly controls whether the UE is allowed to report such information. Therefore, an indication of permission should be placed in UECapabilityEnquiry, enabling the NW to selectively request or suppress the reporting of predicted accuracy based on deployment needs.

Proposal 1:  RAN2 to support:

  - The functionality granularity to be at the sub-use cases level, i.e. each sub-use case in each scenario.

  - NW indicates whether specific information (e.g. Accuracy, Preferred measurement gap, Frequency range, supported associated ID) could be indicated in UECapabilityEnquiry message per functionality/RAT/frequency/cell(group).

  - UE may report the supported associated IDs together with corresponding supported functionalities in the UECapabilityInformation according to NW indication.

- Applicability

At the RAN2 #130 and #131 meeting, the following were agreed for the applicability procedure for AI4Mob and AI4PHY[4,5],

Agreements for AI4Mob at #130 meeting

1.     Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case)

- Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress)

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes.

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress).

- No prohibit timer is introduced.

2      Associated ID should be optionally configurable for training and  inference (i.e. it may not mandatorily required for all training/inference configurations).  FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

- The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain

- PW length (for temporal case A)

- Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional).  MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI

- Measured and predicted beam pattern (optional).   MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI

- Measured and predicted frequency (For inter-freq domain prediction)

Agreements for AI4Mob at #131 meeting

- Confirm that as a baseline following agreements on applicability reporting in AI-based beam management are applicable for AI mobility:

- UE may include “release configuration” flag in applicability reporting to indicate UEs preference to release a non-applicable configuration.

- Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled.

- When UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

- The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported predicted values after UE indicates that an inference configuration is not applicable.

- The UE shall report when an inference configuration becomes non-applicable.

- How to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration.

- Whether Option A and Option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

- RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A).

NOTE: these agreements will be aligned with AI PHY agreements at the end of this week and will not be re-discussed

Agreements for AI4PHY at #131 meeting

- Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

- Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.   FFS is we should have signalling indicating multi-cell.

- We will not define areas.  The Associated ID is 24 bits.

- 4	If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

Agreements for AI4PHY at #131 meeting

- If the UE is ready for inference by end of RRC processing delay, it reports model applicable.  If not, it reports model inapplicable and doesn’t set the release flag.   The network is not expected to release inference configuration (this will not be added to stage 3 specifcation).

Unification of Option A and Option B:

Based on the agreements made in #131 meetings, during the applicability confirmation stage, the NW may configure both Option A and Option B within the same RRCReconfiguration message, and the unified applicability reporting procedure applies regardless of which option is used.

For Option A, the full inference configuration may consist of one of multiple configuration sets. In temporal case A, the prediction window should be explicitly configured for prediction, whereas in temporal case B the prediction could be based on a skipping pattern, an up-sampling rate, or a virtual reference signal configuration. In the inter-frequency case, the inference configuration for prediction may omit the gap configuration, or the UE may ignore it. The inference configuration includes co-located information, such as the association between actual measured cells and predicted measured cells.

For Option B, the configuration is expressed as one of multiple sets of inference related parameters used for applicability checking per serving cell or measurement object. In temporal case A this includes the prediction window length, while in temporal case B the skipping pattern is provided for prediction configuration. In the inter-frequency case, the parameters include both the measured and predicted frequency, together with the same co-located information describing the relationship between actual measured cells and predicted measured cells.

Proposal 2: RAN2 to support Option A and Option B to be configured in the same RRCReconfiguration message with following information:

  - Option A: One of multiple sets of full inference configuration

- Temporal domain case A: The prediction window

- Temporal domain case B: The skipping pattern or up-sampling rate or virtual RS configuration

- Frequency domain:

      - Co-located information

      - Gap-less configuration for prediction

  - Option B: One of multiple sets of inference related parameters

- Temporal case A: PW length

- Temporal case B: Skipping pattern

- Frequency domain: Measured and predicted frequency, co-locate information

Associated ID:

The handling of the Associated ID for AI4PHY was clarified: both single cell specific and multi cell specific allocations are supported; the value is 24 bits and unique within a PLMN; if the NW does not provide the ID the UE determines applicability; and the NW is responsible for maintaining validity, including updating or de-configuring the ID when it becomes out-of-date. For AI4Mob at RAN2 #130, it was agreed that the associated ID is optionally configurable with further details left for the WI phase. To keep AI4Mob consistent with the above AI4PHY agreements, we propose to explicitly capture that AI4Mob shall support the same associated-ID treatment and the configuration applies uniformly under both Option A and Option B.

Proposal 3: RAN2 to support both single-cell specific associated ID and multi-cell specific associated ID with 24bits, which is unique within a PLMN, and can be used for both Option A and Option B.

Applicability reporting:

The benefit of inapplicability cause is proven for AI4PHY at least for the model availability. By knowing this information, the NW could have the freedom on whether release the corresponding inference configuration or not. In addition, except for the model availability, there is some other factor that may dynamically change at the UE side such as the power status, prediction accuracy due to the channel variation, and the buffer status etc. Therefore, the inapplicability cause reporting in general is worthwhile being introduced for AI4Mob.

Since the associated ID is introduced in the training and inference procedure, UE may be able to know the candidate associated IDs which are suitable for specific functionality. In this case, if the candidate associated IDs are provided to the NW and associated ID is unique within a PLMN if supported, NW would be able to make appropriate inference configuration by comparing the candidate associated IDs and the current NW side condition, so as to avoid sending the inapplicable inference configuration again.

Proposal 4: RAN2 to support to report following information in the initial or updated applicability reporting for both Option A and Option B in RRCReconfigurationComplete message:

  - Inapplicability cause

  - Candidate associated IDs

Applicability functionality reporting during handover procedure has already been supported for AI4PHY, and it is straightforward to be inherited to AI4Mob functionalities. In addition, different from beam management which may happen in all the positions within a cell, AI4Mob functionalities may always be used when the UE is at the cell edge where the handover or RLF frequently happens. Therefore, it is also beneficial to support the applicability reporting as soon as possible after RLF or HOF i.e. during RRC reestablishment, to enhance the performance at cell edge by using AI capability. In this case, since the content in RRC configuration is released in RLF or HOF as agreed at the #131 meeting, UE needs to know whether the related inference configurations or inference related parameters are still valid upon initiation of RRC Reestablishment procedure.

Proposal 5: RAN2 to support the applicability reporting during handover procedure and RRC reestablishment procedure.

  - UE needs to know whether the related inference configurations or inference related parameters are still valid upon initiation of RRC Reestablishment.

- Inference configuration

At the RAN2 #129bis, the following were agreed for the inference configurations [2],

Agreements at #129bis meeting

- The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required.    FFS when the UE can perform inference.

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

Inference activation/deactivation:

Because some inference-based functionalities are meaningful only under specific radio or mobility conditions (e.g., fast fading regions, cell-edge states, or mobility thresholds). Allowing the UE to autonomously activate or deactivate the configured inference behavior when a defined event becomes satisfied or unsatisfied avoids unnecessary inference processing and reduces power consumption, without requiring new signaling beyond the initial rules provided by the NW.

Since there are deployment scenarios where the NW , must retain control of when inference is active, for example to react dynamically to load, policy changes, performance monitoring results, or per-UE scheduling strategies, a lightweight L1/L2 trigger (MAC CE or DCI) enables the NW to activate or de-activate inference rapidly without re-issuing full RRC reconfiguration, which minimizes signaling overhead and latency.

When the inter-frequency prediction is activated, the UE is already capable of inferring measurements on a given frequency. Keeping the corresponding measurement gap active on the predicted frequency wastes time-domain resources and may reduce throughput, while disabling the measurement gap configuration by dedicated RRC signaling introduce additional latency and overhead. Allowing the UE to locally deactivate that specific measurement gap implicitly when corresponding inter-frequency inference is activated provides a measurable efficiency gain while keeping the original RRC configured measurement gap available for re-activation if inference later becomes deactivated.

Proposal 6: RAN2 to support

  - The event-triggered activation/deactivation of inference configuration, i.e. UE activates/de-activates the applicable functionalities when certain event condition is satisfied/unsatisfied.

  - The on-demand activation/deactivation of inference configuration by NW through MAC CE or DCI.

  - The autonomous deactivation of the configured measurement gap for a particular frequency band for which the infer-frequency prediction is activated.

How to configure:

At RAN2 #129bis, it was agreed that the inference configuration and reporting for AI/ML mobility can be based on the existing RRM measurement framework. However, the agreement did not yet specify how the inference configuration should be structured in the temporal and frequency domains. For actual measurement and prediction,  some implementations may reuse the same measurement object while others may require a dedicated object for prediction (e.g., when prediction relies on a different sampling granularity, reporting window, or reference signal type). Allowing both same or independent structure prevents unnecessary reconfiguration and enables consistent applicability behavior across products without prescribing a single model.

When inference is used for inter-frequency prediction, supporting a configuration without a gap avoids wasting time-domain resources, reduces interruption to serving cell reception, and improves UE efficiency. If a measurement gap is configured but prediction is activated due to certain reason, allowing the UE to ignore that gap avoids forcing redundant measurement behavior and preserves backward compatibility without requiring new signaling.

Proposal 7: RAN2 to support:

  - Temporal domain: The inference configuration for actual measurement and prediction in the same or independent measurement objects.

  - Frequency domain: The inference configuration for prediction without gap configuration; otherwise UE may ignore the gap configuration.

- Inference reporting

At the RAN2 #129bis and #131 meeting, the following were agreed for inference reporting [2][5],

Agreements at #129bis meeting

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

Agreements at #131 meeting

- For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration.  This list is optional.

- For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

- For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results).

- For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

When to report:

At RAN2 #129bis, it was agreed that, for UE-side RRM prediction, the UE may be configured with periodic or event-triggered reporting of predicted and/or actual RRM measurements, with the detailed conditions left for further study.

Supporting immediate reporting is necessary because the value of the prediction result may degrade if it is forced to wait for a periodic timer or an event trigger, especially when the inference output is generated within a short prediction window. If only periodic or event-triggered reporting were available, it would be difficult to benefit from the timing advantage of temporal domain Case A, where the usefulness of the prediction depends on its freshness. It would also make the behavior hard to distinguish from indirect event-based prediction, in which the inference result is only implied through an existing event rather than explicitly reported.

Event-triggered reporting with inference-specific triggers avoids forcing prediction-based reporting into existing RRM event definitions, which were designed for actual measurements rather than inferred values. Prediction may cross thresholds or indicate performance changes before the corresponding measurement-based event occurs, and allowing a dedicated inference trigger lets the NW react based on prediction rather than waiting for the measured degradation to appear.

Periodic reporting with an independent periodicity allows the NW to configure a reporting cycle that is suitable for prediction, rather than reusing the same periodic configuration as actual measurement reports. The evolution rate of predicted results may differ from that of measured results, and separating the two avoids both unnecessary signalling and delayed reporting when prediction output changes faster than measurement behaviour.

Proposal 8: RAN2 to support:

  - Immediate report when the predicted measurement results is obtained or the inference is finished.

  - Event-triggering with independent new events for inference results.

  - Periodic reporting with independent periodicity configuration for inference results from actual measurement.

What to report:

At RAN2 #131 meeting, the general contents in the inference report were agreed for temporal domain and frequency domain. The details on how the reporting should behave when a prediction result evolves across multiple inference instances or across multiple frequency or beam domains still needs to be addressed.

For temporal domain Case A and sub-use case 1, consolidation of predicted L1 beam-level measurement results across consecutive prediction instances is needed because a single inference execution may generate more than one predicted value, and consolidating them allows the UE to make full use of all available predicted instances rather than reporting only one of them. By aggregating the predicted values before further processing, the reported result becomes more stable across prediction windows and avoids the rapid fluctuation that would otherwise occur if each instance were treated independently, which in turn allows the NW to interpret the prediction outcome as a coherent and reliable value.

After consolidation, how to apply L3 filtering on the derived cell-level result needs to be clarified because the last available actual measurement remains more reliable than any predicted value, and the reliability of predicted results decreases as the prediction point moves further away from the current time. The filtered output based on the most recent actual measurement(Option 1) or on the most recent predicted values(Option 2) could be supported. enables the reporting to reflect the most trustworthy information available at that moment, while still maintaining a single reporting structure. This enables the L3 process to incorporate different reliability levels within a single filtering mechanism, instead of separating actual and predicted measurements into different reporting paths, and allows the NW to interpret the filtered result with an implicit reliability ordering.

Figure 1. L3 filtering for Temporal domain and sub-use case 1

For frequency domain prediction, the ability to include actual and/or predicted measurements of multiple frequencies in a single report avoids fragmentation when more than one inter-frequency prediction target is configured. A single message containing the multi-frequency results reduces overhead and keeps the correlation between frequencies intact, instead of requiring the NW to reconstruct it from several independent reports.

When multiple cells, beams or frequencies are involved, reporting only selected results based on configured rules—such as Top-K ranking, threshold filtering or maximum number limits—ensures that the NW receives only the most relevant values. This prevents oversized reports and enables the inference reporting mechanism to scale without increasing signaling load as the number of prediction candidates increases.

Finally, including L3-filtered quantities such as RSRP and RSRQ together with optional accuracy or confidence indicators allows the NW not only to consume the predicted value itself but also to judge its reliability. This distinction is necessary for cases in which the prediction result may be used for inter-frequency mobility decisions, beam switching or early preparation, since the NW may weight a high-confidence inference differently from a low-confidence one.

Proposal 9: RAN2 to support:

  - Temporal domain and sub-use case 1:

- Consolidation based on predicted L1 beam level measurements across multiple consecutive predicted instances.

- L3 filtering based on current consolidated L1 cell level measurement results and following options:

      - Option 1: The immediate last actual L3 cell level filtered measurement results with independent filter coefficients

      - Option 2: The immediate last L3 cell level filtered measurement results regardless of actual or predicted with independents filter coefficients.

  - Frequency domain:

- Inference report includes actual and/or predicted measurements of multiple frequencies in one measurement report.

  - Selective inference report based on configured priority and/or selection rule e.g. Top K, Threshold, Maximum number

  - Report quantity of L3 filtered RSRP and RSRQ,  and measurement accuracy/confidence level.

- Conclusion

In this contribution, we have the following proposals on functionality management for UE-sided RRM measurement prediction,

- UE capability

Proposal 1:  RAN2 to support:

  - The functionality granularity to be at the sub-use cases level, i.e. each sub-use case in each scenario.

  - NW indicates whether specific information (e.g. Accuracy, Preferred measurement gap, Frequency range, supported associated ID) could be indicated in UECapabilityEnquiry message per functionality/RAT/frequency/cell(group).

  - UE may report the supported associated IDs together with corresponding supported functionalities in the UECapabilityInformation according to NW indication.

- Applicability

Proposal 2: RAN2 to support Option A and Option B to be configured in the same RRCReconfiguration message with following information:

  - Option A: One of multiple sets of full inference configuration

- Temporal domain case A: The prediction window

- Temporal domain case B: The skipping pattern or up-sampling rate or virtual RS configuration

- Frequency domain:

      - Co-located information

      - Gap-less configuration for prediction

  - Option B: One of multiple sets of inference related parameters

- Temporal case A: PW length

- Temporal case B: Skipping pattern

- Frequency domain: Measured and predicted frequency, co-locate information

Proposal 3: RAN2 to support both single-cell specific associated ID and multi-cell specific associated ID with 24bits, which is unique within a PLMN, and can be used for both Option A and Option B.

Proposal 4: RAN2 to support to report following information in the initial or updated applicability reporting for both Option A and Option B in RRCReconfigurationComplete message:

  - Inapplicability cause

  - Candidate associated IDs

Proposal 5: RAN2 to support the applicability reporting during handover procedure and RRC reestablishment procedure.

  - UE needs to know whether the related inference configurations or inference related parameters are still valid upon initiation of RRC Reestablishment.

- Inference configuration

Proposal 6: RAN2 to support

  - The event-triggered activation/deactivation of inference configuration, i.e. UE activates/de-activates the applicable functionalities when certain event condition is satisfied/unsatisfied.

  - The on-demand activation/deactivation of inference configuration by NW through MAC CE or DCI.

  - The autonomous deactivation of the configured measurement gap for a particular frequency band for which the infer-frequency prediction is activated.

Proposal 7: RAN2 to support:

  - Temporal domain: The inference configuration for actual measurement and prediction in the same or independent measurement objects.

  - Frequency domain: The inference configuration for prediction without gap configuration; otherwise UE may ignore the gap configuration.

- Inference reporting:

Proposal 8: RAN2 to support:

  - Immediate report when the predicted measurement results is obtained or the inference is finished.

  - Event-triggering with independent new events for inference results.

  - Periodic reporting with independent periodicity configuration for inference results from actual measurement.

Proposal 9: RAN2 to support:

  - Temporal domain and sub-use case 1:

- Consolidation based on predicted L1 beam level measurements across multiple consecutive predicted instances.

- L3 filtering based on current consolidated L1 cell level measurement results and following options:

      - Option 1: The immediate last actual L3 cell level filtered measurement results with independent filter coefficients

      - Option 2: The immediate last L3 cell level filtered measurement results regardless of actual or predicted with independents filter coefficients.

  - Frequency domain:

- Inference report includes actual and/or predicted measurements of multiple frequencies in one measurement report.

  - Selective inference report based on configured priority and/or selection rule e.g. Top K, Threshold, Maximum number

  - Report quantity of L3 filtered RSRP and RSRQ,  and measurement accuracy/confidence level.

References

- Moderator (CATT), RP-251864, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR,” Jun. 2025.

- RAN2 Chair (InterDigital), “RAN2 #129b Chair Notes”, Apr., 2025.

- Apple, R2-255440,” UE-sided measurement prediction”, Aug., 2025.

- RAN2 Chair (InterDigital), “RAN2 #130 Chair Notes”, May, 2025.

- RAN2 Chair (InterDigital), “RAN2 #131 Chair Notes”, Aug., 2025.