3GPP TSG-RAN WG2 Meeting #132                                                                    R2-2508119

Dallas, USA, Nov. 17th - 21st, 2025

Source:	CATT, CBN

Title:	Discussion on remaining issues for ambient IoT

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

Based on the progress of RAN2#131-bis [1] and the guidance of open issue list [2], this contribution further discusses the remaining open issues suggested by the rapporteur. Section 2 gives our detailed analysis on following aspects:

- Reply to CT1 LS

- New case for no upper layer data available

- Apply the no data available case to more general delayed NAS cases

Section 3 summarizes the contribution of this paper.

2. Discussion

## 2.1 Reply to CT1 LS (Issue 6-1)

CT1 sends LS C1-256624 [3] to RAN2 to ask RAN2 whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens, as shown in highlight part.

1. Overall Description: CT1 notices that in R2-2504935 (LS on parallel service request), RAN2 has agreed on the following on support of parallel service requests: Agreements on parallel service request. - Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification. - The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.. Based on above RAN2 agreement, CT1 has discussed the following two scenarios: a) Collision between inventory procedure and inventory procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the inventory procedure b) Collision between inventory procedure and command procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the command procedure CT1 concluded that there can be an attack scenario of false AIoT pagings that would abort any ongoing valid paging or command, and thereby temporarily or continuously block valid inventory procedure and command procedure execution by the AIoT devices. Hence, CT1 kindly ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

As observed from the CT1 LS, the intention is to avoid the attack scenario of false A-IoT paging that would abort any ongoing valid inventory or command procedure. However, how to handle this kind of attack is under SA3 scope. From RAN2 perspective, to simplify the device complexity, we had agreed that the parallel services are not supported at the device side, and the device always responds to the latest transaction ID upon receiving a paging msg with a new transaction ID. Therefore, it is suggested to reply CT1 with the following information.

Proposal 1 (Issue 6-1): Regarding possibility for the AIoT device to respond to the new A-IoT paging after completion of an ongoing inventory or command when collision happens, RAN2 to reply CT1 with the following information,

- From RAN2 perspective, to simplify the device complexity, the parallel services are not supported at the device side. The device always responds to the latest transaction ID upon receiving a paging msg with a new transaction ID.

## 2.2 New case for no upper layer data available (Issue 3-7)

In last meeting, RAN2 achieved the following agreements about new case with no NAS response:

- RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all. - For cases other than integrity failure, AS will indicate no NAS response expected to reader. FFS how (e.g. using 0 SDU & MDI, or new indication). - For integrity failure, for now RAN2 assumes that there is no AS response to the reader. Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

In the post email discussion [2], there is the following open issue related to new case with no NAS response:

- Issue 3-7: Checking whether there is new case for “no upper layer data available” other than long writing operation which may impact to how to set MDI field.

In the following, we show our view on this topic based on the following Table.

| Use case | Status in CT1 | Remarks |
| --- | --- | --- |
| 6.2.1 Message too short | When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3], and the AIoT device shall provide an indication to the lower layers that the message was ignored. | In this case, since the device cannot recognize the D2R command, it will not generate the D2R response PDU. So it should be differentiated from the long write case so that the gNB will not re-schedule a new D2R resource for re-transmission. The intended gNB behavior is to send the corresponding cause to the A-IoTF so that the A-IoTF will not send this command again to the device. |
| 6.2.2 Message too long | Editor's note: Maximum AIoT NAS message length is FFS and to be defined by RAN3. | - |
| 6.3 Unknown or unforeseen message type | If the AIoT device receives an AIoT NAS message with a message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall return a STATUS message with cause #97 "message type non-existent or not implemented". | Response with NAS Msg |
| 6.4 Non-semantical mandatory information element errors | ### 6.4.1 Common procedures When on receipt of a message, a) an "imperative message part" error; or b) a "missing mandatory IE" error; is diagnosed or when a message containing a syntactically incorrect mandatory IE is received, the AIoT device shall: - if the message is not one of the messages listed in clause 6.4.2, ignore the message and return a STATUS message with cause #96 "invalid mandatory information"; and the network shall either: … ### 6.4.2 Specific AIoT procedures The following procedures shall apply when the AIoT device encounters an error with a mandatory information element in an AIoT NAS message: a) If the message is a READ COMMAND, then a READ COMMAND REJECT message with cause #96 "invalid mandatory information", shall be returned; and b) if the message is a WRITE COMMAND, then a WRITE COMMAND REJECT message with cause #96 "invalid mandatory information", shall be returned. | Response with NAS Msg |
| 6.5 Unknown and unforeseen IEs in the non-imperative message part | ### 6.5.1 IEIs unknown in the message The AIoT device shall ignore all IEs with unknown IEI in a message. ### 6.5.2 Out of sequence IEs The AIoT device shall ignore all out of sequence IEs in a message. ### 6.5.3 Repeated IEs If an information element with format T, TV or TLV is repeated in a message, the AIoT device shall handle only the contents of the information element appearing first and shall ignore all subsequent repetitions of the information element. | Our understanding is the CT1 clarifies how device to handle these unexpected IEs, but eventually the device will send NAS response based on other IE information, i.e., Response with NAS Msg |
| 6.6 Non-imperative message part errors | ### 6.6.2 Syntactically incorrect optional IEs The AIoT device shall treat all optional IEs that are syntactically incorrect in a message as not present in the message. | Our understanding is the CT1 clarifies how device to handle these unexpected IEs, but eventually the device will send NAS response based on other IE information, i.e., Response with NAS Msg |
| 6.7 Messages with semantically incorrect contents | When a message with semantically incorrect contents is received, the AIoT device shall perform the foreseen reactions of the procedural part of clause 5. If, however no such reactions are specified, the AIoT device shall ignore the message. | At least for the current version of 24.369, all R2D commands have the corresponding foreseen reactions, i.e., Response with NAS Msg |

As analysis above, we observe that for the case of “Message too short” as specified in 6.2.1 of 24.369, it is expected to be aware by the gNB so that the gNB will not re-schedule the D2R resource and forward the cause to the A-IoTF. But from our view, we should be more careful about this, because CT1 may put this case together with “Unknown or unforeseen message type” as 24.369 is not stable enough based on the CT1 progress.

Proposal 2 (Issue 3-7): Use MDI=1 and zero SDU to indicate the “Message too short” case to the gNB, provided that RAN2 share the same understanding that the case of “Message too short” is a different than the case of “Unknown or unforeseen message type” as specified by CT1.

## 2.3 Apply the no data available case to more general delayed NAS cases (Issue 6-2)

Another possible case of the delayed NAS response comes from the post email discussion. In CFA inventory procedure, since the timing between CFA paging message and responding D2R message (is about 200us for some chip duration according to TS38.291) is too small for authentication, the A-IoT device may delay the inventory response carried by A-IoT NAS PDU. We echo this is a valid case that may cause no NAS response. But from our view, the reader can be aware of this case by the procedure diversity among the delay NAS caused by long-write operation or CFA paging handling. In details, if the reader does not receive the expected response after sending the write command, it can detect this is due to long write operation by the device; if the reader does not receive the expected response after sending the CFA paging, it can detect this is due to long paging handing by the device. So we can re-use the MDI=0 and 0 SDU without differentiating them by explict signaling.

Proposal 3 (Issue 6-2): Confirm to apply the no data available case to more general delayed NAS cases, e.g., inventory response message.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

Proposal 1 (Issue 6-1): Regarding possibility for the AIoT device to respond to the new A-IoT paging after completion of an ongoing inventory or command when collision happens, RAN2 to reply CT1 with the following information,

- From RAN2 perspective, to simplify the device complexity, the parallel services are not supported at the device side. The device always responds to the latest transaction ID upon receiving a paging msg with a new transaction ID.

Proposal 2 (Issue 3-7): Use MDI=1 and zero SDU to indicate the “Message too short” case to the gNB, provided that RAN2 share the same understanding that the case of “Message too short” is a different than the case of “Unknown or unforeseen message type” as specified by CT1.

Proposal 3 (Issue 6-2): Confirm to apply the no data available case to more general delayed NAS cases, e.g., inventory response message.

4. References

[1]	RAN2#131-bis Chair’s notes.

[2]	Summary of A-IoT MAC open issues

[3]	C1-256624 LS on handling of inventory and command collision