3GPP TSG-RAN WG2 Meeting #132                                                                R2-2508162

Dallas, USA,  Nov. 17th - 21st 2025

Agenda Item:	10.4

Source: 	OPPO

Title:	Initial considerations on 6G Mobility

Document for:	Discussion and Decision

1	Introduction

In RAN#108 meeting, 6G SID [1] has been approved and the mobility related objectives are listed as following:

The detailed objectives of the study are: (4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] (7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: a) 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] b) Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. c)Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

In this contribution, we would like to present our initial considerations on 6G mobility enhancement.

2	Discussion

2.1	 Intra-RAT: Procedures (RRC_CONNECTED)

In 5G, the mobility procedure is categorized into two types of mobility: cell level mobility and beam level mobility.

- Cell Level Mobility: which is mainly L2/L3 oriented, i.e., cell switch which triggered by RRC signaling, MAC CE or executed by UE autonomously (relying on pre-configuration from network).

- Beam Level Mobility: which does not require explicit RRC signaling to be triggered but relies on L1 signaling instead. Beam level mobility can be within a cell, or between cells, the latter is referred to as inter-cell beam management (ICBM).

Figure 1 Mobility procedure

- 5G mobility in RRC_CONNECTED state is categorized into two types of mobility: L2/L3 oriented via cell level mobility and L1-oriented via beam level mobility.

In the evolution of 5G communication system, cell level mobility mechanisms have been continuously improved, with various innovative mechanisms introduced from Rel-15 to Rel-19. These include traditional Handover (HO), Dual Active Protocol Stack (DAPS), Conditional Handover (CHO), L1/L2 Triggered Mobility (LTM), and Conditional Triggered LTM (CLTM), etc. And a serious of PSCell change/addition procedures have been also investigated for multi-connectivity scenario, e.g., PSCell Change, Conditional PSCell Change (CPC), Conditional PSCell addition (CPA), Subsequent Conditional PSCell Addition or Change (subsequent CPAC) etc.

The design and implementation of these handover mechanisms across different protocol versions have led to a certain degree of repetition and redundancy in configurations and processes, while increasing the complexity of system implementation.

Table 1 Mobility Procedure Designs in 5G

| 5G Status | Feature | Details |
| --- | --- | --- |
| Rel-15 | L3 Handover | Handover procedure triggered by RRC signaling. |
|  | Network triggered PSCell Change | PSCell change procedure triggered by RRC signaling. |
| Rel-16 | DAPS | A handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB. |
|  | CHO | A handover that is executed by the UE when one or more handover execution conditions are met. |
|  | CPC | A PSCell change procedure that is executed only when PSCell change execution condition is met. |
| Rel-17 | CPAC | PSCell addition or change procedure that is executed only when PSCell addition or change execution condition is met. |
|  | CHO with SN | For configuration and execution of CHO with SN which includes the cases where the SN is kept, changed or added. |
| Rel-18 | Intra-gNB LTM | gNB receives L1 or L3 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by a cell switch command signaled via a MAC CE. |
|  | SCPAC | A conditional PSCell addition or change procedure that is executed based on pre-configured subsequent CPAC configuration which is not implicitly released by the UE upon (conditional or non-conditional) PSCell addition, (conditional or non-conditional) PSCell change, (conditional or non-conditional) PCell change or SCG release. |
|  | CHO with candidate SCG | A PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met. |
|  | RACH-less HO | Handover procedure allowing UE to transmit first UL message using configured or dynamic signaled UL grant with available timing advance instead of triggering RACH procedure. |
| Rel-19 | Inter-gNB LTM | Support inter-gNB mobility by using LTM mechanism. |
|  | CLTM | LTM cell switch which executed by the UE when L1-based or L3-based LTM cell switch execution conditions are met. |
| Rel-20 | LTM with SCell | Support NW triggered LTM SCell activation with LTM cell switch. |

- In each release of NR, new mobility features were introduced to improve mobility performance and extend the applicable scenarios.

Since the features are introduced in difference releases of NR:

- Separate IEs are used for configuring the candidate/target cells. For example, the conditionalReconfiguration is used for providing the candidate configuration as well as execution conditions for CHO/CPAC/SCPAC, while ltm-Config is used for LTM candidate configuration, among others.

- Separate procedure texts are specified for each handover procedures, including L3 HO, CHO and LTM.

For each row in the table, the yellow-highlighted parts are defined for similar purposes or share significant commonalities according to 5G specifications.

Table 2 Redundancy / Duplication between 5G Mobility Schemes

|  | L3 HO DAPS NW-triggered PSCell addition or change | CHO CHO with SN CHO with candidate SCG CPA, CPC | SCPAC | (C-)LTM |
| --- | --- | --- | --- | --- |
| Target/Candidate Cell Configuration | RRCReconfigurationWith Sync | ConditionalReconfiguration -Candidate cell ID -Candidate cell configuration | ConditionalReconfiguration -Candidate cell ID -Candidate cell configuration -Reference configuration -Complete config indication -sk counter configuration for inter-SN SCPAC -Indication for inter/intra-SN determination for SCPAC | LTM-Config/LTM-ConfigNRDC -Candidate cell ID -Candidate cell configuration -Reference configuration -Complete config indication -sk counter configuration for inter-CU SCG LTM -Indication for inter/intra-CU/DU determination. |
| L3 Meas/Report Configuration | L3 Meas/Report | L3 Meas/Report | L3 Meas/Report | L3 Meas/Report |
| L1 Meas/Report Configuration | N.A. | N.A. | N.A. | L1 Meas/Report |
| Execution condition configuration | N.A. | -Execution conditions | -Execution conditions -Execution conditions for subsequent CPAC | -Execution conditions for CLTM -Execution conditions for subsequent CLTM |
| Other Configuration | N.A. | N.A. | N.A. | -TCI state info -Early UL sync config -Indication for UE-based TA measurement -CSI report config for early CSI acquisition |
| Evaluation | N.A. | Conditional reconfiguration evaluation based on L3 measurement | Conditional reconfiguration evaluation based on L3 measurement | LTM cell switch evaluation based on L1/L3 measurement for CLTM |
| Execution | >Apply RRCReconfiguration | > Apply RRCReconfiguration | SCPAC Execution: > Inter/intra-SN determination > L2 reset handling > Complete config generation > Apply RRCReconfiguration | LTM/CLTM Execution: > Inter/intra-CU/DU determination > L2 reset handling > Complete config generation > Apply RRCReconfiguration |

- Independent procedures are specified for each mobility feature, which bring redundant signalling and procedure text overheads.

To address the above issue, 6G may consider designing a unified mobility management mechanism that integrates the current handover procedures, e.g., (C)LTM and L3 (C)HO, thereby forming a unified mobility management framework for RRC_CONNECTED. This unified mobility management mechanism aims to simplify handover processes, reduce signaling overhead, and further improve handover efficiency through universal configurations and operational steps.

With various mobility features, it is important to identify the most appropriate one for each corresponding scenario. For instance,

- LTM procedure enables the UE to perform an early uplink/downlink synchronization procedure with candidate cell(s), thereby reducing cell switch interruptions. The TCI (Transmission Configuration Indicator) states of the LTM candidate cells can be pre-activated before any of them becomes the serving cell, which facilitates a faster handover to one of these cells when the cell switch is triggered. This makes LTM more suitable for handling typical small coverage scenarios.

- However, compared to traditional L3 handover, the LTM procedure requires the network to prepare more candidate cells in order to facilitate early candidate configuration and subsequent cell switch execution, which results in higher resource reservation. Therefore, L3 HO can effectively mitigate this consumption, particularly in low-frequency and low-mobility scenarios.

I.e., it seems both LTM-based and L3-based HO has its advantage and thus applicable scenario.

- LTM procedure is more proper to handle typically small coverage scenario at higher frequency scenario, while L3 procedure has the advantage of lower resource reservation and more appropriate for lower frequency scenario.

And from the cell switch trigger perspective, both NW-triggered mobility and UE-initiated mobility are supported in 5G.

- Network-triggered handover, leverages the network’s global view of resource utilization, cell loads, and interference levels. With centralized decision-making, NW-triggered mobility guarantees an optimized overall network performance, such as balanced traffic distribution across cells, minimized inter-cell interference, and guaranteed QoS for critical services. Which is particularly useful in scenarios where network-wide coordination is prioritized, such as load balancing in dense urban networks, managing handovers between macro and small cells to offload traffic, or ensuring stable connectivity for stationary or low-mobility users where predictable network behavior is critical.

- In contrast, UE-based mobility empowers the UE to trigger handovers autonomously based on predefined conditions (e.g., signal strength thresholds). Its primary advantages lie in faster response to dynamic channel changes and reduced reliance on network signaling, which lowers handover latency and failure rates. This makes it ideal for high-mobility scenarios, such as UEs on high-speed trains or vehicles, where rapid signal fluctuations demand immediate action. It also excels in edge coverage areas, where UEs frequently transition between cell boundaries, or in networks with high signaling delays, ensuring seamless connectivity by leveraging the UE’s real-time perception of its radio environment.

- NW-triggered mobility and UE-initiated mobility are applicable for different purposes and scenarios.

Furthermore, 5G has studied various mobility features, which can be inherited to 6G unified mobility design.

- RACH-less, which allows UE to skip RACH procedure during cell switch execution to further reduce HO interruption time.

- Early UL/DL sync, which allows UE to perform early UL sync or early TCI state activation before cell switch to reduce the HO interruption time.

- Early CSI measurement/reporting, which allows UE to perform CSI measurement early and report it in very first UL message to target for fast data rate ramp-up.

- Subsequent cell switch, which allows UE to perform subsequent cell switch without new configuration from NW side.

- Early RRC processing, which allow UE to perform early ASN.1 decoding and validity/compliance check for the

RRC configuration of the LTM target cell indicated in the LTM cell switch command.

The purpose is similar, which is aiming to reduce the interruption time of both CP an UP during cell switch execution. All the mobility features above are beneficial for mobility performance and can be inherited to 6G unified mobility design.

- The mobility features studied in 5G (e.g., RACH-less, early UL/DL sync, early CSI, subsequent cell switch, early RRC processing) can be inherited to 6G unified mobility design.

In general, the unified mobility framework should at least support the following aspect:

- L1 measurement based mobility and L3 measurement based mobility

- NW-triggered mobility and UE-initiated mobility

- Immediate and subsequent cell switch

- RACH-less, early UL/DL sync, early CSI, early RRC processing

- For 6G Intra-RAT Mobility of RRC_CONNECTED, RAN2 study to support unified mobility procedure by harmonizing the signalling and configuration, taking 5G mobility features as starting point.

- The 6G unified mobility supports 1) both L1 measurement based mobility and L3 measurement based mobility; 2) both NW-triggered mobility and UE-initiated mobility; 3) Immediate and subsequent cell switch; 4) RACH-less, early UL/DL sync, early CSI and early RRC processing.

Energy efficiency and energy saving is one of the key performances as determined by the RAN requirements in [2]. As agreed in last RAN2 meeting, designing 6G mobility management while taking into account the reduction of power and resource consumption is also one of the key requirements.

RAN2#131bis Agreements

1	    Study mobility with the following requirements in 6G mobility design:

-	     Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	     Robustness of mobility procedures

-	     Energy efficiency for both UE and NW

2	    Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

According to 5G RRC state design, UE may either stay in RRC_CONNECTED or enter RRC_INACTIVE and RRC_IDLE to save power consumption. Compared to RRC_INACTIVE/RRC_IDLE, the drawback of RRC_CONNECTED is the power / resource consumption level when there is no data. If there is no mobility, the long DRX cycle and RRM relaxation can help to reduce the UE power / resource consumption to a minimum level. But when there is mobility, the measurement, report and the associated signaling is unavoidable, for 5G RRC_CONNECTED state.

Take LTM procedure as an example:

- Step-1/2/3/4b/5/6/7 includes signaling exchange as consumer for resource, and

- Measurement for step-1/5, sync for step-1/4a, are all consumer for UE power,

- Reception for step-4b/7 is consumer for network power

- When there is no data is on-going, there could be a concern on keeping the UE in 5G RRC_CONNECTED state, considering the power/resource consumption due to mobility in 5G RRC_CONNECTED state.

Figure 2 5G LTM Procedure

As alternative, releasing UE to RRC_INACTIVE or RRC_IDLE can have some gain on power consumption, while the state transition effort as well as mobility support may cause extra latency which against to the requirement for 6G mobility management design.

The following table re-screens the legacy 5G state transition as well as mobility support in each 5G RRC state upon data arrivals to understand the pros and cons.

Table 3 5G RRC State Handling Schemes

| Index | When there is no data ongoing | Upon data arrival | Consumption when no data ongoing | Latency upon data arrival | Mobility support when there is data on-going | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 5G RRC_IDLE | 5G RRC_ CONNECTED | Low | Long | Y | Typical way |
| 2 | 5G RRC_ INACTIVE | 5G RRC_ CONNECTED | Low | Medium (due to UE context retrieve) | Y | Limited Deployment |
| 3 | 5G RRC_ CONNECTED | 5G RRC_ CONNECTED | Medium if tuning DRX properly (due to measurement and signaling for mobility) | Low | Y |  |
| 4 | 5G RRC_ INACTIVE | 5G RRC_ INACTIVE | Low | Medium (due to UE context retrieve) | N | Limited to small data packet |

Firstly, for Approach-3, the drawback is the power / resource consumption level when there is no data. On the contrary, approach-1/2 solves the concern of power consumption, but ends up with medium/high level of latency upon data arrival.

Specifically, in approach-1, the steps from 1 to 8a are all necessary, covering jobs for NAS to prepare UE AS context, NAS and AS security activation, and SRB/DRB configuration, which is quite long latency.

While for approach-2, although the NAS/AS UE context preparation steps can be saved, but when the serving RAN node is not the anchor RAN node, the reactive UE context retrieve procedure introduce additional latency, which happens between step-1 and step-4.

- There could be a concern to release the UE to 5G RRC_IDLE or 5G RRC_INACTIVE when there is no data activity, considering the latency to resume UP activity when there is DL/UL data arrival.

Figure 3 5G RRC Setup Procedure

Figure 4 5G RRC Resume Procedure

Finally, for approach-4, performance-wise it is quite similar to approach-2, in the sense that UE context retrieval is still needed, although RRCResume message is saved. While the key problem here is the incapability to support mobility during data traffic period.

- 5G small-data-transmission (i.e., SDT) cannot be used as a main approach to handle eMBB traffic, due to incapability to support mobility during data traffic period.

Figure 5 5G SDT Procedure

Given the analysis above, the 6G mobility should aim at improvement on top of the 5G design, where approach-2 and approach-3 seem to be a good starting point.

Table 4 Comparison between Approach-2/3

| Index | When there is no data ongoing | Upon data arrival | Consumption when no data ongoing | Latency upon data arrival | Potential Improvement |
| --- | --- | --- | --- | --- | --- |
| 2 | 5G RRC_ INACTIVE | 5G RRC_ CONNECTED | Low | Medium (due to UE context retrieve) | Change reactive UE context retrieve to proactive UE context retrieve |
| 3 | 5G RRC_ CONNECTED | 5G RRC_ CONNECTED | Medium if tuning DRX properly (due to measurement and signaling for mobility) | Low | Measurement and signalling overhead reduction |

Comparing the two starting points:

If one starts from approach-2, to optimize the latency upon data arrival, the key should be to solve the latency due to RACH procedure and the re-active UE context retrieval. Which can be in various forms to include:

- Enhancement to reduce the RACH latency, e.g., to extend the applicability of RACH-less functionality.

- Enable proactive context relocation, as we did in RRC_CONNECTED. But in order to keep the advantage of low consumption when no data on-going, this proactive context relocation has to be pre-configured / conditionally configured.

- Early CSI acquisition/report, for quick bit-rate ramp-up upon UP resumption.

Where component (1) has less impact on the whole framework, while the component (2)(3) brings large change but also large benefit (with the cost of more additional resource reservation).

Or if one starts from approach-3, to optimize the consumption when there is no data, the key should be to solve the measurement overhead for candidate cell monitoring, the reporting overhead for candidate cell measurement reporting, and unnecessary signaling exchange when there is no data. Which can be in various forms to include:

- Inherit the benefit of (C)LTM, i.e. RACH-less cell switch, and pre-configured candidate configuration.

- Enhance saving unnecessary L1/L3 measurement when there is no-data. For LTM, UE performs L1 measurement for each candidate cell upon reception of the LTM configuration, which aims to track the candidate cell and providing assistance info to NW for cell switch decision. While the UE measurement seems useless from NW perspective for no data period and would cause measurement overheads for UE. Further enhancement to save the measurement overheads can be studied. e.g., remove/relax the L1/L3 measurements on the configured LTM candidate cell(s).

- Enhance saving unnecessary signaling exchange when there is no data. In RRC_CONNECTED, to achieved per cell paging, UE is required to perform complete cell switch execution (HO command, HO confirm) to inform NW of UE’s arrival. For no data period, there is a trade-off between the granularity of UE location tracking area and the overhead of the HO confirmation message. To be energy efficient, simplified UE location tracking and HO confirm can be studied to optimize the unnecessary signaling exchange overhead and enable fast resumption upon data arrival. And the measurement reporting overhead can also be reduced/removed considering the signaling overheads.

For further energy efficiency improvement, approach-3 can be the starting point to improve the energy efficiency of (C)LTM.

- For 6G mobility of RRC_CONNECTED, RAN2 study to further reduce power/resource consumption to optimize when there is no data traffic on-going:

1) Inherit the benefit of (C)LTM, i.e., RACH-less, and pre-configured candidate configuration

2) Saving L1/L3 measurement/reporting when there is no-data, e.g., remove/relax measurement

3) Saving signalling exchange (HO confirm) when there is no data, e.g., simplify/avoid cell-specific UE location tracking.

5G failure detection and recovery mechanism is implemented with two levels:

1）Beam failure detection/recovery (BFD/BFR): For beam failure detection, the gNB configures the UE with beam failure detection reference signals and the UE declares beam failure when the number of beam failure instance indications from the physical layer reaches a configured threshold before a configured timer expires. And the BFD/BFR is performed with the granularity of TRP. After beam failure is detected on an SCell or on a TRP, the UE triggers beam failure recovery by initiating a transmission of a BFR MAC CE for this SCell/TRP. Once BFR is triggered on both TRPs associated with PCell, UE triggers RACH procedure for recovery.

2）Radio link monitoring (RLM): In RRC_CONNECTED, the UE performs radio link monitoring in the active BWP based on reference signals and signal quality thresholds configured by the network. The RLM is only performed in PCell. Upon radio link failure (RLF) declaration, RRC connection re-establishment procedure is executed, where the UE first performs cell reselection and then initiates RRC re-establishment to the selected cell.

- 5G failure detection and recovery mechanism is implemented with two levels: beam level and cell level.

Table 5 5G Failure detection/recovery

|  | PCell |  | SCell |  |
| --- | --- | --- | --- | --- |
|  | Single TRP | Multiple TRPs | Single TRP | Multiple TRPs |
| BFD | - Per cell/TRP detection, Per cell/TRP timer and counter - serving SSB(s)/CSI-RS(s) based (RadioLinkMonitoringRS/TCI state) - Beam failure instance from lower layer (Qout_LR (BLERout < 10% )) |  |  |  |
| BFR | RACH | - BFR MAC CE if BFR triggered for one TRP - RACH if BFR triggered for both TRPs | BFR MAC CE | BFR MAC CE |
| RLM | - Performed for PCell. - Serving SSB(s)/CSI-RS(s) based (RadioLinkMonitoringRS/TCI state) - Consecutive ‘out-of-syn’ indication from lower layer (thresholds Qout (10%)and Qin(2%) ) |  | N.A. |  |
| RLF | RRC re-establishment |  | N.A. |  |

As we can see, with carrier aggregation configured,

- When PCell encounters RLF, UE will initiate RRC re-establishment procedure regardless of the radio condition of SCell, which may cause additional service interruption.

- When PCell encounters beam failure, RACH procedure will be initiated even if SCell is alive, which also causes unnecessary RACH attempts.

- 5G failure detection and recovery mechanism mainly consider the radio condition of primary cell/carrier.

Besides, based on the requirement from 6G SID, the 6G design needs to target diverse device types with diverse bandwidth capability and take into account diverse spectrum allocations:

2. Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] 9. Target scalable and forward compatible design for diverse device types. 10. Improved spectrum utilization and operations taking into account diverse spectrum allocations.

- Improved spectrum utilization will be studied in 6G.

Based on observations above, the 6G failure detection/recovery mechanism can be further enhanced to improve robustness, with considering the following:

- Unified beam level and cell level failure detection/recovery

- Fast failure recovery by alleviating the priority of TRP/carrier

- Taken into account the 6G new spectrum utilization feature

- For 6G mobility of RRC_CONNECTED, RAN2 study to enhance the failure detection/recovery mechanism to improve robustness.

2.2 	Intra-RAT: Procedures (RRC_IDLE, RRC_INACTIVE)

Common signals can be categorized into two groups:

- Dynamic signals, which are influenced by UE density, including Paging and RACH

- Static signals, which are independent of UE density, including SSB and SIBs

The load associated with dynamic signals can vary significantly depending on factors such as UE activity and time of day.

In 5G, to manage dynamic load changes, networks typically deploy additional carriers as different cell(s), each accompanied by cell-specific overhead (e.g., SSB, SIB). However, this load distribution is performed statically, relying on fixed/static cell (re)selection parameters.

- From the network perspective, this requires deploying new cells;

- From the UE perspective, it involves executing cell reselection procedures based on updated parameters.

- This approach is resource-consuming and poorly suited to dynamic load scenarios.

- In 5G RRC_IDLE/RRC_INACTIVE, cell selection/reselection is used for UE to switch between frequencies (when modelled as different cells) for cell camping.

Besides, based on the requirement from 6G SID, the 6G design needs to target diverse device types with diverse bandwidth capability and take into account diverse spectrum allocations:

2. Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] 9. Target scalable and forward compatible design for diverse device types. 10. Improved spectrum utilization and operations taking into account diverse spectrum allocations.

To meet these demands, a unified yet scalable initial access framework is essential, which ensures compatibility across diverse UE types having different bandwidth capabilities. Additionally, to support the anticipated increase in device volume, enhanced initial access capacity is needed. It requires flexible load distribution across multiple frequency layers (e.g., coverage and capacity layers), which expands available resources beyond a single frequency layer and enables more efficient handling of dynamic traffic loads.

- In 6G, the legacy spectrum aggregation scheme can be further enhanced to allow more flexible (than cell (re)selection) load distribution.

- For 6G mobility of RRC_IDLE (FFS for RRC_INACTIVE), instead of relying on cell (re)selection like procedure, RAN2 study mechanism for UE to flexibly switching between frequencies, in order for dynamic load (e.g., RACH and Paging) re-distribution.  Other than this scenario, 5G cell selection/reselection is considered as baseline.

2.3 	Inter-RAT: Scenarios

According to 6G SID [1], one of the objectives is to study migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR.

(1) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: a) 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] b) Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. c) Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Correspondingly, RAN2 should focus on the study of inter-RAT mobility between 6G and 5G, while deferring the discussion on inter-RAT mobility between 6G and 4G to a later phase.

- According to 6G SID, mobility between 5G and 6G should be studied and inclusion of 4G/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary.

In 5G, the following scenarios are supported for inter-RAT mobility between 5G and 4G as specified in TS38.300 and TS 36.300.

- Cell reselection between NR RRC_IDLE and E-UTRA RRC_IDLE.

- Cell reselection from NR RRC_INACTIVE to E-UTRA RRC_IDLE and from E-UTRA RRC_INACTIVE to NR RRC_IDLE.

- Handover between NR RRC_CONNECTED and E-UTRA RRC_CONNECTED.

- In 5G, the following scenarios are supported for inter-RAT mobility between NR and E-UTRA: 1) Cell reselection between NR RRC_IDLE and E-UTRA RRC_IDLE, 2) Cell reselection from NR RRC_INACTIVE/E-UTRA RRC_INACTIVE to E-UTRA RRC_IDLE/NR RRC_IDLE, and 3) Handover between NR and E-UTRA.

The RRC_INACTIVE state was introduced in 5G primarily to reduce control plane latency; however, its commercial deployment has been limited. As outlined in our contribution [3], it is proposed that RAN2 should consider whether the RRC_INACTIVE state is retained in 6G. Consequently, inter-RAT mobility in RRC_CONNECTED state and RRC_IDLE state should be supported in 6G, while RAN2 further study whether to support inter-RAT mobility in the RRC_INACTIVE state.

- For 6G-related inter-RAT mobility, RAN2 study to support inter-RAT mobility between 6G and 5G for RRC_CONNECTED and for RRC_IDLE. FFS on 6G to 4G mobility. FFS for RRC_INACTIVE.

2.4 	Inter-RAT: Procedures

Procedure-wise, inter-RAT mobility in 5G is performed as follows:

For UEs in 5G RRC_CONNECTED, inter-RAT mobility is performed through L3-based handover. The source RAT configures Inter-RAT measurement and reporting for the target RAT, based on which it determines whether to initiate a handover to the target RAT. The UE then performs the handover according to the RRC Reconfiguration message sent by the source RAT, which is prepared by the target RAT. There are two approaches for control plane and user plane handling during this procedure:

Table 6 Inter-RAT Mobility in 5G

|  | Intra-system inter-RAT mobility (i.e., intra-5GC) | Inter-system inter-RAT mobility (i.e., between 5GC and EPC) NOTE: if N26 is unavailable, re-direction is needed |
| --- | --- | --- |
| Control Plane handling | Delta configuration for radio bearer configuration is used to keep SDAP and PDCP configuration | Full configuration is required to reset the radio bearers. |
| User plane handling | In-sequence and lossless handover is supported. | Seamless session continuity (e.g., for voice services) can be supported. |

For UEs in 5G RRC_IDLE or 5G RRC_INACTIVE, inter-RAT mobility relies on the cell reselection procedure. The UE evaluates neighboring cells based on criteria such as signal strength and system information and reselects to a target RAT without NW intervention.

- In 5G, inter-RAT mobility in RRC_CONNECTED relies on L3 handover procedure, inter-RAT mobility in RRC_IDLE relies on cell reselection procedure.

- For intra-system inter-RAT mobility in RRC_CONNECTED, in-sequence and lossless handover is supported by keeping the SDAP and PDCP configurations. For inter-system inter-RAT mobility, handover or redirection is performed based on radio criteria and availability of the N26 interface.

LTM is introduced in 5G to further reduce mobility latency. In this procedure, the UE can be configured with a set of candidate cell configurations for cell switch triggered by L1/L2 signaling or upon fulfilling the evaluation conditions. In LTM, L1 measurement is used for faster cell switch reaction and methods such as early UL/DL sync to the candidate cell(s) are supported for latency reduction. Compared to L3 handover, tighter interworking between the source and target cells is required to achieve the benefits of LTM as detailed in the table below, which may put a higher requirement for inter-RAT scenario.

Table 7 Comparison between Intra-RAT L3 HO and LTM in 5G

| Phases | LTM (e.g., inter-gNB LTM) | L3 handover |
| --- | --- | --- |
| Preparation | S-gNB to C-gNB(s): Request for handover preparation | S-gNB to C-gNB(s): Request for handover preparation |
|  | C-gNB(s) to S-gNB: Candidate cell configuration, CSI resource configuration and early sync information. S-gNB to C-gNB(s): Common CSI resource configuration and LTM configuration ID mapping list. C-gNB(s) to S-gNB: Updated candidate cell configuration. | C-gNB to S-gNB: Target cell configuration |
| Early UL sync | C-gNB to S-gNB: TA value, CFRA resource information, candidate cell ID | N.A. |
| Execution | S-gNB to T-gNB: Target cell ID, TCI state ID. | N.A. |
| Completion | T-gNB to S-gNB: Handover success message. S-gNB to T-gNB: SN status transfer. T-gNB to S-gNB: UE context release. | T-gNB to S-gNB: Handover success message. S-gNB to T-gNB: SN status transfer. T-gNB to S-gNB: UE context release. |
|  | T-gNB to C-gNB(s): LTM configuration update message, e.g., including new security key(s) to be used with the UE. |  |

- Compared to L3 handover, LTM-like procedure requires tight interworking between source cell and target cell which may put a higher requirement for inter-RAT scenario.

The 5G inter-RAT mobility procedure can serve as a baseline for 6G. Further discussion is needed on whether to extend the benefits of LTM to inter-RAT scenarios.

- For 6G-related inter-RAT mobility, RAN2 study the 1) For inter-RAT mobility from/to 6G RRC_CONNECTED, support L3-based procedure as baseline. 2) For inter-RAT mobility from/to 6G RRC_IDLE, 5G cell (re)selection procedure is supported as starting point. FFS on RRC_INACTIVE.

3	Conclusion

Based on the discussion in section2, we have following proposals:

Proposal 1	For 6G Intra-RAT Mobility of RRC_CONNECTED, RAN2 study to support unified mobility procedure by harmonizing the signalling and configuration, taking 5G mobility features as starting point.

Proposal 2	The 6G unified mobility supports 1) both L1 measurement based mobility and L3 measurement based mobility; 2) both NW-triggered mobility and UE-initiated mobility; 3) Immediate and subsequent cell switch; 4) RACH-less, early UL/DL sync, early CSI and early RRC processing.

Proposal 3	For 6G mobility of RRC_CONNECTED, RAN2 study to further reduce power/resource consumption to optimize when there is no data traffic on-going:

1) Inherit the benefit of (C)LTM, i.e., RACH-less, and pre-configured candidate configuration

2) Saving L1/L3 measurement/reporting when there is no-data, e.g., remove/relax measurement

3) Saving signalling exchange (HO confirm) when there is no data, e.g., simplify/avoid cell-specific UE location tracking.

Proposal 4	For 6G mobility of RRC_CONNECTED, RAN2 study to enhance the failure detection/recovery mechanism to improve robustness.

Proposal 5	For 6G mobility of RRC_IDLE (FFS for RRC_INACTIVE), instead of relying on cell (re)selection like procedure, RAN2 study mechanism for UE to flexibly switching between frequencies, in order for dynamic load (e.g., RACH and Paging) re-distribution.  Other than this scenario, 5G cell selection/reselection is considered as baseline.

Proposal 6	For 6G-related inter-RAT mobility, RAN2 study to support inter-RAT mobility between 6G and 5G for RRC_CONNECTED and for RRC_IDLE. FFS on 6G to 4G mobility. FFS for RRC_INACTIVE.

Proposal 7	For 6G-related inter-RAT mobility, RAN2 study the 1) For inter-RAT mobility from/to 6G RRC_CONNECTED, support L3-based procedure as baseline. 2) For inter-RAT mobility from/to 6G RRC_IDLE, 5G cell (re)selection procedure is supported as starting point. FFS on RRC_INACTIVE.

4	Reference

[1] RP-251881   New SID: Study on 6G Radio

[2] RP-250810   RAN requirements for 6G

[3] R2-2506846 Discussion on 6GR control plane