3GPP TSG-RAN WG2 Meeting #132	R2-2508206

Dallas, Texas, USA, Nov 17th – 21st, 2025

Agenda item:		8.7.2

Source:		Samsung

Title:			Discussion on open issues for RLC

Document for:	Discussion & Decision

# 1 Introduction

In RAN2#131bis, following RLC agreements were made, including a few open issues as highlighted:

Agreements for RLC - (RLC-X01) We keep the spec as it is. - (RLC-V01) No need to indicate the SN of obsolete RLC SDUs to the receiving PDCP entity after t-RxDiscard expires. - (RLC-N01) We will clarify RX_Next state variable update when t-RxDiscard expires. FFS an exact wording and where to do the change (e.g. procedure or variable definition) - (RLC-N01) Try to capture it in the procedure “If t-RxDiscard is configured and expires, the receiving side of an AM RLC entity consider the corresponding SDUs as received.”, if problem is identified or companies have strong concern during CR phase, it will be postponed. - (RLC-N01) Add “or t-RxDiscard expires” at the end of definition of RX_Next. - (RLC-N02) In NOTE in 5.3.3.3 we add “If stopReTxDiscardedSDU is configured” at the beginning of the sentence. - (RLC-N02) FFS whether any other changes are needed, e.g. as in R2-2507159 - (RLC-E01) No change is needed. - (RLC-S01) No change needed as per the previous agreement - (RLC-N02) Companies will bring contribution in next meeting to explain the use cases and the TP. - (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it. - RLC-V02 is closed without any spec impact. - RLC-H01 is updated according to the CR in R2-2507016.

In this paper, we discuss (RLC-H02) and (RLC-N01), while other open issue for (RLC-N02) is addressed in our co-sourced contribution (R2-2508571).

# 2 Discussion

## 2.1 (RLC-H02)

Proponent company argues that for some scenario, when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. The scenario makes some assumption on the values configured for t-Reassembly and t-RxDiscard and the order in which they expire. Further, proponent proposes to specify the content of the RLC STATUS PDU triggered by the t-RxDiscard expiry in a separate condition with ACK_SN=RX_Next.

The scenario provided by proponent is described as below:

- SN=4 and 5 are received. t-RxDiscard and t-Reassembly started; RX_Next_Highest, RX_Next_Discard_Trigger and RX_Next_Status_Trigger updated to 6

- SN =7 and 8 are received. RX_Next_Highest updated to 9

- t-Reassembly expires. RX_Highet_Status updated to RX_Next_Status_Trigger (6). RX_Next_Status_Trigger updated to RX_Next_Highest (9). t-Reassembly started

- 10 is received, RX_Next_Highest updated to 11

- t-RxDiscard expires, RX_Next updated to RX_Next_Discard_Trigger (6). RX_Next_Discard_Trigger updated to RX_Next_Highest (11).

- t-Reassembly expires, RX_Highest_Status updated to RX_Next_Status_Trigger (9); RX_Next_Status_Trigger updated to RX_Next_Highest (11). t-Reassembly started

- t-RxDiscard expires. RX_Next updated to RX_Next_Discard_Trigger (11). RX_Highest_Status is updated to 11. Stop and reset T-Reassembly.

However, it seems the scenario description is not correct.

As highlighted in blue in below text-box, any RLC SDU with SN < RX_Next_Discard_Trigger(11) is considered as completely received. This implies RX_Highest_Status should also be considered as updated (11), as it is unreasonable that status can not be reported for “completely received” RLC SDU(s).

##### 5.2.3.2.5 Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - consider the RLC SDU(s) with SN < RX_Next_Discard_Trigger, if any, as completely received; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest.

As highlighted in yellow in below text-box, t-Reassembly is stopped and reset based on the condition (RX_Next_Status_Trigger (11) = RX_Next (11)). The same is reflected to the scenario in red color to make it correct.

- if t-Reassembly is running: - if RX_Next_Status_Trigger = RX_Next; or - if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or - if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size: - stop and reset t-Reassembly.

Observation 1: Contrary to the proponent company’s claim, in the provided scenario, RX_Next is not larger than RX_Highest_Status.

Now, in step 7, given RX_Next (11) and RX_Highest_Status (11), the STATUS PDU shoud be constructed with only including ACK_SN as per the legacy.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing and is not discarded due to the expiry of t-RxDiscard in the resulting STATUS PDU.

We do not see any need to depart from the legacy approach of contructing STATUS PDU and therefore, the changes proposed in (RLC-H02) should not be pursued.

(RLC-H02) Proposal 1: (RLC-H02) is not adopted. Legacy approach of constructing STATUS PDU is only pursued.

## 2.2 (RLC-N01)

##### 5.2.3.2.5 Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - consider the RLC SDU(s) with SN < RX_Next_Discard_Trigger, if any, as completely received; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest.

##### 5.2.3.2.3 Actions when an AMD PDU is placed in the reception buffer When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall: - if x >= RX_Next_Highest: - update RX_Next_Highest to x+ 1. - if all bytes of the RLC SDU with SN = x are received: - reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer; - if x = RX_Highest_Status: - update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received. - if x = RX_Next: - update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

It should be noted that above highlighted text (“consider the RLC SDU(s) with SN < RX_Next_Discard_Trigger, if any, as completely received”) upon t-RxDiscard has impact not only on RX_Next but also on RX_Highest_Status and accordingly, on t-Reassembly operations. That is, RX_Highest_Status would have been updated considering the discard AMD PDU as completely received. As we observed for issue in (RLC-H02) as well, there can be potential misunderstanding and wrong implementation; it is preferable to explicitly specify this behaviour.

Observation 2: There can be potential misunderstanding and wrong implementation with generic text (i.e., “consider the RLC SDU(s) with SN < RX_Next_Discard_Trigger, if any, as completely received”). The impact of t-RXDiscard expiry on RX_Highest_Status should be explicitly specified.

(RLC-N01) Proposal 2: Specifiy the below action when t-RxDiscard expires (adopt TP1):

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received;

TP1

##### 5.2.3.2.5 Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - consider the RLC SDU(s) with SN < RX_Next_Discard_Trigger, if any, as completely received; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following observations and proposals:

Observation 1: Contrary to the proponent company’s claim, in the provided scenario, RX_Next is not larger than RX_Highest_Status.

(RLC-H02) Proposal 1: (RLC-H02) is not adopted. Legacy approach of constructing STATUS PDU is only pursued.

Observation 2: There can be potential misunderstanding and wrong implementation with generic text (i.e., “consider the RLC SDU(s) with SN < RX_Next_Discard_Trigger, if any, as completely received”). The impact of t-RXDiscard expiry on RX_Highest_Status should be explicitly specified.

(RLC-N01) Proposal 2: Specifiy the below action when t-RxDiscard expires (adopt TP1):

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received;

# 4 Text Proposal

TP1

##### 5.2.3.2.5 Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - consider the RLC SDU(s) with SN < RX_Next_Discard_Trigger, if any, as completely received; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest.