3GPP TSG-RAN WG2 Meeting #132												R2-2508214

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.3.1.2

Source: 	Sharp

Title:  	Discussion on QoS, QoE and Service-awareness

Document for: 	Discussion

# Introduction

In RAN2#131bis meeting, RAN2 made the following agreements on 6GR User Plane:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

This document discusses aspects of QoS, QoE, and service-awareness for 6GR.

# Discussion

## HRLLC and Immersive Communications

The 6GR study should be guided by the services of the future. While enhanced Mobile Broadband (eMBB) remains a major requirement, the next generation will be defined by applications needing extreme performance: Hyper-Reliable Low-Latency Communication (HRLLC) and Immersive Communication (e.g., XR). Therefore, the RAN2 study must explicitly target all three (eMBB, HRLLC, and Immersive Communication). HRLLC and Immersive Communication services are transitioning from specialized niches to common and widespread services in the 6G era. HRLLC is essential for industrial automation, intelligent V2X systems, and remote operations, and near-perfect reliability. Similarly, Immersive Communication (VR/AR) requires high throughput combined with extremely low latency to deliver seamless and comfortable immersive experiences.

If 6GR is to succeed in supporting the fully automated and immersive society of the future, its architecture cannot treat HRLLC and Immersive Communication as afterthoughts, which often led to architectural compromises. The initial version of 6GR should fulfill the stringent requirements of HRLLC and Immersive Communication through deep integration of mechanisms for high reliability and Hyper-low latency scheduling.

Observation 1: While eMBB remains a major requirement, the next generation will be defined by applications needing extreme performance.

Proposal 1: RAN2 study aims to support eMBB, HRLLC and Immersive Communications as target services of 6G.

## AIML-related Data Transfer over 6GR UP

In 6GR, AI/ML is a key foundational feature that will drive network optimization and enable innovative services. For effective AI/ML lifecycle management (LCM), both data collection and data transfer are critical UP functionalities. Data collection allows the network to gather diverse, large-scale datasets from distributed sources, including sensing and measurement results and user equipment feedback, which are essential for training, testing, and performance monitoring of the AI/ML models. Simultaneously, efficient and reliable data transfer can fulfill the requirements not only for the collected data delivery, but also for the transmission of AI/ML model, inference result, latent information, and AI token among network entities and devices. Furthermore, AI/ML-related data transfer may require strict QoS conditions to ensure low latency, high reliability, and high throughput. Therefore, both data collection and data transfer must be considered in the 6GR protocol stack design.

Observation 2: AI/ML LCM is a key foundational feature in 6GR, which require specific data collection and data transfer UP functionalities to fulfil the requirements with appropriate QoS support.

Proposal 2: RAN2 to study mechanisms to fulfil the requirement of AI/ML-related data collection and data transfer with appropriate QoS support.

## Multi-Modal Services in 6G

Immersive Communication (e.g. immersive multi-modal VR applications) is one of the key usage scenarios in 6G. The characteristic of Immersive Communication traffic should be considered in 6G UP design. For example, for immersive multi-modal VR applications, how to fulfil the critical synchronization requirements between different QoS flows from different media component as showed in table 1 [1] should be considered.

Table 1: Typical synchronization thresholds for immersive multi-modality VR applications (in TS 22.261)

| Media components | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: 50 ms | tactile delay: 25 ms |
| visual-tactile | visual delay: 15 ms | tactile delay: 50 ms |
| NOTE 1: for each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

In 5G, the inter-QoS flow synchronization is fulfilled by setting appropriate QoS parameters via coordination between AF, PCF and SMF. Due to the backward compatibility constraint, some limitation on inter-QoS flow synchronization exists. As the start of 6G does not have any requirement of backward compatibility, 6G UP design need to comprehensively investigate synchronization between QoS flows from different media component into consideration by cooperation with SA2.

Observation 3: In 5G, inter-QoS flow synchronization replies only on CN coordination.

Proposal 3: Finer granularity and synchronization for multi-model service should be considered by cooperation with SA2.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observations and agree the following proposals:

Observation 1: While eMBB remains a major requirement, the next generation will be defined by applications needing extreme performance.

Observation 2: AI/ML LCM is a key foundational feature in 6GR, which require specific data collection and data transfer UP functionalities to fulfil the requirements with appropriate QoS support.

Observation 3: In 5G, inter-QoS flow synchronization replies only on CN coordination.

Proposal 1: RAN2 study aims to support eMBB, HRLLC and Immersive Communications as target services of 6G.

Proposal 2: RAN2 to study mechanisms to fulfil the requirement of AI/ML-related data collection and data transfer with appropriate QoS support.

Proposal 3: Finer granularity and synchronization for multi-model service should be considered by cooperation with SA2.

# References

[1] 3GPP TS 22.261, “Service requirements for the 5G system; Stage 1”.