3GPP TSG-RAN WG2 Meeting #132										    R2-2508216

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.3.3.2

Source: 	Sharp

Title:  	Energy efficiency in 6GR

Document for: 	Discussion

Introduction

In RAN2#131bis meeting [1], energy efficiency in 6GR was discussed and the following design goal and understanding were achieved:

Design goal

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose.   Focus on solutions that have RAN2 impacts.

In this paper, network energy efficiency and UE power saving are discussed.

Discussion

NW energy saving

In LTE, network energy-saving solution was not considered as macro cells in most of cases and micro cells in dense urban environments are major deployment scenarios. In NR, high-frequency usage e.g., FR2 mandates denser cell-site deployments where network energy consumption is not negligible anymore. In 6GR, we see that high-frequency usage and dense cell-site deployments will not be precluded, so the network energy saving will be also very important.

In NR, cell DTX/DRX is introduced for network energy saving and only supported in RRC_CONNECTED. We can consider to redesign and generalize cell DTX/DRX in 6GR so that it is not limited to RRC_CONNECTED UEs.

Proposal 1: RAN2 to study Cell DTX/DRX across all RRC states.

UE power saving

For 6GR, UE power saving should be considered in the initial version. Both RRC_CONNECTED and RRC_IDLE/INACTIVE modes need to be studied. In NR, many solutions are introduced for UE power saving, including I-DRX, RRM measurement relaxation, RLM/BFD relaxation, DCP, LP-WUS, PEI, etc. Some solutions are for special UEs e.g. (e)RedCap UE, UE with LR. Then, which solutions to be included or reused in the initial version of 6GR should be discussed.

RRM measurement relaxation, RLM/BFD relaxation and I-DRX don’t depend on RAN1 and can be discussed first in RAN2. In NR, RRC_IDLE/INACTIVE RRM measurement relaxation is applied for all the UEs, but RRC_CONNECTED RRM measurement relaxation is only applied for stationary (e)RedCap UE. We think stationary status should not be limited to (e)RedCap UE in 6GR, and all the UEs can relax RRM measurements if they are stationary.

Proposal 2: RAN2 to reuse RRM measurement relaxation, RLM/BFD relaxation and I-DRX for all the UEs as baseline in the initial version of 6GR.

In NR, for RRC_IDLE/INACTIVE, both PEI and LP-WUS are used to indicate paging earlier. For RRC_CONNECTED, DCP and LP-WUS have functional overlap. And there are two options for LP-WUS in RRC_CONNECTED for the same purpose, one depends on C-DRX and the other one doesn’t depend on C-DRX. In 6GR, RAN1 has agreed to study and evaluate OFDM-based sequence DL WUS. If the UEs support such DL WUS, there seems no need to support PEI and DCP. RAN2 should study and simplify DL WUS procedures.

Proposal 3: RAN2 to study DL WUS procedures in 6GR to avoid multiple options for the same purpose.

Alignment of NW energy saving and UE power saving

Importance of network energy saving does not mean that increased UE energy consumption is acceptable. The 6GR network energy saving mechanisms shall not sacrifice UE energy consumption. In RAN2#131bis meeting, RAN2 agreed the design goal that RAN2 strives to develop solutions that consider network energy efficiency and UE power saving. We see that a baseline of UE energy saving can be NR C-DRX. Also, for network energy saving, cell DTX/DRX mechanisms should be discussed together. In NR, UE C-DRX and cell DTX/DRX are standardized separately, so it produced some complexities. RAN2 should discuss how to simply 6GR network/UE energy saving mechanism.

Proposal 4: RAN2 to study on simplification of UE C-DRX and cell DTX/DRX to keep comparable UE energy saving performance with NR DRX.

Conclusion

This paper discusses energy efficiency in 6GR. Based on the discussion, the following proposals are provided:

Proposal 1: RAN2 to study Cell DTX/DRX across all RRC states.

Proposal 2: RAN2 to reuse RRM measurement relaxation, RLM/BFD relaxation and I-DRX for all the UEs as baseline in the initial version of 6GR.

Proposal 3: RAN2 to study DL WUS procedures in 6GR to avoid multiple options for the same purpose.

Proposal 4: RAN2 to study on simplification of UE C-DRX and cell DTX/DRX to keep comparable UE energy saving performance with NR DRX.

References

[1] Draft chairman notes on RAN2#131bis meeting