3GPP TSG-RAN WG2 Meeting #132												R2-2508217

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.3.1.1

Source: 	Sharp

Title:  	6GR User Plane Functionality

Document for: 	Discussion

# Introduction

In RAN2#131bis meeting, RAN2 made the following agreements on 6GR User Plane:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

This document discusses essential User Plane (UP) functionalities and initial considerations for 6GR.

# Discussion

## Essential UP Functionalities

| MAC | - Mapping between logical channels and transport channels; - Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels; - Scheduling information reporting; - Error correction through HARQ (one HARQ entity per cell in case of CA); - Priority handling between UEs by means of dynamic scheduling; - Priority handling between logical channels of one UE by means of logical channel prioritisation; - Priority handling between overlapping resources of one UE; - Padding. |
| --- | --- |
| RLC | - Transfer of upper layer PDUs; - Sequence numbering independent of the one in PDCP (UM and AM); - Error Correction through ARQ (AM only); - Segmentation (AM and UM) and re-segmentation (AM only) of RLC SDUs; - Reassembly of SDU (AM and UM); - Duplicate Detection (AM only); - RLC SDU discard (AM and UM); - RLC re-establishment; - Protocol error detection (AM only). |
| PDCP | - Transfer of data (user plane or control plane); - Maintenance of PDCP SNs; - Header compression and decompression using the ROHC protocol; - Header compression and decompression using EHC protocol; - Compression and decompression of uplink PDCP SDUs: DEFLATE based UDC only; - Ciphering and deciphering; - Integrity protection and integrity verification; - Timer based SDU discard; - For split bearers, routing; - Duplication; - Reordering and in-order delivery; - Out-of-order delivery; - Duplicate discarding. |
| SDAP | - Mapping between a QoS flow and a data radio bearer; - Marking QoS flow ID (QFI) in both DL and UL packets |

Table 1. Functions supported by NR User Plane protocols [1].

Table 1 shows the list of functions supported by NR UP sublayers. Most of functions are considered essential and should be kept to connect between upper layers above L2 and physical layer. However, the following functions are just to support the completeness of the UP protocol itself, so we can discuss later after RAN2 concludes essential functions and sublayer structures:

- Mapping between channels (e.g., LCH, transport channel, radio bearer, etc)

- Padding.

We do not see that there is no existing redundant function in NR UP except SDAP functions. Also, duplication and split bearer operation may depend on the conclusion of Dual-Connectivity, so RAN2 should postpone the decision on those functions. Hence, we propose to keep the current NR functions as required baseline functions.

Observation 1: Most of NR UP functions are considered essential to connect between upper layers above L2 and physical layer.

Proposal 1: The following NR UP functions are considered as required essential 6GR UP functions:

- Data transfer

- Sequence numbering

- Duplicate discarding/duplicate detection

- PDU discard

- Retransmission (HARQ, ARQ)

- Segmentation/reassembly

- MAC multiplexing and de-multiplexing

- Priority handling (between UE, between LCHs/RBs, between resources, etc)

- Compression (ROHC, UDC, EHC)

- Security protocol (both integrity protection and ciphering)

- In-order delivery (including reordering) and out-of-order delivery

Proposal 2: RAN2 to study the necessity of packet duplication and routing in split bearer after Dual-Connectivity is agreed.

UP protocols are related to data processing. For serving high data rate expected for 6G communications, 6G UP protocols should be lean and efficient. Table 1, duplicate or similar functions are highlighted with colour codes. We can easily find multiple sets of duplicate or similar functions in NR UP. In 6G UP, this inefficiency should be avoided. We see that one function is located only in one place to avoid the unnecessary complications.

Observation 2: NR UP protocol stack has multiple sets of duplicated functions.

Proposal 3: For each purpose, only one function is supported in 6GR UP, i.e. No duplicate functions for the same purpose.

## QoS Flow to DRB Mapping

SDAP is a sublayer to support QoS flow to DRB mapping. The main reason to introduce SDAP sublayer is a case that so many QoS flows are being served by a UE and 1-to-1 mapping between QoS flow and DRB may require a significantly large number of DRBs. However, most of UEs serve a limited number of QoS flows simultaneously. QoS flow to DRB mapping seems to be redundant. Even without SDAP sublayer, the increased number of DRBs supported by the UE will not be changed drastically. Considering one more independent layer, UE process due to SDAP does not have a big gain. Also, in-band QFI marking is considered as additional header overhead. RAN2 should consider no in-band QFI marking.

Observation 3: Most of UEs serve a limited number of QoS flows simultaneously. QoS flow to DRB mapping seems to be redundant.

Proposal 4: 6GR supports only 1-to-1 QoS flow to DRB mapping with QFI marking.

  - Radio Bearer/Logical Channel Concept

Figure 1. DL Layer 2 structure in NR [1]

Figure 1 shows Layer 2 structure in NR downlink. The concept of Radio Bearer/Logical Channel is supported in NR and LTE which realizes the management of UP data with different characteristics and priorities. Considering that various service types will be supported by 6GR, radio bearer and logical channel could still be helpful to fulfil the specific requirements like latency, error recovery, etc. for different services. Although RLC entity is associated with a radio bearer/logical channel in NR, we don’t expect that it implies an additional requirement for RLC if we reuse this concept in 6GR.

Observation 4: Considering the service types will be supported by 6GR, radio bearer and logical channel could still be helpful to fulfil the specific requirements like latency, error recovery, etc.

Proposal 5: 6GR reuses concepts of radio bearer and logical channel as in NR.

  - Unified Compressions

NR standards employ multiple compression mechanisms across various releases i.e., ROHC, UDC, EHC. ROHC is a Rel-15 basic feature mainly targeted to voice packets. UDC is a specific UL compression mechanism transplanted from LTE UDC. EHC is for Ethernet-based applications like industrial IoT. All those mechanisms were standardized independently, but those are all compression mechanisms which have commonalities. For the sake of 6GR simplification, it is advisable to adopt a unified compression mechanism framework, to avoid independent features with overlapping goals.

Observation 5: In NR, multiple compression mechanisms, i.e., ROHC, UDC, EHC. ROHC were introduced independently, but those are all compression mechanisms which have commonalities.

Proposal 6: RAN2 to study a unified compression framework including ROHC, UDC, and EHC.

  - Hardware-friendly Processing

In RAN2#131bis meeting, RAN2 discussed hardware-friendly design different from traditional software-based implementation which can be easily modified by the software code updates. Hardware-based implementation should be very simple and avoid multiple types of information structure and parsing operations. An example of the hardware processing is security processing (i.e., ciphering and integrity protection) which performs only a few mathematical operations including XOR based on the generated security key. If we extend the hardware-friendly processing in 6G UP, a candidate should be header processing. However, NR UP headers are variable size especially in MAC and RLC and carry a lot of information across the multiple sublayers. The information size may be reduced by merging similar functions. Also, RAN2 should consider fixed size UP header as much as possible.

Observation 6. NR UP headers are variable size especially in MAC and RLC which is not hardware-friendly.

Proposal 7: For hardware-friendly processing, 6GR UP protocol pursues fixed size header for each upper layer SDU.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: The following NR UP functions are considered as required essential 6GR UP functions:

- Data transfer

- Sequence numbering

- Duplicate discarding/duplicate detection

- PDU discard

- Retransmission (HARQ, ARQ)

- Segmentation/reassembly

- MAC multiplexing and de-multiplexing

- Priority handling (between UE, between LCHs/RBs, between resources, etc)

- Compression (ROHC, UDC, EHC)

- Security protocol (both integrity protection and ciphering)

- In-order delivery (including reordering) and out-of-order delivery

Proposal 2: RAN2 to study the necessity of packet duplication and routing in split bearer after Dual-Connectivity is agreed.

Proposal 3: For each purpose, only one function is supported in 6GR UP, i.e. No duplicate functions for the same purpose.

Proposal 4: 6GR supports only 1-to-1 QoS flow to DRB mapping with QFI marking.

Proposal 5: 6GR reuses concepts of radio bearer and logical channel as in NR.

Proposal 6: RAN2 to study a unified compression framework including ROHC, UDC, and EHC.

Proposal 7: For hardware-friendly processing, 6GR UP protocol pursues fixed size header for each upper layer SDU.

# Reference

[1] TS 38.300	NR and NG-RAN Overall Description, Stage-2