3GPP TSG-RAN WG2 Meeting #132												R2-2508218

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	9.7.2

Source: 	Sharp

Title:  	Voice Support over NB-IoT NTN

Document for: 	Discussion

# Introduction

In IoT NTN Phase 4 WID [1], down-selection between CP and UP solutions is captured as follows:

- Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection

This document discusses how to address the issue above.

# Discussion

The main difference between UP solution and CP solution is which radio bearer carries the voice data. Voice data is considered as user traffic. Traditionally, DRB is being used to carry user traffic. In this context, UP solution is desirable.

- Voice data is considered as user traffic which is being served by DRB.

TR 23.700-19 [2] already provided rough overhead analysis for both UP solution and CP solution. Both solutions assume RTP/UDP/IP on top of 3GPP protocols, so ROHC will be common for both solutions. MAC header is also common. The only difference in header overhead may occur due to PDCP/RLC headers in UP solution and NAS/RRC/RLC headers in CP solution.

Based on the current standards without consideration of any enhancements, the UP solution seems to have total 2-byte header (1 byte in PDCP and 1-byte in RLC) and CP solution seems to have 15 bytes (11 bytes in NAS, 2-bytes in RRC and 2-byte in RLC). UP solution looks better from the overhead perspective. However, we do not see it is a big difference as we may consider enhancements for further overhead reductions for both solutions, e.g., NAS overhead reduction in other WGs or AS overhead reduction in RAN2.

- UP solution has smaller overhead, but the difference can be minimized by further enhancements.

For CP solution, we expect more work in RAN2. For example, dedicated SRBs for voice traffic and potentially IMS control signaling should be defined. Also, voice data is usually served by RLC UM. SRB with RLC UM should be defined whereas DRB already allows it. Moreover, UP solution can just reuse existing PDCP PDU format and signaling for ROHC whereas CP solution assumes PDCP bypass.

- CP solution requires more RAN2 work whereas UP solution can reuse existing features.

Considering the observations above, we see that UP solution can be selected from the RAN2 point of view.

- RAN2 to select UP solution for voice support over NB-IoT-NTN.

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposal:

- RAN2 to select UP solution for voice support over NB-IoT-NTN.

# Reference

[1] RP-252473	Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4	vivo (Moderator)

[2] 3GPP TR 23.700-19	Study on Integration of satellite components in the 5G architecture Phase 4	v1.0.0 (2025-09)